IEEE P802.11
Wireless LANs

Status of TGa (5 GHz high speed PHY) of 802.11 after January 98 Interim Meeting

Naftali Chayat, TGa Chair

In January 1998 Task Group A of 802.11 gathered in Lynnwood, WA, with 73 people attending. During the meeting the 6 PHY proposals, presented previously in November 97 meeting, were presented in greater detail and initial performance data was presented. The Task Group agreed on a common format of presenting performance data in March 1998 meeting for comparison and selection. The comparison criteria include sensitivity and multipath robustness, spectrum utilization (number of channels, Co-Channel and Adjacent channel interference sensitivity), complexity and power consumption, RF power amplifier efficiency and robustness to phase noise.

Initial assessment of members’ priorities shows bias towards robust operation in multipath typical to low/medium ranges, with low-power consumption and non-excessive complexity. Of smaller priority come radio related parameters and sensitivity. In March 1998 Task Group A intends to select the preferred modulation method and to have the Working Group Draft Standard avaliable after its May 1998 meeting.

Comparison of proposals

| Company | Lucent Tech. | NTT | Micrilor | NEC | BreezeCom | RadioLAN |
| --- | --- | --- | --- | --- | --- | --- |
| Modulation method | OFDM QPSK or 16-QAM in each subcarrier | OFDM DQPSK in each subcarrier | DBOK (Differential BiOrtho. Keying ) + Overlaid signaling | QPSK | Offset Quadrature Modulation (OQPSK/OQAM) | PPM (16-PPM, 4-PPM) |
| Pulse shaping features | Crest-factor reduction | Crest-factor reduction | MSK based waveforms | 50% Square-Root Raised Cosine | GMSK-derived pulse shape | 50% Square-Root Raised Cosine |
| Error Correction Coding | Convolutional K=7, Inter-carrier interleaving | Convolutional K=7, Inter-carrier interleaving | Optional RS(13,15) | Uncoded, BCH(31,26) considered | Hamming (31,26), uncoded option | Uncoded |
| Rates supported | 10 Mbit/s 20 Mbit/s 30 Mbit/s | 20 Mbit/s | 10 Mbit/s (16DBOK) 18 Mbit/s (4x4DBOK) 24 Mbit/s (4x4DBOK) | 20 Mbit/s | 21 Mbit/s (coded) 25 Mbit/s (uncoded) 42 Mbit/s (coded) 50 Mbit/s (uncoded) | 10 Mbit/s (4-PPM) 20 Mbit/s (16-PPM) |
| Number of channels in U-NII band | 5 in 100 MHz 11 in 200 MHz | 3 in 100 MHz 7 in 200 MHz | 2 in 100 MHz | 4-5 in 100 MHz 10 in 200 MHz | 3 in 100 MHz 7 in 200 MHz | 3 in 100 MHz |
| Applicable documents | 97/123, 98/12, 97/92 | 97/137,98/02,98/03 97/89 | 97/130, 98/17, 98/19 | 97/121, 98/34, 98/35 | 97/111, 98/21 | 97/145r1, 98/38 |