May 2003

doc.: IEEE 802.11-03/402r1

IEEE P802.11
Wireless LANs

Interpretation Response 2-05/03
Date:
May 14, 2003

Author:
Terry Cole

AMD

5900 E Ben White Blvd, M/S PCS-4
                                                             Austin, Texas, 78731

Phone: +1 512 602 2454

Fax: +1 512 602 5051

e-Mail: terry.cole@amd.com

Interpretation Number: 2-05/03 (status codes)

Topic: 802.11 reason codes

Relevant Clauses: 7.3.1.7

Classification: unambiguous

Interpretation Request

In clause 7 of IEEE Std. 802.11-1999, the content of a Reason Code field is defined that appears in several different 802.11 frames.  There is not any definition of the use of each of the different reason codes on transmission nor any definition of the behavior of a station upon reception of these reason codes.  Please define the use of these reason codes for both transmission and reception.

Interpretation for IEEE std 802.11-1999 (reaffirmed 2003)

The standard defines as valid a set of reason codes for use in management frames defined in clause 7 and in MLME SAP primitives defined in clause 10.  Transmission of some reason codes during authentication, association and re-association processes are described in Annex C. The standard does not define normative behavior for transmitting other reason codes. Reception of reason codes is defined by the standard in that the code is passed across the MLME SAP interface to an external entity outside the scope of the standard. This item is being brought to the attention of the 802.11 working group for the possibility of action in a future revision or maintenance change.
Submission
page 1
Terry Cole, AMD

