| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | HR-MS to HR-MS Direct Communication without Infrastructure Stations |  |
| Date Submitted | 2011-03-06 |  |
| Source(s) | Haiguang Wang, Hoang Anh Tuan, Jaya Shankar, Shoukang Zheng, Yeow Wai Leong Institute for Infocomm Research 1 Fusionopolis Way, #21-01, Connexis (South Tower) Singapore 138632 | Voice: +65 6408-2256 E-mail: hwang@i2r.a-star.edu.sg |
| Re: | Call for contributions for 802.16n AWD |  |
| Abstract | In this document, we propose a neighbor discovery method for the HR-MSs in the |  |
| Purpose | To discuss and adopt the proposed text in the 802.16n draft Text |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Copyright Policy | The contributor is familiar with the IEEE-SA Copyright Policy <http://standards.ieee.org/IPR/copyrightpolicy.html>. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Direct HR-MS Communication without Infrastructure Stations

Haiguang Wang, Hoang Anh Tuan, Jaya Shankar,

Shoukang Zheng, Yeo Wai Leong

Institute for Infocomm Research (I2R)

1 Fusionopolis Way, #21-01, Connexis South Tower

Singapore 138632

# 1. Introduction

HR-MS to HR-MS direct communication is new feature for IEEE 802.16 networks. It will be defined in the IEEE 802.16n amendment.  This feature will significantly improve the capability of HR-MS in various harsh environment and increase network throughput when local traffic dominates the network.

In 802.16n System Requirement Document [1] section 6.1.3.1, it says that “HR-MS shall provide direct communication i.e. the origination and termination of the data are at the HR-MS. Association establishment procedure of an HR-MS to another HR-MS shall be supported.”.

Since the requirement does not set constraint on whether HR-MSs have associated to an infrastructure stations, in this contribution, we assume HR-BS/HR-RS is not presented in the network and the HR-MSs communicate to each other directly. They may also help each other in data forwarding.

# 2. HR-MS to HR-MS Direct Communication

Supporting the HR-MS to HR-MS direct communication without infrastructure node is required by IEEE 802.16n network. In this contribution, we focus on the use cases of HR-MSs communicate to each other without getting support from infrastructure stations as figure 1 shows. A few HR-MSs connect to each other and form a communication network with two hops.

Figure 1: Use cases of HR-MS to HR-MS direct communication without infrastructure stations

Since the communication of HR-MS with and without the supporting from infrastructure node are significantly different, we propose to classify the communication mode of HR-MS into two modes, infrastructure mode and ad hoc mode, the same as IEEE 802.11 standard. In infrastructure mode, HR-MSs should associate to a infrastructure node, an HR-BS/HR-RS, before they can communicate. While in ad hoc node, HR-MSs are allowed to transmit before they associate to any infrastructure nodes.

In this contribution, we focus on the ad hoc mode only.   Figure 2 shows a state machine for HR-MS in direct communication without infrastructure node.  After an HR-MS is turned into ad hoc mode, it does some initialization work first. After that, it enters a discovery state. In this state, the node transmits system configuration information from time to time to let other nodes to know the current node. Once there are HR-MSs that have intention to communicate to each other, the HR-MSs should enter communication mode and transmit data to each other.  Once data communication finishes, the HR-MSs may transit back to discovery mode.

Figure 2: System state of HR-MS in ad hoc mode

# 3. Summary

In this contribution, we provide a method for 802.16n HR-MSs in setting up HR-MS direct communication network by themselves without turning any node into base station or relay station.

# 4. Text Proposal for IEEE 802.16n AWD

Xxx

Note:

The text in BLACK color: the existing text in AWD

The text in RED color: the removal of existing AWD text

The text in BLUE color: the new text added to the Multi-Carrier DG Text

[-------------------------------------------------Start of Text Proposal---------------------------------------------------]

[Adopt the following text in the 802.16n Document (XXX --- document number)]

17.2.2 Direct communication between HR-MSs

In ad hoc mode, an HR-MS can be in three states as Figure XXX shows.

- Initialization state: initialize the system for ad hoc mode.

- Discovery state: only transmit node configuration information from time to time.

- Communication state: setup communication links with other HR-MS and transmit data to each other.

Figure XXX: State machine for HR-MS in Ad Hoc Mode

17.3.2 Direct communication between HR-MSs

In ad hoc mode, a node can be in three states as Figure XXX shows.

- Initialization state: initialize the system for ad hoc mode.

- Discovery state: only transmit node configuration information from time to time.

- Communication state: setup communication links with other HR-MS and transmit data to each other.

[-------------------------------------------------End of Text Proposal----------------------------------------------------]

# References

- IEEE 802.16n-10/0048, “802.16n System Requirements Document including SARM annex”, January 2011.

- IEEE 802.16n-10/0049, “802.16n Table of Contents for Amendment Working Draft”, January 201.