| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Downlink Control for Multicast in IEEE 802.16n |  |
| Date Submitted | 2011-03-06 |  |
| Source(s) | Eunkyung Kim, Sungcheol Chang, Sungkyung Kim, Hyun Lee, Chulsik Yoon ETRI | E-mail: ekkim@etri.re.kr scchang@etri.re.kr |
| Re: | “IEEE 802.16n-10/0051,” in response to the CFC for IEEE 802.16n AWD |  |
| Abstract | Downlink control supporting multicast in IEEE 802.16n |  |
| Purpose | To discuss and adopt the proposed text in the AWD of 802.16n |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Downlink Control for Multicast in IEEE 802.16n

Eunkyung Kim, Sungcheol Chang, Sungkyung Kim, Hyun Lee, Chulsik Yoon

ETRI

# Introductions

HR-Network shall provide optimized MAC protocols for unicast and multicast transmission. In addition, section 6.2.1 in SRD (i.e., IEEE 802.16n-10/0048[1]) of IEEE 802.16n describes some requirements related to service provided on HR-Network to support unicast and multicast communication.

This document provides a new downlink control channel (i.e., DL Assignment A-MAP) supporting multicast operation meeting the requirements [1][4] in IEEE 802.16n.

Figure 1 shows an example of multicast service using a new DL Assignment A-MAP (i.e., HR-Multicast DL Assignment A-MAP). To allocate multicast resource during a time periodically, HR-Multicast DL Assignment A-MAP includes following parameters:

- Allocation Period: indicates a period of persistent allocation of multicast resource.

- Lifetime: a timer indicating the next instance of HR-Multicast DL-Assignment A-MAP IE. Unless the Lifetime expires, this HR-Multicast DL Assignment A-MAP does not change during the allocation duration. At the time the Lifetime expires, the HR-Multicast DL Assignment A-MAP may change or release the allocation.

In addition, 16bit CRC is generated based on the randomized contents of the HR-Multicast DL Assignment A-MAP IE. The CRC is masked by the 16-bit CRC mask generated by masking prefix (1bit; 0b0) + type indicator (3bits; 0b010) + Multicast Group ID(12bits).

Note: In 802.16m, after CRC check, whether received Broadcast A-MAP is recognized multicast or not by the parameter of Broadcast A-MAP IE (i.e., Function Index). However, proposed CRC masking generated by masking by multicast group ID can recognize the received A-MAP is multicast or not right after the CRC check.

Figure 1 – Multicast service using HR-Multicast DL Assignement A-MAP

# References

- IEEE 802.16n-10/0048, “802.16n System Requirements Document including SARM annex,” January 2011.

- IEEE Std. 802.16-2009, “IEEE Standard for Local and metropolitan area networks; Part 16: Air Interface for Broadband Wireless Access Systems,” May 2009.

- IEEE 802.16m-09/0034r3, “IEEE 802.16m System Description Document (SDD),” June 2010.

- IEEE C802.16gman-10/0014, “Group Calls and Multicast Operation for Public Safety and Public Protection,” May 2010.

# Proposed Text for the 802.16n Amendment Working Document (AWD)

Note:

The text in BLACK color: the existing text in the 802.16n Amendment Draft Standard

The text in RED color: the removal of existing 802.16n Amendment Draft Standard Text

The text in BLUE color: the new text added to the 802.16n Amendment Draft Standard Text

[-------------------------------------------------Start of Text Proposal---------------------------------------------------]

[Remdy1: Adapt the following change in section 16.3.5.5.2.4 Assignment A-MAP in the 802.16n AWD]

[Change section 16.3.5.5.2.4 as indicated:]

16.3.5.5.2.4 Assignment A-MAP IE

Table 842 describes Assignment A-MAP IE Types.

Table 842 – Assignment A-MAP IE Types

| A-MAP IE Type | Usage | Property |
| --- | --- | --- |
| 0b0000 | DL Basic Assignment A-MAP IE | Unicast |
| 0b0001 | UL Basic Assignment A-MAP IE | Unicast |
| 0b0010 | DL Subband Assignment A-MAP IE | Unicast |
| 0b0011 | UL Subband Assignment A-MAP IE | Unicast |
| 0b0100 | Feedback Allocation A-MAP IE | Unicast |
| 0b0101 | UL Sounding Command A-MAP IE | Unicast |
| 0b0110 | CDMA Allocation A-MAP IE | Unicast |
| 0b0111 | DL Persistent Allocation A-MAP IE | Unicast |
| 0b1000 | UL Persistent Allocation A-MAP IE | Unicast |
| 0b1001 | Group Resource Allocation A-MAP IE | Multicast |
| 0b1010 | Feedback Polling A-MAP IE | Unicast |
| 0b1011 | BR-ACK A-MAP IE | Multicast |
| 0b1100 | Broadcast Assignment A-MAP IE | Broadcast/Multicast |
| 0b1101 | Reserved HR-Multicast DL Assignment A-MAP IE | NA. Multicast |
| 0b1110 | Reserved | NA. |
| 0b1111 | Extended Assignment A-MAP IE | NA. |

CRC Mask

A 16-bit CRC is generated based on the randomized contents of assignment A-MAP IE and is masked by 16-bit CRC mask using the bitwise XOR operation.

The 16-bit masked CRC is constructed using a 1 bit masking prefix, a 3 bit message type indicator, and 12 bit Masking Code as described in Table 843.

Table 843 – Description of CRC Mask

| Masking Prefix (1 bit MSB) | Remaining 15 bit LSBs |  |
| --- | --- | --- |
| 0b0 | Type Indicator | Masking Code |
|  | 0b000 | 12 bit STID or TSTID |
|  | 0b001 | Refer to Table 844 |
|  | 0b010 | Refer to Table 845 |
| 0b1 | 15 bit RA-ID: The RA-ID is derived from the AMS' random access attributes (i.e., superframe number (LSB 5bits), frame_index (2 bits), preamble code index for ranging or BR (6 bits) and opportunity index for ranging or BR (2 bits)) as defined below: RA-ID = (LSB 5bits of superframe number ¦ frame_index ¦ preamble_code_index ¦ opportunity_index) |  |

……

Table 845 – Description of Masking Code for type indicator 010

| Decimal Value | Description |
| --- | --- |
| 4095 | Used to mask Broadcast A-MAP IE for multicast assignment |
| Others | Reserved 12 bit MGID is used to make HR-Multicast DL Assignment A-MAP IE for high reliable multicast assignment |

……

[Remedy2: Add the following text into Section 17.3.9 Multicast Support in the 802.16n AWD]

## 17. WirelessMAN-High Reliability Network

### 17.3 WirelessMAN HR Advanced air interface

#### 17.3.9 Multicast Support

##### 17.3.9.x Multicast protocol features and functions

###### 17.3.9.x.y Downlink control channel for multicast communication

HR-Multicast DL Assignment A-MAP IE carries configuration information for multicast communication for one Multicast Zone in an HR-BS. In HR-Multicast DL Assignment A-MAP, allocation period indicates a period of persistent allocation of multicast resource and Lifetime is a timer indicating the next instance of HR-Multicast DL-Assignment A-MAP IE. Unless the Lifetime expires, this HR-Multicast DL Assignment A-MAP does not change during the allocation duration. At the time the Lifetime expires, the HR-Multicast DL Assignment A-MAP may change or release the allocation.

Table xx – HR-Multicast DL Assignment A-MAP IE*

| Syntax | Size (bit) | Description/Notes |
| --- | --- | --- |
| HR-Multicast_DL_Assignment_A-MAP_IE() { |  |  |
| A-MAP IE Type | 4 | HR-Multicast DL Assignment A-MAP IE |
| Allocation period | 2 | Period of persistent allocation of multicast resource. If (Allocation Period==0b00), it indicates the deallocation of persistent resource. 0b00: deallocation 0b01: 2 frames 0b10: 4 frames 0b11: 6 frames |
| If (Allocation Period == 0b00) { |  |  |
| Resource Index | 11 | 5 MHz: 0 in first 2 MSB bits + 9 bits for resource index 10 MHz: 11 bits for resource index 20 MHz: 11 bits for resource index Resource index includes location and allocation size. |
| Long TTI Indicator | 1 | Indicates number for AAI subframes spanned by the allocated resource. 0b0: 1 AAI subframe (default TTI) 0b1: 4 DL AAI subframe for FDD or all DL AAI subframes for TDD (long TTI) |
| Reserved | 22 |  |
| } else if(Allocation Period != 0b00) { |  |  |
| Isizeoffset | 5 | Offset used to compute burst size index |
| MEF | 2 | MIMO encoder format 0b00: SFBC 0b01: Vertical encoding 0b10: Multi-layer encoding 0b11: CDR |
| If (MEF ==0b01) { |  | Parameter for vertical encoding |
| Mt | 3 | Number of streams in transmission Mt<=Nt Nt: Number of transmit antennas at the HR-BS 0b000: 1 stream 0b001: 2streams 0b010: 3streams 0b011: 4streams 0b100: 5streams 0b101: 6streams 0b110: 7streams 0b111: 8streams |
| Reserved | 1 |  |
| } else if (MEF == 0b10) { |  | Parameters for multi-layer encoding |
| Si | 4 | Index to identify the combination of the number of streams and the allocated pilot stream index in a transmission with MU-MIMO, and the modulation constellation of paired user in the case of 2 stream transmission 0b0000: 2 streams with PSI=stream1 and other modulation = QPSK 0b0001: 2 streams with PSI=stream1 and other modulation = 16QAM 0b0010: 2 streams with PSI=stream1 and other modulation = 64QAM 0b0011: 2 streams with PSI=stream1 and other modulation information not available 0b0100: 2 streams with PSI=stream2 and other modulation =QPSK 0b0101: 2 streams with PSI=stream2 and other modulation =16QAM 0b0110: 2 streams with PSI=stream2 and other modulation =64QAM 0b0111: 2 streams with PSI=stream2 and other modulation information not available 0b1000: 3 streams with PSI=stream1 0b1001: 3 streams with PSI=stream2 0b1010: 3 streams with PSI=stream3 0b1011: 4 streams with PSI=stream1 0b1100: 4 streams with PSI=stream2 0b1101: 4 streams with PSI=stream3 0b1110: 4 streams with PSI=stream4 0b1111: n/a |
| } |  |  |
| Resource Index | 11 | 5 MHz: 0 in first 2 MSB bits + 9 bits for resource index 10 MHz: 11 bits for resource index 20 MHz: 11 bits for resource index Resource index includes location and allocation size. |
| Long TTI Indicator | 1 | Indicates number for AAI subframes spanned by the allocated resource. 0b0: 1 AAI subframe (default TTI) 0b1: 4 DL AAI subframe for FDD or all DL AAI subframes for TDD (long TTI) |
| Lifetime(L) | 4 | Indicates the time to transmit next HR-Multicast DL Assignment A-MAP and the information of this HR-Multicast DL Assignment A-MAP does not change during the allocation duration. The next HR-Multicast DL Assignment A-MAP is at the superframe whose superframe number, Nsuperframe, satisfies the following condition. Nsuperframe modulo L+1 = 0 |
| Reserved | 7 |  |
| } |  |  |
| } |  |  |

*A 16bit CRC is generated based on the randomized contents of the HR-Multicast DL Assignment A-MAP IE. The CRC is masked by the 16-bit CRC mask (with masking prefix = 0b0 and message type indicator = 0b010) generated according to Table 843 as describe in 16.3.5.5.2.4.

[-------------------------------------------------End of Text Proposal----------------------------------------------------]