| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | [Text Proposal of Multi-Mode Operation in IEEE 802.16n Networks] |  |
| Date Submitted | [2011-05-16] |  |
| Source(s) | Liru Lu (Alina), Ming-Tuo Zhou Xin Zhang, Vinh Dien Hoang, Masayuki Oodo, Hiroshi Harada(NICT) | Voice: [65-67711006] E-mail: [liru@nict.com.sg; mingtuo@nict.com.sg; zhangxin@nict.com.sg; hvdien@nict.com.sg; moodo@nict.go.jp; harada@nict.go.jp |
| Re: | [Call for contributions for the 802.16n Amendment Working Draft Base Contribution: IEEE 802.16n-10/0048] |  |
| Abstract | [This document proposes some mechanisms to be used in multi-mode operations.] |  |
| Purpose | [To be discussed in 802.16n Task Group.] |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Copyright Policy | The contributor is familiar with the IEEE-SA Copyright Policy <http://standards.ieee.org/IPR/copyrightpolicy.html>. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Mechanisms for Multi-Mode Operation

Liru Lu (Alina), Ming-Tuo Zhou, Xin Zhang, Hoang Vinh Dien, Masayuki Oodo, Hiroshi Harada

NICT

# Introduction

Document IEEE 802.16n-10/0048 specifies the function requirements of HR-Network. Section 6.1.1 defines the requirements for multi-mode operation.

In this file, we propose the mechanisms for multi-mode operation to address the following topics.

# Relay Function for HR-BS

# Relay Function for HR-MS (Relay Mode)

# Base Station Function for HR-MS (BS Mode)

# Proposed Text

----------------------------------------------------- Start of Proposed Text -----------------------------------------------------

[Insert new sections after 17.3.1 as follows:]

17.3.1 Multi-mode operation

# 17.3.1.1 Relay Function for HR-BS

An HR-BS (affected HR-BS) may operate as a relay station to communicate with another HR-BS (serving HR-BS) that has connection to backhaul. HR-BS shall maintain its BS functionality such as central scheduling even when it is operating its relay function.

# 17.3.1.2 Relay Function for HR-MS (Relay Mode)

Relay function shall be supported by HR-MS. HR-MS may initialize its relay mode by the following two schemes:

(1) Centralized scheme (Initialized by HR-BS)

(2) Distributed scheme (Initialized by HR-MS)

# 17.3.1.3 Base Station Function for HR-MS (BS Mode)

HR-MS shall only start its BS mode when its superordinate HR-BS is unavailable in the HR network. There are two options for HR-MS to be enabled as an HR-BS: Proactive mode and Reactive mode. In proactive mode, the mode switch to HR-BS is directed by the superordinate HR-BS of the HR-MS; In reactive mode, the mode switch is initiated by the HR-MS itself.

----------------------------------------------------- End of Proposed Text -----------------------------------------------------