| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Multicast Security : Join and Leave |  |
| Date Submitted | 2011-03-06 |  |
| Source(s) | Joseph Chee Ming Teo, Jaya Shankar, Yeow Wai Leong, Hoang Anh Tuan, Zheng Shoukang, Mar Choon Hock Institute for Infocomm Research 1 Fusionopolis Way, #21-01, Connexis (South Tower) Singapore 138632 | Voice: +65 6408-2292 E-mail: cmteo@i2r.a-star.edu.sg |
| Re: | Call for contributions for 802.16n AWD |  |
| Abstract | In this document, we first consider the multicast key management scenario where there is network infrastructure and it is assumed that each node shares a unicast security key MSKi with the base station HR-BS. Next, we focus on the situation where there is no backbone infrastructure and all multicast group members possess a private/public key pair and public key certificate (X.509 certificate) signed by a certification authority (e.g. Manufacturer), that is, we assume that there is X.509 PKI infrastructure in place. We propose multicast key management procedures for Initial Group Formation, Node Join events and Node leave events to secure multicast communications in 802.16n HR-networks for the above two scenarios. |  |
| Purpose | To discuss and adopt the proposed text in the 802.16n draft Text |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Copyright Policy | The contributor is familiar with the IEEE-SA Copyright Policy <http://standards.ieee.org/IPR/copyrightpolicy.html>. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Multicast Security: Join and Leave

Joseph Chee Ming Teo, Jaya Shankar, Yeow Wai Leong, Hoang Anh Tuan, Zheng Shoukang, Mar Choon Hock

Institute for Infocomm Research (I2R)

1 Fusionopolis Way, #21-01, Connexis South Tower

Singapore 138632

# 1. Introduction

# In this contribution, we propose multicast key management procedures for Node Join events and Node leave events to secure multicast communications in 802.16n HR-networks.

# 4. Text Proposal for IEEE 802.16n AWD

Note:

The text in BLACK color: the existing text in AWD

The text in RED color: the removal of existing AWD text

The text in BLUE color: the new text added to the AWD Text

[-------------------------------------------------Start of Text Proposal---------------------------------------------------]

[Adopt the following text in the 802.16n AWD Document (C802.16x-xx/xxxx)]

# 17.2.10.2 Multicast Key Management

### 17.2.10.2.x.2 Join Event Procedure

The Join Event Procedure takes place whenever there are nodes that join the multicast group and the multicast key has to be changed in order to prevent the new joining nodes from obtaining previous secure multicast communications. MSKj denotes the unicast shared key between HR-MSj with HR-BS.

The Join Event Procedure includes the following steps:

Step 1: New mobile station HR-MSj first generates nonce NHR-MSj. Next, HR-MSj computes the MAC θHR-MSj = MAC(MSKj|MulticastGrpID|THR-MSj|NHR-MSj|HR-BSAddr|HR-MSjAddr) and sends Join_MG_MSG_#1 message to HR-BS, where Join_MG_MSG_#1 = MulticastGrpID| THR-MSj|NHR-MSj|HR-BSAddr| HR-MSjAddr|θHR-MSj.

Step 2: HR-BS first verifies the received timestamps and nonces for freshness and θHR-MSj. If the verifications are correct, then HR-BS generates nonce NHR-BS', MAK' and computes θHR-BS = MAC(MAK'|NHR-BS'|HR-BSAddr|MulticastGrpID) and θHR-MSj' =MAC(MSKj|MAK'|NHR-BS'|NHR-MSj|HR-BSAddr|HR-MSjAddr). HR-BS then encrypt and obtain cj = EMSKj(MAK', MAK'_lifetime, HR-MSjAddr, HR-BSAddr). HR-BS also encrypts using the existing MAK and obtains c' = EMAK(MAK', MAK'_lifetime,  HR-BSAddr, NHR-BS') Finally, HR-BS sends Multicast_Join_MSG_#2 message to HR-MSi for 1 ≤ i ≤ n and Multicast_Join_MSG_#3 message to HR-MSj respectively, where Multicast_Join_MSG_#2 = MulticastGrpID|THR-BS'|NHR-BS'|HR-BSAddr| c'|θHR-BS and Multicast_Join_MSG_#3 = MulticastGrpID|THR-BS'|NHR-BS'|HR-MSjAddr|HR-BSAddr|NHR-MSj|cj| θHR-MSj'.

Step 3a: Each HR-MSi for 1 ≤ i ≤ n first verifies the received timestamp and nonce for freshness. If the verifications are correct, then each HR-MSi decrypts c' and obtains the new MAK', and MAK'_lifetime. Next, each HR-MSi verifies θHR-BS. If the verification is correct, then HR-MSi can commence secure multicast.

Step 3b:  HR-MSj first verifies the received timestamp and nonces for freshness. If the verifications are correct, then HR-MSj uses MSKj to decrypt c_j and obtains MAK' and MAK'_lifetime. Next, HR-MSj verifies θHR-MSj'. If the verification is correct, then HR-MSj can commence secure multicast.

### 17.2.10.2.x.3 Leave Event Procedure

The Leave Event Procedure takes place whenever there are nodes leaving the multicast group and the multicast key has to be changed in order to prevent the leaving nodes from obtaining future secure multicast communications. MSKi denotes the unicast shared key between HR-MSi with HR-BS.

The Leave Event Procedure includes the following steps:

Step 1: HR-BS generates nonce NHR-BS', new group key MAK' and computes θHR-MSi' = MAC(MSKi|MAK'|NHR-BS'|HR-BSAddr|HR-MSiAddr|MulticastGrpID) for 1 ≤ i \!= L ≤ n. HR-BS then uses the shared key MSKi with the remaining HR-MSi for 1 ≤ i != L ≤ n to encrypt and obtain ci' =  EMSKi(MAK',MAK'_lifetime, HR-BSAddr, NHR-BS'). Finally, HR-BS sends Multicast_Leave_MSG_#1 messages to remaining HR-MSi for 1 ≤ i != L ≤ n, where Multicast_Leave_MSG_#1 = MulticastGrpID|THR-BS'|NHR-BS'|HR-BSAddr|ci'|θHR-MSi'.

Step 2: Each remaining HR-MSi for 1 ≤ i != L ≤ n first verifies the received timestamp and nonce for freshness. If the verifications are correct, then each remaining HR-MSi uses its shared key MSKi to decrypt ci ' and obtains the new MAK' and MAK'_lifetime. Next, each remaining HR-MSi verifies θHR-MSi'. If the verification is correct, then each remaining HR-MSi can commence secure multicast.

Figure X3 - Flow Diagram for Leave Procedure

Figure Y3 - Flow Chart for Leave Procedure

[-------------------------------------------------End of Text Proposal----------------------------------------------------]

# References

- IEEE 802.16n-10/0048, “802.16n System Requirements Document including SARM annex”, January 2011.

- IEEE 802.16n-10/0049, “802.16n Table of Contents for Amendment Working Draft”, January 2011.

# Use of IEEE-Copyrighted Conference and Journal Papers

The IEEE 802.16 Working Group has obtained permission to post IEEE-copyrighted material from IEEE conferences and publications when received in an official contribution to the Working Group. IEEE requires two conditions:

- A full copyright and credit notice must be posted at the top of the paper in the following format:

Copyright ©2002 Institute of Electrical and Electronics Engineers, Inc.  Reprinted, with permission, from [all relevant journal info].

This material is posted here with permission of the IEEE. Internal or personal use of this material is permitted. However, permission to reprint/republish this material for advertising or promotional purposes or for creating new collective works for resale or redistribution must be obtained from the IEEE (contact pubs–permissions@ieee.org).

By choosing to view this document, you agree to all provisions of the copyright laws protecting it.

- If the author is not the submitter of the contribution, the author’s written approval for the reuse of the material must be attached.