| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Security Procedures for Direct Communication in IEEE 802.16n |  |
| Date Submitted | 2011-06-11 |  |
| Source(s) | Joseph Teo Chee Ming, Jaya Shankar, Yeow Wai Leong, Hoang Anh Tuan, Wang Haiguang, Zheng Shoukang, Mar Choon Hock Institute For Infocomm Research | E-mail: cmteo@i2r.a-star.edu.sg |
| Re: | in response to the Contribution for Security Procedures in IEEE 802.16n |  |
| Abstract | Detail description of Security procedures for Direct Communications to be discussed and adopted to IEEE 802.16n AWD |  |
| Purpose | To discuss and adopt the proposed text in the 802.16n draft Text |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Security Procedure for Direct Communication in IEEE 802.16n

Joseph Teo Chee Ming, Jaya Shankar, Yeow Wai Leong, Hoang Anh Tuan, Wang Haiguang, Zheng Shoukang, Mar Choon Hock

Institute for Infocomm Research

# Introduction

This contribution contains the detailed proposed text from Security Procedures for Secure Direct Communications in IEEE 802.16n. It is to be proposed to placed into the AWD text for the IEEE 802.16n GridMAN standards. This contribution covers both Section 17.2.10 Security (for 802.16e) and also Section 17.3.10 Security (for 802.16m) as the Security Procedure is meant for both 16e and 16m standard.

It should be noted that:

- Proposed texts are placed in the Section they address.  If the text in the contribution addresses more than one section, then the text is split and placed under the appropriate Section numbers (or categories).

- All proposed texts from different contributions that address a particular section are together.

- Existing Section numbers are in black color with a bold face.

- Existing text are colored black

- All proposed sections are colored blue with an underline and bold face.

- All proposed text are initially colored blue with an underline.

[-------------------------------------------------Begin of Text Proposal----------------------------------------------------]

# 17.2.10 Security

17.2.10.1 Security Procedure for Direct Communication Data Security

17.2.10.1.1 Security Procedure for BS-coordinated Secure Direct Communication

17.2.10.1.1.a BS-coordinated Key Management Procedure for Secure Direct Communication

In order to support BS-coordinated secure direct communication, the security procedure described in this subsection shall be executed between HR-MS, HR-BS, Authenticator, and AAA Server. Upon successful completion of the security procedure, HR-MSs received the security key from the HR-BS and use this security key for secure direct communication between/among HR-MSs. This security key may be used as the pre-established shared key for secure direct communications in Section 17.2.10.1.1.1.1.

The HR-BS/Authenticator is used to denote that the HR-BS may pass the messages to the AAA-server via the Authenticator for verification and the AAA-server may compute the direct communication security key DMK and send it to the HR-BS via the Authenticator. The flow diagram is shown in Figure AAA while the flow chart is shown in Figure BBB.

The BS-coordinated security procedure includes the following steps:

Step 1: Once it is determined that secure direct communications is allowed between HR-MS1 and HR-MS2, HR-BS generates the security key DMK, selects NHR-BS and encrypts EHR-MS1_KEK(DMK, key_lifetime, HR-MS1Addr, HR-MS2Addr) and computes θHR-BS = MACCMAC1(“DC_REPLY_OK_BS”|THR-BS|NHR-BS|EHR-MS1_KEK(DMK, key_lifetime, HR-MS1Addr, HR-MS2Addr)|HR-MS1Addr|HR-MS2Addr) and sends Key-Transfer-MSG#1 message to HR-MS1, where Key-Transfer-MSG#1 = “DC_REPLY_OK_BS”|THR-BS|NHR-BS|EHR-MS1_KEK(DMK, key_lifetime, HR-MS1Addr, HR-MS2Addr)|HR-MS1Addr|HR-MS2Addr|θHR-BS. HR-BS also encrypts EHR-MS2_KEK(DMK, key_lifetime, HR-MS2Addr, HR-MS1Addr) and computes θHR-BS = MAC CMAC2(“DC_REPLY_OK_BS”|THR-BS|NHR-BS|EHR-MS2_KEK(DMK, key_lifetime, HR-MS2Addr, HR-MS1Addr)|HR-MS2Addr|HR-MS1Addr) and sends Key-Transfer-MSG#2 message to HR-MS2, where Key-Transfer-MSG#2 = “DC_REPLY_OK_BS”|THR-BS|NHR-BS|EHR-MS2_KEK(DMK, key_lifetime, HR-MS2Addr, HR-MS1Addr)|HR-MS2Addr|HR-MS1Addr|θHR-BS.

Step 2a: If HR-MS1 received Key-Transfer-MSG#1 message from HR-BS, HR-MS1 first checks THR-BS, NHR-BS for freshness and θHR-BS for message authentication. If the verifications fail, then HR-MS1 shall ignore Key-Transfer-MSG#1 message. If the verifications are correct, then HR-MS1 decrypts EHR-MS1_KEK(DMK, key_lifetime, HR-MS1Addr, HR-MS2Addr) and obtains the security key DMK and its lifetime key_lifetime.

Step 2b: Upon receiving the Key-Transfer-MSG#2 message, HR-MS2 first checks THR-BS, NHR-BS for freshness and θHR-BS for message authentication. If the verifications fail, HR-MS2 shall ignore the Key-Transfer-MSG#2 message. If the verifications are correct, then HR-MS2 decrypts EHR-MS2_KEK(DMK, key_lifetime, HR-MS2Addr, HR-MS1Addr) and obtains the security key DMK and its lifetime key_lifetime.

Figure AAA: Flow Diagram of Authentication and Key Establishment of Network Aided Direct Communication scenario.

Figure BBB: Flow Chart of Network Aided Direct Communication Authentication and Key Establishment Security Procedure.

## 17.2.10.1.1.a.a Message Type

Table aaa – Message Type

| Code | Message Type | MAC control message name |
| --- | --- | --- |
| a | Key-Transfer-MSG#1 | AAI-PKM-RSP |
| b | Key-Transfer-MSG#2 | AAI-PKM-RSP |

# 17.2.10.1.1.a.b Message Attributes

Table eee – Key-Transfer-MSG#1 message attribute

| Attribute | Contents |
| --- | --- |
| “DC_REPLY_OK_BS” | HR-BS response to HR-MS1 that HR-MS2 accepted direct communications |
| THR-BS | Timestamp generated by HR-BS |
| NHR-BS | Freshly generated random number of 64bits by HR-BS |
| EHR-MS1_KEK(DMK, key_lifetime, HR-MS1Addr, HR-MS2Addr) | Encryption of DMK, key lifetime by HR-BS using HR-MS1's KEK |
| HR-MS1Addr | Address of HR-MS1 |
| HR-MS2Addr | Address of HR-MS2 |
| θHR-BS | Message digest calculated using CMAC key by HR-BS |

Table fff – Key-Transfer-MSG#2 message attribute

| Attribute | Contents |
| --- | --- |
| “DC_REPLY_OK_BS” | HR-BS response to HR-MS1 that HR-MS2 rejected direct communications |
| THR-BS | Timestamp generated by HR-BS |
| NHR-BS | Nonce generated by HR-BS |
| EHR-MS2_KEK(DMK, key_lifetime, HR-MS2Addr, HR-MS1Addr) | Encryption of DMK, key lifetime by HR-BS using HR-MS2's KEK |
| HR-MS2Addr | Address of HR-MS2 |
| HR-MS1Addr | Address of HR-MS1 |
| θHR-BS | Message digest calculated using CMAC key by HR-BS |

## 17.2.10.1.1.1b Autonomous Mutual Authentication of HR-MS and data security for Direct Communications

### 17.2.10.1.1.1b.1 Secure direct communication using pre-established shared key

In order to support secure direct communication between two or among more HR-MSs, pre-established shared key is used.

The pre-established shared key is established prior to the start of this direct communications.

The pre-established shared key may be established using the procedure mentioned in Section 17.2.10.1.1.

The key agreement handshake procedure described below shall be used for HR-MSs to mutually authenticate themselves (without access to a security server) using the pre-established shared key and to derive data security keys for secure direct communications. Figure CCC shows the flow diagram while Figure DDD shows the flow chart for this scenario.

The key agreement handshake procedure using pre-established shared key includes the following steps:

Step 1: HR-MS1 selects nonce NHR-MS1 and uses the pre-established shared key DMK to compute DAK, DCMAC key and θHR-MS1 = MACDCMAC(N HR-MS1|DMK_Sequence_No|DAKID|Key_lifetime). Finally, HR-MS1 sends the DirectComms_KeyAgreement_MSG_#1 message to HR-MS2, where DirectComms_KeyAgreement_MSG_#1 = NHR-MS1|DMK_Sequence_No|DAKID|Key_lifetime|θHR-MS1.

Step 2: HR-MS2 first verifies the received nonce is fresh and uses the pre-established shared key DMK to compute DAK =Dot16KDF (DMK, HR-MS1Addr|HR-MS2Addr|”DAK”, 160), the DCMAC = Dot16KDF(DAK,  “DCMAC_KEYS”, 128) and uses DCMAC key to checks θHR-MS1. If the verification fails, HR-MS2 shall ignore the DirectComms_KeyAgreement_MSG_#1 message. If the verification is correct, HR-MS2 selects NHR-MS2 and computes θHR-MS2 = MACDCMAC(N HR-MS1|N HR-MS2|DAKID|DMK_Sequence_No|DC_Security_Parameters). Finally, HR-MS2 sends DirectComms_KeyAgreement_MSG_#2 message to HR-MS1, where DirectComms_KeyAgreement_MSG_#2 = N HR-MS1|N HR-MS2|DAKID|DMK_Sequence_No|DC_Security_Parameters|θHR-MS2.

Step 3: HR-MS1 receives the DirectComms_KeyAgreement_MSG_#2 message from HR-MS2 and checks the received nonces for freshness and also checks DAKID and θHR-MS2. If the verifications fail, HR-MS1 shall ignore the DirectComms_KeyAgreement_MSG_#2 message. If the verifications are correct, HR-MS1 computes θHR-MS1' = MACDCMAC(NHR-MS1|N HR-MS2|DMK_Sequence_No|DC_SAID|DC_Security_Parameters). Finally, HR-MS1 sends DirectComms_KeyAgreement_MSG_#3 message to HR-MS2, where DirectComms_KeyAgreement_MSG_#3 = NHR-MS1|N HR-MS2|DMK_Sequence_No|DC_SAID| DC_Security_Parameters|θHR-MS1'. If HR-MS1 does not receive DirectComms_KeyAgreement_MSG_#2 message from HR-MS2 within DirectComms_KeyAgreement_MSG_#1 Timeout, it shall resend the DirectComms_KeyAgreement_MSG_#1 message up to DirectComms_KeyAgreement_MSG_#1 MaxResends times. If HR-MS1 reaches its maximum number of resends, it shall initiate another authentication or drop the request.

Step 4: Upon receiving the DirectComms_KeyAgreement_MSG_#3 message, HR-MS2 checks the received nonces for freshness and θHR-MS1'. If the verifications are invalid, then HR-MS2 shall ignore the DirectComms_KeyAgreement_MSG_#3 message. If the verifications are correct, HR-MS2 applies the negotiated security parameters. Otherwise, if θHR-MS1' is invalid, then HR-MS2 shall ignore the DirectComms_KeyAgreement_MSG_#3 message. If HR-MS2 does not receive DirectComms_KeyAgreement_MSG_#3 message from HR-MS1 within DirectComms_KeyAgreement_MSG_#2 Timeout, it shall resend the DirectComms_KeyAgreement_MSG_#2 message up to DirectComms_KeyAgreement_MSG_#2 MaxResends times. If HR-MS2 reaches its maximum number of resends, it shall initiate another authentication or drop the request. HR-MS1 and HR-MS2 can now derive DTEK = DOT16KDF(DAK, “DTEK_KEY”, 128) and commence secure direct communications.

Figure CCC: Flow Diagram of Authentication and Key Establishment of Direct Communication without Infrastructure (Pre-shared key case).

HR-MS2 verifies the received CMAC tuple. If the verification is correct, HR-MS2 generates its nonce and uses the pre-shared DMK to compute DAK and DCMAC key. HR-MS2 then computes the CMAC tuple and sends DirectComms_KeyAgreement_MSG_#2 message to HR-MS1.

HR-MS1 verifies the received CMAC tuple and checks the DAKID. If the verifications are correct, then HR-MS1 computes its new CMAC tuple and sends DirectComms_KeyAgreement_MSG_#3 message to HR-MS2.

HR-MS1 selects its nonce and uses the pre-shared DMK to compute DAK and DCMAC key. HR-MS1 then computes the CMAC tuple and sends DirectComms_KeyAgreement_MSG_#1 message to HR-MS2.

HR-MS2 receives the above message and verifies the CMAC tuple. If the verification is correct, then HR- MS2 confirms that HR-MS1 has computed the correct keys and both HR-MS1 and HR-MS2 can now derive DTEK and commence secure direct communications.

Figure DDD: Flow Chart of Pre-shared key-based Autonomous Direct Communication Authentication and Key Establishment Security Procedure.

# 17.2.10.1.1.b.1.a Message Type

Table aaa – DC_Request_MSG#1 message attribute

| Code | Message Type | MAC control message name |
| --- | --- | --- |
|  | DirectComms_KeyAgreement_MSG #1 | AAI-PKM-RSP |
|  | DirectComms_KeyAgreement_MSG #2 | AAI-PKM-REQ |
|  | DirectComms_KeyAgreement_MSG #3 | AAI-PKM-RSP |

# 17.2.10.1.1.b.1.b Message Attributes

Table ggg – DirectComms_KeyAgreement_MSG_#1 message attribute

| Attribute | Contents |
| --- | --- |
| NHR-MS1 | Freshly generated random number of 64bits by HR-MS1 |
| DMK_Sequence_No | new DMK sequence number |
| DAKID | identifies the direct communications authorization key |
| Key_lifetime | DMK key lifetime |
| θHR-MS1 | Message digest calculated using DCMAC key |

Table hhh – DirectComms_KeyAgreement_MSG_#2 message attribute

| Attribute | Contents |
| --- | --- |
| NHR-MS1 | Nonce generated by HR-MS1 in DirectComms_KeyAgreement_MSG_#1 message |
| N HR-MS2 | Freshly generated random number of 64bits by HR-MS2 |
| DAKID | identifies the direct communications authorization key |
| DMK_Sequence_No | new DMK sequence number |
| DC_Security_Parameters | The requesting HR-MS's security capabilities |
| θHR-MS2 | Message digest calculated using DCMAC key |

Table iii – DirectComms_KeyAgreement_MSG_#3 message attribute

| Attribute | Contents |
| --- | --- |
| NHR-MS1 | Nonce generated by HR-MS1 in DirectComms_KeyAgreement_MSG_#1 message |
| N HR-MS2 | Nonce generated by HR-MS1 in DirectComms_KeyAgreement_MSG_#2 message |
| DMK_Sequence_No | new DMK sequence number |
| DC_SAID | identifies the direct communications authorization key for protecting this message |
| DC_Security_Parameters | The supporting HR-MS's security capabilities |
| θHR-MS1' | Message digest calculated using DCMAC key |

### 17.2.10.1.1.1b.2 Secure direct communication using Public Key Infrastructure

When pre-established shared key is not used for direct communication, Public Key Infrastructure shall be used.

Each HR-MS has a public/private key pair and digital certificate (e.g. X.509) issued by a certification authority for mutual authentication and key exchange prior to the start of this direct communications.

The key agreement handshake procedure described below shall be used for HR-MSs to mutually authenticate themselves (without access to a security server) using Public Key Infrastructure and to derive data security keys for secure direct communications. The flow diagram for this scenario is depicted in Figure EEE and the Flow Chart for this scenario is shown in Figure FFF.

The key agreement handshake procedure using Public Key Infrastructure includes the following steps:

Step 1: HR-MS1 first generates nonce NHR-MS1. Next, HR-MS1 computes the signature σHR-MS1 = SIGN(T HR-MS1|N HR-MS1|HR-MS2Addr|HR-MS1Addr) and sends DirectComms_KeyAgreement_MSG_#1 message to HR-MS2, where DirectComms_KeyAgreement_MSG_#1 = T HR-MS1|NHR-MS1|HR-MS2Addr|HR-MS1Addr|σHR-MS1|Cert(HR-MS1).

Step 2: HR-MS2 first verifies the received timestamp and nonce for freshness and the certificate Cert(HR-MS1) and signature σHR-MS1. If the verifications fail, then HR-MS2 ignores the DirectComms_KeyAgreement_MSG_#1 message. If the verifications are correct, then HR-MS2 generates nonce NHR-MS2 and security key DMK and computes DAK =Dot16KDF ( DMK, HR-MS1Addr|HR-MS2Addr| “DAK”, 160) and the DCMAC = Dot16KDF(DAK, “DCMAC_KEYS”, 128) and θHR-MS2 = MACDCMAC(N HR-MS2|NHR-MS1|HR-MS2Addr|HR-MS1Addr). HR-MS2 then uses HR-MS1's public key to encrypt and obtain EHR-MS1_PK(DMK, key_lifetime, HR-MS1Addr, HR-MS2Addr). Finally, HR-MS2 computes signature σHR-MS2 = SIGN(THR-MS2|NHR-MS2|HR-MS1Addr|HR-MS2Addr|NHR-MS1|EHR-MS1_PK(DMK, key_lifetime, HR-MS1Addr, HR-MS2Addr)|θHR-MS2) and sends DirectComms_KeyAgreement_MSG_#2 message to HR-MS1, where DirectComms_KeyAgreement_MSG_#2  = THR-MS2|NHR-MS2|HR-MS1Addr|HR-MS2Addr|NHR-MS1|EHR-MS1_PK(DMK, key_lifetime, HR-MS1Addr, HR-MS2Addr)|θHR-MS2|σHR-MS2|Cert({HR-MS2).

Step 3: HR-MS1 first verifies the received timestamp and nonces for freshness and the certificate Cert(HR-MS2) and signature σHR-MS2. If the verification is invalid, then HR-MS1 ignores the DirectComms_KeyAgreement_MSG_#2 message. If the verifications are correct, then HR-MS1 decrypts EHR-MS1_PK(DMK, key_lifetime, HR-MS1Addr, HR-MS2Addr) and obtains security key DMK and key_lifetime. Next, HR-MS1 computes DAK and DCMAC and verifies θHR-MS2. If the verification is invalid, then HR-MS1 ignores the DirectComms_KeyAgreement_MSG_#2 message. If the verification is correct, then HR-MS1 computes θHR-MS1 = MACDCMAC(N HR-MS1|N HR-MS2|HR-MS1Addr|HR-MS2Addr) and sends DirectComms_KeyAgreement_MSG_#3 message to HR-MS2, where DirectComms_KeyAgreement_MSG_#3 = NHR-MS2|HR-MS2Addr|HR-MS1Addr| θHR-MS1. If HR-MS1 does not receive DirectComms_KeyAgreement_MSG_#2 message from HR-MS2 within DirectComms_KeyAgreement_MSG_#1 Timeout, it shall resend the DirectComms_KeyAgreement_MSG_#1 message up to DirectComms_KeyAgreement_MSG_#1 MaxResends times. If HR-MS1 reaches its maximum number of resends, it shall initiate another authentication or drop the request.

Step 4: HR-MS2 receives the DirectComms_KeyAgreement_MSG_#3 message and verifies received nonce and the CMAC tuple. If the verification fails, then HR-MS2 ignores DirectComms_KeyAgreement_MSG_#3 message. If the verification is correct, then HR-MS2 confirms that HR-MS1 has computed the correct keys and commence secure direct communications. If HR-MS2 does not receive DirectComms_KeyAgreement_MSG_#3 message from HR-MS1 within DirectComms_KeyAgreement_MSG_#2 Timeout, it shall resend the DirectComms_KeyAgreement_MSG_#2 message up to DirectComms_KeyAgreement_MSG_#2 MaxResends times. If HR-MS2 reaches its maximum number of resends, it shall initiate another authentication or drop the request. HR-MS1 and HR-MS2 can now derive DTEK = DOT16KDF(DAK, “DTEK_KEY”, 128) and commence secure direct communications.

Figure EEE: Flow Diagram of Authentication and Key Establishment of Direct Communication without Infrastructure (HR-MS becomes HR-BS*case).

HR-MS1 sends a DirectComms_KeyAgreement_MSG_#1 message to HR-MS2 to request for a direct communication authentication and key establishment. The message contains a nonce, signature generated by HR-MS1, HR-MS1 and HR-MS2 addresses and HR-MS1’s digital certificate.

HR-MS2 verifies the received certificate and signature. If the verification is correct, HR-MS2 generates its nonce and DMK. Next, HR-MS2 computes DAK, DCMAC key and the CMAC tuple of the message. HR-MS2 then uses the public key of HR-MS1 to encrypt the DMK. Finally, HR-MS2 computes the signature and sends DirectComms_KeyAgreement_MSG_#2 message to HR-MS1. The message also contains HR-MS2’s digital certificate.

HR-MS1 verifies the received certificate and signature. If the verification is correct, then HR-MS1 decrypts the received encrypted message and obtains DMK. Next, HR-MS1 computes DAK and DCMAC key and verifies the received CMAC tuple. If the verification is correct, then HR-MS1 computes its CMAC tuple and sends DirectComms_KeyAgreement_MSG_#3 message to HR-MS2.

HR-MS2 receives the above message and verifies the CMAC tuple. If the verification is correct, then HR-MS2 confirms that HR-MS1 has computed the correct keys and both HR-MS1 and HR-MS2 derives DTEK and commence secure direct communications.

Figure FFF: Flow Chart of PKI-based Autonomous Direct Communication Authentication and Key Establishment Security Procedure.

# 17.2.10.1.1.b.2.a Message Type

Table aaa – DC_Request_MSG#1 message attribute

| Code | Message Type | MAC control message name |
| --- | --- | --- |
|  | DirectComms_KeyAgreement_MSG #1 | AAI-PKM-RSP |
|  | DirectComms_KeyAgreement_MSG #2 | AAI-PKM-REQ |
|  | DirectComms_KeyAgreement_MSG #3 | AAI-PKM-RSP |

# 17.2.10.1.1.b.2.b Message Attribute

Table jjj – DirectComms_KeyAgreement_MSG_#1 message attribute

| Attribute | Contents |
| --- | --- |
| T HR-MS1 | Timestamp generated by HR-MS1 |
| NHR-MS1 | Freshly generated random number of 64bits by HR-MS1 |
| HR-MS2Addr | Address of HR-MS2 |
| HR-MS1Addr | Address of HR-MS1 |
| σHR-MS1 | Signature of message generated by HR-MS1 using its RSA private key |
| Cert(HR-MS1) | Digital certificate of HR-MS1 |

Table kkk – DirectComms_KeyAgreement_MSG_#2 message attribute

| Attribute | Contents |
| --- | --- |
| THR-MS2 | Timestamp generated by HR-MS2 |
| NHR-MS2 | Freshly generated random number of 64bits by HR-MS2 |
| HR-MS1Addr | Address of HR-MS1 |
| HR-MS2Addr | Address of HR-MS2 |
| NHR-MS1 | Nonce generated by HR-MS1 in DirectComms_KeyAgreement_MSG_#1 message |
| EHR-MS1_PK(DMK, key_lifetime, HR-MS1Addr, HR-MS2Addr) | Public key encryption using HR-MS1's Public key where DMK = DirectComms Master Key generated by HR-MS and key_lifetime = lifetime of DMK |
| θHR-MS2 | Message digest calculated using DCMAC key by HR-MS2 |
| σHR-MS2 | Signature of message generated by HR-MS2 using its RSA private key |
| Cert(HR-MS2). | Digital certificate of HR-MS2 |

Table lll – DirectComms_KeyAgreement_MSG_#3 message attribute

| Attribute | Contents |
| --- | --- |
| NHR-MS2 | Nonce generated by HR-MS2 in DirectComms\_KeyAgreement\_MSG\_\#2 message |
| HR-MS2Addr | Address of HR-MS2 |
| HR-MS1Addr | Address of HR-MS1 |
| θHR-MS1 | Message digest calculated using DCMAC key by HR-MS1 |

17.2.10.1.2 Security Procedure for Secure talk-around Direct Communication using dedicated resource

In order to support secure direct communication between two or among more HR-MSs, pre-established shared key is used.

The key agreement handshake procedure described in Section 17.2.10.1.1.b.1 may be used for HR-MSs to mutually authenticate themselves using the pre-established shared key and to derive data security keys for secure talk-around direct communications.

# 17.2.10.1.x Security Context for Direct Communication Security

The direct communciations security context describes the set of parameters that links the direct communication security keys for secure secure direct communications.

### 17.2.10.1.x.a DMK context

The DMK context includes all parameters associate with the DMK. This context is created when the DMK is derived.

The DMK context is described in Table xxx.

Table xxx – The DMK context

| Parameter | Size (bit) | Usage |
| --- | --- | --- |
| DMK | 160 | Multicast Master Key shared by HR-BS and HR-MSs in a multicast group |
| DMK SN | 4 | DMK sequence number |
| DMK Lifetime | 32 | MMK Lifetime |
| DAK_COUNT | 16 | Counter to ensure freshness of computed CMAC key and prevent replay attacks. |

### 17.2.10.1.x.b DAK context

The DAK context includes all parameters associate with the DAK. This context is created whenever a new DAK is derived. This context shall be deleted when the DAK is not in used.

The DAK context is described in Table yyy.

Table xxx – The DAK context

| Parameter | Size (bit) | Usage |
| --- | --- | --- |
| DAK | 160 | Direct Communications Authentication Key derived from DMK. |
| DAK Lifetime | 32 | DAK Lifetime |
| DAKID | 64 | Identifies the DAK key. |
| DCMAC_KEY | 128 | Key which is used for signing Direct Communications MAC control messages. |
| DCMAC_PN | 24 | Used to avoid multicast replay attack on the control connection. The initial value of DCMAC_PN is zero. |
| DAK_COUNT | 16 | Counter to ensure freshness of computed CMAC key and prevent replay attacks. |

### 17.2.10.1.x.c DSA context

The DSA context is the set of parameters managed by each DSA in order to ensure DTEK management and usage in a secure way for secure direct communications.

The DSA holds the DTEK context and additional information that belongs to the DSA itself.

### 17.2.10.1.x.d DTEK context

The DTEK context includes all parameters of the DTEK and is described in Table zzz.

Table zzz – The DTEK context

| Parameter | Size (bit) |  | Usage |  |  |
| --- | --- | --- | --- | --- | --- |
| DTEK | 128 |  | Key used for encryption or decryption of direct communications messages |  |  |
| DMK SN |  | 4 |  | DMK sequence number |  |
| COUNTER_DTEK | 16 |  | The counter used to derive this DTEK |  |  |
| DTEK lifetime | 32 |  | DTEK lifetime=DMK lifetime |  |  |
| DTEK_PN | 22 |  | The PN used for encrypting multicast packets. After each Multicast MAC PDU transmission, the value shall be increased by 1. (0x000000-0x1FFFFF) |  |  |

### 17.2.10.1.x.e DSA context

The DSA context is described in Table aaa.

Table aaa – The DSA context

| Parameter | Size (bit) | Usage |
| --- | --- | --- |
| DSAID | 8 | The identifier of this DSA, which decribes the applied encryption/decryption method and DTEK contexts. |
| DTEK context | Sizeof(DTEK context) | DTEK context for encryption and decryption |

# 17.2.10.1.y Key Derivation

The key hierarchy defines what keys are present in the system for secure direct communication and how the keys are generated.

17.2.10.1.y.a DMK Derivation

The DMK is the security key/pre-established shared key that is randomly generated by HR-BS or HR-MS or a network entity (e.g. an AAA Server etc). The DMK is a 160-bit key.

The DMK may be used as a source for keying materials required by upper layers.

The DMK is used to derive the Direct Communication Authentication Key (DAK).

17.2.10.1.y.b DAK Derivation

DAK is derived from DMK and belongs to a pair of HR-MSs. The DAK is used for Direct Communications in the event of failure in the backbone.

The DAK derivation is as follows:

DAK =Dot16KDF (DMK, HR-MS1Addr|HR-MS2Addr|”DAK”, 160)

where: HR-MS1Addr and HR-MS2Addr are the addresses of HR-MS1 and HR-MS2 respectively.

The DCMAC-DTEK prekey is derived from DAK and is used to derive other keys:

- Direct Communication Cipher-based Message Authentication Code (DCMAC) key

- Direct Communication Traffic Encryption (DTEK) Key

The DCMAC-DTEK prekey derivation is done as follows:

DCMAC-DTEK prekey = Dot16KDF (DAK, DAK_COUNT|”DCMAC-DTEK prekey”, 160)

17.2.10.1.y.c DCMAC Key Derivation

DCMAC key is derived from DAK and used for message authentication for the messages sent during direct communications.

DCMAC key is derived as follows:

DCMAC key = Dot16KDF(DCMAC-DTEK prekey, “DCMAC_KEYS”, 128)

17.2.10.1.y.d DTEK Derivation

DTEK is the transport encryption key used to encrypt data in direct communications.

DTEK is derived as follows:

DTEK = Dot16KDF(DCMAC-DTEK prekey, DSAID|COUNTER_DTEK|”DTEK_KEY”, 128)

Where

SAID is the security association to which the TEK belongs.

COUNTER_DTEK is a counter used to derive different TEKs for the same SAID, the value of the counter is changed everytime a new DTEK needs to be derived within the same AK and AK_COUNT pair is valid. Everytime a new DCMAC-DTEK prekey is derived, this counter is reset.

# 17.3.10 Security

17.3.10.1 Security Procedure for Direct Communication Data Security

17.3.10.1.1 Security Procedure for BS-coordinated Secure Direct Communication

Refer to Section 17.2.10.1.1.

17.3.10.1.2 Security Procedure for Secure talk-around Direct Communication using dedicated resource

Refer to Section 17.2.10.1.2.

# 17.3.10.1.x Security Context for Direct Communication Security

Refer to Section 17.2.10.1.x

# 17.3.10.1.y Key Derivation

Refer to Section 17.2.10.1.y

[-------------------------------------------------End of Text Proposal----------------------------------------------------]