| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | MAC control messages to support multimode operation |  |
| Date Submitted | 2011-07-20 |  |
| Source(s) | Eunkyung Kim, Sungcheol Chang, Sungkyung Kim, Won-Ik Kim, Miyoung Yun, Hyun Lee, Chulsik Yoon, Kwangjae Lim ETRI Anh Tuan Hoang, Haiguang Wang, Jaya Shankar, Wai Leong Yeow, Shoukang Zheng, Choong Hock Mar I2R | E-mail: ekkim@etri.re.kr scchang@etri.re.kr athoang@i2r.a-star.edu.sg hwang@i2r.a-star.edu.sg jshankar@i2r.a-star.edu.sg |
| Re: | “IEEE 802.16n-11/0008,” in response to the 802.16n (GRIDMAN) AWD Call for Comments |  |
| Abstract | Multimode MAC control messages for IEEE 802.16n |  |
| Purpose | To discuss and adopt the proposed text in the AWD of 802.16n |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# MAC control messages to support multimode operation

Eunkyung Kim, Sungcheol Chang, Sungkyung Kim, Won-Ik Kim, Miyoung Yun, Hyun Lee, Chulsik Yoon, Kwangjae Lim

ETRI

Anh Tuan Hoang, Haiguang Wang, Jaya Shankar, Wai Leong Yeow, Shoukang Zheng, Choong Hock Mar

I2R

# Introductions

IEEE 802.16n AWD [5] describes multimode operation based on the 802.16n SRD [1].

This document provides (modified/new) MAC control messages to support multimode operation in detail as follows:

- (modified) AAI-SBC-REQ

transmitted by MS to report multimode basic capability during network entry.

- (modified) AAI-ARS-CONFIG-CMD message format

An ABS shall use AAI-ARS-CONFIG-CMD message to configure the TTR mode ARS PHY layer operational parameters.

An HR-BS shall use AAI-ARS-CONFIG-CMD message to configure the multimode HR-MS acting as HR-RS PHY layer operational parameters.

- (new) AAI-MMRS-REQ

To establish relay link between a multimode station and superordinate HR-BS, AAI-MMRS-REQ message is transmitted by the station or the superordinate HR-BS.

- (new) AAI-MMRS-RSP

An AAI-MMRS-RSP message is transmitted by multimode station or superordinate HR-BS in response to AAI-MMRS-REQ message.

- (new) AAI-MMRL-REQ message

HR-MS transmits AAI-MMRL-REQ message for the purpose as follows:

  - to release its relay mode and to return its original role

  - to response or reject the unsolicited AAI-MMRL-RSP message by the HR-BS

- (new) AAI-MMRL-RSP message

An AAI-MMRL-RSP message is transmitted by multimode station or superordinate HR-BS in response to AAI-MMRL-REQ message.

- (new) MM-ADV message

Infrastructure stations and HR-MS acting as HR-BS or HR-RS may transmit AAI-MM-ADV message to support multimode operation in the case as follows:

  - When the backhaul link is down or up from down

  - During maintaining relay link due to unavailable backhaul link, PHY layer parameters need be reconfigured such as power and FA

  - During release multimode service (return to original mode), multimode HR-MS needs announce subordinate HR-MSs to perform handover to any infrastructure station.

In addition, procedure to establish/maintain/release relay link between HR-BSs and between HR-BS and HR-MS capable of acting as RS is provided based on the [6][7][8].

# References

- IEEE 802.16n-10/0048r1, “802.16n System Requirements Document including SARM annex,” March 2011.

- IEEE Std. 802.16TM-2009, “IEEE Standard for Local and metropolitan area networks; Part 16: Air Interface for Broadband Wireless Access Systems,” May 2009.

- IEEE 802.16mTM-2011, IEEE Standard for Local and metropolitan area networks; Part 16: Air Interface for Broadband Wireless Access Systems; Amendment 3: Advanced Air Interface,” May 2011.

- IEEE 802.16m-09/0034r3, “IEEE 802.16m System Description Document (SDD),” June 2010.

- IEEE 802.16n-11/0009r1, “802.16n Amendment Working Draft,” May 2011.

- IEEE C802.16n-11/0139r2, “TTR Mode for HR-BS Acting as HR-RS,” July 2011.

- IEEE C802.16n-11/0122, “Relay function for HR-BS,” July 2011.

- IEEE C802.16n-11/0123, “Relay function for HR-MS,” July 2011.

# Proposed Text for the 802.16n Amendment Working Document (AWD)

Note:

The text in BLACK color: the existing text in the 802.16n Amendment Draft Standard

The text in RED color: the removal of existing 802.16n Amendment Draft Standard Text

The text in BLUE color: the new text added to the 802.16n Amendment Draft Standard Text

[-------------------------------------------------Start of Text Proposal---------------------------------------------------]

[Remedy1: Add the following rows in line 173, section 16.2.3 of 802.16n AWD.]

| xx | Multimode | AAI-MM-ADV | Multimode advertisement | N/A | Broadcast |
| --- | --- | --- | --- | --- | --- |
| xx+1 | Multimode | AAI-MMRS-REQ | Multimode Relay request | Encrypted/ICV | Unicast |
| xx+2 | Multimode | AAI-MMRS-RSP | Multimode Relay response | Encrypted/ICV | Unicast |
| xx+3 | Multimode | AAI-MMRL-REQ | Multimode releasement request | Encrypted/ICV | Unicast |
| xx+4 | Multimode | AAI-MMRL-RSP | Multimode releasement response | Encrypted/ICV | Unicast |

[Remedy2: Add the following text into section 16.2.3 of 802.16n AWD.]

16.2.3 MAC Control messages

[Change Section 16.2.3.5 as indicated:]

16.2.3.5 AAI-SBC-REQ

An AAI-SBC-REQ message, to which HARQ operation is applied, is transmitted by AMS to negotiate basic capability during network entry.

Table 688 – AAI-SBC-REQ message field description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| If (AMS requests transmittal of NSP information) { |  |  |  |
| SIQ (Service Information Query) | 2 | Bit 0: Indicates that the AMS requests transmittal of the NSP List for the list of NSP IDs supported by the Operator Network that includes the current ABS; Bit 1: Indicates that the AMS requests transmittal of the Verbose NSP Name List in addition to the NSP List; bit 1 shall not be set to a value of ‘1’ unless bit 0 is set to 1 |  |
| } else { |  |  |  |
| CAPABILITY_INDEX | 5 | It refers to the “Capability Class” that the AMS can support. Value: 0~31 |  |
| DEVICE_CLASS | 5 | It refers to the “Device Class” that the AMS can support. Value: 0~31 |  |
| CLC Request | variable | See Table 700 | Present if AMS requests to activate one Type I or II CLC class for fast CLC class activation during initial network entry |
| Long TTI for DL | 1 | If Bit 0=1, it supports | Present as needed |
| UL sounding | 2 | If Bit 0=1, decimation separation based sounding (FDM) supports If Bit 1=1, cyclic shift separation based sounding (CDM) supports | Present as needed |
| OL Region | 3 | If Bit 0=1, OL Region type 0 supports If Bit 1=1, OL Region type 1, CDR and CoFIP supports If Bit 2=1, OL Region type 2 supports | Present as needed |
| DL resource metric for FFR | 1 | If Bit 0=1, it supports | Present as needed |
| Max. Number of streams for SU-MIMO in DL MIMO | 3 | The number in the range 1 through 8 that is higher by 1 than this field | Present as needed |
| Max. Number of streams for CL MU-MIMO (MIMO mode 4) in AMS point of view in DL MIMO | 1 | The number in the range 1 through 2 that is higher by 1 than this field | Present as needed |
| DL MIMO mode | 6 | If Bit 0 =1, mode0 supports If Bit 1 =1, mode1 supports If Bit 2 =1, mode 2 supports If Bit 3 =1, mode 3 supports If Bit 4 =1, mode 4 supports If Bit 5=1, mode 5 supports | Present as needed |
| feedback support for DL | 11 | If Bit 0 =1, differential mode supports If Bit 1 =1, MIMO feedback mode 0 supports If Bit 2 =1, MIMO feedback mode 1 supports If Bit 3=1, MIMO feedback mode 2 supports If Bit 4 =1, MIMO feedback mode 3 supports If Bit 5 =1, MIMO feedback mode 4 supports If Bit 6 =1, MIMO feedback mode 5 supports If Bit 7 =1, MIMO feedback mode 6 supports If Bit 8 =1, MIMO feedback mode 7 supports If Bit 9 =1, Long-term reporting disabling support for MFM 0,4,7 If Bit 10 =1, Short-term reporting disabling support for MFM 2,3,5,6 | Present as needed |
| Subband assignment A-MAP IE support | 1 | If Bit 0=1, DL/UL Subband assignment A-MAP IE supports | Present as needed |
| DL pilot pattern for MU MIMO | 2 | If Bit 0 =1, DL 4 stream pilot pattern for DL MU MIMO support If Bit 1 =1, DL 8 stream pilot pattern for DL MU MIMO support | Present as needed |
| Number of Tx antenna of AMS | 2 | The number in the range {1, 2, 4} that is higher by 1 than this field | Present as needed |
| Max. Number of streams for SU-MIMO in UL MIMO(1/2/3/4) | 2 | The number in the range 1 through 4 that is higher by 1 than this field | Present as needed |
| Max. Number of streams for MU-MIMO in AMS point of view in UL MIMO(1/2/3/4) | 2 | The number in the range 1 through 4 | Present as needed |
| UL pilot pattern for MU MIMO | 3 | If Bit 0 =1, UL 2 stream pilot pattern support If Bit 1 =1, UL 4 stream pilot pattern support If Bit 2 =1, UL 8 stream pilot pattern support | Present as needed |
| UL MIMO mode | 5 | If Bit 0 =1, mode0 supports If Bit 1 =1, mode1 supports If Bit 2 =1, mode 2 supports If Bit 3 =1, mode 3 supports If Bit 4 =1, mode 4 supports | Present as needed |
| Modulation scheme | 2 | If Bit 0=1, DL 64 QAM supports If Bit 1=1, UL 64 QAM supports | Present as needed |
| UL HARQ buffering capability | 7 | Bit 0–6: The number that is higher by 1 than this field is the amount of information bits in 4800 bytes units the AMS can buffer in the UL. | Present as needed |
| DL HARQ buffering capability | 7 | Bit 0–6: The number that is higher by 1 than this field, is the steady amount of aggregated DL HARQ information bits per frame in units of 4800 bytes, at which the aimed combining gain or better is obtained in the benchmark scenario, as defined in 16.2.14.2.1.3. | Present as needed |
| AMS DL processing capability per subframe | 7 | Bit 0–6: The number that is higher by 1 than this field, is the steady amount of aggregated DL data information bits per subframe in units of 600 bytes that the AMS can process. | Present as needed |
| AMS UL processing capability per subframe | 7 | Bit 0–6: The number that is higher by 1 than this field, is the steady amount of aggregated UL data information bits per subframe in units of 600 bytes that the AMScan process. | Present as needed |
| FFT size(2048/1024/512) | 3 | If Bit 0 = 1, FFT 2048 supports If Bit 1 = 1, FFT 1024 supports If Bit 2 = 1, FFT 512 supports | Present as needed |
| Authorization policy support | 1 | If Bit 0 = 0, No authorization; If Bit 0 = 1, EAP-based authorization is supported. | Present as needed |
| Inter-RAT Operation Mode | 2 | 0b00: single radio mode operation for inter RAT handover 0b01: multi radio mode operation for inter RAT handover 0b10–0b11: Reserved | Present as needed |
| Supported Inter-RAT type | 8 | 1 indicates support, 0 indicates not supported: Bit 0: IEEE 802.11 Bit 1: GERAN(GSM/GPRS/EGPRS) Bit 2: UTRAN Bit 3: E-UTRAN Bit 4: CDMA 2000 Bit 5–7: Reserved, set to zero | Present as needed |
| MIH Capability Supported | 1 | If Bit 0=1, the capability of IEEE 802.21 Media Independent Handover Services supports. | Present as needed |
| MAX Tx Power | 24 | The maximum available power of the carrier for initial network entry. Bit 0–7: Maximum transmitted power for QPSK. Bit 8–15: Maximum transmitted power for 16-QAM Bit 15–23: Maximum transmitted power for 64-QAM. Each unsigned 8-bit integer specifies the maximum transmitted power value in dBm. The maximum transmitted power is quantized in 0.5 dBm steps ranging from –64 dBm (encoded 0x00) to 63.5 dBm (encoded 0xFF). Values outside this range shall be assigned the closest extreme. If AMS does not support 64-QAM, the AMS shall report the value of 0x00 for Bit 15–23. | Present as needed |
| If (ARS is a sender of AAI-SBC- REQ) { |  |  | //only available during ARS network entry phase |
| Relay mode | 1 | 0b0: TTR relay mode 0b1: STR relay mode |  |
| if (Relay mode == 0b0){ |  |  |  |
| ARSTTG | 6 | ARSTTG value (μs). It shall be less than 50 μs. |  |
| ARSRTG | 6 | ARSRTG value (μs). It shall be less than 50 μs. |  |
| } |  |  |  |
| } |  |  |  |
| Visited NSP ID | 24 | NSP ID of the Network Service Provider the AMS intends to be the conduit for authentication to the AMS home network | Present as needed |
| Multimode capability supported | 3 | If bit0 = 1, the capability of TTR relay mode supports If bit1 = 1, the capability of STR relay mode supports If bit2 = 1, the capability of base station function supports | Present as needed in HR-Network |
| } |  |  |  |

[Change the text in 16.2.3.57 as follows:]

16.2.3.57 AAI-ARS-CONFIG-CMD message format

An ABS shall use AAI-ARS-CONFIG-CMD message to configure the TTR mode ARS PHY layer operational parameters.

An HR-BS shall use AAI-ARS-CONFIG-CMD message to configure the multimode HR-MS acting as HR-RS PHY layer operational parameters.

[Change the table 757 in 16.2.3.57 as follows:]

Table 757—AAI-ARS-CONFIG-CMD message field description

| Field | Size (bits) | Value/Description | Conditions |
| --- | --- | --- | --- |
| If(subordinate RS (including HR-MS acting as RS) is TTR relay mode in HR-Network) { |  |  | // TTR mode |
| AAI_Relay_zone_AMS_allocation_indicator | 1 | 0b0: The ABS does not allocate resources to the AMS in the AAI DL Relay zone; 0b1: The ABS may allocate resources to the AMS in the AAI DL Relay zone | Always present |
| MIMO Midamble indication in AAI DL Relay zone | 1 | 0b0: MIMO midamble is not transmitted in AAI DL Relay zone 0b1: MIMO midamble is transmitted in AAI DL Relay zone If AAI_Relay_zone_AMS_allocation_indicator == 0b0, this field is set to 0b1. | Always present |
| Superframe Number Action | 4 | LSBs of the superframe number when ARS start ARS operation and apply the PHY operational parameters. | Always present |
| R_IdleTime | 11 | Unit is 0.1 μs | Always present |
| If(ABS allocates resource for periodic ranging in AAI UL Relay zone) { |  |  |  |
| Allocation periodicity of the S-RCH | 2 | Indicates the periodicity of the S-RCH allocation. 0b00: Every frame 0b01: The second frame in every superframe 0b10: The second frame in every 4th superframe, i.e., mod(superframe number, 4) = 0 0b11: The second frame in every 8th superframe, i.e., mod(superframe number, 8) = 0 | Present when ABS allocates resource for periodic ranging in AAI UL Relay zone |
| Subframe offset of the S-RCH | 2 | Indicates the subframe offset (OSF) of the S-RCH allocation. The range of values is 0 ≤OSF ≤3. S-RCH is allocated in the (OSF +UAZ) subframe. | Present when ABS allocates resource for periodic ranging in AAI UL Relay zone |
| Start RP code information of the S-RCH | 4 | Indicates the ks which is the parameter controlling the start root index of the RP codes (rs0). rs0=6ks+1 The range of values is 0≤ks≤15. | Present when ABS allocates resource for periodic ranging in AAI UL Relay zone |
| NPE | 2 | Indicates the number of periodic code (NPE) according to the Table 917. | Present when ABS allocates resource for periodic ranging in AAI UL Relay zone |
| } |  |  |  |
| If(ABS allocates resource for BR channel in AAI UL Relay zone) { |  |  |  |
| UL BW REQ channel information | 2 | Indicates the number and the location of UL AAI subframe where the UL BW REQ channels are allocated. 0b00: i-th UL AAI subframe of UL relay zone in the first frame in every superframe 0b01: i-th UL AAI subframe of UL relay zone in the first and second frame in every superframe 0b10: i-th UL AAI subframe of UL relay zone in every frame 0b11: i-th and (i+1)-th UL AAI subframes of UL relay zone in every frame Where i-th is “first” if UL R-RTI = 0, and i-th is “second” if UL R-RTI=Ts. | Present when ABS allocates resource for BR channel in AAI UL Relay zone |
| UL BW REQ channel allocation | 4 | The DRU index for UL BW REQ channel within FPi defined by “Frequency partition location for UL control channels” in S-SFH SP1. | Present when ABS allocates resource for BR channel in AAI UL Relay zone |
| Bandwidth request backoff start | 4 | Initial backoff window size for contention BRs, expressed as a power of 2. Values of n range 0–15 (the highest order bits shall be unused and set to 0) | Present when ABS allocates resource for BR channel in AAI UL Relay zone |
| Bandwidth request backoff end | 4 | Final backoff window size for contention BRs, expressed as a power of 2. Values of n range 0–15 | Present when ABS allocates resource for BR channel in AAI UL Relay zone |
| } |  |  |  |
| If(AAI_Relay_zone_AMS_allocation_indicator == 0b0){ |  |  |  |
| R_DCASSB0 | 5/4/3 | Indicates the number of subband-based CRUs in FP0 for AAI DL Relay zone. See 16.6.3.3.2 Cell-specific resource mapping For 2048 FFT size, 5 bits For 1024 FFT size, 4 bits For 512 FFT size, 3 bits | Present when AAI_Relay_zone_AMS_allocation_indicator ==0b0 |
| R_DCASMB0 | 5/4/3 | Indicates the number of miniband-based CRUs in FP0 for AAI DL Relay zone. See 16.6.3.3.2 Cell-specific resource mapping For 2048 FFT size, 5 bits For 1024 FFT size, 4 bits For 512 FFT size, 3 bits | Present when AAI_Relay_zone_AMS_allocation_indicator ==0b0 |
| R_DCASi | 3/2/1 | Indicates the number of total allocated CRUs, in a unit of a subband, for FPi (i > 0) for AAI DL Relay zone. See 16.6.3.3.2 Cell-specific resource mapping For 2048 FFT size, 3 bits For 1024 FFT size, 2 bits For 512 FFT size, 1 bit | Present when AAI_Relay_zone_AMS_allocation_indicator ==0b0 |
| R_UCASSB0 | 5/4/3 | Indicates the number of total allocated CRUs, in a unit of a subband, for FPi (i > 0) for AAI DL Relay zone. See 16.6.3.5.1 Cell-specific resource mapping For 2048 FFT size, 5 bits For 1024 FFT size, 4 bits For 512 FFT size, 3 bits | Present when AAI_Relay_zone_AMS_allocation_indicator ==0b0 |
| R_UCASMB0 | 5/4/3 | Indicates the number of miniband-based CRUs in FP0 for AAI UL Relay zone. See 16.6.3.5.1 Cell-specific resource mapping For 2048 FFT size, 5 bits For 1024 FFT size, 4 bits For 512 FFT size, 3 bits | Present when AAI_Relay_zone_AMS_allocation_indicator ==0b0 |
| R_UCASi | 3/2/1 | Indicates the number of total allocated CRUs, in a unit of a subbands, for FPi (i > 0) for AAI UL Relay zone. See 16.6.3.5.1 Cell-specific resource mapping For 2048 FFT size, 3 bits For 1024 FFT size, 2 bits For 512 FFT size, 1 bit | Present when AAI_Relay_zone_AMS_allocation_indicator ==0b0 |
| } |  |  |  |
| } |  |  | // TTR mode only |
| If (subordinate HR-MS is multimode MS acting as HR-RS in HR-Network) { |  |  |  |
| SA-PREAMBLE index | 10 |  | Always present |
| MS functionality maintenance indication | 1 | 0b0: MS functionality is maintained after role change 0b1: MS functionality is not maintained | Always present |
| Cell bar information | 1 | If Cell bar bit == 0b1, this cell shall not be allowed for network entry or reentry | Always present |
| If (subordinate HR-MS is acting as STR relay mode) { |  |  |  |
| Frame configuration index | 6 | The mapping between value of this index and frame configuration is listed in Table 806, Table 807, and Table 808. | Always present |
| FFT size indication | 2 | 0b00: 2048 FFT 0b01: 1024 FFT 0b10: 512 FFT 0b11: reserved | Always present |
| DL carrier frequency for BS and RS (FBR_DL) | 10 | Indicates the DL carrier frequency in unit of 100KHz for MS acting as RS. Used to receive from HR-BS in the DL relay zone. | Present if needed |
| UL carrier frequency for BS and RS (FBR_UL) | 10 | Indicates the UL carrier frequency in unit of 100KHz for MS acting as RS. Used to transmit to HR-BS in the UL relay zone. | Present if needed |
| DL carrier frequency for RS and MS (FRM_DL) | 10 | Indicates the DL carrier frequency in unit of 100KHz for MS acting as RS in FDD. If the duplex mode is TDD, this carrier is used for DL/UL Used to transmit to subordinate HR-MS in the DL in FDD. Used to transmit/receive to/from subordinate HR-MS in TDD. | Shall be present if FRM_DL is different from that of HR-BS’ DL access zone |
| UL carrier frequency for RS and MS (FRM_UL) | 10 | Indicates the UL carrier frequency in unit of 100KHz for MS acting as RS in FDD. Used to transmit to subordinate HR-MS in the UL in FDD. | Shall be present if FRM_UL is different from that of HR-BS’ UL access zone |
| Superframe Number Action | 4 | LSBs of the superframe number when HR-RS start RS operation and apply the PHY operational parameters. | Always present |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

[Remedy3: Add the following text at the end of 16.2.3 of 802.16n AWD.]

16.2.3.xa AAI-MM-ADV message

Infrastructure stations and HR-MS acting as HR-BS or HR-RS may transmit AAI-MM-ADV message to support multimode operation in the case as follows:

- When the backhaul link is down or up

- During maintaining relay link due to unavailable backhaul link, PHY/MAC layer parameters need be reconfigured such as

  - Power down

  - Power reduction

  - FA change

- Multimode service establish/release/change to inform subordinate stations to perform handover

Table xyz—Parameters for AAI-MM-ADV message

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| Action Type | 3 | Used to indicate the purpose of this message 0b000: Reconfiguration of HR-BS/RS including multimode BS/RS 0b001: Restart of HR-BS/RS including multimode BS/RS 0b010: Power down (including FA down) of HR-BS/RS including multimode BS/RS 0b011: Power reduction of HR-BS/RS including multimode BS/RS 0b100: Backhaul link down of HR-BS 0b101: Backhaul link up of HR-BS 0b110: FA change of HR-BS/RS including multimode BS/RS 0b111: Multimode service end of HR-MS | Mandatory |
| If (Action Type == 0b000) { |  |  | // reconfiguration |
| New IDcell | 10 | New IDcell that the ABS will use after the reconfiguration process. | Optional |
| Frame configuration index | 6 | New mapping between value of this index and frame configuration is listed in Table 806, Table 807, and Table 808. | Optional |
| Unavailable Start Time (UST) | 8 | Start of unavailable time in unit of frame | Mandatory |
| Unavailable Time Interval (UTI) | 8 | Interval of unavailable time in unit of superframe | Mandatory |
| } else if (Action Type == 0b001) { |  |  | // restart |
| Unavailable Start Time (UST) | 8 | Start of unavailable time in unit of frame | Mandatory |
| Unavailable Time Interval (UTI) | 8 | Interval of unavailable time in unit of superframe | Mandatory |
| } else if (Action Type == 0b010) { |  |  | // power down |
| Time of Power Down | 8 | Expected time when the HR-BS will be powered down in units of frame | Mandatory |
| Expected uptime of BS | 8 | Expected uptime of BS in units of superframe | Optional |
| } else if (Action Type == 0b011) { |  |  | // power reduction |
| Tx Power Reduction | 10 | dB value of Tx power reduction | Mandatory |
| Expected time of power reduction | 8 | Expected resource adjustment time in units of frame |  |
| } else if (Action Type == 0b100) { |  |  | // backhaul link down |
| Time of backhaul link down | 8 | Expected time when the backhaul link will be down in units of superframe | Optional |
| Expected time of backhaul link available | 8 | Expected time in unit of LSB of superframe when backhaul link will be available of HR-BS either itself or via neighbor HR-BS | Optional |
| } else if (Action Type == 0b101) { |  |  | // backhaul link up |
| Expected time of backhaul link up | 8 | Expected time in unit of LSB of superframe when the HR-BS restarts service without any help of neighbor BS using relay link but the HR-BS’ backhaul link | Optional |
| } else if (Action Type == 0b011) { |  |  | // power reduction |
| Tx Power Reduction | 10 | dB value of Tx power reduction | Mandatory |
| Expected time of power reduction | 8 | Expected resource adjustment time in units of frame |  |
| } else if (Action Type == 0b111) { |  |  | // multimode service end |
| Expected time of backhaul link up | 8 | Expected time in unit of LSB of superframe when the HR-MS release the multimode service and to allow subordinate MS to perform handover to other infrastructure | Optional |
| } |  |  |  |

16.2.3.xb AAI-MMRS-REQ

To establish relay link between a multimode station and superordinate HR-BS, AAI-MMRS-REQ message is transmitted by the multimode station or the superordinate HR-BS.

Table xxx—AAI-MMRS-REQ message field description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| Request Relay mode | 1 | 0b0: TTR relay mode 0b1: STR relay mode | Always present |
| If(this request is subordinate station initiated request) { |  |  | Shall be present when subordinate station initiates AAI-MMRS-RSP |
| If (request relay mode == 0b0) { |  |  | // TTR |
| ST-TTG | 6 | Transmit-to-receive turnaround gap of subordinate station, i.e., HR-MS or HR-BS, in units of μs. It shall be less than 50 μs. |  |
| ST-RTG | 6 | Receive-to-transmit turnaround gap of subordinate station, i.e., HR-MS or HR-BS, in units of μs. It shall be less than 50 μs. |  |
| If (subordinate station is HR-BS) { |  |  |  |
| Ta | 11 | Proposed value of timing advance Ta , in units of 0.1 μs |  |
| Tbs | 5 | Proposed duration of the BS Operation mode, in units of frames |  |
| Trs | 5 | Proposed duration of the BS Operation mode, in units of frames |  |
| } |  |  |  |
| 11Proposed value of timing advance Ta , in units of 0.1 μs5Proposed duration of the BS Operation mode, in units of frames5Proposed duration of the BS Operation mode, in units of frames} else if (request relay mode == 0b1) { |  |  | // STR |
| Duplex mode support indication | 2 | If bit0 = 1, FDD supports If bit1 = 1, TDD supports |  |
| for(i=1; i<=N-frequency; i++) { |  | N-frequency is the number of available frequency to communicate[1..16] |  |
| Carrier frequency | 10 | Indicates the carrier frequency in unit of 100KHz. |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| If (this request is superordinate station initiated request && received subordinate station is HR-BS) { |  |  | Shall be present when Superordinate HR-BS initiates AAI-MMRS-REQ |
| If (request relay mode == 0b0) { |  |  | // TTR |
| Ta | 11 | Proposed value of timing advance Ta , in units of 0.1 μs |  |
| Tbs | 5 | Proposed duration of the BS Operation mode, in units of frames |  |
| Trs | 5 | Proposed duration of the BS Operation mode, in units of frames |  |
| } |  |  |  |
| } |  |  |  |

16.2.3.xc AAI-MMRS-RSP

An AAI-MMRS-RSP message is transmitted by multimode station or superordinate HR-BS in response to AAI-MMRS-REQ message.

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| If(the response is transmitted by superordinate HR-BS) { |  |  | Present when superordinate HR-BS responds the subordinate station initiated request |
| Response code | 2 | 0b00: in response to the AAI-MMRS-REQ message to accept the request 0b01: in response to the AAI-MMRS-REQ message to allow to transmit subordinate station initiated AAI-ARE-REQ after action time expires 0b10: in response to the AAI-MMRS-REQ message to reject the request 0b11: reserved |  |
| If(Response code == 0b00 and the request was sent by an HR-BS wishing to establish TTR relay link){ |  |  |  |
| Ta | 11 | Confirmed value of timing advance Ta , in units of 0.1 μs |  |
| Tbs | 5 | Confirmed duration of the BS Operation mode, in units of frames |  |
| Trs | 5 | Confirmed duration of the BS Operation mode, in units of frames |  |
| } |  |  |  |
| If(Response code == 0b01) { |  |  |  |
| Action time | 4 | LSBs of the superframe number when the subordinate station transmits AAI-MMRS-REQ message. | Always present |
| } |  |  |  |
| } else { |  |  | Present when subordinate station responds to the superordinate HR-BS initiated request |
| If(received request relay mode == 0b0) { |  |  | // TTR mode |
| ST-TTG | 6 | Transmit-to-receive turnaround gap of subordinate station, i.e., HR-MS or HR-BS, in units of μs. It shall be less than 50 μs. | Shall be present if action code == 0b0 in AAI-MMRS-REQ. |
| ST-RTG | 6 | Receive-to-transmit turnaround gap of subordinate station, i.e., HR-MS or HR-BS, in units of μs. It shall be less than 50 μs. | Shall be present if action code == 0b0 in AAI-MMRS-REQ. |
| If (requested subordinate station is HR-BS) { |  |  | Shall be present if the subordinate station is HR-BS and action code == 0b0 in AAI-MMRS-REQ. |
| Ta | 11 | Confirmed value of timing advance Ta , in units of 0.1 μs |  |
| Tbs | 5 | Confirmed duration of the BS Operation mode, in units of frames |  |
| Trs | 5 | Confirmed duration of the BS Operation mode, in units of frames |  |
| } |  |  |  |
| } else if (received request relay mode == 0b1) { |  |  | // STR mode |
| Duplex mode support indication | 2 | If bit0 = 1, FDD supports If bit1 = 1, TDD supports | Always present |
| for(i=1; i<=N-frequency; i++) { |  | N-frequency is the number of available frequency to communicate[1..16] |  |
| Carrier frequency | 10 | Indicates the carrier frequency in unit of 100KHz. |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

16.2.3.xd AAI-MMRL-REQ message

HR-MS transmits AAI-MMRL-REQ message for the purpose as follows:

- to release its relay mode and to return its original role

- to response or reject the unsolicited AAI-MMRL-RSP message by the HR-BS

Table yyy—AAI-MMRL-REQ message field description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| Release_Request_Code | 2 | Used to indicate the purpose of this message 0b00: multimode releasement 0b01: response for the unsolicited AAI-MMRL-RSP message by the HR-BS 0b10: reject for the unsolicited AAI-MMRL-RSP message by the HR-BS. This code is applicable only when UL data is pending to transmit. 0b11: reserved | Always present |

16.2.3.xe AAI-MMRL-RSP message

An AAI-MMRL-RSP message is transmitted by multimode station or superordinate HR-BS in response to AAI-MMRL-REQ message.

Table yyy+1—AAI-MMRL-RSP message format

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| Action code | 2 | Used to indicate the purpose of this message 0b00: HR-MS shall immediately terminate multimode service and return its original HR-MS mode. 0b01: HR-MS shall terminate multimode service and return its original HR-MS mode at the action time expires 0b10: In response to an AAI-MMRL-REQ message to allow HR-MS to transmit MS-initiated request after action time expires. 0b11: In response to an AAI-MMRL-REQ message to reject the request of HR-MS. | Always present |
| If (action code == 0b01 or 0b10) { |  |  |  |
| Action time | 4 | LSBs of the superframe number when HR-RS start releasing the multimode or transmit AAI-MMRL-REQ message. | Always present |
| } |  |  |  |

[Remedy4: Adopt the following change in Section 17.3.1 in the 802.16n AWD.]

17.3.1 Multi-mode operation

17.3.1.1 Relay function for HR-BS

An HR-BS (affected HR-BS) may operate as a relay station to communicate with another HR-BS (serving HR-BS) that has connection to backhaul.

An HR-BS acting as RS mode operates in either TTR mode or STR mode.

17.3.1.1.1 STR mode for HR-BS acting as HR-RS

To support STR mode, the affected HR-BS maintains base station functionality.

The procedures for RS mode change consists of following activities:

- establishing a relay link with a serving HR-BS

- if necessary, informing some of its subordinate stations to perform handover

- if necessary, reconfigureing the physical frame and commence operation in relay mode

- commencing the new operation.

The affected HR-BS establishes relay link with a serving HR-BS as described in 17.3.1.1.1. The procedure applies to both STR and TTR relay modes.

When supporting STR relay mode, the affected HR-BS maintains base station functionality.

When supporting TTR relay mode, the affected HR-BS maintains connectivity with its subordinate HR-RS by performing a dual-role BS/RS operation described in 17.3.1.1.2.2.

17.3.1.1.2 TTR mode for HR-BS acting as HR-RS

To support TTR mode, the affected HR-BS can maintain connectivity with subordinate HR-RS. How to maintain is FFS.

The procedures for RS mode change consist of following activities:

a) establish a relay link with a serving HR-BS

b) if necessary, inform some subordinate stations to perform handover

c) if necessary, reconfigure the physical frame and commence operation in relay mode

17.3.1.1.1 Relay link establishment

The HR-BS having no connection to backhaul transmits MM-ADV message with action type = 0b100 described in 16.2.3.xa including expected time of backhaul link available. Based on the expected time, HR-MS handovers to neighbor infrastructure station or staying in the HR-BS until restarting service with available backhaul link.

To establish relay link with a serving HR-BS, the HR-BS having no connection to backhaul follows network entry and initialization for relay link described in 16.2.15 and 16.6.2.10. In addition, the HR-BS shall perform the relay link establishment procedure as follows:

- Scan for DL channel and establish synchronization with the HR-BS having connection to backhaul

- Obtain DL/UL parameters (from SuperFrameHeader)

- Perform ranging

- Basic capability negotiation, if needed

- Authorization, authentication, and key exchange, if needed

- Registration with the HR-BS, if needed

- Configuration operational parameters including initiating relay link using AAI-MMRS-REQ/RSP and AAI-ARS-CONFIG-CMD messages

To establish relay link with another HR-BS (serving HR-BS), HR-BS having no connection to backhaul transmits AAI-MMRS-REQ message described in 16.2.3.xb including relay mode, i.e., either TTR or STR mode. In response to AAI-MMRS-REQ, the serving HR-BS transmits AAI-MMRS-RSP message described in 16.2.3.xc to inform whether the request is accepted or rejected. Upon receiving the AAI-MMRS-RSP message, the affected HR-BS starts establishing the relay link with serving HR-BS immediately or retransmits AAI-MMRS-REQ message at the action time expires. If the serving HR-BS rejects the request, the serving HR-BS informs the HR-BS having no connection to backhaul the rejection of the request. Upon receiving the AAI-MMRS-RSP message with rejection information, the HR-BS either tries to establish relay link with another HR-BS or follows standalone network operation described in 17.3.4.

To support handover as a part of robustness against SPOF as described in 17.3.7.2, an indication of whether MAC context information of the subordinate HR-MS is being shared by infrastructure stations shall be transmitted to HR-MS.

For the case of affected HR-BS establishing a TTR relay link with a serving HR-BS, the following actions shall be carried out:

- In the AAI-MMRS-REQ message, the affected HR-BS reports its required switching gaps BSTTG and BSRTG to the serving HR-BS. Here, BSTTG is the minimum transmit-to-receive turnaround gap while BSRTG is the minimum receive-to-transmit turnaround gap required at the affected HR-BS. Based on the values of BSTTG and BSRTG, the two HR-BSs agree on the timing advance Ta of the frame boundary of the affected HR-BS, relative to that of the serving HR-BS. The affected HR-BS can propose a value for Ta in the AAI-MMRS-REQ message, and the serving HR-BS can reply with a confirmed Ta value in the AAI-MMRS-RSP message. With the values of BSTTG, BSRTG, and Ta, the two HR-BSs calculate shared values for the switching time R-TTI and R-RTI, based on the following equations:

R-TTI = 0 if RTD/2 + Ta > BSTTG and R-TTI = Ts if RTD/2 +Ta < BSTTG

and

R-RTI = 0 if Ta – RTD/2 > BSRTG and R-RTI = Ts if Ta – RTD/2 < BSRTG,

where RTD is the round trip delay between the affected HR-BS and the serving HR-BS and Ts is the OFDMA symbol duration.

- Also included in the AAI-MMRS-REQ message sent by affected HR-BS is the proposed dual-mode switching pattern (Tbs, Trs), as described in 17.3.1.1.1.2. This pattern shall be confirmed in the corresponding AAI-MMRS-RSP message sent by the serving HR-BS.

As an alternative to what described above, certain parts of the signaling between the two HR-BSs can be carried out through backhaul, i.e., prior to (and in preparation for) the backhaul failure at affected HR-BS.

17.3.1.1.2 Maintaining connectivity for subordinate HR-RS

17.3.1.1.2.1 Affected HR-BS supporting STR relay mode

When supporting STR relay mode, the affected HR-BS maintains its base station functionality and therefore continues to support its subordinate HR-RS.

17.3.1.1.2.2 Affected HR-BS supporting TTR relay mode

The affected HR-BS shall be able to switch between BS Operation and RS Operation in a frame-by-frame basis. The role switching pattern shall be periodic, with the dual-role HR-BS assuming BS Operation for Tbs consecutive frames, followed by RS Operation for Trs consecutive frames. Tbs can be set to 0. The values of Tbs , Trs shall be negotiated between the affected/dual-role HR-BS and its serving HR-BS. This negotiation can happen when the affected/dual-role HR-BS starts associating with the serving HR-BS, e.g., through AAI-MMRS-REQ/RSP and AAI-ARS-CONFIG-CMD messages. The configuration can be altered during operation, e.g., through AAI-MMRS-REQ/RSP, AAI-ARS-CONFIG-CMD. The dual-role operation of affected HR-BS is illustrated in Figure xx1.

Figure xx1: Affected/dual-role HR-BS performs BS/RS role-switching in a frame-by-frame basis.

The operation of affected HR-BS in each mode, i.e., BS Operation and RS Operation, depends on the value of switching interval R-TTI and is specified in 17.3.1.1.2.2.1 and 17.3.1.1.2.2.2.

17.3.1.1.2.2.1 When R-TTI = 0

When R-TTI = 0, the affected HR-BS shall keep its original PHY-layer configuration, including IDCell, frame configuration, and FFR pattern. In addition, the affected HR-BS shall set the AAI_Relay_zone_AMS_allocation_indicator field in AAI-SCD and AAI-ARS-CONFIG-CMD messages to 0b0. The operation of the affected/dual-role HR-BS can be described as follows.

In BS Operation Mode:

- The affected/dual-role HR-BS shall only communicate with its subordinate MS/AMS/HR-MS/HR-RS stations and shall not be available to receive from or transmit to its serving HR-BS.

- The manner in which the affected/dual-role HR-BS control and communicate with its subordinate HR-MSs/HR-RSs shall be the same as that of a normal HR-BS. The serving HR-BS is not expected to know the specific configuration of the dual-role HR-BS during BS Operation. When the affected/dual-role HR-BS transmits to or receives from its subordinate MS/AMS/HR-MS/HR-RS during BS Operation, it does so independently to the serving HR-BS.

- The affected/dual-role HR-BS transmits control messages regarding its role-switching behaviors toward its subordinate HR-RSs. Essentially, these role-switching messages tell the subordinate HR-RSs when the HR-BS will switch to RS Operation and what are the specific behaviors of the HR-BS during RS Operation.

In RS Operation Mode:

- The affected/dual-role HR-BS shall communicate with the serving HR-BS and with the subordinate MS/AMS/HR-MS. It may or may not communicate with its subordinate HR-RS during this mode of operation. The frame structure of the affected HR-BS is divided into DL Access zone, DL Relay zone, UL Access zone, and UL Relay zone. Note that as R-TTI = 0, no time gap need to be inserted into the last OFDM symbol of the last subframe in the DL Access zone.

- As the affected HR-BS still transmits the same SA-Preamble, the subordinate MS/AMS/HR-MS are oblivious to the mode change of the affected HR-BS. The affected HR-BS continue to transmit to its subordinate MS/AMS/HR-MS in the DL Access zone, and receive from its subordinate MS/AMS/HR-MS in the UL Access zone.

- The affected/dual-role HR-BS receives from and transmits to its serving HR-BS during the DL Relay zone and UL Relay zone, respectively. The PHY-layer configuration for DL/UL Relay zones shall be sent by the serving HR-BS toward the affected HR-BS in the AAI-ARS-CONFIG-CMD message.

- The affected/dual-role HR-BS can communicate with its subordinate HR-RSs in the following ways:

  - The affected/dual-role HR-BS can instruct its subordinate HR-RSs to transmit UL data during the DL Relay zone, i.e., when the affected/dual-role HR-BS also receives from the serving HR-BS. While doing so, the affected/dual-role HR-BS shall instruct the transmitting HR-RSs to use the same PHY-layer configuration as used in the DL Relay zone of the serving HR-BS.

  - The affected/dual-role HR-BS can instruct its subordinate HR-RSs to receive DL messages during the UL Relay zone, i.e., when the affected/dual-role HR-BS also transmits to the serving HR-BS. While doing so, the HR-BS shall instruct the transmitting HR-RSs to use the same PHY-layer configuration as used in the UL Relay zone of the serving HR-BS. Furthermore, if an R-RTI = Ts is inserted in the first OFDMA symbol of the first subframe of the UL Relay zone, the dual-role HR-BS shall let its subordinate HR-RSs to be aware of this insertion.

17.3.1.1.2.2.2 When R-TTI = Ts

When R-TTI = Ts, the affected HR-BS shall change its IDCell, i.e., it shall pick one of the SA-Preamble sequences (and possibly new preamble carrier index) that are allocated for TTR ARS. The operation of the affected/dual-role HR-BS can be described as follows.

In BS Operation Mode:

- The affected/dual-role HR-BS shall only communicate with its subordinate AMS/HR-MS/HR-RS stations and shall not be available to receive from or transmit to its serving HR-BS.

- The affected/dual-role HR-BS shall behave like a normal HR-RS for its subordinate AMS/HR-MS. The subordinate AMS/HR-MS detect the SA-preamble transmitted by the affected/dual-role HR-BS and classify the HR-BS as a TTR HR-RS. In response, the affected HR-BS shall only transmit DL data toward its subordinate AMS/HR-MS in the DL Access zone and receive UL data from its subordinate AMS/HR-MS in the UL Access zone. Furthermore, as R-TTI = Ts, the affected HR-BS shall not transmit on the last OFDM symbol of the last subframe in the DL Access zone. The information regarding R-TTI = Ts  shall be transmitted in the SFH SP2 toward subordinate AMS/HR-MS.

- The affected HR-BS shall behave like a normal HR-BS for its subordinate HR-RS. That means the affected HR-BS shall transmit to its subordinate HR-RS in the DL Relay zone, and receive from its subordinate HR-RS in the UL Relay zone.

In RS Operation Mode:

- The affected HR-BS shall communicate with its serving HR-BS. It may or may not communicate with its subordinate HR-RS in the mode of operation, and the specifications are as described in 17.3.1.1.2.2.

- The affected/dual-role HR-BS shall behave like a normal HR-RS for its subordinate AMS/HR-MS. The subordinate AMS/HR-MS detect the SA-preamble transmitted by the affected/dual-role HR-BS and classify the HR-BS as a TTR HR-RS. In response, the affected HR-BS shall only transmit DL data toward its subordinate AMS/HR-MS in the DL Access zone and receive UL data from its subordinate AMS/HR-MS in the UL Access zone. Furthermore, as R-TTI = Ts, the affected HR-BS shall not transmit on the last OFDM symbol of the last subframe in the DL Access zone. The information regarding R-TTI = Ts  shall be transmitted in the SFH SP2 toward subordinate AMS/HR-MS.

17.3.1.1.3 Relay link configuration

During establishing relay link, serving HR-BS transmits AAI-ARS-CONFIG-CMD message described in 16.2.3.57 to configure PHY layer parameter set including superframe number indicating the time to establish relay link.

While HR-BS is maintaining relay link, the serving HR-BS shall send AAI-ARS-ESI message described in 16.2.3.58 in the DL relay zone when the essential system information in SFH is changed. The HR-BS also shall send AAI-ARS-CONFIG-CMD message in the DL relay zone when PHY layer parameter needs to be reconfigured.

HR-BS acting as relay may transmit AAI-MM-ADV message with action type described in 16.2.3.xa to update PHY/MAC layer parameter after receiving AAI-ARS-ESI or AAI-ARS-CONFIG-CMD message.

17.3.1.1.4 Relay link release

If the HR-BS recovers from failure of backhaul, it may inform network or notify the current serving HR-BS of the HR-BS having recovered backhaul link through the backhaul network interface. The superordinate serving HR-BS may then initiate HR-MS handover back to the HR-BS in which the recovered HR-BS should be listed in the first priority. The HR-BS having recovered backhaul may store MAC context information of the serving MSs (basic capabilities, security capabilities, etc.). Such context information allows HR-MS to perform optimized network reentry when returning back to the HR-BS upon its recovery.

HR-BS transmits AAI-MM-ADV message with action type = 0b101 described in 16.2.3.xa including expected time of backhaul link up. When receiving the AAI-MM-ADV message, HR-MS performs either handover to neighbor infrastructure station and returns to the HR-BS at the expected time or waiting in the HR-BS until restarting service with available backhaul link.

[Remedy5: Adopt the following change in Section 17.3.1 in the 802.16n AWD.]

17.3.1.2 Relay function for HR-MS

An HR-MS may operate as an HR-RS to provide connectivity for multiple out-of-coverage HR-MSs. During basic capability negotiation within network entry, an HR-MS that is capable of role change to HR-RS shall report such capability to the super-ordinate HR-BS/HR-RS.

While operating as HR-RS, the station may maintain certain HR-MS functionalities. A mode switch to HR-RS shall be commanded by its superordinate HR-BS.

17.3.1.2.1 Relay link establishment

To support relay function for HR-MS, HR-MS capable of relay function shall establish relay link with HR-BS.

An HR-MS acting as HR-RS is operated in either TTR mode or STR mode and its relay mode is determined by HR-BS.

To request subordinate HR-MS to change its role as HR-RS, HR-BS transmits AAI-MMRS-REQ message described in 16.2.3.xb including relay mode (i.e., either TTR or STR mode).

In response to AAI-MMRS-REQ, the HR-MS transmits AAI-MMRS-RSP message described in 16.2.3.xc.

During establishing relay link, HR-BS transmits AAI-ARS-CONFIG-CMD message described in 16.2.3.57 to configure PHY layer parameter set including superframe number indicating the time to start acting as HR-RS.

To support handover as a part of robustness against SPOF as described in 17.3.7.2, an indication of whether MAC context information of the subordinate HR-MS is being shared by infrastructure stations shall be transmitted to HR-MS.

17.3.1.2.2 Relay link configuration

While HR-MS is acting as relay mode, the superordinate HR-BS shall send AAI-ARS-ESI message described in 16.2.3.58 in the DL relay zone when the essential system information in SFH is changed. The HR-BS also shall send AAI-ARS-CONFIG-CMD message in the DL relay zone when PHY layer parameter needs to be reconfigured.

While an HR-MS operating as HR-BS, any communication is performing with superordinate HR-BS in DL/UL relay zone to maintain HR-MS functionalities.

HR-MS acting as relay mode may transmit AAI-MM-ADV message described in 16.2.3.xa to update PHY/MAC layer parameter after receiving AAI-ARS-ESI or AAI-ARS-CONFIG-CMD message.

17.3.1.2.3 Relay link release

If the HR-MS release its role from the relay mode, HR-MS may perform handover to the any infrastructure station.

An HR-MS acting as RS may end its relay service and remove the relay link from the HR-BS. During the HR-MS’ relay mode releasement process, all subordinate HR-MSs of the HR-MS acting as RS shall be transferred to another infrastructure station prior to HR-MS’ relay mode releasement. The HR-MS acting as RS sets Cell Bar bit to 1 in order to prevent HR-MS (re)entry and transmits AAI-MM-ADV message to transfer all subordinate HR-MSs to another infrastructure station. An HR-MS acting as RS may transmit an AAI-MMRL-REQ message described in 16.2.3.xd in UL relay zone to an HR-BS so that it initiates the releasement procedure and requests handover of all its subordinate HR-MSs. Upon receiving the AAI-MMRL-REQ message, the HR-BS decides whether it allows the HR-MS’ relay mode releasement. If the request is accepted, the HR-BS may transmit the AAI-MMRL-RSP message described in 16.2.3.xe in DL relay zone to inform the acceptance and start BS-initiated handover process for the requested HR-MSs. After handover procedures between the HR-BS and HR-MS acting as RS’ subordinate HR-MSs are completed, the HR-BS informs the HR-MS acting as RS that handover is completed by transmitting an AAI-MMRL-RSP message in DL relay zone. Upon receiving the AAI-MMRL-RSP message, the HR-MS acting as RS starts relay mode releasement process immediately or at action time expires. If the HR-BS rejects the request, the HR-BS informs the HR-MS acting as RS the rejection of the request by transmitting the AAI-MMRL-RSP message in DL relay zone. Upon receiving the AAI-MMRL-RSP message with rejection information, the HR-MS acting as RS continues operating in relay mode. After action time expires, the HR-MS acting as RS retransmits an AAI-MMRL-REQ message in UL relay zone to the HR-BS.

The mode releasement process may be initiated by an HR-BS through transmitting an unsolicited AAI-MMRL-RSP message in DL relay zone.

After mode releasement process, all the relay-related connections and resource are released between the HR-BS and the HR-MS.

[-------------------------------------------------End of Text Proposal----------------------------------------------------]