| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | MAC Control Operations for Talk-around Direct Communications |  |
| Date Submitted | 2011-09-14 |  |
| Source(s) | Sungcheol Chang, Seokki Kim, Eunkyung Kim, Miyoung Yun, Won-Ik Kim, Sungkyung Kim, Hyun Lee, Chulsik Yoon, Kwangjae Lim ETRI | E-mail: scchang@etri.re.kr |
| Re: | Call for Comments on the 802.16n Amendment Working Document |  |
| Abstract | This provides AWD text proposals of MAC control operations for talk-around direct communication in 16n |  |
| Purpose | To be discussed and adopted by 802.16 TGn |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# MAC Control Operations for Talk-around Direct Communications

Sungcheol Chang, Seokki Kim, Eunkyung Kim, Miyoung Yun, Won-Ik Kim, Sungkyung Kim, Hyun Lee, Chulsik Yoon, Kwangjae Lim

ETRI

# Introductions

The following set of contributions provide ETRI’s proposals for talk-around direct communication:

- Physical channels including synchronization, dedicated, and supplementary channels [1-3,6].

- Physical control operations of synchronization, dedicated channel allocation, HARQ, etc. [4]

- MAC Control procedures including one of PTT based one-way communication, and two hop direct communication[5].

This contribution provides AWD text proposals of MAC control procedures to support one-way unicast, two-way unicast, and one-way multicast communications for talk-around direct communication. One-way unicast communication is that one HR-MS sends packets on the dedicated channels and the other HR-MS receives the packets. Two-way unicast communications is that two one-way unicast communications are used for two HR-MSs. One HR-MS sends packets on a dedicated channel and the other HR-MS sends packets on other dedicated channel. Multicast communication is that one HR-MS sends packets on a dedicated channel and the other HR-MSs receive the packets on the dedicated channel. Table 1 summarizes MAC control messages for MAC procedures of talk-around direct communication.

Table 1 MAC control messages

MAC control messages are piggybacked by RTS and CTS messages which are used for reservation of a dedicated channel. These messages include DM-LEST-REQ/RSP and DM-TLK-REQ/RSP messages.

A two-way communication consists of two one-way unicast communications. One HR-MS sends packets on a dedicated channel. The other HR-MS sends packets on a dedicated channel. MAC procedures in one-way communication follows ones of one-way unicast communication.

# Proposed Text for the 802.16n Amendment Working Document (AWD)

Note:

The text in BLACK color: the existing text in the 802.16n Amendment Draft Standard

The text in RED color: the removal of existing 802.16n Amendment Draft Standard Text

The text in BLUE color: the new text added to the 802.16n Amendment Draft Standard Text

[-------------------------------------------------Start of Text Proposal---------------------------------------------------]

## [Remedy1: Modify the following text before Section 17.3.2.6.1.3 in the 802.16n AWD]

17.3.2.6.1.3 MAC control messages

The peer-to-peer protocols of MAC layers in two HR-MSs communicate using the MAC control messages to perform the control plane function. MAC control messages shall be carried in a MAC PDU to be transported in a unicast control connection. In addition, the point-to-multipoint protocols of MAC layers in a multicast group of HR-MSs communicate using MAC control messages to perform the control plane functions. Table xxx lists the MAC control messages that shall be defined in the ASN.1 format.

Table 1216 – MAC control messages

| No. | Functional Areas | Message names | Message description | Secuirty | Connection |
| --- | --- | --- | --- | --- | --- |
| n+1 | Link Establishment | DM-LEST-REQ | Link Establishment Request |  | Unicast |
| n+2 | Link Establishment | DM-LEST-RSP | Link Establishment Response |  | Unicast |
| n+3 | Link Release | DM-LREL-REQ | Link Release Request |  | Unicast |
| n+4 | Link Release | DM-LREL-RSP | Link Release Response |  | Unicast |
| n+5 | Flow Management | DM-DSA-REQ | Dynamic Service Addition Request |  | Unicast |
| n+6 | Flow Management | DM-DSA-RSP | Dynamic Service Addition Response |  | Unicast |
| n+7 | Flow Management | DM-DSA-ACK | Dynamic Service Addition Acknowledgement |  | Unicast |
| n+8 | Flow Management | DM-DSC-REQ | Dynamic Service Change Request |  | Unicast |
| n+9 | Flow Management | DM-DSC-RSP | Dynamic Service Change Response |  | Unicast |
| n+10 | Flow Management | DM-DSC-ACK | Dynamic Service Change Acknowledgement |  | Unicast |
| n+11 | Flow Management | DM-DSD-REQ | Dynamic Service Deletion Request |  | Unicast |
| n+12 | Flow Management | DM-DSD-RSP | Dynamic Service Deletion Response |  | Unicast |
| n+13 | Flow Management | DM-DSD-ACK | Dynamic Service Deletion Acknowledgement |  | Unicast |
| n+14 | Measurement | DM-MES-REQ | Measurement Request |  | Unicast |
| n+15 | Measurement | DM-MES-RSP | Measurement Response |  | Unicast |
| n+163 | Measurement | DM-MES-REP | Measurement Report |  | Unicast |
| n+174 | Resource Management | DM-RCHG-REQ | Resource Change Request |  | Unicast |
| n+185 | Resource Management | DM-RCHG-RSP | Resource Change Response |  | Unicast |
| n+196 | Token Management | DM-TKN-REQ | Token Request |  | Unicast |
| n+207 | Token Management | DM-TKN-RSP | Token Response |  | Unicast |
| n+218 | Token Management | DM-TKN-HO | Token Handover |  | Unicast or Multicast |
| n+229 | Link Establishment | DM-LEST-CMD | Link Establishment Command |  | Multicast |
| n+2310 | Link Release | DM-LREL-CMD | Link Release Command |  | Unicast or Multicast |
| n+2411 | Flow Management | DM-DSA-CMD | Dynamic Service Addition Command |  | Unicast or Multicast |
| n+2512 | Flow Management | DM-DSC-CMD | Dynamic Service Change Command |  | Unicast or Multicast |
| n+2613 | Flow Management | DM-DSD-CMD | Dynamic Service Delete Command |  | Unicast or Multicast |
| n+2714 | Measurement | DM-MES-CMD | Measurement Command |  | Unicast or Multicast |
| n+2815 | Resource Management | DM-RCHG-CMD | Resource Change Command |  | Unicast or Multicast |
| n+2916 | Token Management | DM-TKN-ADV | Token Advertisement |  | Multicast |

17.3.2.6.1.3.1 DM-LEST-REQ

An HR-MS transmits a DM-LEST-REQ message to establish a peer-to-peer link.

Table 1217— DM-LEST-REQ message field description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| Link Change Count | 4 | The change count of this transaction assigned by the sender. If new transaction is started, Link Change Count is incremented by one (modulo 16) by the sender. | Shall always be present |
| For (i=0; i<N_Flow_Est; i++) { |  | N_Flow_Est is the number of flows on which the receiver sender of this message sends MAC PDUs. Range [0..1] |  |
| FID | 4 | Flow identifier assigned by the sink of packets on the flow |  |
| Traffic priority | 3 | 0 to 7: Higher numbers indicate higher priority Default: 0 |  |
| CS Specification Parameters | 8 | 0–15: Reserved 16: Voice Codec G.729A 17–255: Reserved |  |
| MAC Header Type | 1 | Indicates whether AGMH or SPMH is presented at the start of MAC PDUs of the service flow. 0 : AGMH (Advanced Generic MAC Header) 1 : SPMH (Short-Packet MAC header) default value is 0. |  |
| } |  |  |  |
| For (i=0; i<N_Flow_Req; i++) { |  | N_Flow is the number of flows which the sender requests the receiver to establish. Range [0..1] |  |
| Traffic priority | 3 | 0 to 7: Higher numbers indicate higher priority Default: 0 |  |
| CS Specification Parameters | 8 | 0–15: Reserved 16: Voice Codec G.729A 17–255: Reserved |  |
| } |  |  |  |
| Reserved |  |  |  |

17.3.2.6.1.3.2 DM-LEST-RSP

An HR-MS transmits a DM-LEST-RSP message in response to a received DM-LEST-REQ.

Table 1218— DM-LEST-RSP message field description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| Link Change Count | 4 | Link Change Count from corresponding the DM-LEST-REQ | Shall always be present |
| Confirmation Code | 4 | Zero indicates the request was successful. Nonzero indicates failure | Shall always be present |
| For (i=0; i<N_Flow_Est; i++) { |  | N_Flow_Est is the number of flows on which the receiver of this message sends MAC PDUs. Range [0..1] |  |
| FID | 4 | Flow identifier assigned by the sink of packets on the flow |  |
| Traffic priority | 3 | 0 to 7: Higher numbers indicate higher priority Default: 0 |  |
| CS Specification Parameters | 8 | 0–15: Reserved 16: Voice Codec G.729A 17–255: Reserved |  |
| MAC Header Type | 1 | Indicates whether AGMH or SPMH is presented at the start of MAC PDUs of the service flow. 0 : AGMH (Advanced Generic MAC Header) 1 : SPMH (Short-Packet MAC header) default value is 0. |  |
| } |  |  |  |
| Reserved |  |  |  |

17.3.2.6.1.3.3 DM-LREL-REQ

An HR-MS transmits a DM-LREL-REQ message to release either a peer-to-peer link or a point-to-multipoint link.

Table 1219— DM-LREL-REQ message field description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| Link Release Request Code | 8 | Used to indicate the purpose of this message 0x00: Link release request. 0x01–0xff: Reserved | Shall always be present |

17.3.2.6.1.3.4 DM-LREL-RSP

An HR-MS transmits a DM-LREL-RSP message in response to a received DM-LREL-REQ.

Table 1220— DM-LREL-RSP message field description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| Link Release Confirm Code | 8 | Used to indicate the purpose of this message 0x00: Link release confirm. All the established flows including traffic flows are terminated immediately. 0x01: The sender of this message rejects to release the established link. 0x02–0xff: Reserved | Shall always be present |

17.3.2.6.1.3.5 DM-DSA-REQ

An HR-MS transmits a DM-DSA-REQ message to create a new service flow.

Table 1221— DM-DSA-REQ message field description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| FID Change Count | 4 | The change count of this transaction assigned by the sender. If new transaction is started, FID Change Count is incremented by one (modulo 16) by the sender. | Shall always be present |
| For (i=0; i<N_Flow_Est; i++) { |  | N_Flow_Est is the number of flows on which the receiver of this message sends MAC PDUs. Range [0..1] |  |
| FID | 4 | Flow identifier assigned by the sink of packets on the flow |  |
| Traffic priority | 3 | 0 to 7: Higher numbers indicate higher priority Default: 0 |  |
| CS Specification Parameters | 8 | 0–15: Reserved 16: Voice Codec G.729A 17–255: Reserved |  |
| MAC Header Type | 1 | Indicates whether AGMH or SPMH is presented at the start of MAC PDUs of the service flow. 0 : AGMH (Advanced Generic MAC Header) 1 : SPMH (Short-Packet MAC header) default value is 0. |  |
| } |  |  |  |
| For (i=0; i<N_Flow_Req; i++) { |  | N_Flow is the number of flows which the sender requests the receiver to establish. Range [0..1] |  |
| Traffic priority | 3 | 0 to 7: Higher numbers indicate higher priority Default: 0 |  |
| CS Specification Parameters | 8 | 0–15: Reserved 16: Voice Codec G.729A 17–255: Reserved |  |
| MAC Header Type | 1 | Indicates whether AGMH or SPMH is presented at the start of MAC PDUs of the service flow. 0 : AGMH (Advanced Generic MAC Header) 1 : SPMH (Short-Packet MAC header) default value is 0. |  |
| } |  |  |  |
| Reserved |  |  |  |

17.3.2.6.1.3.6 DM-DSA-RSP

An HR-MS transmits a DM-DSA-RSP message in response to a received DM-DSA-REQ.

Table 1222— DM-DSA-RSP message field description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| FID Change Count | 4 | FID Change Count from corresponding the DM-DSA-REQ | Shall always be present |
| Confirmation Code | 4 | Zero indicates the request was successful. Nonzero indicates failure | Shall always be present |
| For (i=0; i<N_Flow_Est; i++) { |  | N_Flow_Est is the number of flows on which the receiver of this message sends MAC PDUs. Range [0..1] |  |
| FID | 4 | Flow identifier assigned by the sink of packets on the flow |  |
| Traffic priority | 3 | 0 to 7: Higher numbers indicate higher priority Default: 0 |  |
| CS Specification Parameters | 8 | 0–15: Reserved 16: Voice Codec G.729A 17–255: Reserved |  |
| MAC Header Type | 1 | Indicates whether AGMH or SPMH is presented at the start of MAC PDUs of the service flow. 0 : AGMH (Advanced Generic MAC Header) 1 : SPMH (Short-Packet MAC header) default value is 0. |  |
| } |  |  |  |
| Reserved |  |  |  |

17.3.2.6.1.3.7 DM-DSA-ACK

An HR-MS may transmit a DM-DSA-ACK message in response to a received DM-DSA-RSP.

Table 1223— DM-DSA-ACK message field description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| FID Change Count | 4 | FID Change Count from corresponding the DM-DSA-REQ | Shall always be present |
| Confirmation Code | 4 | Zero indicates the request was successful. Nonzero indicates failure | Shall always be present |

17.3.2.6.1.3.8 DM-DSC-REQ

An HR-MS transmits a DM-DSC-REQ message to change the parameters of an existing service flow

Table 1224— DM-DSC-REQ message field description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| FID Change Count | 4 | The change count of this transaction assigned by the sender. If new transaction is started, FID Change Count is incremented by one (modulo 16) by the sender. | Shall always be present |

17.3.2.6.1.3.9 DM-DSC-RSP

An HR-MS transmits a DM-DSC-RSP message in response to a received DM-DSC-REQ.

Table 1225— DM-DSC-RSP message field description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| FID Change Count | 4 | The change count of this transaction assigned by the sender. If new transaction is started, FID Change Count is incremented by one (modulo 16) by the sender. | Shall always be present |

17.3.2.6.1.3.10 DM-DSC-ACK

An HR-MS may transmit a DM-DSC-ACK message in response to a received DM-DSC-RSP.

Table 1226— DM-DSC-ACK message field description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| FID Change Count | 4 | FID Change Count from corresponding the DM-DSA-REQ | Shall always be present |
| Confirmation Code | 4 | Zero indicates the request was successful. Nonzero indicates failure | Shall always be present |

17.3.2.6.1.3.11 DM-DSD-REQ

An HR-MS transmits a DM-DSD-REQ message to delete an existing service flow.

Table 1227— DM-DSD-REQ message field description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| FID Change Count | 4 | The change count of this transaction assigned by the sender. If new transaction is started, FID Change Count is incremented by one (modulo 16) by the sender. | Shall always be present |
| For (i=0; i<N_Flow_Rel; i++) { |  | N_Flow_Rel is the number of flows which the sender of this message is to release. Range [0..12] |  |
| FID | 4 | Flow identifier assigned by the sink of packets on the flow |  |
| } |  |  |  |
| For (i=0; i<N_Flow_Req; i++) { |  | N_Flow is the number of flows which the sender requests the receiver to release. The sender of this message is used to send MAC PDUs on the flow. Range [0..12] |  |
| FID | 4 | Flow identifier assigned by the sink of packets on the flow |  |
| } |  |  |  |

17.3.2.6.1.3.12 DM-DSD-RSP

An HR-MS transmits a DM-DSD-RSP message in response to a received DM-DSD-REQ.

Table 1228— DM-DSD-RSP message field description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| FID Change Count | 4 | The change count of this transaction assigned by the sender. If new transaction is started, FID Change Count is incremented by one (modulo 16) by the sender. | Shall always be present |
| For (i=0; i<N_Flow_Rel; i++) { |  | N_Flow_Rel is the number of flows which the sender of this message is to release. Range [0..12] |  |
| FID | 4 | Flow identifier assigned by the sink of packets on the flow |  |
| } |  |  |  |

17.3.2.6.1.3.13 DM-DSD-ACK

An HR-MS may transmit a DM-DSD-ACK message in response to a received DM-DSD-RSP.

Table 1229— DM-DSD-ACK message field description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| FID Change Count | 4 | FID Change Count from corresponding the DM-DSD-REQ | Shall always be present |
| Confirmation Code | 4 | Zero indicates the request was successful. Nonzero indicates failure | Shall always be present |

17.3.2.6.1.3.14 DM-MES-REQ

An HR-MS transmits a DM-MES-REQ message to request a radio measurement and reporting the measurement results.

Table 1230— DM-MES-REQ message field description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |

17.3.2.6.1.3.15 DM-MES-RSP

An HR-MS transmits a DM-MES-RSP message in response to a received DM-MES-REQ.

Table 1231— DM-MES-RSP message field description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |

17.3.2.6.1.3.163 DM-MES-REP

An HR-MS transmits a DM-MES-REP message to report the measurement results if a report trigger condition is met.

Table 1232— DM-MES-REP message field description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |

17.3.2.6.1.3.174 DM-RCHG-REQ

An HR-MS transmits a DM-RCHG-REQ message to change radio resource for dedicated channel.

Table 1233— DM-RCHG-REQ message field description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| For (i=0; i<N_Resource_Change; i++) { |  | N_Resource_Change is the number of dedicated channels Range [0..12] |  |
| Old DC Frame Number | 4 | Indicates a DC frame number with the dedicated channel is on the frame. |  |
| Old Dedicated Channel Number | 4 | Indicates a number of dedicated channel with DC Frame Number. |  |
| New DC Frame Number | 4 | Indicates a DC frame number with the dedicated channel is on the frame. |  |
| New Dedicated Channel Number | 4 | Indicates a number of dedicated channel with DC Frame Number. |  |
| } |  |  |  |

17.3.2.6.1.3.185 DM-RCHG-RSP

An HR-MS transmits a DM-RCHG-RSP message in response to a received DM-RCHG-REQ.

Table 1234— DM-RCHG-REQ message field description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| Confirmation Code | 4 | Zero indicates the request was successful. Nonzero indicates failure | Shall always be present |

17.3.2.6.1.3.196 DM-TKN-REQ

An HR-MS transmits a DM-TKN-REQ message to change a token for half duplex communication.

Table 1235— DM-TKN-REQ message field description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| DC Frame Number | 4 | Indicates a DC frame number with the dedicated channel is on the frame. |  |
| Dedicated Channel Number | 4 | Indicates a number of dedicated channels with DC Frame Number. |  |

17.3.2.6.1.3.207 DM-TKN-RSP

An HR-MS transmits a DM-TKN-RSP message in response to a received DM-TKN-REQ.

Table 1236— DM-TKN-RSP message field description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| Confirmation Code | 4 | Zero indicates the request was successful. Nonzero indicates failure | Shall always be present |

17.3.2.6.1.3.218 DM-TKN-HO

An HR-MS transmits a DM-TKN-HO message to handover a token for half duplex communication

Table 1237— DM-TKN-HO message field description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| DCTID | 24 | DC Terminal Identifier. Indicate the HR-MS which takes a PTT token. | Shall always be present |
| DC Superframe Number | 4 | Indicates a DC Superframe number in which the HR-MS sends packets on the flows |  |

17.3.2.6.1.3.229 DM-LEST-CMD

An HR-MS transmits a DM-LEST-CMD message to establish a point-to-multipoint link.

Table 1238— DM-LEST-CMD message field description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| Link Change Count | 4 | The change count of this transaction assigned by the sender. If new transaction is started, Link Change Count is incremented by one (modulo 16) by the sender. | Shall always be present |
| For (i=0; i<N_Flow_Est; i++) { |  | N_Flow_Est is the number of flows on which the sender of this message sends MAC PDUs. Range [0..1] |  |
| FID | 4 | Flow identifier assigned by the source of packets on the flow |  |
| Traffic priority | 3 | 0 to 7: Higher numbers indicate higher priority Default: 0 |  |
| CS Specification Parameters | 8 | 0–15: Reserved 16: Voice Codec G.729A 17–255: Reserved |  |
| MAC Header Type | 1 | Indicates whether AGMH or SPMH is presented at the start of MAC PDUs of the service flow. 0 : AGMH (Advanced Generic MAC Header) 1 : SPMH (Short-Packet MAC header) default value is 0. |  |
| } |  |  |  |

17.3.2.6.1.3.2310 DM-LREL-CMD

An HR-MS transmits a DM-LREL-CMD message to release a point-to-multipoint link one-way point-to-point and point-to-multipoint links.

Table 1239— DM-LREL-CMD message field description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| Link Release Command Code | 8 | Used to indicate the purpose of this message 0x00: Link release command. 0x01–0xff: Reserved | Shall always be present |

17.3.2.6.1.3.2411 DM-DSA-CMD

An HR-MS transmits a DM-DSA-CMD message to create a new service flow on a point-to-multipoint link one-way point-to-point and point-to-multipoint links.

Table 1240— DM-DSA-CMD message field description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| FID Change Count | 4 | The change count of this transaction assigned by the sender. If new transaction is started, FID Change Count is incremented by one (modulo 16) by the sender. | Shall always be present |
| For (i=0; i<N_Flow_Est; i++) { |  | N_Flow_Est is the number of flows on which the sender of this message sends MAC PDUs. Range [0..1] |  |
| FID | 4 | Flow identifier assigned by the source of packets on the flow |  |
| Traffic priority | 3 | 0 to 7: Higher numbers indicate higher priority Default: 0 |  |
| CS Specification Parameters | 8 | 0–15: Reserved 16: Voice Codec G.729A 17–255: Reserved |  |
| MAC Header Type | 1 | Indicates whether AGMH or SPMH is presented at the start of MAC PDUs of the service flow. 0 : AGMH (Advanced Generic MAC Header) 1 : SPMH (Short-Packet MAC header) default value is 0. |  |
| } |  |  |  |
| Reserved |  |  |  |

17.3.2.6.1.3.2512 DM-DSC-CMD

An HR-MS transmits a DM-DSC-CMD message to change the parameters of an existing service flow on a point-to-multipoint link one-way point-to-point and point-to-multipoint links.

Table 1241— DM-DSC-CMD message field description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| FID Change Count | 4 | The change count of this transaction assigned by the sender. If new transaction is started, FID Change Count is incremented by one (modulo 16) by the sender. | Shall always be present |

17.3.2.6.1.3.2613 DM-DSD-CMD

An HR-MS transmits a DM-DSD-CMD message to delete an existing service flow on a point-to-multipoint link one-way point-to-point and point-to-multipoint links.

Table 1242— DM-DSD-CMD message field description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| FID Change Count | 4 | The change count of this transaction assigned by the sender. If new transaction is started, FID Change Count is incremented by one (modulo 16) by the sender. | Shall always be present |
| For (i=0; i<N_Flow_Rel; i++) { |  | N_Flow_Rel is the number of flows which the sender of this message is to release. Range [0..12] |  |
| FID | 4 | Flow identifier assigned by the source of packets on the flow |  |
| } |  |  |  |

17.3.2.6.1.3.2714 DM-MES-CMD

An HR-MS transmits a DM-MES-CMD message to request a radio measurement and reporting the measurement results on a point-to-multipoint link one-way point-to-point and point-to-multipoint links.

Table 1243— DM-MES-CMD message field description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |

17.3.2.6.1.3.2815 DM-RCHG-CMD

An HR-MS transmits a DM-RCHG-CMD message to change communication resource on a point-to-multipoint link one-way point-to-point and point-to-multipoint links.

Table 1244— DM-RCHG-CMD message field description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| For (i=0; i<N_Resource_Change; i++) { |  | N_Resource_Change is the number of dedicated channels Range [0..12] |  |
| Old DC Frame Number | 4 | Indicates a DC frame number with the dedicated channel is on the frame. |  |
| Old Dedicated Channel Number | 4 | Indicates a number of dedicated channel with DC Frame Number. |  |
| New DC Frame Number | 4 | Indicates a DC frame number with the dedicated channel is on the frame. |  |
| New Dedicated Channel Number | 4 | Indicates a number of dedicated channel with DC Frame Number. |  |
| } |  |  |  |

17.3.2.6.1.3.2916 DM-TKN-ADV

An HR-MS transmits a DM-TKN-ADV message to advertise status of a token for half duplex communication on a point-to-multipoint link

Table 1245— DM-TKN-REQ message field description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| DC Frame Number | 4 | Indicates a DC frame number with the dedicated channel is on the frame. |  |
| Dedicated Channel Number | 4 | Indicates a number of dedicated channel with DC Frame Number. |  |
| PTT Token Status | 4 | Zero indicates that the PTT token is available. Nonzero indicates unavailable |  |

17.3.2.6.1.4 Security

Talk-around direct communication key is managed as described in 17.3.10.1.2.

17.3.2.6.1.5 Connection management

A peer-to-peer connection is a mapping between two MAC peers of HR-MSs, which is defined as a unicast connection. The unicast connection is defined in one way and identified by an UTID DCTID and an FID.

A point-to-multipoint connection is a mapping among MAC peers of a group of HR-MSs, which is defined as a multicast connection. HR-MSs in a multicast group share the multicast connection. When an HR-MS has a right to send a packet on the multicast connection, all the other HR-MSs in the multicast group shall receive the packet from the sending HR-MS.

Two types of connections are used: control connections and transport connections. Control connections are used to carry MAC control messages. Transport connections are used to carry user data packet. These two types of connections are applicable to both unicast and multicast connections.

17.3.2.6.1.5.1 Control connections

One pair of bi-directional unicast control connections are automatically established when two HR-MSs perform two-way unicast link establishment with two-way handshake of control messages.

One multicast control connection is established when HR-MSs perform multicast link establishment multicasting one-way control message.

17.3.2.6.1.5.2 Transport connections

A unicast transport connection is unidirectional and identified by an FID between two HR-MSs. The unicast transport connection is established during the service flow creation procedure. A 4-bit FID along with sender DCTID is unique within a network.

One multicast transport connection is established and identified by an FID among a group of HR-MSs. The FID is assigned during the service flow creation procedure. A 4-bit FID along with sender DCGID is unique within a network.

17.3.2.6.1.6 Link management

HR-MS establishes a one-way unicast link of direct communication between two peer-to-peer HR-MSs.

HR-MS establishes a two-way unicast link of direct communication between two peer-to-peer HR-MSs, in which two one-way unicast links are made and work independently.

HR-MS establishes a multicast link of direct communication among a multicast group of HR-MSs.

17.3.2.6.1.6.1 Synchronization

Before link establishment, all the HR-MSs involved in direct communication shall be synchronized. The HR-MS shall acquire the PHY synchronization of direct communication on Synchronization channel. The detail synchronization procedure is described in section 17.3.2.6.2.x5.

17.3.2.6.1.6.2 Link establishment

When HR-MSs need to communicate directly, the HR-MSs shall establish a one of one-way unicast or two-way unicast or multicast links of direct communication. During link establishment, a transport connection shall be established so that HR-MSs communicate directly. During link establishment, multiple transport connections may be established.

Two HR-MSs establish an one-way unicast link with two-way handshake of DM-LEST-REQ/RSP messages. The DM-LEST-REQ message is piggybacked by a RTS message, which is an initial message for dedicated channel reservation. The DM-LEST-RSP message is piggybacked by a CTS message, which is in response to the RTS message.

Two HR-MSs establish a two-way unicast link which consists of two one-way unicast links. One communication of a direction uses one-way unicast signaling procedure independent to the one-way unicast signaling procedure of the opposite direction.

HR-MSs establish a multicast link with one-way DM-LEST-CMD message.

The radio resource for a dedicated channel is allocated during link establishment. A sending HR-MS shall send QoS parameters of traffic connection and the receiving HR-MS selects radio resources of a dedicated channel.

Figure 904- Message procedures of link establishment for unicast(left) and multicast(right) one-way links

17.3.2.6.1.6.2 Link release

The unicast or multicast link is terminated with link release. On link release, all the connections built on a direct communication link are terminated automatically.

Two HR-MSs release an one-way unicast link using DM-LREL-REQ/RSP DM-LREL-CMD messages.

Two HR-MSs release a two-way unicast link by releasing two one-way unicast links independently.

HR-MSs release a multicast link using DM-LREL-CMD message.

Figure 905- Message procedures of link release for unicast(left) and multicast(right) one-way links

17.3.2.6.1.7 QoS management

QoS concept of direct communication shall be the same as described as in section 16.2.12 with the exception of QoS described in this section.

17.3.2.6.1.7.1 Service Flow Management

Service flows may be created, changed, or deleted. This is accomplished through a series of MAC management messages referred to as DM-DSA-CMD, DM-DSC-CMD, and DM-DSD-CMD messages. The DM-DSA-CMD messages creates a new service flow. The DM-DSC-CMD messages changes an existing service flow. The DM-DSD-CMD messages deletes an existing service flow.

Figure 906- Message procedures of dynamic service flow addition for unicast(left) and multicast(right) one-way links

Figure 907- Message procedures of dynamic service flow deletion for unicast(left) and multicast(right) one-way links

17.3.2.6.1.8 Token management

When an HR-MS has a token, the HR-MS sends packets on a shared link for half-duplex transmission. The DM-TKN-HO message passes the token over to other HR-MS. The DM-TKN-REQ message makes a request of token handover. The DM-TKN-RSP message makes a response to the DM-TKN-REQ message.

Figure 908- Message procedures of token management for unicast and multicast one-way links

If the receiving HR-MS requests the token, TKN-REQIND code on supplementary channel is sent to the sending HR-MS. If accepting it, the sending HR-MS send DM-TKN-HO message to pass the token over to the receiver HR-MS in response to TNK-REQIND code. When the receiving HR-MS gets the token, it becomes the sending HR-MS and sends packets on the dedicated channel.

For multicast one-way link, an HR-MS requests a token by sending DM-TKN-REQ message. If the sending HR-MS accepts the token request, it sends DM-TKN-RSP message in response to the DM-TKN-REQ message. DM-TKN-REQ/RSP messages are sent in a frame that the sending HR-MS does not send data packets.

Figure 909- Message procedures of token management with token request for unicast(left) and multicast(right) one-way links

17.3.2.6.1.9 Resource management

When a receiving HR-MS needs to change resource, the HR-MS requests it to the sending HR-MS. The receiving HR-MS recommends a candidate resource at least and the sending HR-MS may use new resource to send packets toward the receiving HR-MS.

When a receiving HR-MS needs to change transmission modulation, the HR-MS requests it to the sending HR-MS.

Figure 910- Message procedures of resource management for unicast and multicast links

17.3.2.6.1.10 Measurement

An HR-MS requests that the receiving HR-MS measures signals and interferences on a dedicated channel and the measured values are reported periodically or at a trigger event. The sending HR-MS sends the DM-MES-REQCMD messages to the receiving HR-MS. The receiving HR-MS sends the DM-MES-RSPREP message or CQI report on supplementary channel in response and the DM-MES-REP message to DM-MES-CMD message.

Figure 911- Message procedures of measurement for unicast(left) and multicast(right) links

[-------------------------------------------------End of Text Proposal----------------------------------------------------]

# References

[1] IEEE C802.16n-11/0152, “ Frame Structure for Talk-around Direct Communications.”

[2] IEEE C802.16n-11/0153, “ Changes on Synchronization Channel for Talk-around Direct Communications.”

[3] IEEE C802.16n-11/0154, “ Supplementary Channel for Talk-around Direct Communications.”

[4] IEEE C802.16n-11/0155, “ PHY Control Operations for Talk-around Direct Communications.”

[5] IEEE C802.16n- 11/0156, “ MAC Control Operations for Talk-around Direct Communications.”

[6] IEEE C802.16n- 11/0160, “ Dedicated Channel for Talk-around Direct Communications.”