| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text and Control Messages for MAC-layer Support for BS-controlled HR-MS Forwarding to Network (Rev3-based) |  |
| Date Submitted | 2011-11-05 |  |
| Source(s) | Anh Tuan Hoang, Haiguang Wang, Jaya Shankar, Wai Leong Yeow, Shoukang Zheng, Joseph Teo Chee Ming Institute for Infocomm Research 1 Fusionopolis Way, #21-01, Connexis (South Tower) Singapore 138632 Eldad Zeira InterDigital | Voice: +65 6408-2271 E-mail: athoang@i2r.a-star.edu.sg |
| Re: | In response to the 802.16n (GRIDMAN) Call for Comments to AWD in 802.16n-11/0024 |  |
| Abstract | This contribution proposed modifications for text and control messages for MAC-layer support for BS-controlled HR-MS forwarding to network. |  |
| Purpose | To discuss and adopt the proposed text in the 802.16n draft Text |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Copyright Policy | The contributor is familiar with the IEEE-SA Copyright Policy <http://standards.ieee.org/IPR/copyrightpolicy.html>. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text and Control Messages for MAC-layer Support for BS-controlled HR-MS Forwarding to Network (Rev3-based)

Anh Tuan Hoang, Haiguang Wang, Jaya Shankar, Wai Leong Yeow,

Shoukang Zheng, Joseph Teo Chee Ming

Institute for Infocomm Research (I2R)

Eldad Zeira (InterDigital)

Introduction

This contribution proposed modifications for text and control messages for MAC-layer support for BS-controlled HR-MS forwarding to network.

# References

- IEEE 802.16n Amendment Working Draft, IEEE 802.16n-11/0024.

Text Proposal for the 802.16n Amendment Working Document (AWD)

Note:

The text in BLACK or BLUE colors: the existing text in the 802.16n Amendment Draft Standard

The text in RED color: the removal of existing 802.16n Amendment Draft Standard Text

The text in GREEN color: the new text added to the 802.16n Amendment Draft Standard Text

[-----------------------------------------------Start of Text Proposal---------------------------------------------------]

{Remedy 1: Insert the following new rows into Table 54}

| Type | Message names | Message description | Connection |
| --- | --- | --- | --- |
| … | … | … | … |
| TBD | FN-CONFIG-CMD | BS-controlled FTN configuration | Primary management |
| TBD | NCI | Network Configuration Information (for BS-controlled FTN) | Broadcast by forwarding HR-MS |
| TBD | F-MAP | Forwarding MAP (for BS-controlled FTN) | Broadcast by forwarding HR-MS |
| … | … | … | … |

{Remedy 2: Insert the following new section}

6.3.2.3.98.x FN-CONFIG-CMD Message

An HR-BS/RS transmits FN-CONFIG-CMD message to instruct an HR-MS or group of HR-MS to start the coverage extension process. This involves the scheduled HR-MS to transmit frame preamble, M-FCH, and network configuration information so that an HR-MS outside of HR-BS/RS coverage may start network entry (through the forwarding HR-MS).

Table xx1 – FN-CONFIG-CMD message format

| Syntax | Size (bit) |  | Notes |  |
| --- | --- | --- | --- | --- |
| FN-CONFIG-CMD_Message_Format{ |  |  |  |  |
| Management Message Type = [TBD] | 8 |  | — |  |
| Frame number action (nstart) | 4 |  | LSBs of frame number in which the coverage-extending cycle should be started |  |
| Preamble index | 7 |  | Identifies IDCell, together with the segment |  |
| Number of ranging opportunities (mrng) | 4 |  | Ranging opportunities are allocated in every frame that immediately follows a frame in which the forwarding HR-MS transmits preamble, M-FCH, and NCI |  |
| Used subchannel bitmap |  | 6 |  | Bit 0: Subchannel group 0 Bit 1: Subchannel group 1 Bit 2: Subchannel group 2 Bit 3: Subchannel group 3 Bit 4: Subchannel group 4 Bit 5: Subchannel group 5 |
| Repetition_Coding_Indicator |  | 2 |  | 0b00: No repetition coding on NCI 0b01: Repetition coding of 2 used on NCI 0b10: Repetition coding of 4 used on NCI 0b11: Repetition coding of 6 used on NCI |
| Coding_Indicator |  | 2 |  | 0b00: CC encoding used on NCI 0b01: BTC encoding used on NCI 0b10: CTC encoding used on NCI 0b11: ZT CC encoding used on NCI |
| NCI_Length |  | 6 |  |  |
| NCI_Slot_Offset |  | 3 |  |  |
| Subchannel offset for ranging feedback |  | 7 |  | This value indicates start subchannel offset. Number of OFDMA slots allocated for ranging feedback in this case is fixed at 2. |
| Subchannel offset for ranging |  | 5 |  |  |
| Ranging Method |  | 1 |  | 0b0: Initial ranging over two symbols 0b1: BR/periodic ranging over one symbol |
| Initial ranging codes |  | 5 |  | Indicates the CDMA codes used for initial ranging toward the forwarding HR-MS. 2 MSBs indicate index of starting code: 0b00: starting code index = 0 0b01: starting code index = 4 0b10: starting code index = 8 0b11: starting code index = 16 3 LSBs indicate number of codes used. The codes are contiguous from the starting code. |
| Forwarding HR-MS EIRP |  | 7 |  | Signed in units of 1dBm. |
| EIRxPIR,max |  | 16 |  | Initial ranging maximum equivalent isotropic received power at forwarding HR-MS. Signed in units of 1 dBm. |
| F-MAP Length |  | 4 |  | In OFDMA slots |
| F-MAP Offset |  | 5 |  | In OFDMA slots from the end of the NCI |
| } |  |  |  |  |

{Remedy 3: Modify Section 6.3.2.3.98.13 as indicated}

6.3.2.3.98.13 NCI Message

The NCI message is transmitted by the forwarding HR-MS so that an HR-MS outside the coverage of serving HR-BS can access the network through HR-MS with the FTN operationscheme provided in this section.

Table 1 ---- NCI message field and description format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| NCI_Message_Format() { | — | — |
| Management Message Type = [TBD] | 8 | — |
| PHY Synchronization Field | 32 8 | Specified in Table 319, 8 bits for frame duration and 24 bits for frame number. |
| Base StationOperator ID | 48 8 | 8 LSBs of the Operator ID |
| Allocation Start Time | 8 | In number of OFDMA symbols, should include the RTG/TTG gap in the calculation. |
| Reduced_NCI | 1 |  |
| If(Reduced_NCI == 0b0){ |  |  |
| No. OFDMA symbols | 8 | For TDD, the number of OFDMA symbols in the UL subframe |
| UL_PermBase | 7 | UL_PermBase parameter for the subcarrier permutation to be used on the UL channel. |
| UL allocated subchannels bitmap | 70/ 22 | This is a bitmap describing the physical subchannels allocated to the segment in the UL, when using the UL PUSC permutation. The LSB shall correspond to subchannel 0. For any bit that is not set, the corresponding subchannel shall not be used by the SS on that segment. Equal 70 for 2048, 1024 FFT Equal 22 for 512, 256 FFT |
| Subchannel offset for ranging feedback | 7 | This value indicates start subchannel offset. Number of OFDMA slots allocated for ranging feedback in this case is fixed at 2. |
| }else{ |  |  |
| Subchannel offset for ranging | 75 |  |
| No. OFDMA Symbols for ranging | 7 |  |
| No. Subchannels for ranging | 7 |  |
| Ranging Method | 21 | 0b000b0: Initial ranging over two symbols 0b01: Initial ranging over four symbols 0b100b1: BR/periodic ranging over one symbol 0b11: BR/periodic ranging over three symbols |
| Initial ranging codes | 5 | Indicates the CDMA codes used for initial ranging toward the forwarding HR-MS. 2 MSBs indicate index of starting code: 0b00: starting code index = 0 0b01: starting code index = 4 0b10: starting code index = 8 0b11: starting code index = 16 3 LSBs indicate number of codes used. The codes are contiguous from the starting code. |
| Forwarding HR-MS EIRP | 7 | Signed in units of 1dBm. |
| EIRxPIR,max | 16 | Initial ranging maximum equivalent isotropic received power at forwarding HR-MS. Signed in units of 1 dBm. |
| Subchannel offset for ranging feedback | 7 | This value indicates start subchannel offset. |
| Duration for ranging feedback | 2 | In OFDMA slots |
| Reserved | 1 |  |
| } |  |  |
| F-MAP Length | 4 | In OFDMA slots |
| F-MAP Offset | 5 | In OFDMA slots from the end of the NCI |
| F-MAP repetition coding indicator | 2 |  |
| } |  |  |

{Remedy 4: Insert the following new sections}

6.3.2.3.98.14 F-MAP Message

Forwarding HR-MS transmits F-MAP in the FCZ zone to let its attached forwarded HR-MS know of the OFDMA resource allocations in the uplink subframe. The format of F-MAP is described in Table xx1.

Table xx1 – F-MAP message format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| F-MAP_Message_Format() { | — | — |
| Management Message Type = [TBD] | 8 | — |
| No. of OFDMA symbols | 8 | For TDD, the number of OFDMA symbols in the UL subframe |
| Allocation Start Time | 30 |  |
| Frame_Usage_Code | 2 | 0b00: Both next frame and the one following it are used for forwarding direction 0b01: Next frame is used for forwarding direction while the one following it is used for reversed direction 0b10: Next frame is used for reversed direction while the one following it is used for forwarding direction 0b11: Both next frame and the one following it are used for reversed direction |
| for (i=1; i <= n; i++){ |  |  |
| F-MAP_IE() |  |  |
| } |  |  |
| if !(byte boundary){ |  |  |
| Padding Nibble | 4 | Padding to reach byte boundary |
| } |  |  |
| } |  |  |

Each F-MAP IE has the format as described in Table xx2.

Table xx2 –F-MAP IE format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| F-MAP_IE() { | — | — |
| Frame Indicator | 1 | 0b0: The burst is scheduled in the next frame 0b1: The burst is scheduled in the frame that follows the next frame. |
| CID | 16 | — |
| Extended_F-MAP_IE | 1 |  |
| if(Extended_F-MAP_IE == 0b1){ |  |  |
| Extended F-MAP IE |  |  |
| }else{ |  |  |
| FEC Code type and Modulation type | 5 | Values from 0 to 31 are interpreted as defined in Table 574. |
| if ((Frame_Indicator == 0b0 && Frame_Usage_Code == 0b10 ¦¦ 0b11) ¦¦ (Frame_Indicator == 0b1 && Frame_Usage_Code == 0b00 ¦¦ 0b01)) { |  | This allocation is for forwarded HR-MS |
| Ranging data ratio | 5 | Reducing factor in units of 1 dB, between the power used for this burst and power should be used for CDMA ranging. It shall be encoded as signed integer. |
| OFDMA symbol offset indicator | 6 | Represent number k which is used to calculate OFDMA symbol offset using the formula: Symbol_offset = 3*Round(k*4/3), where Round(x) returns the integer nearest to x. |
| Subchannel offset | 7 | This value indicates start subchannel offset of this F-MAP allocation. |
| Duration | 7 | In OFDMA slots |
| }else{ |  |  |
| OFDMA symbol offset | 8 | This value indicates start symbol offset of this F-MAP allocation. The reference point of this offset is the start of UL subframe. |
| Subchannel offset | 7 | This value indicates start subchannel offset of this F-MAP allocation. |
| Duration | 10 | In OFDMA slots |
| } |  |  |
| } |  |  |
| } |  |  |

Each Extended F-MAP IE has the format as described in Table xx3.

Table xx3 – Extended F-MAP IE format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| Extended-F-MAP_IE() { | — | — |
| EFUC | 4 |  |
| Length | 4 | — |
| Unspecified data | variable |  |
| } |  |  |

Table xx4 – Extended code assignment

| Extended code (hexadecimal) | Usage |
| --- | --- |
| 0x0 | Fast-feedback for forwarded HR-MS |
| 0x1 … 0xF | Reserved |

Table xx5 – Fast Feedback IE

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| Fast-Feedback_IE() { | — | — |
| EFUC | 4 | 0x0 |
| Length | 4 | 0x2 |
| Subchannel offset | 7 | This value indicates start subchannel offset of this F-MAP allocation. |
| Reserved | 1 |  |
| } |  |  |

{Remedy 5: Modify Section 16.3.2 as indicated}

16.3.2 BS-Controlled HR-MS Forwarding to Network

The general operation of BS-controlled HR-MS forwarding to network (FTN) can be described as follows:

- At the beginning of the process, the serving HR-BS/RS instructs one or a group of its associated HR-MS to broadcast discoverable signals so that an out-of-coverage HR-MS can start network entry and get associated to the HR-BS/RS.

- After the out-of-coverage HR-MS has completed network entry and become an attached forwarded HR-MS, the forwarding HR-MS helps forward control and data messages between the forwarded HR-MS and the serving HR-BS/RS.

16.3.2.1 Medium access control

16.3.2.1.1 HR-MS Discover Network Infrastructure

For use case 2, The HR-BS may instruct HR-MS that are associated with it to transmit access information at pre-defined resources relative to the preambles transmitted by the HR-MS. The access information defines resources for access by the HR-MS that is not under HR-BS coverage. Access information may be omitted. If access information is omitted then access resources are defined by the index and the sub-carrier set index of the SA-Preamble. All or a group of the directly associated HR-MS may or may not transmit the same access information on the same or different resources.

An unassociated HR-MS that detects the associated HR-MS preamble(s) shall subsequently receive access information to determine the access resource. If access information is omitted then access resources are determined from the SA-Preamble. The unassociated HR-MS transmits a CDMA preamble.

The associated HR-MS that received the CDMA preamble responds with sufficient access information to complete the association procedure.

16.3.2.1.1 Network entry involving forwarding HR-MS

For an HR-MS that is inside-of-coverage of an HR-BS/RS, the network entry process is as described in 6.3.9.

For an HR-MS that is outside-of-coverage of any HR-BS/RS but within reach of a forwarding HR-MS, the process of network entry can be described as follows.

16.3.2.1.2 Discovery for HR-MS Forwarding to Network

To enable coverage extension, a serving HR-BS/HR-RS shall schedule some of its directly-associated HR-MSs to transmit preambles so that an outside-of-coverage HR-MSs who are out of coverage of HR-BS/HR-RS can detect and start network entry. The directly-associated HR-MS shall transmit frame preamble at the first OFDM symbol and followed by the network configuration information (NCI).

The HR-BS may instruct HR-MS that are associated with it to transmit MS-Frame Control Header (M-FCH) and network configuration information (NCI) at pre-defined resources relative to the preamble transmitted by the HR-MS. The M-FCH and NCI, when transmitted, define resources for access by the HR-MS that is not under HR-BS coverage. This corresponds to the coverage extension procedure defined in 16.3.2.1.1.1. If M-FCH and NCI are omitted then access resources are defined by the index and the sub-carrier set index of the transmitted preamble. This corresponds to the coverage extension procedure defined in 16.3.2.1.1.2.

16.3.2.1.1.1 Coverage Extension with M-FCH and NCI Preceding Initial Ranging

The procedure starts when serving HR-BS/RS transmits an FN-CONFIG-CMD message to instruct one or a group of its directly-associated HR-MS to carry out a coverage-extending process. The transmitted FN-CONFIG-CMD message includes the following information:

- Preamble Index: which identify the preamble to be transmitted by the forwarding HR-MS, together with the frequency segment.

- nstart : the frame number in which the new coverage-extending process should be started.

- mrng: the number of ranging opportunities, together with their particular configurations.

- The parameters determining the transmissions of NCI.

- The parameters determining the transmissions of F-MAP.

- The parameters determining the allocation of resources for ranging response

- The parameters determining the allocation of resources for fast-feedback by the forwarded HR-MS.

16.3.2.1.2.2 Scheduling of Preamble and NCI

Prior to sending out the FN-CONFIG-CMD message, HR-BS/RS should When HR-BS/HR-RS schedules some of HR-MSs to transmit the FTN discovery message, it shall reserve a Forwarding Control Zone (FCZ) of three OFDMA symbols at the end of uplink sub-frame for the transmission of preamble and NCI. This zone reservation is achieved by defining a with UL_Zone_IE() as specified in Table 471. The zone shall have a minimum length of three symbols.

Upon receiving the FN-CONFIG-CMD message, the scheduled HR-MS (or group of HR-MS), from now on simply referred to as forwarding HR-MS, shall start transmitting the specified preamble in the first OFDMA symbol of the FCZ, followed by M-FCH and NCI in the remaining two OFDMA symbols. The formats of M-FCH and NCI are described in 16.3.2.2.1.1 and 6.3.2.3.98.13, respectively. The M-FCH carries similar information to the FCH transmitted by HR-BS, and allows the new HR-MS to receive the NCI, which follows M-FCH.

When a new HR-MS detects preamble, M-FCH, and NCI transmitted by the forwarding HR-MS and decides to start initial ranging through the forwarding HR-MS, the following process shall be carried out:

- The new HR-MS, after acquiring synchronization and uplink transmission parameters shall choose one ranging CDMA code from those specified in NCI using a uniform random process. The HR-MS shall send the selected ranging CDMA code on the ranging slot specified in NCI.

- Upon successfully receiving the ranging code from the new HR-MS, the forwarding HR-MS is supposed to respond with an FN-RNG-ACK message. As specified in NCI, mrng ranging opportunities shall be allocated. If the new HR-MS has previously transmitted a ranging CDMA code in a ranging opportunity which is not the last opportunity, the HR-MS shall expect to receive an FN-RNG-ACK message from the forwarding HR-MS before the next ranging opportunity. If the new HR-MS does not receive such an FN-RNG-ACK message, it shall pick one ranging CDMA code from those specified in the NCI with equal probability and transmit on the coming ranging opportunity. The new HR-MS may ramp-up its transmit power as specified in NCI. If the new HR-MS transmits a ranging code in the last ranging opportunity specified in NCI, it shall expect to receive an FN-RNG-ACK message within Tx1 Timer.

- The FN-RNG-ACK message provides responses to all the successfully received CDMA ranging codes in the ranging opportunity preceding its transmission. There are four possible ranging status responses from the forwarding HR-MS to the new HR-MS in the FN-RNG-ACK message, i.e.,

  - Continue: The forwarding HR-MS informs the new HR-MS of needed adjustments (e.g., time, power, and possibly frequency corrections) and a status notification of “continue”.

  - Success: The forwarding HR-MS informs the new HR-MS of status “success”, but may have adjustment suggestions to the new HR-MS if necessary. With status success, the forwarding HR-MS shall inform the new HR-MS of a BW allocation in the uplink for the new HR-MS to send RNG-REQ message.

  - Abort: The forwarding HR-MS informs the new HR-MS to abort the current initial ranging process.

  - Continue with 2nd Phase: This status notification is only applicable if the current coverage extension process follows the Two-Phase approach, as specified in FN-CONFIG-CMD message. The forwarding HR-MS informs the new HR-MS to prepare to receive post-access preambles and post-access NCI. This status shall only be sent once, i.e., it shall not be given when the new HR-MS has already been carrying out ranging using post-access preambles and post-access NCI from forwarding HR-MS.

- Upon receiving a Continue status notification and parameter adjustments in FN-RNG-ACK message, the new HR-MS shall adjust its parameters accordingly and continue the ranging process by selecting one ranging CDMA code from those specified in NCI with equal probability and transmitting in the next available ranging opportunity, or as instructed in the FN-RNG-ACK message.

- Upon receiving a Success status notification, the new HR-MS shall wait for the forwarding HR-MS to inform it of a BW allocation in the UL to send RNG-REQ message. The forwarding HR-MS forwards the corresponding request to the serving HR-BS, and subsequently acts on behalf of the HR-BS in sending an RNG-RSP message to the new HR-MS. Initial ranging process for the new HR-MS is over after receiving RNG-RSP message, which includes a valid Basic CID. If the new HR-MS does not receive bandwidth allocation for RNG-REQ within Tx or the RNG-REQ/RSP is not completed in [TBD] frames, it stops the process and try to associate with another HR-MS or a HR-BS/RS.

- Upon receiving an Abort status notification, the new HR-MS shall stop the ranging process. It shall not attempt to restart initial ranging with a forwarding HR-MS transmitting the same preamble in the next nclosed coverage extending process.

- Upon receiving Continue with 2nd Phase status notification, the new HR-MS waits for post-access preamble and post-access NCI to be transmitted by forwarding HR-MS. Upon receiving post-access preamble and NCI, the new HR-MS starts a new round of initial ranging as follows steps as described above. The initial transmission power for the ranging is determined based on the last ranging prior to detection of post-access preamble and NCI.

The behavior of the forwarding HR-MS and serving HR-BS/RS during initial ranging of the new HR-MS shall be as follows.

- After transmitting NCI, the forwarding HR-MS shall monitor the specified ranging opportunities for any initial-ranging attempt by new HR-MS. If no such ranging attempt is detected, the coverage extension process terminates.

- When the forwarding HR-MS detects one or more CDMA ranging codes transmitted on the allocated ranging slots, it shall calculate signal strength and necessary adjustments (time, power, frequency corrections). An FN-RNG-ACK message shall be transmitted to the new HR-MS.

- FN-RNG-ACK message shall be transmitted in the FCZ, with the location specified inside the NCI. The HR-BS shall also monitor the pre-allocated resource for the FN-RNG-ACK in order to carry out any further proactive resource allocation for the coverage extending process.

- The forwarding HR-MS can also transmit an FN-RNG-REP report to the HR-BS. The FN-RNG-REP message contains signal strength and possibly necessary adjustments for the new HR-MS. The HR-BS may receive FN-RNG-REP messages from multiple scheduled forwarding HR-MS. The HR-BS is supposed to follow up with an FN-RNG-FLU message before the next allocated ranging opportunity of the coverage extending process if there is any. Otherwise, if this is the last ranging opportunity as specified in NCI, the FN-RNG-FLU message is supposed to be transmitted within Tx2 Timer, where Tx2 < Tx1. The FN-RNG-FLU message specifies the forwarding HR-MS that shall transmit an FN-RNG-ACK message to the new HR-MS (if such a message need to be transmitted).

- After the ranging status has become Success, HR-BS shall allocate bandwidth in the uplink to allow the new HR-MS to transmit RNG-REQ message. The forwarding HR-MS shall recognize this allocation IE in the UL-MAP and inform the new HR-MS accordingly (using a CDMA Allocation IE). The resource allocation for RNG-REQ message shall be preceded by RNG-RCV-IE which allocates uplink resource for the forwarding HR-MS to relay ranging, registration, capability negotiation, security exchanges from the new HR-MS to serving HR-BS.

- The HR-BS may request the forwarding HR-MS to report any ranging attempt, whether they have crossed the response threshold or not, for management purposes.

16.3.2.1.1.1 Coverage Extension with Initial Ranging Preceding NCI

The procedure starts when serving HR-BS (or serving HR-RS) transmits an FN-CONFIG-CMD message to instruct one or a group of its directly-associated HR-MS to carry out a coverage-extending process.

Upon receiving the FN-CONFIG-CMD message, the scheduled HR-MS (or group of HR-MS), from now on simply referred to as forwarding HR-MS, shall start transmitting pre-access frame preambles.

Both pre-access and post-access preambles are assigned by the serving HR-BS. The HR-BS may assign the same preamble to multiple forwarding HR-MS. The HR-BS should assign pre-access preamble to groups of HR-MS based on their service characteristics. The HR-BS should assign post-access preamble such that they will be unique within physical proximity.

When a new HR-MS detects preamble sequences transmitted by the forwarding HR-MS and decides to start initial ranging through the forwarding HR-MS, the process can be described as follows:

- The new HR-MS, after acquiring downlink synchronization and uplink transmission parameters shall pick one of the ranging codes derived from the preamble ID using a uniform random process. The HR-MS shall send the selected ranging code on the first allocated ranging channel.

- Upon receiving the ranging code from the new HR-MS, the forwarding HR-MS is supposed to respond with  post-access preamble and NCI. If the new HR-MS does not receive a post access preamble and NCI it may ramp-up the transmit power for the next ranging attempt, if necessary.

- The forwarding HR-MS responds to ranging that has exceeded a threshold determined by the HR-BS.

If the new HR-MS detects a post-access preamble and NCI it shall pick one of the ranging codes (specified in NCI) using a uniform random process. The HR-MS shall send the selected ranging code on the first allocated ranging channel. The initial transmission power for the ranging is determined based on the last ranging prior to detection of post-access preamble and NCI.

- There are three possible ranging status responses from the forwarding HR-MS to the new HR-MS in the FN-RNG-ACK message, i.e.,

  - Continue: The forwarding HR-MS informs the new HR-MS of needed adjustments (e.g., time, power, and possibly frequency corrections) and a status notification of “continue”.

  - Success: The forwarding HR-MS informs the new HR-MS of status “success”, but may have adjustment suggestions to the new HR-MS if necessary. With status success, the forwarding HR-MS shall inform the new HR-MS of a bandwidth allocation in the uplink for the new HR-MS to send RNG-REQ message.

  - Abort: The forwarding HR-MS informs the new HR-MS to abort the current initial ranging process.

- Based on the received response of ranging status, the new HR-MS perform the following:

  - Upon receiving a Continue status notification and parameter adjustments in FN-RNG-ACK message, the new HR-MS shall adjust its parameters accordingly and continue the ranging process by randomly select one ranging code and transmit in the next available ranging opportunity, or as instructed in the FN-RNG-ACK message.

  - Upon receiving a Success status notification, the new HR-MS shall wait for the forwarding HR-MS to inform it of a BW allocation in the UL to send RNG-REQ message.

  - Upon receiving an Abort status notification, the new HR-MS shall stop the ranging process.

After a successful initial ranging, the forwarding HR-MS shall assist HR-BS and the new HR-MS to exchange control messages to complete basic capability negotiation, HR-MS authorization and key exchange, and registration.

The behavior of the forwarding HR-MS and serving HR-BS/RS during initial ranging of the new HR-MS shall be as follows.

- After transmitting NCI, the forwarding HR-MS shall monitor the specified ranging opportunities for any initial-ranging attempt by new HR-MS until told to stop by the HR-BS.

- When the forwarding HR-MS detects one or more ranging codes transmitted on the allocated ranging opportunities, it shall calculate signal strength and necessary adjustments (time, power, frequency corrections). An FN-RNG-ACK message shall be transmitted to the new HR-MS.

- After the ranging status has become Success, HR-BS shall allocate bandwidth in the uplink to allow the new HR-MS to transmit RNG-REQ message. The forwarding HR-MS shall recognize this allocation IE in the UL-MAP and inform the new HR-MS accordingly.

- The HR-BS may request the forwarding HR-MS to report any ranging attempt (whether they have crossed the response threshold or not).

16.3.2.1.2  Synchronization

See  16.2.2.1.3.

16.3.2.1.3  Bandwidth Requests sent from Forwarded HR-MS

For use case 2, an out-of-coverage forwarded HR-MS can request bandwidth by transmitting some known sequences (Bandwidth Request (BR) preambles) toward the forwarding HR-MS.

The process can be described as follows.

-	Serving HR-BS/RS schedules resources in an uplink subframe for forwarded HR-MSs to transmit BR messages to their corresponding forwarding HR-MS.

- The resource allocation information is conveyed to the forwarded HR-MS.

-	The forwarding HR-MS listens to bandwidth requests at times and resources indicated by the HR-BS. The forwarded HR-MS may transmit bandwidth requests using these resources.

-	The forwarding HR-MS, upon receiving BR messages from one of its forwarded HR-MS, forwards the requests to serving HR-BS/RS.

-	Any resource assignment from the HR-BS is forwarded to the forwarding HR-MS.

16.3.2.1.2 Connection management involving forwarding HR-MS

[TBD]

16.3.2.1.3 Construction and transmission of MAC PDUs by forwarding HR-MS

Forwarding and forwarded HR-MS shall construct and transmit MAC PDUs in the same way as described in 6.3.3.

16.3.2.1.4 Bandwidth request involving forwarding HR-MS

A forwarding HR-MS carries out bandwidth request to the serving HR-BS/RS as a normal HR-MS, i.e., as specified in 6.3.6. The way a forwarded HR-MS carries out bandwidth request depends on whether it is inside or outside of the coverage of the HR-BS/RS.

16.3.2.1.4.1 BR by an inside-of-coverage forwarded HR-MS

When a forwarded HR-MS is inside the coverage of the serving HR-BS/RS, it carries out bandwidth request to the serving HR-BS/RS as a normal HR-MS, i.e., as specified in 6.3.6.  The corresponding forwarding HR-MS does not need to be aware of the bandwidth request from the forwarded HR-MS.

16.3.2.1.4.2 BR by an outside-of-coverage forwarded HR-MS

An out-of-coverage forwarded HR-MS can request bandwidth by transmitting some known sequences (Bandwidth Request (BR) preambles) toward the forwarding HR-MS.

The process can be described as follows.

- Serving HR-BS/RS schedules resources in uplink for forwarded HR-MSs to transmit BR messages to their corresponding forwarding HR-MS.

- The resource allocation information is conveyed to the forwarded HR-MS.

- The forwarding HR-MS listens to bandwidth requests at times and resources indicated by the HR-BS. The forwarded HR-MS may transmit bandwidth requests using these resources.

- The forwarding HR-MS, upon receiving BR messages from one of its forwarded HR-MS, forwards the requests to serving HR-BS/RS.

Any resource assignment from the HR-BS is forwarded to the forwarding HR-MS.

16.3.2.1.5 Synchronization between forwarding and forwarded HR-MS

When the forwarded HR-MS is inside the coverage of the serving HR-BS/RS, synchronization between itself and the corresponding forwarding HR-MS is achieved as described in 16.2.2.1.3.1.

When the forwarded HR-MS is out-of-coverage of the serving HR-BS/RS, synchronization between itself and the corresponding forwarding HR-MS is achieved as described in 16.2.2.1.3.2.

16.3.2.1.6 Changes in FTN operation modes

Figure xxx – Transitions to/from different (FTN) operating modes.

Figure 927 illustrates possible changes in HR-MS operation modes. Normal mode refers to the scenario when HR-MS communicates directly with its serving HR-BS/RS, FTN use case 1 is when both forwarding and forwarded HR-MS are within the coverage of HR-BS/RS, and use case 2 is when the forwarded HR-MS is outside-of-coverage of HR-BS/RS.

For the transition from normal operation to FTN use case 1, serving HR-BS/RS is the entity that makes the decision and informs the two involved HR-MS of the FTN connection setup.

For the transition from normal operation to FTN use case 2, serving HR-BS/RS first sends FN-CONFIG-CMD message to instruct the forwarding HR-MS to start relevant transmissions, e.g., of Preamble, M-FCH, NCI …, unless the HR-MS has already been doing so. Next the HR-BS/RS informs the other HR-MS, the one that is about to go out of HR-BS/RS coverage, of the potential forwarding HR-MS, together with relevant operation parameters such as IDCell, ranging opportunities. This may be follow by ranging between the two HR-MS before the FTN connection setup.

For the transitions from FTN use case 1 to normal operation, the serving HR-BS/RS makes the decision and initiates FTN connection release. For the transition from FTN use case 2 to normal operation, the forwarded HR-MS can monitor the channel to see if it can receive the frame preamble transmitted by the serving HR-BS/RS. If such preamble is detected, and the forwarded HR-MS can subsequently achieve DL synchronization, the normal network entry process with the HR-BS/RS shall be applied. The transition from FTN use case 1 to FTN use case 2 is similar to the transition from normal operation to FTN use case 2.

In both FTN use cases 1 and 2, the forwarded HR-MS carries out handover to a neighboring HR-BS/RS in the same manner as in normal operation.

16.3.2.1.4 16.3.2.1.7 Paging by HR-MS

The HR-BS may instruct an HR-MS or a group of HR-MS to broadcast a Paging Indicator (PI) message or IE. The PI contains ID’s of paged HR-MSs or groups of HR-MSs. The resources for the PI are signaled in the S-NCI. PI format is TBD.

An HR-MS that receives its ID or an assigned group ID in the PI shall access the paging HR-MS as is done for HR-MS neighbor discovery.

[-----------------------------------------------End of Text Proposal---------------------------------------------------]

# Use of IEEE-Copyrighted Conference and Journal Papers

The IEEE 802.16 Working Group has obtained permission to post IEEE-copyrighted material from IEEE conferences and publications when received in an official contribution to the Working Group. IEEE requires two conditions:

- A full copyright and credit notice must be posted at the top of the paper in the following format:

Copyright ©2002 Institute of Electrical and Electronics Engineers, Inc.  Reprinted, with permission, from [all relevant journal info].

This material is posted here with permission of the IEEE. Internal or personal use of this material is permitted. However, permission to reprint/republish this material for advertising or promotional purposes or for creating new collective works for resale or redistribution must be obtained from the IEEE (contact pubs–permissions@ieee.org).

By choosing to view this document, you agree to all provisions of the copyright laws protecting it.

- If the author is not the submitter of the contribution, the author’s written approval for the reuse of the material must be attached.