| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed changes to clause 15.4.2.3 |  |
| Date Submitted | 2007-09-19 |  |
| Source(s) | Mariana Goldhamer Alvarion Ltd. 21A, HaBarzel St, 69710, Tel Aviv, Israel Harry Bims Apple, Inc. 1 Infinite Loop, Cupertino, CA 95014 | Voice: +972544225548 E-mail:mariana.goldhamer@alvarion.com Voice: +1-650-283-4174 harrybims@att.net |
| Re: | Task Group Review |  |
| Abstract |  |  |
| Purpose | Approval |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Proposed changes to clause 15.4.2.3

Mariana Goldhamer, Harry Bims

Alvarion Ltd., Apple Inc.

### Comment 102 by Mariana

Comment:

Fix text and figure relative to Radio Signature for consistency with CX Frame; define the suitable MAC support

Comments of the Ah-Hoc:

- the contribution needs to be split in two parts, one for interferer identification and another one for interference control

- the Radio Signature scheduling should not overlap with CSI

- needs to explain that the interferer identification using CXP is not used during CXCC

# Proposed reorganization and text changes

| Existing clause | Title | Text |  |
| --- | --- | --- | --- |
| 15.4.2.3 | Interference control in sub-frames | Move to 15.4.5 and make the changes proposed below | P131/r8 to 134/r34 |

# Proposed text for 15.4.5 based on text from 15.4.2.3

15.3.6 Interferer identification

The interferer may be identified using CXCC procedures or using Radio Signature procedures.

15.3.6.1 Interferer identification using CXCC

The interfering Base Station may be identified from the BSID which is transmitted within the BSD transmitted during the CXCC sub-channel 2 or BS_NURBC messages transmitted during CXCC sub-channel 3 or the CSI sub-channel.

The interfering SS may be identified from the SSID included in the SSURF message, transmitted during CXCC sub-channel 2.

15.3.6.2 Interferer identification using Radio Signature

According to this procedure, the interferers may be identified by the time position their Radio Signature, which may be for example a short preamble for OFDM/OFDMA cases. The radio signature is a priory scheduled using the CXP for inter-system communication (15.5) and the CX-DL-MAP and CX-UL-MAP messages.  The Radio Signature is defined by the following parameters:

- Peak power

- Relative spectral density

- Direction of transmission.

Every transmitter should send the radio signature during an interference-free slot. The time position of this slot (frame_number, sub-frame, time-shift) should be used for identification. No other transmitters in the coexistence community should operate during the time slots scheduled for radio signature transmission. The scheduling of the Radio Signature transmission shall not use the CSI allocation.

In order to identify the interferer, a receiver should:

listen to the media during the radio signature slot and determine which interferers are the strongest; By scanning the BS databases it will be possible to identify, by the frame number, sub-frame number and offset, which BS is associated with the interferer. Based on time-shift information, the Base Station will be able to identify the Subscriber Station ID. During the allocated radio-signature transmit opportunity no other radio transmitters will operate. The silent transmitters will schedule a GAP DIUC or UIUC IE.

The Base Station data base shall keep the following information regarding the usage of “non-interfering sub-frame” or Master sub-frames belonging to other systems:

- BS power, relative to the radio signature power, when using each of the sub-frames;

- List of SSs and their power, relative to the radio signature power, when using each of the sub-frames.

The received power during other sub-frames can be obtained by using the radio signature measurement and suitable calculations, according to data-base information on used powers.

Radio Signature scheduling

Take the text from p132/r58 to p134/r33

End 15.3.6

Modify 15.4.2.3 and move to 15.4.5 accordingly:

15.4.5 Interference Control in sub-frames

The interference control may operate during Slave or Shared sub-frames. The interference control procedures should use interferer identification as described in clause 15.3.6

A BS has the right to request any interferer to reduce its power by P dB, for transmissions during the time in which this Base Station is the Master. If the requested transmitter cannot execute the request, it has to cease the operation during the Master sub-frame of the requesting Base Station; The system which uses the sub-frame as a Master can use the maximum capable and allowed operating EIRP with no limitation. If allowed by the first system, A newly entering system can use this sub-frame at its maximum capable and allowed operating EIRP,, otherwise it has to reduce its power to a level acceptable by the first system using the sub-frame as Master. The first system has the right to not allow the operation of any other system in parallel, if this causes harmful interference to it.

Messages as Stop_Operating_Request and Reduce_Power_Request can be used for controlling the interference levels.

An acceptable interference level during a Master sub-frame should be lower than 3dB above the noise or 12dB below the signal.

[editor’s note: the following 2 paragraph is redundant with the text in Error: Reference source not found.]

- The procedures for controlling the interference to BS are:

- Identification of the source of interference, using the procedures defined in 15.3.6;

- Request for transmit power reduction using CXP messages in clause 15.5. If the sources of the interference are identified, the BS should send message to the BSs of the interfering systems, asking them to drop the power of the specified transmitter inside the neighbor systems by P dB.

- Request for stopping the operation. Alternatively, the BS could send message to the BSs of the interference source systems, asking the interference source system to stop operating during this BS's master slot.

The interference status information of this BS in the system and the interference status information in the coexistence message from its neighbor systems should be stored and updated regularly into the distributed database of this system.

The following text is the continuation of 15.3.6.2