| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | ## Discussion on SSURF density and sub-carrier composition within the context of IEEE 802.16e OFDMA modulation/coding formats |  |
| Date Submitted | 2007-11-09 |  |
| Source(s) | John Sydor Communications Research Centre 3701 Carling Avenue Ottawa, Ontario | Voice: 613-998-2388 Fax: [Fax Number] john.sydor@crc.ca |
| Re: | Notes based on CMI ad-hoc |  |
| Abstract | Discussion on the number of SSURF messages to be installed inside the CX-CMI_U(n). |  |
| Purpose | Work toward providing more information regarding the SSURF in the OFDMA |  |
| Notice | This document has been prepared to assist IEEE 802.16. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy and Procedures | The contributor is familiar with the IEEE 802.16 Patent Policy and Procedures <http://ieee802.org/16/ipr/patents/policy.html>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard." Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication. Please notify the Chair <mailto:chair@wirelessman.org> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.16 Working Group. The Chair will disclose this notification via the IEEE 802.16 web site <http://ieee802.org/16/ipr/patents/notices>. |  |

Discussion on SSURF density and sub-carrier composition within the context of IEEE 802.16e OFDMA modulation/coding formats

John Sydor

Communincations Research Centre, Ottawa

Introduction

IEEE 802.16e OFDMA offers a number of modulation and coding rates for 512 (5 MHz) and 1024 (10 MHz) systems. SSURF messages need to be universally demodulated and decoded by all same-PHY interfered-with systems. Ideally, to be identified in worst case operating environments,  such SSURFs should be sent at a rate somewhat more robust than the standard operating modulation/coding format of the data channel. This format impacts the number of SSURFs that can be contained in the CXC_CMI_U(n).

Additionally we are also left with the question on the sub-channel/sub-carrier distribution and permutations of the SSURF. Sub-carrier sets used with SSURF messages should reflect the sub-carrier sets used by the mobile’s data link. This is important since the purpose of the SSURFs is to indicate the activity  of a specific mobile located on specific channels (and sub-carrier frequency domain slots).

Tabulated Results

Table 1 ( derived from Ref. 1) indicates the data rates for the lower order, high repetition modulation and coding schemes anticipated for use by WiMAX (IEEE 802.16e) systems. This table is used because it reflects commonly accepted uplink and downlink data rates that are anticipated for WiMAX mobile 5 and 10 MHz bandwidth allocations.

Table 2 gives the information content that is available in a CX_CMI_U(n). The calculation is based upon the fact that the CMI slots occupy 1.9 msec of CXCC time (Tcc_s). Based on this information content, calculations are undertaken as to the number of SSURFs that can occupy the given CMI at the given modulation/coding rate. The calculation is based upon the SSURF messages, as currently specified, containing  either 280 bits of data for IPV4 or 360 bits for IPV6.

Table 2 also gives the S/N ratio at which the given modulation/coding rate has a BER of 10(-6). Some of the values given are estimated.

| Modulation | Code Rate |  |  | 5 MHz Channel |  |  |  | 10 MHz Channel |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Downlink Rate Mbps |  | Uplink Rate Mbps |  | Downlink Rate Mbps | Uplink Rate Mbps |
| QPSK | ½ CTC 6X |  |  | 0.53 |  | 0.38 |  | 1.06 | 0.78 |
|  | ½ CTC 4X |  |  | 0.79 |  | 0.57 |  | 1.58 | 1.18 |
|  | ½ CTC 2X |  |  | 1.58 |  | 1.14 |  | 3.17 | 2.35 |
|  | ½ CTC 1 X |  |  | 3.17 |  | 2.28 |  | 6.34 | 4.70 |
|  | ¾ CTC |  | 4.75 |  | 3.43 |  | 9.50 |  | 7.06 |
| 16 QAM |  | ½ CTC |  | 6.34 |  | 4.57 |  | 12.07 | 9.41 |
|  |  | ¾ CTC |  | 9.50 |  | 6.85 |  | 19.01 | 14.11 |

Table 1 Mobile WiMAX PHY Data Rate with PUSC Sub-Channel

| Modulation | Code Rate (Min S/N for BER 10-6) |  | 5 MHz Channel |  |  |  |  |  |  | 10 MHz Channel |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Bit content in CX_CMI_U(n) Interval (1.9 Msec) |  |  | SSURF Content Ipv4 |  | SSURF Content Ipv6 |  | Bit content in CX_CMI_U(n) Interval (1.9 Msec) | SSURF Content Ipv4 | SSURF Contet Ipv6 |
| QPSK | ½ CTC 6X (-4.9 dB) |  | 722 |  |  | 2 |  | 2 |  | 1482 | 5 | 4 |
|  | ½ CTC 4X |  | 1083 |  |  | 3 |  | 3 |  | 2242 | 8 | 6 |
|  | ½ CTC 2X |  | 2166 |  |  | 7 |  | 6 |  | 4465 | 15 | 12 |
|  | ½ CTC 1 X (2.9 dB) |  | 4332 |  |  | 15 |  | 12 |  | 8930 | 31 | 24 |
|  | ¾ CTC (6.9 dB) |  |  | 6517 | 23 |  | 18 |  | 13414 |  | 47 | 37 |
| 16 QAM |  | ½ CTC (8.6 dB) | 8683 |  |  | 31 |  | 24 |  | 17879 | 63 | 49 |
|  |  | ¾ CTC | 13015 |  |  | 46 |  | 36 |  | 26809 | 95 | 74 |

Table 2: Approximate calculation of SSURF message occupancy within CX_CMI_U(n) conforming to expected WiMAX Mobile OFDMA modulation/coding rates

Discussion and Recommendations

The lowest, most robust modulation and coding rates indicate that anywhere from 2-15 SSURFs can be placed inside the CX_CMI_U(n) for 5 MHz channel widths, for example. In order to maintain an adequate SSURF reporting rate is is recommended that the modulation/coding rate for the SSURFs be chosen to be either at  or one level below the lowest data rate that would normally be used by the mobile. What this rate is remains to be discussed.

In order to ensure that a SSURF is representative of the sub-carrier content of the data messages sent by the mobile, it is also recommended that the SSURFs sub-carrier content be spread across the available bandwidth and that contiguous permutations not be used. Such sub-carrier distributions will provide the best frequency diversity and interference averaging capability. It is recommended that the SSURFs consist of the smallest number of sub-carriers (48) allowed by the standard.

Ref 1: Mobile WiMAX-Part 1: A technical overview and performance evaluation. Copyright by WiMAX Forum, Feb 21, 2006.