| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | SZK Definition and Management |  |
| Date Submitted | 2007-09-17 |  |
| Source(s) | Sheng Sun; Guo-Qiang Wang; Hang Zhang; Peiying Zhu, Mo-Han Fong, Wen Tong, David Steer, Gamini Senarath, , Derek Yu, Israfil Bahceci, and Mark Naden Nortel 3500 Carling Avenue Ottawa, Ontario K2H 8E9 Sergey Seleznev, Hyoung Kyu Lim Samsung Electronics Rep. of Korea, Gyonggi-do, Suwon | Voice: +613-763-4460 E-mail: shengs @nortel.com Voice: +613-765-4159 E-mail: guoqiang@nortel.com Voice: +82312795968 E-mail: s.sergey@samsung.com |
| Re: | IEEE 802.16j-07/019: “Call for Technical Comments Regarding IEEE Project 802.16j” |  |
| Abstract |  |  |
| Purpose | To incorporate the proposed text into the P802.16j Baseline Document (IEEE 802.16j-06/026r4) |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Security Zone Key Management

Sheng Sun, G.Q Wang ,Hang Zhang, Peiying Zhu, Mo-Han Fong, Wen Tong, David Steer, Gamini Senarath, , Derek Yu, Israfil Bahceci, and Mark Naden

Nortel

Sergey Seleznev, Hyoung Kyu Lim

Samsung Electronics

# 1. Introduction

This contribution is to specify the definition of  SZK( Security Zone Key) concept and its operation in current .16j baseline document

# 2. Proposed text change

++++++++++++++++++++++ Start Text ++++++++++++++++++++++++++++++++

[Insert the following text at the end of 7.1]

MR-BS and a group of RSs in MR cell maintain a set of trusted relationships, called Security Zone, in order to satisfy requirements of multi-hop relay system operation (see 7.1.8).

[Insert the following section 7.1.8]

7.1.8 Multi-hop relay operation security

The Security Zone consist of a MR-BS and number of the RSs. All RSs within security zone maintain trusted relations, i.e. share common security context for the protection of relay management traffic. Security zone has the following properties:

- RS becomes eligible to join security zone by the fact of RSs successful authentication to a network.

- RS becomes a member of security zone after security zone key material is provided to that RS by the MR-BS.

- The MR-BS employs PKMv2 procedures to securely deliver key material to RS.

- Once left, RS should not be able to operate in a security zone, without reauthentication and key redistribution (i.e. RS must not have knowledge of keys before it joins and after it leaves the security zone).

[Add the following section at end of 7.2.2.4.5]

7.2.2.4.5 SZK Context

The SZK context includes all the parameters associated with the SZK. SZK is randomly generated by the MR-BS and transferred to the RS under KEK or SZKEK. It is used to sign relay management messages within security zone. The SZK context is described in Table Xx.

Table Xx—SZK context

| Parameter | Size (bits) | Usage |
| --- | --- | --- |
| SZK | 160 | Randomly generated by the MR-BS and transmitted to RS under KEK or SZKEK (in multicast update). |
| SZK SN | 4 | The sequence number of SZK. The new SZK SN shall be one greater than the preceding SZK SN. |
| HMAC_KEY_SZU or CMAC_KEY_SZU | 160 or 128 | The key which is used for signing UL management messages. |
| HMAC_PN_SZU, CMAC_PN_SZU, HMAC_PN_RLU or CMAC_PN_PLU | 32 | Used to avoid UL replay attack on the management connection—when this expires new SZK shall be distributed by the MR-BS. |
| HMAC_KEY_SZD or CMAC_KEY_SZD | 160 or 128 | The key which is used for signing UL management messages. |
| HMAC_PN_SZD, CMAC_PN_SZD, HMAC_PN_RLD, CMAC_PN_RLD | 32 | Used to avoid DL replay attack on the management connection—when this expires new SZK shall be distributed by the MR-BS. |

[Insert the following section 7.2.2.3.4]

7.2.2.3.4 Security zone SA

Security zone SA is a group SA, which contains keying material to secure relay control within a security zone. The primary keying material is SZK and SZKEK, which are provisioned by the MR-BS. Members of a security zone are considered trusted and share common SZK. SZKEK may also be common within security zone.

The contents of security zone SA are as follows:

—The SZK, a 160 or 128-bit Security Zone Key, serves as a root for HMAC/CMAC keys derivation.

—The SZKEK, a 128-bit Security Zone Key Encryption Key, serves the same function as SA KEK but for GSA.

The SAID shall be unique within a MR-BS and all RSs.