| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Changes to Data Subcarrier Randomization (15.3.11.1.3) for the IEEE 802.16m/D2 |  |
| Date Submitted | 2009-11-05 |  |
| Source(s) | Alexei Davydov, Tom Harel Intel | alexei.davydov@intel.com tom.harel@intel.com |
| Re: | Category: LB30a / Area: Chapter 15.3.11 (Channel Coding - HARQ) |  |
| Abstract | The contribution proposes details on initial state for data randomization |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m Advanced Air Interface (Draft 3) |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Proposed Changes to Data Subcarrier Randomization (15.3.11.1.3) for the IEEE 802.16m/D2

Harel Tom, Alexei Davydov

Intel Corporation

# Motivation

# The initial state for data randomization is not defined. In this contribution we provide missing details.

# Text proposal for inclusion in the 802.16m D2

< Insert the following text in the subsection 15.3.11.1.3 Randomization on page 532 line 59>

--------------------------------------------------  Text Start  ------------------------------------------------

b0…b11 = 12 bits of STID for data channel and [1 0 1 1 1 0 1 0 1 0 1 1] for downlink control channel.

b12…b14= 101. The value of these bits is set for avoiding initial state of all zeros.

---------------------------------------------------  Text End  ---------------------------------------------------