| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification on MCRC masking operations |  |
| Date Submitted | 2009-11-10 |  |
| Source(s) | Daniel Cohn Alvarion Kelvin Chou MediaTek Inc. | Daniel.cohn@alvarion.com kelvin.chou@mediatek.com |
| Re: | Letter Ballot #30a on the Draft Amendment (IEEE P802.16m/D2) |  |
| Abstract | IEEE 802.16m/D2 specifies a masking operation to construct the MCRC which is not defined in the standard |  |
| Purpose | To be discussed and adopted by IEEE 802.16m TG |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clarification on MCRC masking operations

Daniel Cohn, Kelvin Chou

Alvarion, Mediatek

Motivation

Several A-MAP IE messages include the MCRC field, which is defined as the message 16-bit CRC masked by either the STID or by the RA-ID and masking indicator (depending on the IE type). The standard does not define the masking operation, although it seems clear that intention was to refer to a logical exclusive OR operation which would allow the destination MS to recover original CRC after applying the masking operation again. This is especially confusing since masking is usually referred to a logical AND operation.

Suggested Modifications

In table 733:

CRC masked by the reserved STID for BR ACK A-MAP, using the bitwise exclusive OR operation

In section 15.2.11.2:

AMS decodes the IE and checks the MCRC field by performing the bitwise exclusive OR operation with its specific 12-bit RA-ID and 4-bit Masking Indicator.

In table 734:

CRC masked by RA-ID and Masking Indicator, using the bitwise exclusive OR operation

In table 818:

16 bit CRC masked by Station ID, using the bitwise exclusive OR operation

In table 821:

16 bit CRC masked by Station ID, using the bitwise exclusive OR operation