| Project | IEEE 802.16 Broadband Wireless Access Working Group |  |
| --- | --- | --- |
| Title | Proposed text related to Restricted Connection between Non-CSG-Closed Users and Femtocell Basestation in IEEE 802.16m/D3 (16.4.2) |  |
| Date Submitted | 2009-01-12 |  |
| Source(s) | Linghang Fan, Andreas Maeder, Hassan Al-kanani, Nader Zein, Tetsu Ikeda NEC | E-mail: [Linghang.fan, nader.zein, hassan.alkanani] @eu.nec.com andreas.maeder@nw.neclab.eu, t-ikeda@ap.jp.nec.com |
| Re: | LB comment to 802.16m Amendment Working Document D3 |  |
| Abstract | This contribution provides text addition to AWD to enhance the definition of femtocell subscriber type CSG-Closed in IEEE 802.16m |  |
| Purpose | For discussion and approval by IEEE 802.16m TG |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Proposed text related to Restricted Connection between Non-CSG-Closed Users and Femtocell Basestation in IEEE 802.16m/D3 (16.4.2)

Linghang Fan, Andreas Maeder, Hassan Al-kanani, Nader Zein, Tetsu Ikeda

NEC

# Introduction

In D3, femto base station subscriber types are categorised into CSG-Closed, CSG-Open and OSG, and a closed subscriber group CSG-Closed femtocell base station is accessible only to AMSs which are members of this base station except for emergency services.  If the macrocell and femtocell use different frequencies, it is not a big issue. However, if the macrocell and femtocell uses the same frequency, it can cause big problem. In this case, when a non-CSG-Closed AMS enters an area, which is covered only by one/several CSG-Closed Femtocell ABSs, its communication with its macrocell ABS may be severely interfered, or even totally interrupted. Specifically, if a macro AMS is switched on in a CSG-closed only area or enters a CSG-closed only area, the interference from the surrounding Femtocell ABSs may interrupt the communication between the macro AMS and the nearest macro ABS or its serving macro ABS.

However, it is a general principle that the deployment of a femtocell should cause as less interference to the overlapping macrocell as possible. According to the current definition of CSG-Closed, this problem can not be solved.

Exchanging information between femtocell ABS and non-CSG AMSs may help to reduce such kind of interference and thus improve the performance for both non-CSG AMSs and the femtocell ABS.

# Proposed Text

[Insert text in subclause 16.4.2 as follows]

-------------------------------------------------------Start of the Text 1-------------------------------------------------------

16.4 Support for Femto ABS

16.4.2 Femto base station subscription types

# A Femtocell ABS may belong to one of the following subscriber types.

# CSG-Closed Femtocell ABS: a CSG-Closed Femtocell ABS is accessible only to the AMSs, which are in its CSG, except for emergency services. AMSs which are not the members of the CSG, should not try to access CSG-Closed Femtocell ABSs for data traffic. In case the communication between a macro AMS and the macro ABS is interrupted by interference from the CSG-Closed Femto ABS, which share  frequency assignment with the overlay macrocell, the macro AMS can signal related information (e.g. AMS ID and interference level indicator) to the Femto ABS, which can help bridge the communication between the macro AMS and the macro ABS.

--------------------------------------------------------End of the Text 1-------------------------------------------------------

[Insert text in subclause 16.4.11 as follows]

-------------------------------------------------------Start of the Text 2-------------------------------------------------------

16.4.11 Interference Avoidance and Interference Mitigation

When a macro AMS, which is not a member of the nearby CSG-Closed ABS but is severely interfered by the CSG-Closed ABS, which share frequency assignment with the overlay macrocell, The macro AMS may request the CSG-Closed ABS to mitigate the interference. The macro AMS may send an AAI_RNG-REQ message to the CSG-closed femto ABS. The RNG-REQ message from the AMS contains an interference level bit to indicate that the AMS was experiencing severe interference to the overlay macro ABS cell due to this CSG-Closed femto ABS.

--------------------------------------------------------End of the Text 2-------------------------------------------------------