| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposal to Add one bit in Assignment IE to specify whether Midamble can be used for channel estimation or not for this burst assignment |  |
| Date Submitted | 2009-12 |  |
| Source(s) | Hanqing Lou, Genyuan Wang, Li Guo, Yanxin Na, Hang Jin, Herbert Ruck Cisco Systems | halou@cisco.com |
| Re: | P802.16m/D3 LB30b |  |
| Abstract | The contribution proposes text to describe the usage of pilots for downlink measurements and burst demodulation. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m Advanced Air Interface (Draft 4) |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposal to Add one bit in Assignment IE to specify whether Midamble can be used for channel estimation or not for this burst assignment

Hanqing Lou, Genyuan Wang, Li Guo, Yanxin Na, Hang Jin, Herbert Ruck

Cisco Systems

- Introduction

In IEEE802.16m D3, the usage of pilot for channel estimation in DLRU is very fuzzy. In Section 16.3.5.4.3 Usage of Downlink pilots, for DLRU, it is only stated that “ the 2 streams non-adaptively precoded common pilots across the DLRU should be used for channel estimation by all AMSs allocated a burst in the DLRU.” It only means that DLRU can use common pilot for channel estimation. However, it is not clear whether the AMS can use midamble for DLRU channel estimation or not. In the reality, many reasons would result in that midamble is not feasible for channel estimation in DLRU. For example, in high mobility, the channel changes so fast such that the midamble and DLRU burst experience totally different channel. So we need a mechanism such that ABS is able to inform the AMS that it could use the midamble for the current burst or not.

The mechanism proposed here is a very simple and low-overhead scheme. Adding one bit in DL_ Basic_ Assignment_ A-MAP IE such that ABS can inform AMS through this bit.

The contribution proposes the text to describe the usage of midamble for downlink channel estimation. It provides the mechanism so that ABS can inform the AMS whether the midamble can be used for channel estimation for this burst or not.

Proposed AWD text has been underlined in blue, AWD text that has been deleted has been marked in red and struck through and existing AWD text is shown in black.

- References

[1] P802.16m DRAFT Amendment to IEEE Standard for Local and metropolitan area networks, P80216m/D3, Dec 2009.

- Text proposal for inclusion in the 802.16m AWD

< Make change in Table 812 as marked below >

-------------------------------  Start of text proposal #1 --------------------------------------------

| Syntax | Size in bits | Description/Notes |
| --- | --- | --- |
| DL_Basic_Assignment_A-MAP_IE() { |  |  |
| A-MAP IE Type | 4 | DL Basic Assignment A-MAP IE |
| ISizeOffset | 5 | Offset used to compute burst size index |
| MEF | 2 | MIMO encoder format 0b00: SFBC 0b01: Vertical encoding 0b10: Horizontal encoding 0b11: CDR |
| if (MEF == 0b01){ |  | Parameters for vertical encoding |
| Mt | 3 | Number of streams in transmission Nt : Number of transmit antennas at the ABS 0b000: 1 stream 0b001: 2 streams 0b010: 3 streams 0b011: 4 streams 0b100: 5 streams 0b101: 6 streams 0b110: 7 streams 0b111: 8 streams |
| Reserved | 1 | Reserved bit |
| } else if(MEF == 0b10) { |  | Parameters for horizontal encoding |
| Si | 4 | Index used to identify the combination of the number of streams and the allocated pilot stream index in a transmission with MU-MIMO , and the modulation constellation of paired user in the case of 2 stream transmission 0b0000: 2 streams with PSI=stream1 and other modulation =QPSK 0b0001: 2 streams with PSI=stream1 and other modulation =16QAM 0b0010: 2 streams with PSI=stream1 and other modulation =64QAM 0b0011: 2 streams with PSI=stream1 and other modulation information not available 0b0100: 2 streams with PSI=stream2 and other modulation =QPSK 0b0101: 2 streams with PSI=stream2 and other modulation =16QAM 0b0110: 2 streams with PSI=stream2 and other modulation =64QAM 0b0111: 2 streams with PSI=stream2 and other modulation information not available 0b1000: 3 streams with PSI=stream1 0b1001: 3 streams with PSI=stream2 0b1010: 3 streams with PSI=stream3 0b1011: 4 streams with PSI=stream1 0b1100: 4 stream with PSI=stream2 0b1101: 4 streams with PSI=stream3 0b1110: 4 streams with PSI=stream4 0b1111: n/a |
| } |  |  |
| Resource Index | 11 | 5 MHz: 0 in first 2 MSB bits + 9 bits for resource index 10 MHz: 11 bits for resource index 20 MHz: 11 bits for resource index Resource index includes location and allocation size. |
| Long TTI Indicator | 1 | Indicates number of AAI subframes spanned by the allocated resource. 0b0: 1 AAI subframe (default) 0b1: 4 DL AAI subframes for FDD or all DL AAI subframes for TDD |
| HFA | 3 | HARQ Feedback Allocation |
| AI_SN | 1 | HARQ identifier sequence number |
| ACID | 4 | HARQ channel identifier |
| SPID | 2 | HARQ subpacket identifier for HARQ IR 0b00: 0 0b01: 1 0b10: 2 0b11: 3 |
| CRV | 1 | Constellation Rearrangement Version 0b0: 0 0b1: 1 |
| Channel Estimation Aided by Midamble | 1 | This bit only applicable to DLRU allocations. 0b0: AMS should use pilots associated with burst allocation for channel estimation 0b1: AMS could use midamble and pilots associated with burst allocation for channel estimation |
| Reserved | 2 | Reserved bits |
| } |  |  |

-------------------------------  End of text proposal #1 --------------------------------------------

< Insert the following paragraph on page 397 on line 53  (16.3.6.5.2.4.1) >

-------------------------------  Start of text proposal #2 --------------------------------------------

Channel Estimation Aided by Midamble: This bit is only applicable to DLRU allocations. It is an indicator to specify whether AMS should use pilots associated with burst allocation only for the channel estimation.

-------------------------------  End of text proposal #2 --------------------------------------------