| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Modifications to Service Specific BR Header (16.2.2) |  |
| Date Submitted | 2010-01-13 |  |
| Source(s) | Jaesun Cha, Soojung Jung, Eunkyung Kim, Sungcheul Chang, Hyun Lee, Kwangjae Lim, Chulsik Yoon ETRI | jscha@etri.re.kr, *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE 802.16 Working Group Letter Ballot Recirculation #30b on P802.16m/D3 |  |
| Abstract | This contribution proposes to modify some parameters in service specific BR header |  |
| Purpose | To be discussed and adopted by TGm for 802.16m draft. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Modifications to Service Specific BR Header (16.2.2)

Jaesun Cha, Soojung Jung, Eunkyung Kim, Sungcheol Chang, Hyun Lee, Kwangjae Lim, Chulsik Yoon

ETRI

# Introduction

Sleep Cycle change request may be piggybacked with bandwidth request in service specific BR without STID header as shown in Table 659.

In Table 659, even though SCID change indicator is included if scheduling type is equal to aGP service, decision on whether Sleep Cycle information is included or not is made only if QoS parameter change indicator is not equal to 1.

Sleep cycle shall be designed considering traffic patterns of service flows established between AMS and ABS, which means that any change in QoS parameters may impact on the efficiency of active Sleep Cycle. We can’t see any reason why Sleep Cycle change request is valid only when QoS parameter set is changed.

In addition, we propose one more editorial modification regarding padding fields. In service specific BR without STID header, many padding fields are used to fit the header into 6-byte signaling header format. But, because their lengths are different from each other, it makes readability poor. It’s better to use one padding field of variable length instead of multiple padding fields.

# Text Proposal

---------------------------------------------------------Start of the Text----------------------------------------------------------

[Modify Table 659 as follows]

Table 659 – Service Specific BR without STID Header Format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| Service specific BR header without STID header () { |  |  |
| FID | 4 | Flow Identifier. This field indicates MAC signaling header |
| Type | 4 | MAC signaling header type = 0b0010 |
| BR Type | 1 | Indicates whether the requested bandwidth is incremental or aggregate. 0: incremental 1: aggregate |
| BR Size | 19 | Bandwidth request size in bytes. |
| BR FID | 4 | The FID for which UL bandwidth is requested. |
| if (scheduling type == aGP service) { |  |  |
| QoS parameter change indicator | 1 | QoS parameter change indicator 0: no change in QoS parameter 1: having changes in QoS parameter |
| SCID change indicator | 1 |  |
| if (QoS parameter change indicator = 1) { |  |  |
| Running Grant Polling Interval (GPI) | 6 | Indicating new GPI (frames) to use for future allocation. |
| } else { |  |  |
| QoS parameter set switch | 1 | 0: primary QoS parameter set. 1: secondary QoS parameter set |
| } |  |  |
| If (SCID change indicator == 1) { |  |  |
| SCID | 4 |  |
| } |  |  |
| } |  |  |
| else if (scheduling type == BE) { |  |  |
| Minimum grant delay | 6 | Indicating minimum delay (frames) of the requested grant |
| } |  |  |
| Padding | variable | This field shall be filled by ‘0’ so that the total length of this header is equal to 6 bytes. |
| } |  |  |

---------------------------------------------------------End of the Text----------------------------------------------------------