| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Indication of the MIMO OL region – Proposed Amendment Text |  |
| Date Submitted | 2010-01-12 |  |
| Source(s) | Sudhir Ramakrishna David Mazzarese Kwanhee Roh Taeyoung Kim Hyunjeong Kang Heewon Kang Samsung Electronics | sudhir.r@samsung.com |
| Re: | “Contribution in support of a comment on LB30”: Category – AWD Comments Area – Chapter 16.2.3 (MAC Control Messages) Chapter 16.3.5 (Downlink physical structure) Chapter 16.3.6 (Downlink Control Structure) Chapter 16.3.7 (Downlink MIMO) |  |
| Abstract | This contribution proposes some modifications to the parameters indicating the OL MIMO region configuration in the AAI System Configuration Descriptor message in Section 15.2.3.28, and also proposes the interpretation rules for these parameters. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Indication of the MIMO OL region – Proposed Amendment Text

Sudhir Ramakrishna, David Mazzarese, Kwanhee Roh, Taeyoung Kim, Hyunjeong Kang, Heewon Kang

Samsung Electronics

- Introduction

In [1], the definition of the MIMO OL region, and the method of indicating its structure, is incomplete. Also, the sub-channelization in [1] is such that some of the OL regions cannot be implemented. This contribution completes the specification of the OL region. This contribution makes the following changes to the following sections in [1].

- The AAI System Configuration Description (AAI_SCD) message, defined in Section 15.2.3.28, carries some parameters that indicate the configuration of the MIMO open-loop region. This contribution modifies/optimizes these parameters,

- The rules as to how the OL region related parameter in the AAI_SCD message are to be interpreted to get the structure of the OL region are provided in Section 16.3.7.2.5.1, “Open Loop Region”.

- In [1], due to the cell-specific CRU/DRU allocation rules defined in Equation (194) in Section 16.3.5.3.1 (“CRU/DRU allocation”), it is not possible in general to get DLRUs and NLRUs to be aligned, thus affecting the implementability of OL Regions of Types 0 and the NLRU component of the OL Region of Type 1. This contribution proposes to add a bit to the S-SFH SP2, via which we may switch off the cell-specific permutation in Equation (194) for frequency partition 0.

There are 4 text modifications proposed in this contribution, each in a different section, and appear in the section order in [1]. Modification 1 makes changes to the parameters in the AAI_SCD message in Section 16.2.3.29. Modification 2 adds text to Section 16.3.5.3.1 (“CRU/DRU allocation”). Modification 3 adds an addition bit to the S-SFH SP2 IE, while Modification 4 adds text to the MIMO Open Loop Region section, describing how the AAI_SCD parameters are to be interpreted to derive the structure of the OL region.

- In this document, text to be deleted has been struck out and marked with red font. Text to be added is in blue underlined font. Existing text to be retained un-changed is in black font.

- Proposed Text Modification 1

- The contents of the AAI System Configuration Description (AAI_SCD) message, which is defined starting on Line 50 on Page 94, and ending on Line 33 on Page 95 in Section 16.2.3.29, are proposed to be modified as follows.

- **************** Begin Proposed Modification 1 *****************************************

-- ASN1START

AAI_SCDMessage :: SEQUENCE {

Change Configuration Change			INTEGER (0 ~ 15)

BS_Restart_Count					INTEGER (0 ~ 15)

SA_PreamblePartitionforBStype

Trigger TLV encoding :: = SEQUENCE{}

DefaultHORSSI_CINRaveraggingparameters :: = SEQUENCE {}

NormalizedCINR :: = SEQUENCE {}

OL_MIMO_Parameters :: = SEQUENCE {

OL-Region-ON				INTEGER (0..1) OPTIONAL

OL-Rank1-Configuration			INTEGER (0..7) OPTIONAL

SB-OL-Region-2Size			INTEGER (0..15) OPTIONAL

}

MIMO_OL_Region _Parameters :: = SEQUENCE {

OL-Region-Type0-ON INTEGER (0, 1) OPTIONAL

OL-Region-Type1-NLRU-Size INTEGER (0, 31) OPTIONAL

OL-Region-Type1-SLRU-Size INTEGER (0, 31) OPTIONAL

OL-Region-Type2-SLRU-Size INTEGER (0, 31) OPTIONAL

}

Parameters_GRA :: = SEQUENCE {}

PeriodicRNGParameters :: = SEQUENCE {}

GAMMA_IOT_FP0					INTEGER (0..15) OPTIONAL

GAMMA_IOT_FP1					INTEGER (0..15) OPTIONAL

GAMMA_IOT_FP2					INTEGER (0..15) OPTIONAL

GAMMA_IOT_FP3					INTEGER (0..15) OPTIONAL

Alpha (a)					INTEGER (0.. 7) OPTIONAL

Beta (ß)					INTEGER (0..1) OPTIONAL

SINRmin						INTERGER (0..15) OPTIONAL

BR_Channel Configuration MIN Access Class	INTERGER (0..4) OPTIONAL

Sounding_multiplexing_type 					CHOICE {

fdm		FDM

cdm		CDM

}

Decimation_value_D/Max_Cyclic_Shift_Index_P_for_sounding		ENUMERATED {n4, n9, n16,

n18, n36}

RELAY_Support_Parameters :: = SEQUENCE {

16m_Relay_zone_AMS_allocation_indicator	INTEGER (0..1) OPTIONAL

}

}

-- ASN1STOP

**************** End Proposed Text Modification 1 *****************************************

- Proposed Text Modification 2

- The content on Lines 32 to 35 on Page 329 of Section 16.3.5.3.1 (“CRU/DRU allocation”) is proposed to be modified as follows.

- **************** Begin Proposed Modification 2 *****************************************

- In Equation(194), PermSeq() is the permutation sequence of length (FPSi-LSB-CRU,FPi) and is determined by SEED = {IDcell*343} mod 210. The permutation sequence is generated by the random sequence generation algorithm specified in 16.3.5.3.3. DL_PermBase is set to preamble IDcell. If the OLRegion parameter in the S-SFH SP2 is set to 0b0, then the SEED and DL_PermBase in FP0 shall both be set to 0 in Equation(194).

- **************** End Proposed Modification 2 *****************************************

- Proposed Text Modification 3

- Table 802 (“S-SFH SP2 IE format”) on Page 379 of Section 16.3.6.5.1.2 is proposed to be modified as follows.

**************** Begin Proposed Text Modification 3 *****************************************

| - S-SFH SP2 IE format |  |  |
| --- | --- | --- |
| Syntax | Size (bit) | Notes |
| S-SFH SP2 IE format () { |  |  |
| Start superframe offset where new SP2 information is used | 2 |  |
| Frame configuration index | 6 | The mapping between value of this index and frame configuration is listed in Table Error: Reference source not found, Error: Reference source not found,and Error: Reference source not found |
| If (Duplexing mode == FDD) { |  |  |
| UL carrier frequency | 6 |  |
| UL bandwidth | 3 |  |
| } |  |  |
| MSB bytes of 48 bit ABS MAC ID | 36 | Specifies 36 MSB of BS ID |
| MAC protocol revision | 4 | Version number of AAI supported on this channel |
| OLRegion | 1 | Provides indication about the structure of the MIMO OL Region. Further details in Section 16.3.5.3.1. |
| DSAC | 5/4/3 | See Error: Reference source not found DL Frequency partitioning For 20 MHz, 5 bits For 10 MHz, 4 bits For 5 MHz, 3 bits |
| DFPC | 4/3/3 | See Error: Reference source not found DL Frequency partitioning For 20 MHz, 4 bits For 10 MHz, 3 bits For 5 MHz, 3 bits |
| DFPSC | 3/2/1 | See Error: Reference source not found DL Frequency partitioning For 20 MHz, 3 bits For 10 MHz, 2 bits For 5 MHz, 1 bit |
| USAC | 5/4/3 | See Error: Reference source not found UL Frequency partitioning For 20 MHz, 5 bits For 10 MHz, 4 bits For 5 MHz, 3 bits |
| UFPC | 4/3/3 | See Error: Reference source not found UL Frequency partitioning For 20 MHz, 4 bits For 10 MHz, 3 bits For 5 MHz, 3 bits |
| UFPSC | 3/2/1 | See Error: Reference source not found UL Frequency partitioning For 20 MHz, 3 bits For 10 MHz, 2 bits For 5 MHz, 1 bit |
| AMS Transmit Power Limitation Level | 5 | Unsigned 5-bit integer. Specifies the maximum allowed AMS transmit power. Values indicate power levels in 1 dB steps starting from 0 dBm |
| EIRxPIR,min | 5 |  |
| reserved |  |  |
| } |  |  |

**************** End Proposed Text Modification 3 *****************************************

- Proposed Text Modification 4

- The following text is proposed to be inserted on the beginning of Page 455 in Section 16.3.7.2.5.1 (“Open Loop Region”), following Table 837 (“Types of open-loop regions”) in [1].

- **************** Begin Proposed Text Modification 4 *****************************************

The structure of the MIMO OL Region is indicated by the MIMO_OL_Region_Parameters parameter in the AAI_SCD message, which shall be interpreted as follows.

The OL Region of Type 0 is present if OL-Region-Type0-ON is set to 0b1 in the AAI_SCD message. In that case, all DLRUs in frequency partition 0 shall be in OL region Type 0.

The decimal value of the OL-Region-Type1-NLRU-Size parameter shall indicate the number of NLRUs in the Type 1 region, in units of N1; N1 has been defined in Section 16.3.5.2.1. The Type 1 region shall be interpreted to consist of the NLRUs with contiguous logical indices, from index 0 up-to and including index N1* OL-Region-Type1-NLRU-Size -1, in frequency partition 0.

The decimal value of the OL-Region-Type1-SLRU-Size parameter shall indicate the number of SLRUs in the Type 1 region, in units of N1. The Type 1 region shall be interpreted to consist of the SLRUs with contiguous logical indices, from index 0 up-to and including index N1* OL-Region-Type1-SLRU-Size -1, in frequency partition 0.

The decimal value of the OL-Region-Type2-SLRU-Size parameter shall indicate the number of SLRUs in the Type 2 region, in units of N1. The Type 2 region shall be interpreted to consist of the SLRUs with contiguous logical indices, from index N1* OL-Region-Type1-SLRU-Size up-to and including index N1* (OL-Region-Type1-SLRU-Size + OL-Region-Type2-SLRU-Size) -1, in frequency partition 0.

- ***************** End Proposed Text Modification 4 *******************************************

- References

[1] IEEE P802.16m/D3, “Part 16: Air Interface for Broadband Wireless Access Systems”, December 2009.