| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed text for E-MBS zone configuration (P802.16m/D3-16.9.3.2) |  |
| Date Submitted | 2009-12-31 |  |
| Source(s) | Kaiying Lv, Nan Li, Bo Sun ZTE Corporation | Voice: [Telephone Number (optional)]] E-mail: lv.kaiying@zte.com.cn |
| Re: | Category: LB30b / Area: Chapter 16.9.3.2.( E-MBS zone configuration) |  |
| Abstract | This contribution proposes modifications on E-MBS zone configuration |  |
| Purpose | To discuss and adopt the proposed text in TGm. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

- Proposed text for E-MBS Zone Configuration

- Kaiying Lv，Nan Li，Bo Sun

- ZTE Corporation

- Proposed Text Changes

-------------------------------------------------------Start of the Text --------------------------------------------------------

[Make the following changes to Section16.9.3.2 in page644 beginning Line 3 in P802.16m/D3. ]

16.9.3.2 E-MBS Zone Configuration

Different MSTIDs and FIDs may be used in different E-MBS Zones for the same multicast and broadcast service flow. A multicast and broadcast zone identifier (E-MBS_ZONE_ID) is used to indicate a service area in which a MSTID and FID for a broadcast and multicast service flow are valid. An ABS that supports E-MBS shall include the E-MBS Zone identifier(s) to which it belongs in the AAI-E-MBS_CFG message. The E-MBS Zone identifier shall not be '0'.

When the E-MBS Zone identifier list appears in AAI_NBR_ADV message with only one value of '0' for some of ABS's, it indicates that the corresponding neighbor ABSs are not affiliated with any E-MBS zone.

When the ABS sends AAI_DSA message for establishment of connection for E-MBS_ZONE_ID, the E-MBS_ZONE_ID shall be encoded in the AAI-DSA message. One ABS may have multiple E-MBS_ZONE_IDs for different E-MBS services.

To support inter-MBS zone transition, AMSs need to get E-MBS_Zone_IDs to which the neighboring ABS's belong as well as any flow continuity across such neighboring zone. When the neighboring E-MBS Zone identifier list appears in AAI-E-MBS_CFG message, it indicates that some neighbor ABSs are affiliated with E-MBS zone(s) which provide inter-MBS zone transition with service continuity. In this case, the physical carrier index should be indicated if the carrier(s) carrying E-MBS services in the neighboring E-MBS zone has(have) changed. When an AMS in normal mode or idle mode transits to the new E-MBS zone, it will detect and decode the E-MBS services on the carrier directed by the physical carrier index.

---------------------------------------------------------End of the Text---------------------------------------------------------

Reference

[1] IEEE P802.16m/D3