| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed text on transmission of DL resource assignment information for AAI_RNG-RSP (D4 - 16.2.3.2) |  |
| Date Submitted | 2010-03-16 |  |
| Source(s) | Heejeong Cho, Youngsoo Yuk, Ronny (Yongho) Kim, Jin Sam Kwak LG Electronics | Voice : +82-31-450-7828 E-mail: hj77cho@lge.com |
| Re: | IEEE 802.16-10/0011, IEEE 802.16 Working Group Letter Ballot #31 |  |
| Abstract | This contribution proposes methods of transmitting DL resource assignment information for AAI_RNG-RSP before ABS assigns TSTID. |  |
| Purpose | To be adopted by TGm for the 802.16m Amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed text on transmission of DL resource assignment information for AAI_RNG-RSP (D4 - 16.2.3.2)

Heejeong Cho, Youngsoo Yuk, Ronny (Yongho) Kim, Jin Sam Kwak

LG Electronics

- Introduction

This contribution includes the proposed text for the DL resource assignment information of AAI_RNG-RSP in the IEEE 802.16m DRAFT Amendment [1].

- Modifications and key-descriptions

- A-MAP IE for resource assignment of AAI_RNG-RSP

  - In contention-based random access ranging procedure, an AAI_RNG-RSP including a TSTID or STID is transmitted in response to the AAI_RNG-REQ when an ABS does not assign TSTID or STID to an AMS.

  - What identifier is CRC in an A-MAP IE for the AAI_RNG-RSP masked by?

    - Method 1 : To use Broadcast / Multicast ID

      - If DL basic assignment A-MAP IE or Broadcast assignment A-MAP IE is reused, AMS should decode all of the AAI_RNG-RSPs.

    - Method 2 : To use RA-ID

      - If CDMA Allocation A-MAP IE is reused and the CRC in the A-MAP IE is masked by its own RA-ID, AMS should decode the AAI_RNG-RSP.

      - In order to support transmission of DL assignment information, we need to add Uplink/Downlink indicator field in the CDMA Allocation A-MAP IE.

Figure 1 - Example of the contention-based random access ranging procedure when an ABS does not assign TSTID or STID to an AMS

- References

[1] IEEE P802.16m/D4, “DRAFT Amendment to IEEE Standard for Local and metropolitan area networks”

[2] Remedy 2 in IEEE C80216m-10/0295r5, “Signaling Support for 16m ULPC Initial Stage (IEEE 802.16m/D4 Amendment)”

- Text proposal for the 802.16m DRAFT amendment

-----------------------------------------------------------Start of the Text---------------------------------------------------------

[On the IEEE P802.16m/D4, page 66, line 42, make the following changes to subclause 16.2.3.2]

16.2.3.2 AAI_RNG-RSP

An AAI_RNG-RSP, to which HARQ operation is applied, shall be transmitted by the ABS in response to a received AAI_RNG-REQ. In contention-based random access ranging procedure, the AAI_RNG-RSP message is allocated by CDMA Allocation A-MAP IE only if the ABS sends the message to an AMS which is not assigned STID/TSTID yet.

[On the IEEE P802.16m/D4, page 258, line 48, make the following changes to subclause 16.2.11.2]

16.2.11.2 Grant

In the contention-based random access bandwidth request procedure, the grant for BR header is allocated by CDMA Allocation A-MAP IE, which is shown in Table 755832. CDMA Allocation A-MAP IE is used for allocation of UL bandwidth to a user that requested bandwidth using a random ranging preamble code or BR preamble code. In contention-based random access ranging procedure, ABS transmits AAI_RNG-RSP message to a user which is not assigned STID/TSTID through DL resource assigned by CDMA Allocation A-MAP IE.

[On the IEEE P802.16m/D4, page 503, line 35, make the following changes to subclause 16.3.6.5.2.4.7]

Table 832—CDMA Allocation A-MAP IE format

| Syntax | Size (bits) | Notes |
| --- | --- | --- |
| CDMA_Allocation_A-MAP IE() { | — | — |
| A-MAP IE type | 4 | CDMA Allocation A-MAP IE |
| CDMA allocation indication | 1 | 0b0: Bandwidth allocation for bandwidth request. 0b1: Bandwidth allocation for ranging |
| If (CDMA allocation indication == 0b0) { |  |  |
| Resource Index | 11 | 5 MHz: 0 in first 2 MSB bits + 9 bits for resource index 10 MHz: 11 bits for resource index 20 MHz: 11 bits for resource index Resource index includes location and allocation size. |
| Long TTI Indicator | 1 | Indicates number of AAI subframes spanned by the allocated resource. 0b0: 1 AAI subframe (default) 0b1: 4 UL AAI subframes for FDD or all UL AAI subframes for TDD If number of DL AAI subframes, D, is less than number of UL AAI subframes, U, Long TTI Indicator= 0b1 |
| HFA | 3 | HARQ Feedback Allocation |
| Reserved | 20 | Reserved bits |
| } |  |  |
| Else if (CDMA allocation indication == 0b1) { |  |  |
| Uplink/Downlink Indicator | 1 | Indicates whether the following fields are for resource assignment in the uplink or in the downlink. 0b0: Uplink 0b1: Downlink |
| Resource Index | 11 | 5 MHz: 0 in first 2 MSB bits + 9 bits for resource index 10 MHz: 11 bits for resource index 20 MHz: 11 bits for resource index Resource index includes location and allocation size. |
| Long TTI Indicator | 1 | Indicates number of AAI subframes spanned by the allocated resource. 0b0: 1 AAI subframe (default) 0b1: 4 UL AAI subframes for FDD or all UL AAI subframes for TDD If number of DL AAI subframes, D, is less than number of UL AAI subframes, U, Long TTI Indicator= 0b1 |
| ISizeOffset | 5 | Offset used to compute burst size index |
| HFA | 3 | HARQ Feedback Allocation |
| If (Uplink/Downlink Indicator == 0b0) { |  |  |
| iotFP | 7 | IoT value of Frequency Partition used for AMS resource assignment, quantized in 0.5 dB steps as IoT level from 0 dB to 63.5 dB., detail reference to 16.2.3.30 AAI_ULPC_NI message |
| offsetControl | 6 | offsetControl is the transmit power adjustment value transmitted by the ABS. It represents the value among -15.5 to 16 dB with 0.5 dB step |
| Reserved | 1 | Reserved bits |
| } Else { |  |  |
| ACID | 4 | HARQ channel identifier |
| Reserved | 150 | Reserved bits |
| } |  |  |
| } |  |  |
| } | — | — |

[On the IEEE P802.16m/D4, page 83, line 7, make the following changes to subclause 16.2.3.11]

Table 686—Parameters for AAI_HO-CMD message

| Name | Value | Usage |
| --- | --- | --- |
| … |  |  |
| Ranging initiation deadline |  | Shall be included for each target ABS when Mode=0b00 |
| Preassigned STID | Pre-assigned STID which will be used at the target ABS. | Shall be included for each target ABS when Mode = 0b00 |

-----------------------------------------------------------End of the Text---------------------------------------------------------