| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Changes to ranging preamble code partition information table (16.3.9.2.4.2) |  |
| Date Submitted | 2010-03-05 |  |
| Source(s) | Chiwoo Lim, Youngkyo Beak, Hwasun Yoo, Jaeweon Cho, Hokyu Choi, Heewon Kang Samsung Electronics Co., Ltd. | chiwoo.lim@samsung.com |
| Re: | “IEEE 802.16 Working Group Letter Ballot#31” Target topic: “IEEE P802.16m/D4, section 16.3.9.2.4.2”. |  |
| Abstract | The contribution proposes the text change to ranging preamble code partition information table of the 802.16m draft amendment, IEEE P802.16m/D4. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Changes to ranging preamble code partition information table (16.3.9.2.4.2)

Chiwoo Lim, Youngkyo Beak, Hwasun Yoo, Jaeweon Cho, Hokyu Choi, Heewon Kang
Samsung Electronics Co., Ltd.

- Introduction

The contribution proposes the text change of ranging preamble code partition information table to be included in the 802.16m amendment. In the current draft amendment [1], code index is used in RA-ID and AAI_RNG-ACK. The number of code index is considered as less than 64 in RA-ID and AAI_RNG-ACK. So we set the size of code index in RAID by 6bits.

However, there is an index which has the total number of codes is larger than 64 in ranging preamble code partition information table. We need to clarify this.

- References

[1] IEEE P802.16m/D4, “P802.16m DRAFT Amendment to IEEE Standard for Local and metropolitan area networks Part 16: Air Interface for Broadband Wireless Access Systems”, 2010-02-03

- Proposed changes for 802.16m draft amendment

---------------------------------------------------  Text Start  ---------------------------------------------------

[Remedy-1: Modify the Table 900 in page 628 as following]

Table 900 – Ranging preamble code partition information table for Femtocell, NIN, NHO and NPE

| Partition Index | Number of initial ranging preamble codes, NIN | Number of handover ranging preamble codes, NHO | Number of periodic ranging preamble codes, NPE |
| --- | --- | --- | --- |
| 0 | 4 | 4 | 4 |
| 1 | 4 | 8 | 4 |
| 2 | 4 | 16 | 4 |
| 3 | 4 | 24 | 4 |
| 4 | 8 | 4 | 8 |
| 5 | 8 | 8 | 8 |
| 6 | 8 | 16 | 8 |
| 7 | 8 | 24 | 8 |
| 8 | 16 | 4 | 16 |
| 9 | 16 | 8 | 16 |
| 10 | 16 | 16 | 16 |
| 11 | 16 | 24 | 16 |
| 12 | 24 | 4 | 24 |
| 13 | 24 | 8 | 24 |
| 14 | 24 | 16 | 24 |
| 15 | 24 | 24 | 24 16 |

---------------------------------------------------  Text End  ---------------------------------------------------