| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed text related to E-MBS Scheduling Interval (MSI) (16.9) |  |
| Date Submitted | 2010-03-05 |  |
| Source(s) | Jeongki Kim, Youngsoo Yuk, Kiseon Ryu, Ronny (Yongho) Kim, Jin Sam Kwak LG Electronics | E-mail: jeongki.kim@lge.com, youngsoo.yuk@lge.com |
| Re: | Comments on IEEE P802.16m/D4 for IEEE 802.16 WG Letter Ballot Recirc #31 |  |
| Abstract | This contribution proposes the texts related to MSI (E-MBS Scheduling Interval). |  |
| Purpose | To be adopted by TGm for the 802.16m Amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed text related to E-MBS Scheduling Interval (MSI) (16.9)

Jeongki Kim, Youngsoo Yuk, Kiseon Ryu, Ronny (Yongho) Kim, Jin Sam Kwak

LG Electronics

- Introduction

According to D4 [1], the MSI begins at the next superframe after the AAI_E-MBS-CFG is received. So, AMS subscribing E-MBS does not need to calculate the start superframe of MSI based on the Nsuperframe and NMSI.

And the minimum MSI length (2 superframes) is hardly used. So, delete the 2 superframes and add the 32 superframes in MSI length.

- References

[1] IEEE P802.16m/D4, “DRAFT Amendment to IEEE Standard for Local and metropolitan area networks”

- Text proposal for the 802.16m DRAFT amendment

---------------------------------------------------------Start of the Proposed Text ---------------------------------------------------------

[Remedy #1: On page 751, modify Table 949 as follows:]

Table 949—AAI_E-MBS-CFG_Message format

| Syntax | Size in bits | Notes |
| --- | --- | --- |
| AAI-E-MBS-CFG_Message_Format() () { | - | - |
| ... | ... | ... |
| MSI Length (NMSI) | 2 | The length of an MSI in units of the number of superframes 0b00: 2 superframes, 480 ms (NMSI = 24) 0b01: 4 superframes, 8160 ms (NMSI = 48) 0b10: 8 superframes, 16320 ms (NMSI = 816) 0b11: 16 superframes, 32640 ms (NMSI = 1632) |
| … | ... | ... |
| } |  |  |

[Remedy #2: Modify “16.9.3.3 E-MBS Scheduling Interval” section as follows:]

16.9.3.3 E-MBS Scheduling Interval (MSI)

For each E-MBS Zone there is an E-MBS Scheduling Interval (MSI), which refers to a number of successive super-frames for which the access network may schedule traffic for the streams associated with the MBS Zone prior to the start of the interval. The MSI can span several super-frames and the length of this interval, denoted by NMSI, depends on the particular use case of E-MBS. The MSI can be, NMSI = 2, 4, 8, 16 and 1632 superframes long. The E-MBS_MAP message addresses the mapping of E-MBS data associated with an E-MBS Zone for an entire MSI. The MBS MAP message is structured such that it may be used to efficiently define multiple transmission instances for a given stream within an MSI. The MSI for a particular E-MBS zone is transmitted in the AAI-E-MBS-CFG message. The MSI begins at the next superframe after the AAI_E-MBS-CFG message is received. Using the superframe number, Nsuperframe from SFH, and NMSI from AAI-E-MBS-CFG message, the AMS computes the beginning of the MSI as follows:

The MSI begins at the superframe when its Nsuperframe meets the following condition.

Nsuperframe modulo NMSI == 0

An AMS decodes only the E-MBS data bursts associated with user selected content. The AMS wakes up in each MSI in order to check whether there are E-MBS data bursts to be decoded. -----------------------------------------------------------End of the Proposed Text   --------------------------------------------------------