| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text on the E-MBS MAP in IEEE P802.16m (16.3.6) |  |
| Date Submitted | 2010-03-05 |  |
| Source(s) | Eunkyung Kim, Soojung Jung, Jaesun Cha, Sungcheol Chang, Hyun Lee, Chulsik Yoon ETRI 138 Gajeongno, Yusong-gu, Daejeon, 305-700, KOREA | Voice: +82-42-860-5415 E-mail: ekkim@etri.re.kr |
| Re: | “P802.16m/D4,” in response to the IEEE 802.16 Working Group Letter Ballot #31: Announcement, IEEE 802.16-10/0011 |  |
| Abstract | E-MBS CFG Change Indication to support E-MBS on IEEE 802.16m Amendment Draft Standard |  |
| Purpose | To discuss and adopt the proposed text in the next revision of 802.16m draft |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Proposed Text on the E-MBS MAP in IEEE P802.16m (16.3.6)

Eunkyung Kim, Soojung Jung, Jaesun Cha, Sungcheol Chang, Hyun Lee, Chulsik Yoon

ETRI

# Instructions

IEEE 802.16m Amendment Draft Standard [1] describes the Enhanced Multicast Broadcast Service (E-MBS).

According to the IEEE 802.16m Amendment Draft Standard [1], AAI_E-MBS-CFG Change Indication in the E-MBS MAP has been proposed and it is an indication to indicate whether any change in the parameters of AAI_E-MBS-CFG MAC Control Message at its next transmission instance. If it is set (AAI_E-MBS-CFG Change Indication = 1), parameters in the AAI_E-MBS-CFG message will change and AMS shall decode the next AAI_E-MBS-CFG message. Otherwise, the AAI_E-MBS-CFG message does not change and the AMS resets the Allocation Lifetime as defined in the previous AAI_E-MBS-CFG message and may not decode the next transmission.

Even though Lifetime is useful information to decode the AAI_E-MBS-CFG message, it could be ambiguous information to reset the timer when the timer is expired. It is also some overhead some reason. Furthermore, the change indication in E-MBS MAP can cover the Lifetime and it is more efficient way to indicate to AMS.

S-SFH SP2 update indication in E-MBS MAP indicates whether zone related information will change or not. However, zone related information will be included in other SFH in addition to SP2.

Therefore, in this contribution, AAI_E-MBS-CFG message and E-MBS MAP are modified to support E-MBS efficiently.

# Limit of LIFETIME

Even though Lifetime is useful information to decode the AAI_E-MBS-CFG message, it could be ambiguous information to reset the timer when the timer is expired. It is also some overhead according to the following reason.

- Whenever LIFETIME expires, all AMS should receive the AAI_E-MBS-CFG message regardless it changes or not.

- AAI_E-MBS-CFG message is one of broadcast messages, which means it does not change frequently.

  - LIFETIME changes every time to transmit AAI_E-MBS-CFG message

  - Changed broadcast could be overhead in terms of coding operation in PHY layer.

  - AMS should receive the AAI_E-MBS-CFG every whenever the LIFETIME expires even though any parameter does not change in AAI_E-MBS-CFG.

# Usage of AAI_E-MBS-CFG Change Indication without LIFETIME

Figure 1 shows the example of AAI_E-MBS-CFG changing indication without LIFETIME resolving the problem of LIFETIME as described in Error: Reference source not found. As shown Figure 1, the ABS sets the AAI_E-MBS-CFG Change indication only if parameters in the AAI_E-MBS-CFG message change. Therefore, AMS shall decode the next AAI_E-MBS-CFG message. Otherwise, the AAI_E-MBS-CFG message does not change and the AMS maintains the parameter as defined in the previous AAI_E-MBS-CFG message and may not decode the next transmission.

Figure 1 – Usage of AAI_E-MBS-CFG change indication without LIFETIME

# Text Proposal for the 802.16m Amendment Draft Standard

Note:

The text in BLACK color: the existing text in the 802.16m Amendment Draft Standard

The text in RED color: the removal of existing 802.16m Amendment Draft Standard Text

The text in BLUE color: the new text added to the 802.16m Amendment Draft Standard Text

[-------------------------------------------------Start of Text Proposal---------------------------------------------------]

[Modify the Table 808-E-MBS Map Structure in 16.3.6.2.3 of 802.16m/D4]

Table 808 – E-MBS MAP Structure

| Syntax | Size (bits) | Notes |
| --- | --- | --- |
| E-MBS-MAP () { | - | - |
| AAI_E-MBS-CFG Change Indication | 1 | AAI_E-MBS-CFG Change Indication indicates any change in the parameters of AAI_E-MBS-CFG MAC Control Message at its next transmission instance. If it is set (AAI_E-MBS-CFG Change Indication = 1), parameters in the AAI_E-MBS-CFG message will change and AMS shall decode the next AAI_E-MBS-CFG message. Otherwise, the AAI_E-MBS-CFG message does not change and the AMS resets the Allocation Lifetime maintains the parameters as defined in the previous AAI_E-MBS-CFG message and may not decode the next transmission. |
| S-SFH SP2 update indicator Zone related information Change Indication | 1 | Indicates whether the changed S-SFH SP2 zone related information via SFH will be transmitted in this MSI will change before the next transmission instance. If this field is set to 1, the time offset which the changed S-SFH SP2 is transmitted, which zone related information via SFH will be transmitted at, will be included in this E-MBS MAP. |
| If (S-SFH SP2 update indicator Zone related information Change Indication ==1) { |  | AMS shall wake up every S-SFH SP2 transmission period until the AMS receives the changed S-SFH SP2. |
| S-SFH SP2 Zone related information transmission time offset | Variable | Indicates the superframe offset which the changed S-SFH SP2 is changed when zone related information via SFH will be transmitted. The size of this field depends on MSI MSI == 0b00: 1 bits MSI == 0b01: 2 bits MSI == 0b10: 3 bits MSI == 0b11: 4 bits |
| } |  |  |
| E-MBS_DATA_IE() | - | Refer to Table 841 |
| Padding | variable | Padding to reach byte boundary |
| } |  |  |

[Modify the Table 949-AAI_E-MBS-CFG message format in 16.9.3.1 of 802.16m/D4]

Table 949—AAI_E-MBS-CFG message Format

| Syntax | Size (bits) | Notes |
| --- | --- | --- |
| AAI-E-MBS-CFG_Message_Format() { |  |  |
| MAC Control Message Type | 8 | AAI_E-MBS-CFG |
| E-MBS_CFG_LIFETIME (m) | 4 | Indicates the duration of E-MBS_CFG_LIFETIME for which the E-MBS configuration information of the zone do not change. Duration of E-MBS_CFG_LIFETIME: 16(m+1) superframes |
| Zone_Allocation Bit-MAP | variable | Indicates the zone configuration. The size is as below. 20 bits for 20 MHz 9 bits for 10MHz 3 bits for 5 MHz |
| ZF | 1 | Zone Flag Bit. Indicates the use of the last zone. 0b0: Unicast 0b1: E-MBS |
| for (i = 0; i< Num_E-MBS_Zones; i++) { |  |  |
| E-MBS_Zone_ID | 7 | The EMBS_Zone_ID to which this EMBS MAP applies. |
| MSI Length (NMSI) | 2 | The length of an MSI in units of the number of superframes 0b00: 2 superframes, 40 ms (NMSI = 2) 0b01: 4 superframes, 80 ms (NMSI = 4) 0b10: 8 superframes, 160 ms (NMSI = 8) 0b11: 16 superframes, 320 ms (NMSI = 16) |
| E-MBS MAP Resource Index | 11 | Resource index includes location and allocation size. |
| E-MBS MAP ISizeOffsetISizeOffset | 5 | Offset used to compute burst size index |
| MSTID and FID Mappings List inclusion | 1 | Indicates whether MSTID and FID Mapping list is included or not 0b0: No Zone boundary or Zone boundary but no inter-zone service continuity in neighbor ABS 0b1: Zone boundary and inter-zone service continuity |
| if (MSTID and FID Mappings List inclusion ==1) { |  |  |
| MSTID and FID Mappings List | Variable | MSTID and FID Mappings List between serving and neighbor E-MBS Zones |
| } |  |  |
| } |  |  |
| } |  |  |

[-------------------------------------------------End of Text Proposal----------------------------------------------------]

# References

- IEEE P802.16m/D4, “DRAFT Amendment to IEEE Standard for Local and metropolitan area networks; Part 16: Air Interface for Broadband Wireless Access systems; Advanced Air Interface,” February 2010