| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text Ranging channel allocation to support UTDOA (16.3.9) |  |
| Date Submitted | 2010-03-05 |  |
| Source(s) | Xiangying Yang Xinrong Wang Etemad Kamran Yang-seok Choi Muthaiah Venkatachalam Intel | Xiangying.yang@intel.com |
| Re: | Commets on P802.16m/D4 for recirculation #31 |  |
| Abstract | The contribution provides the proposed text of SA-Preamble partitioning for ABS type to be included in IEEE 802.16m amendment document. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text Ranging channel allocation to support UTDOA (16.3.6)

Xiangying Yang, Xinrong Wang, Etemad Kamran, Yang-seok Choi, Muthaiah Venkatachalam

Intel

- Introduction

UTDOA is a simple, established LBS method as shown in Figure.

1

1

2

3

3

3

- Serving BS negotiates ranging opportunity with neighbor BSs, so that other BSs will block that resource for monitoring UL ranging transmission. This is feasible when all BSs are deployed on the same frequency, regardless of reuse pattern.

- Serving BS informs MS about dedicated ranging code/opportunity

- MS sends dedicated ranging code in the designated resource

In IEEE802.16e-2009, Section “6.3.10.4.1 Dedicated ranging and automatic adjustments” defines

- A dedicated ranging is an optional initial ranging that can be used to expedite the ranging process when the ranging is performed as an initial step of a certain procedure such as location determination, coordinated association during scanning, location update in idle mode, etc. For a dedicated ranging, BS will provide dedicated ranging information and allocate the dedicated ranging region at a pre-defined “rendezvous time,” in terms of relative frame number. The BS will also assign the following:

— A unique code number (from within the initial ranging codeset)

— A transmission opportunity within the allocated region (in terms of offset from the start of the region)

- the ranging region and ranging method defined could be used for the purpose of ranging using dedicated CDMA code and transmit opportunity assigned in the unsolicited RNG-RSP message (for location determination of MS) or in the MOB_SCN-RSP message(for coordinated association).

The following TLV parameters may be included in an unsolicited RNG-RSP message:

- Rendezvous time This is the offset, measured in units of frame duration, when the BS is expected to provide a non-contention-based ranging opportunity for the MS. The offset is calculated from the frame where RNG-RSP message is transmitted. The BS is expected to provide the non-contentionbased ranging opportunity at the frame specified by the rendezvous time parameter.

- CDMA code A unique code assigned to the MS, to be used for dedicated ranging. The code is from the initial ranging codeset.

- Transmission Opportunity Offset A unique transmission opportunity assigned to the MS, to be used for dedicated ranging, in units of symbol duration.

- References

[1] IEEE 802.16m-09/1902r3, “Proposed Text Proposal of SA-Preamble Partition in IEEE 802.16m/D1”

[2] IEEE 802.16m-07/0034r2, “802.16m System Requirements”

[3] IEEE 802.16m/D4, “Advanced Air Interface (Draft)”

[4] IEEE 802.16m-08/043, “Style guide for writing the IEEE 802.16m amendment”

- Text proposal for inclusion in the 802.16m amendment

Modify P802.16m/D4 as following

-------------------------------------------------------  Text Start  --------------------------------------------------------------

16.3.9.2.4 Ranging channel

[Note for Editor: Page 625, line 49]

The ranging channel for handover ranging can also be allocated by A-MAP based on ABS scheduling decision in any AAI subframe, except the AAI subframe that has already been used for a regular allocation. Dedicated ranging codes and opportunities may also be assigned in AAI_HO-CMD or AAI_RNG-ACK in an unsolicited fashion. The frequency resource allocation for dedicated ranging channel is the same as that for the regular ranging allocation, as specified in equation 279.

[Note for Editor: Insert the following fields in Table 678—Parameters for AAI_RNG-ACK]

| Name | Value | Usage |
| --- | --- | --- |
| Dedicated Ranging Assignment Flag | 1 | If set to 0, it indicates that no dedicated ranging assignment is provided. If set to 1, it indicates that dedicated ranging assignment is provided. |
| Dedicated ranging code | 5 | Indicates the dedicated ranging code. Shall be included when Dedicated Ranging Assignment Flag =1 |
| Subframe Index | 3 | Indicates the subframe index of the allocated ranging opportunity. Shall be included when Dedicated Ranging Assignment Flag=1 |

-------------------------------------------------------  Text End  ----------------------------------------------------------------