| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Cleanup texts for S-SFH SPx IEs (16.3.6.5.1.2) |  |
| Date Submitted | 2010-03-05 |  |
| Source(s) | Mihyun Lee, Hyunkyu Yu, Jaeweon Cho, Hokyu Choi Samsung Electronics Co., Ltd. | mihyun.mac.lee@samsung.com |
| Re: | IEEE 802.16-10/0011, “IEEE 802.16 Working Group Letter Ballot#31” Target topic: “IEEE P802.16m/D4, section 16.3.6.2.1”. |  |
| Abstract | The contribution provides the clean up text for S-SFH SPx IE |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m/D5 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Cleanup texts for S-SFH SPx IE (16.3.6.5.1.2)

Mihyun Lee, Hyunkyu Yu, Jaeweon Cho, Hokyu Choi

Samsung Electronics

- Introduction

This contribution provides the cleanup text to S-SFH IE in 16.3.6.5.1.2

1. Use the unified terminology for DL/UL control channels

- UL ACK/NACK channel  UL HARQ feedback channel

- DL ACK/NACK channel  DL HARQ Feedback A-MAP

2. Add the location of UL sounding symbol for WirelessMAN-OFDMA support

3. Re-arrangement of the fields: move common fields to the upper row.

4. Correct the reserved bits in case of WirelessMAN-OFDMA systems. Table 1 shows the bit calculation of S-SFH SP1 IE.

Table 1. S-SFH SP1 IE

| # | Channel | Parameters | 16m BS (Macro) |  |  | mixed mode |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | DRU | fields | 2k | 1k | 512 | fields | 1k | 512 |
| 1 | SP1 | MSB of superframe number | 8 | 8 | 8 | MSB of superframe number | 8 | 8 |
| 2 | SP1 | LSBs of 48 bit ABS MAC ID | 12 | 12 | 12 | LSBs of 48 bit ABS MAC ID | 12 | 12 |
| 3 | SP1 | Number of UL ACK/NACK channels per HARQ feedback region | 2 | 2 | 2 | Number of UL ACK/NACK channels per HARQ feedback region | 2 | 2 |
| 4 | SP1 | Number of DL ACK/NACK channels per HF-A-MAP region | 2 | 2 | 2 | Number of DL ACK/NACK channels per HF-A-MAP region | 2 | 2 |
| 5 | SP1 | Power control channel resource size indicator | 2 | 2 | 2 | Power control channel resource size indicator | 2 | 2 |
| 6 | SP1 | Primary frequency partition location | 1 | 1 | 1 | Primary frequency partition location | 1 | 1 |
| 7 | SP1 | A-A-MAP MCS selection | 1 | 1 | 1 | A-A-MAP MCS selection | 1 | 1 |
| 8 | SP1 | ABS EIRP | 7 | 7 | 7 | ABS EIRP | 7 | 7 |
| 9 | SP1 | Cell bar information | 1 | 1 | 1 | Cell bar information | 1 | 1 |
| 10 | SP1 | UL_N_MAX_ReTx | 1 | 1 | 1 | UL_N_MAX_ReTx | 1 | 1 |
| 11 | SP1 | DL_N_MAX_ReTx. | 1 | 1 | 1 | DL_N_MAX_ReTx. | 1 | 1 |
| 12 | SP1 |  | 1 | 1 | 1 |  | 1 | 1 |
| 13 | SP1 | DCASSB0 | 5 | 4 | 3 | DCASSB0 | 4 | 3 |
| 14 | SP1 | DCASMB0 | 5 | 4 | 3 | DCASMB0 | 4 | 3 |
| 15 | SP1 | DCASi | 3 | 2 | 1 | DCASi | 2 | 1 |
| 16 | SP1 | Frame configuration index | 6 | 6 | 6 | Frame configuration index | 6 | 6 |
| 17 | SP1 | WirelessMAN-OFDMA support | 1 | 1 | 1 | WirelessMAN-OFDMA support | 1 | 1 |
| 18 | SP1 | Allocation periodicity of ranging channel for non-synchronized AMSs | 2 | 2 | 2 | Allocation periodicity of ranging channel | 2 | 2 |
| 19 | SP1 | Subframe offset of ranging channel | 2 | 2 | 2 | Subframe offset of ranging channel | 2 | 2 |
| 20 | SP1 | Start code information of ranging channel for non-synchronized AMSs | 4 | 4 | 4 | Start code information of ranging channel | 4 | 4 |
| 21 | SP1 | Ranging preamble code partition information for non-synchronized AMSs | 4 | 4 | 4 | Ranging preamble code partition information | 4 | 4 |
| 22 | SP1 | Number of cyclic shifted ranging preamble codes per root index for non-synchronized AMSs | 2 | 2 | 2 | UL_Permbase | 7 | 7 |
| 23 | SP1 | Ranging channel formats for non-synchronized AMSs | 1 | 1 | 1 | location of UL sounding symbol | 2 | 2 |
| 24 | SP1 | UL sounding | 3 | 3 | 3 | Reserved | 7 | 4 |
| 25 | SP1 | UCASSB0 | 5 | 4 | 3 |  |  |  |
| 26 | SP1 | UCASMB0 | 5 | 4 | 3 |  |  |  |
| 27 | SP1 | UCASi | 3 | 2 | 1 |  |  |  |
| 28 | SP1 | reserved | TBD | TBD | TBD |  | TBD | TBD |
|  |  | Total | 90 | 84 | 78 |  | 84 | 78 |

Removed text

Added text

- References

[1] IEEE P802.16m/D4, “P802.16m DRAFT Amendment to IEEE Standard for Local and metropolitan area networks Part 16: Air Interface for Broadband Wireless Access Systems”, 2010-02-03

- Proposed Text Changes

[Remedy #1: Adopt the following modification inTable 811, page 448, section 16.3.6.5.1.2]

--------------------------------------------------Start of Remedy #1 ------------------------------------------------

| - S-SFH SP1 IE format |  |  |
| --- | --- | --- |
| Syntax | Size (bit) | Notes |
| S-SFH SP1 IE format () { |  |  |
| MSB of superframe number | 8 | Remaining bit of SFN except LSB of SFN in P-SFH |
| LSBs of 48 bit ABS MAC ID | 12 | Specifies the 12 least significant bits of ABS ID |
| Number of UL ACK/NACK HARQ feedback channels per HARQ feedback region | 2 | Describes LHFB in Error: Reference source not found. Channel numbers represented by the two bits (0, 1,2, 3) are as follows. For 5 MHz band, 6, 12, 18, 24 For 10 MHz band, 6, 12, 24, 30 For 20 MHz band, 12, 24, 48, 60 |
| Number of DL ACK/NACK channels HARQ Feedback A-MAPs per HF-A-MAP region | 2 | Channel numbers represented by the two bits (0, 1, 2, 3) are as follows. For 5 MHz band, 4, 8, 12, 16 For 10 MHz band, 8, 16, 24, 32 For 20 MHz band, 16, 32, 48, 64 |
| Power control channel resource size indicator | 2 |  |
| Primary frequency partition location | 1 | 0b0: Reuse 1 partition 0b1: Power-boosted reuse 3 partition |
| A-A-MAP MCS selection | 1 | 0b0: QPSK 1/2 and QPSK 1/4 can be used for assignment A-MAP in reuse-1 partition. QPSK 1/2 is used for assignment A-MAP in the power-boosted reuse 3 partition of FFR. 0b1: QPSK 1/2 and QPSK 1/8 can be used for assignment A-MAP in reuse-1 partition. QPSK 1/4 is used for assignment A-MAP in the power-boosted reuse 3 partition of FFR. |
| DCASSB0 | 5/4/3 | See Error: Reference source not found DL CRU/DRU allocation For 2048 FFT size, 5 bits For 1024 FFT size, 4 bits For 512 FFT size, 3 bits |
| DCASMB0 | 5/4/3 | See Error: Reference source not found DL CRU/DRU allocation For 2048 FFT size, 5 bits For 1024 FFT size, 4 bits For 512 FFT size, 3 bits |
| DCASi | 3/2/1 | See Error: Reference source not found DL CRU/DRU allocation For 2048 FFT size, 3 bits For 1024 FFT size, 2 bits For 512 FFT size, 1 bit |
| ABS EIRP | 7 | Signed in units of 1 dBm |
| Cell bar information | 1 | If Cell Bar bit = 1, this cell is not allowed for network entry or re-entry |
| UL_N_MAX_ReTx | 1 | Specifies the maximum retransmission number for UL HARQ 0b0: 4 0b1: 8 |
| DL_N_MAX_ReTx | 1 | Specifies the maximum retransmission number for DL HARQ 0b0: 4 0b1: 8 |
|  | 1 | Specifies the ABS's Rx processing time for UL HARQ for F = 8 in FDD or D + U = 8 in TDD 0b0: 3 AAI subframes 0b1: 4 AAI subframes |
| Frame configuration index | 6 | The mapping between value of this index and frame configuration is listed in Table Error: Reference source not found, Error: Reference source not found,and Error: Reference source not found |
| WirelessMAN-OFDMA support | 1 | Indicates whether frame configuration supports WirelessMAN-OFDMA systems or not 0b0 : No support of WirelessMAN-OFDMA with FDM-based UL PUSC zone 0b1 : Support of WirelessMAN-OFDMA with FDM-based UL PUSC zone |
| If (WirelessMAN-OFDMA support = 0b1){ |  |  |
| Allocation periodicity of ranging channel | 2 | Indicates the periodicity of ranging channel allocation according to the Error: Reference source not found. |
| Subframe offset of ranging channel | 2 | Indicates the subframe offset (OSF) of ranging channel allocation related to the Error: Reference source not found 898. The range of values is |
| Start code information of ranging channel | 4 | Indicates the kns which is the parameter for start of code group (S). The range of values is . |
| Ranging preamble code partition information | 4 | Indicates the number of initial, handover and periodic codes (N, O and M) according to the Error: Reference source not found. |
| UL_Permbase | 7 | Indicate UL_Permbase used in WirelessMAN-OFDMA system |
| UL sounding location | 2 | Indicates the index of UL subframe where UL sounding symbol is located 0b00: no sounding symbol 0b01: 0th UL subframe 0b10: 1st UL subframe 0b11: 2nd UL subframe |
| Reserved | 4/73 | For 1024 FFT size, 7 bits For 512 FFT size, 4 bits |
| } else if (WirelessMAN-OFDMA support = 0b0) { |  |  |
| if(Femtocell) { |  | for 16m Femtocell |
| Allocation periodicity of ranging channel for synchronized AMSs | 2 | Indicates the periodicity of ranging channel allocation according to the Error: Reference source not found. |
| Subframe offset of ranging channel | 2 | Indicates the subframe offset (OSF) of ranging channel allocation related to the Error: Reference source not found901. The range of values is |
| Start code information of ranging channel for synchronized AMSs | 4 | Indicates the which is the parameter controlling the start root index of ranging preamble codes (rs0). The range of values is |
| Ranging preamble code partition information | 4 | Indicates the number of initial, handover and periodic codes (N, O and M) according to the Error: Reference source not found. |
| Reserved | 3 |  |
| } else { |  |  |
| Allocation periodicity of ranging channel for non-synchronized AMSs | 2 | Indicates the periodicity of ranging channel allocation according to the Error: Reference source not found 898. |
| Subframe offset of ranging channel | 2 | Indicates the subframe offset (OSF) of ranging channel allocation related to the Error: Reference source not found898. The range of values is |
| Start code information of ranging channel for non-synchronized AMSs | 4 | Indicates the kns which is the parameter controlling the start root index of ranging preamble codes (rns0). for ranging channel format 0. for ranging channel format 1. The range of values is |
| Ranging preamble code partition information for non-synchronized AMSs | 4 | Indicates the number of initial and handover ranging preamble codes (NIN and NHO) according to the Error: Reference source not found897. |
| Number of cyclic shifted ranging preamble codes per root index for non-synchronized AMSs | 2 | Indicates the number of cyclic shifted codes per root index (Mns) for ranging preamble codes according to the Error: Reference source not found. |
| Ranging channel formats for non-synchronized AMSs | 1 | Indicates the ranging channel formats number of Error: Reference source not found |
| } |  |  |
| UCASSB0 | 5/4/3 | See Error: Reference source not foundUL CRU/DRU allocation For 2048 FFT size, 5 bits For 1024 FFT size, 4 bits For 512 FFT size, 3 bits |
| UCASMB0 | 5/4/3 | See Error: Reference source not found UL CRU/DRU allocation For 2048 FFT size, 5 bits For 1024 FFT size, 4 bits For 512 FFT size, 3 bits |
| UCASi | 3/2/1 | See Error: Reference source not found UL CRU/DRU allocation For 2048 FFT size, 3 bits For 1024 FFT size, 2 bits For 512 FFT size, 1 bits |
| } |  |  |
| Uplink AAI subframes for sounding | 3 | This value represents the number of uplink AAI subframes with sounding symbols. 0b000 – no sounding symbols 0b001 – 1 AAI subframe 0b010 – 2 AAI subframes 0b011 – 3 AAI subframes 0b100 – 4 AAI subframes 0b101-111 – reserved The sounding symbols shall be placed in AAI subframes in accordance to their type. First, sounding symbols shall be allocated in uplink AAI subframes of type 2 starting from the first in time AAI subframe of type 2. If the number of uplink AAI subframe of type 2 is less than the number of AAI subframes for sounding, sounding symbols shall be allocated in the AAI subframes of other types in the following order: type 1. For these types of uplink AAI subframes sounding symbols shall be allocated in the similar way as for type 2. Type 3 uplink AAI subframes are not used for sounding. |
| } |  |  |
| ABS EIRP | 7 | Signed in units of 1 dBm |
| Cell bar information | 1 | If Cell Bar bit = 1, this cell is not allowed for network entry or re-entry |
| UL_N_MAX_ReTx | 1 | Specifies the maximum retransmission number for UL HARQ 0b0: 4 0b1: 8 |
| DL_N_MAX_ReTx | 1 | Specifies the maximum retransmission number for DL HARQ 0b0: 4 0b1: 8 |
|  | 1 | Specifies the ABS's Rx processing time for UL HARQ for F= 8 in FDD or D + U = 8 in TDD 0b0: 3 AAI subframes 0b1: 4 AAI subframes |
| Reserved | TBD |  |
| } |  |  |

----------------------------------------------------------- End of Remedy #1 --------------------------------------------------------------------------

[Remedy #2: Adopt the following modification inTable 813, page 453, section 16.3.6.5.1.2]

--------------------------------------------------Start of Remedy #2 ------------------------------------------------

Table 813—S-SFH SP3 IE format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| … |  |  |
| FFR partition resource metrics | 0 or 4 or 8 | 0 bit: if FPCT=1. 4 bits or 8 bits: as As defined in Table 772 |
| UL BW REQ channel information | 3 | 0b000: First UL AAI subframe in every superframe 0b001: First UL AAI subframe in every frame 0b010: Every UL AAI subframes in every frame. If the number of UL AAI subfrmaes are more than four, UL BW REQ channels are allocated only to first four UL AAI subframes (i.e., maximum number of UL BW REQ channel in a frame is four) 0b011-0b111: reserved |
| … |  |  |

----------------------------------------------------------- End of Remedy #2 --------------------------------------------------------------------------