| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed modifications for support of 8 streams in DL MU-MIMO and UL CSM (16.3.7.1.3) |  |
| Date Submitted | 2010-03-05 |  |
| Source(s) | Sangheon Kim, Taeyong Kim, Hokyu Choi, and Heewon Kang Samsung Electronics | sangheon.kim@samsung.com ty33.kim@samsung.com |
| Re: | P802.16m/D4 LB31 |  |
| Abstract | This contribution proposes change for 8 stream support in DL MU-MIMO and UL CSM |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m Advanced Air Interface (Draft 4) |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed modification to support 8 streams of DL MU-MIMO and UL CSM
in IEEE P802.16m/D4 (16.3.7.1.3)

Sangheon Kim, Taeyong Kim, Hokyu Choi, and Heewon Kang

Samsung Electronics

- Introduction

This contribution proposes the modification for support of 8streams in Downlink MU-MIMO and in uplink CSM. For 8streams in DL MU-MIMO, Si field in DL basic assignment A-MAP IE needs to be modified. For 8stream CSM, some fields of UL basic assignment A-MAP IE should be changed and 8stream pilot pattern should be defined.

Proposed AWD text has been underlined in blue, AWD text that has been deleted has been marked in red and struck through and existing AWD text is shown in black.

- References

[1] DRAFT Amendment to IEEE Standard for Local and metropolitan area networks, P80216m/D4, Feb. 2010.

Text Proposal

<Modify the section 16.3.6.5.2.4.1 on line 39, page 461 as shown >

_______________________________Begin proposed text change #1 _________________________________

# 16.3.6.5.2.4.1 DL basic assignment A-MAP IE

Table 821 describes the fields in a DL Basic Assignment A-MAP IE used for resource assignment in the DL.

A-MAP IE Type: Defines the structure of the assignment A-MAP IE for the bits in the assignment A-MAP IE following the A-MAP IE type field. A-MAP IE Type distinguishes between assignment A-MAP IEs used for the UL/DL, and assignment A-MAP IEs used for resource allocation and control signaling. Additional A-MAP IE types are reserved for future use.

Mt : Number of streams in transmission. The DL pilot pattern with Mt streams shall be used in the allocated resource.

Si: Index used to identify the combination of the number of streams and the allocated pilot stream index in a transmission with MU-MIMO, and the modulation constellation of paired user in the case of 2 stream transmission. For Co-MIMO transmission to two users, the ABS shall indicate to one AMS a total number of

| Table 821 - DL basic assignment A-MAP IE |  |  |
| --- | --- | --- |
| Syntax | Size in bits | Description/Notes |
| DL_Basic_Assignment_A-MAP_IE() { |  |  |
| A-MAP IE Type | 4 | DL Basic Assignment A-MAP IE |
| ISizeOffset | 5 | Offset used to compute burst size index |
| MEF | 2 | MIMO encoder format 0b00: SFBC 0b01: Vertical encoding 0b10: Multi-layer encoding 0b11: CDR |
| if (MEF == 0b01){ |  | Parameters for vertical encoding |
| Mt | 3 | Number of streams in transmission Nt : Number of transmit antennas at the ABS 0b000: 1 stream 0b001: 2 streams 0b010: 3 streams 0b011: 4 streams 0b100: 5 streams 0b101: 6 streams 0b110: 7 streams 0b111: 8 streams |
| Reserved | 2 | Reserved bit |
| } else if(MEF == 0b10) { |  | Parameters for Multi-layer encoding |
| Si | 5 | Index used to identify the combination of the number of streams and the allocated pilot stream index in a transmission with MU-MIMO , and the modulation constellation of paired user in the case of 2 stream transmission 0b00000: TNS = 2 streams with PSI=stream0 and other modulation =QPSK 0b00001: TNS = 2 streams with PSI=stream0 and other modulation =16QAM 0b00010: TNS = 2 streams with PSI=stream0 and other modulation =64QAM 0b00011: TNS = 2 streams with PSI=stream0 and other modulation information not available 0b00100: TNS = 2 streams with PSI=stream1 and other modulation =QPSK 0b00101: TNS = 2 streams with PSI=stream1 and other modulation =16QAM 0b00110: TNS = 2 streams with PSI=stream1 and other modulation =64QAM 0b00111: TNS = 2 streams with PSI=stream1 and other modulation information not available 0b01000: TNS = 3 streams with PSI=stream0 0b01001: TNS = 3 streams with PSI=stream1 0b01010: TNS = 3 streams with PSI=stream2 0b01011: TNS = 4 streams with PSI=stream0 0b01100: TNS = 4 streams with PSI=stream1 0b01101: TNS = 4 streams with PSI=stream2 0b01110: TNS = 4 streams with PSI=stream3 0b01111: TNS = 3 streams with PSI=stream0 and stream1 0b10000: TNS = 4 streams with PSI=stream0 and stream1 0b10001: TNS = 4 streams with PSI=stream2 and stream3 0b10010~0b11111: n/a 0b10010: TNS = 8 with PSI=stream0 0b10011: TNS = 8 with PSI=stream1 0b10100: TNS = 8 with PSI=stream2 0b10101: TNS = 8 with PSI=stream3 0b10110: TNS = 8 with PSI=stream4 0b10111: TNS = 8 with PSI=stream5 0b11000: TNS = 8 with PSI=stream6 0b11001: TNS = 8 with PSI=stream7 0b11010: TNS = 8 with PSI=stream0 and stream1 0b11011: TNS = 8 with PSI=stream2 and stream3 0b11100: TNS = 8 with PSI=stream4 and stream5 0b11101: TNS = 8 with PSI=stream6 and stream7 0b11110: reserved 0b11111: reserved |
| } else { |  |  |
| Reserved | 5 | Reserved bit |
| } |  |  |
| Resource Index | 11 | 5 MHz: 0 in first 2 MSB bits + 9 bits for resource index 10 MHz: 11 bits for resource index 20 MHz: 11 bits for resource index Resource index includes location and allocation size. |
| Long TTI Indicator | 1 | Indicates number of AAI subframes spanned by the allocated resource. 0b0: 1 AAI subframe (default) 0b1: 4 DL AAI subframes for FDD or all DL AAI subframes for TDD |
| HFA | 3 | HARQ Feedback Allocation |
| AI_SN | 1 | HARQ identifier sequence number |
| ACID | 4 | HARQ channel identifier |
| SPID | 2 | HARQ subpacket identifier for HARQ IR 0b00: 0 0b01: 1 0b10: 2 0b11: 3 |
| CRV | 1 | Constellation Rearrangement Version 0b0: 0 0b1: 1 |
| Reserved | 1 | Reserved bits |
| } |  |  |

________________________________End proposed text change #1 _________________________________

<Modify the section 16.3.6.5.2.4.2 on line 34, page 467 as shown >

_______________________________Begin proposed text change #2 _________________________________

# 16.3.6.5.2.4.2 UL basic assignment A-MAP IE

Table 822 describes the fields in a UL Basic Assignment A-MAP IE used for resource assignment in the UL.

In the UL, the amount of LRUs that the ABS allocates to a specific AMS, starting in a specific AAI subrame, shall not be larger than the amount of LRUs in that AAI subframe over all used carriers. This limitation considers both bursts that are long TTI and bursts that are not.

TNS: Total number of streams in the LRU for CSM

SI: First pilot index for CSM.

PF: Precoding flag to indicate adaptive or non-adsptive precoding.

PMI: Precoding matrix index .

SU-PMI Indicator: Flag to indicate if both SU-MIMO and PMI are signaled

CSM: Flag to indicate if CSM is signaled

Table 813 UL basic assignment A-MAP IE*

| Syntax | Size in bits | Description/Notes |
| --- | --- | --- |
| UL_Basic_Assignment_A-MAP_IE() { | - | - |
| A-MAP IE Type | 4 | UL Basic Assignment A-MAP IE |
| ISizeOffset | 5 | Offset used to compute burst size index |
| Mt | 2 | Number of streams in transmission (Mt <= Nt ), up to 4 streams per AMS supported. Nt is the number of transmit antennas at the AMS. 0b00: 1 stream 0b01: 2 streams 0b10: 3 streams 0b11: 4 streams |
| SU-PMI Indicator | 1 | Flag to indicate if both SU-MIMO and PMI are signaled. 0b0: SU MIMO without PMI indication, or CSM without PMI indication 0b1: SU MIMO with PMI indication |
| if (SU-PMI Indicator == 0b0){ |  |  |
| CSM | 1 | Flag to indicate CSM 0b0: SU-MIMO 0b1: CSM |
| if (CSM == 0b0){ |  |  |
| MEF | 1 | MIMO encoder format - 0b0: SFBC - 0b1: Vertical encoding Non-adaptive precoding shall be used at the AMS with SFBC. Mt shall be ignored with SFBC. |
| if (MEF == 0b0){ |  |  |
| Reserved | 4 6 | Reserved bits |
| else if (MEF == 0b1){ |  |  |
| Mt | 2 | Number of streams in transmission (Mt <= Nt ), up to 8 streams per AMS supported. Nt is the number of transmit antennas at the AMS. 0b00: 1 stream 0b01: 2 streams 0b10: 3 streams 0b11: 4 streams |
| PF | 1 | Precoding flag for SU-MIMO when PMI is not signaled 0b0: Non-adaptive precoding 0b1: Adaptive precoding using the precoder of rank Mt of the AMS's choice |
| Reserved | 3 | Reserved bits |
| } |  |  |
| }else if (CSM == 0b1){ |  | Parameters for CSM |
| PF | 1 | Precoding flag for CSM when PMI is not signaled 0b0: Non-adaptive precoding 0b1: Adaptive precoding using the precoder of rank Mt of the AMS's choice |
| CSM Format | 6 | 0b000000: TNS = 2, Mt =1, SI =1 0b000001: TNS = 2, Mt =1, SI =2 0b000010: TNS = 3, Mt =1, SI =1 0b000011: TNS = 3, Mt =1, SI =2 0b000100: TNS = 3, Mt =1, SI =3 0b000101: TNS = 3, Mt =2, SI =1 0b000110: TNS = 3, Mt =2, SI =2 0b000111: TNS = 4, Mt =1, SI =1 0b001000: TNS = 4, Mt =1, SI =2 0b001001: TNS = 4, Mt =1, SI =3 0b001010: TNS = 4, Mt =1, SI =4 0b001011: TNS = 4, Mt =2, SI =1 0b001100: TNS = 4, Mt =2, SI =2 0b001101: TNS = 4, Mt =2, SI =3 0b001110: TNS = 4, Mt =3, SI =1 0b001111: TNS = 4, Mt =3, SI =2 0b010000: TNS = 8, Mt =1, SI =1 0b010001: TNS = 8, Mt =1, SI =2 0b010010: TNS = 8, Mt =1, SI =3 0b010011: TNS = 8, Mt =1, SI =4 0b010100: TNS = 8, Mt =1, SI =5 0b010101: TNS = 8, Mt =1, SI =6 0b010110: TNS = 8, Mt =1, SI =7 0b010111: TNS = 8, Mt =1, SI =8 0b011000: TNS = 8, Mt =2, SI =1 0b011001: TNS = 8, Mt =2, SI =2 0b011010: TNS = 8, Mt =2, SI =3 0b011011: TNS = 8, Mt =2, SI =4 0b011100: TNS = 8, Mt =2, SI =5 0b011101: TNS = 8, Mt =2, SI =6 0b011110: TNS = 8, Mt =2, SI =7 0b011111: TNS = 8, Mt =3, SI =1 0b100000: TNS = 8, Mt =3, SI =2 0b100001: TNS = 8, Mt =3, SI =3 0b100010: TNS = 8, Mt =3, SI =4 0b100011: TNS = 8, Mt =3, SI =5 0b100100: TNS = 8, Mt =3, SI =6 0b100101: TNS = 8, Mt =4, SI =1 0b100110: TNS = 8, Mt =4, SI =2 0b100111: TNS = 8, Mt =4, SI =3 0b101000: TNS = 8, Mt =4, SI =4 0b101001: TNS = 8, Mt =4, SI =5 0b101010~0b111111: reserved |
| TNS | 2 | - Total number of streams in the LRU for CSM 0b00: reserved 0b01: 2 streams 0b10: 3 streams 0b11: 4 streams |
| SI | 2 | First pilot index for CSM For TNS=2 0b00: SI=1 0b01: SI=2 0b10: N/A 0b11: N/A For TNS=3 0b00: SI=1 0b01: SI=2 0b10: SI=3 0b11: N/A For TNS=4 0b00: SI=1 0b01: SI=2 0b10: SI=3 0b11: SI=4 |
| } |  |  |
| }else if (SU-PMI Indicator == 0b1){ |  | Parameters for SU-MIMO with PMI ( adaptive precoding) |
| Mt | 2 | Number of streams in transmission (Mt <= Nt ), up to 4 streams per AMS supported. Nt is the number of transmit antennas at the AMS. 0b00: 1 stream 0b01: 2 streams 0b10: 3 streams 0b11: 4 streams |
| PMI | 6 | 4 bits PMI for Nt = 2, first 2 MSB set to 0 6 bits PMI for Nt = 4 |
| } |  |  |
| Resource Index | 11 | 5 MHz: 0 in first 2 MSB bits + 9 bits for resource index 10 MHz: 11 bits for resource index 20 MHz: 11 bits for resource index Resource index includes location and allocation size |
| Long TTI Indicator | 1 | Indicates number of subframes spanned by the allocated resource. 0b0: 1 subframe (default) 0b1: 4 UL subframes for FDD or all UL subframes for TDD If number of DL subframes, Dis less than number of UL subframes, U, Long TTI Indicator= 0b1 |
| HFA | 3 | HARQ Feedback Allocation |
| AI_SN | 1 | HARQ identifier sequence number |
| ACID | 4 | HARQ channel identifier |
| Reserved | 2 | Reserved bit |
| } | - | - |

- ________________________________End proposed text change #2 _________________________________

<Modify the section 16.3.7.1.3 on line 62, page 533 as shown >

_______________________________Begin proposed text change #3 _________________________________

# 16.3.7.1.3 Downlink MIMO modes

There are six MIMO transmission modes for unicast DL MIMO transmission as listed in Table 844.

| Table 844 - Downlink MIMO modes |  |  |  |
| --- | --- | --- | --- |
| Mode index | Description | MIMO encoding format (MEF) | MIMO precoding |
| Mode 0 | OL SU-MIMO (Tx diversity) | SFBC | non-adaptive |
| Mode 1 | OL SU-MIMO (SM) | VE | non-adaptive |
| Mode 2 | CL SU-MIMO (SM) | VE | adaptive |
| Mode 3 | OL MU-MIMO (SM) | ME | non-adaptive |
| Mode 4 | CL MU-MIMO (SM) | ME | adaptive |
| Mode 5 | OL SU-MIMO (Tx diversity) | CDR | non-adaptive |

.

The allowed values of the parameters for each DL MIMO mode are shown in Table 845.

| Table 845 - DL MIMO parameters |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Number of transmit antennas | STC rate per MIMO layer | Number of MIMO streams | Number of subcarriers | Number of MIMO layers |
|  | Nt | R | Mt | NF | L |
| MIMO mode 0 | 2 | 1 | 2 | 2 | 1 |
|  | 4 | 1 | 2 | 2 | 1 |
|  | 8 | 1 | 2 | 2 | 1 |
| MIMO mode 1 and MIMO mode 2 | 2 | 1 | 1 | 1 | 1 |
|  | 2 | 2 | 2 | 1 | 1 |
|  | 4 | 1 | 1 | 1 | 1 |
|  | 4 | 2 | 2 | 1 | 1 |
|  | 4 | 3 | 3 | 1 | 1 |
|  | 4 | 4 | 4 | 1 | 1 |
|  | 8 | 1 | 1 | 1 | 1 |
|  | 8 | 2 | 2 | 1 | 1 |
|  | 8 | 3 | 3 | 1 | 1 |
|  | 8 | 4 | 4 | 1 | 1 |
|  | 8 | 5 | 5 | 1 | 1 |
|  | 8 | 6 | 6 | 1 | 1 |
|  | 8 | 7 | 7 | 1 | 1 |
|  | 8 | 8 | 8 | 1 | 1 |
| MIMO mode 3 and MIMO mode 4 | 2 | 1 | 2 | 1 | 2 |
|  | 4 | 1 | 2 | 1 | 2 |
|  | 4 | 1 | 3 | 1 | 3 |
|  | 4 | 1 | 4 | 1 | 4 |
|  | 8 | 1 | 2 | 1 | 2 |
|  | 8 | 1 | 3 | 1 | 3 |
|  | 8 | 1 | 4 | 1 | 4 |
| MIMO mode 4 | 4 | 2 and 1* | 3 | 1 | 2 |
|  | 4 | 2 and 1** | 4 | 1 | 3 |
|  | 4 | 2 | 4 | 1 | 2 |
|  | 8 | 2 and 1* | 3 | 1 | 2 |
|  | 8 | 2 and 1** | 4 | 1 | 3 |
|  | 8 | 2 | 4 | 1 | 2 |
|  | 8 | 1 | 8 | 1 | 8 |
|  | 8 | 2 and 1*** | 8 | 1 | 7 |
|  | 8 | 2 and 1**** | 8 | 1 | 6 |
|  | 8 | 2 and 1***** | 8 | 1 | 5 |
|  | 8 | 2 | 8 | 1 | 4 |
| MIMO mode 5 | 2 | 1/2 | 1 | 2 | 1 |
|  | 4 | 1/2 | 1 | 2 | 1 |
|  | 8 | 1/2 | 1 | 2 | 1 |

* 2 streams to one AMS and 1 stream to another AMS, with 1 layer each.

** 2 streams to one AMS and 1 stream each to the other two AMSs, with 1 layer each.

*** 2 streams to one AMS and 1 stream each to the other six AMSs, with 1 layer each.

**** 2 streams each to two AMSs and 1 stream each to the other four AMSs, with 1 layer each.

***** 2 streams each to three AMSs and 1 stream each to the other two AMSs, with 1 layer each.

Mt refers to the number of MIMO streams transmitted to one AMS with MIMO modes 0, 1, 2 and 5.

Mt refers to the total number of MIMO streams transmitted to multiple AMS on the same RU with MIMO modes 3 and 4.

________________________________End proposed text change #3 _________________________________

<Modify the section 16.3.7.2.5.4 on line 18, page 543 as shown >

- _______________________________Begin proposed text change #4 _________________________________

| Table 850 - DL MIMO control parameters |  |  |  |
| --- | --- | --- | --- |
| Parameter | Description | Value | Notes |
| Mt | Number of MIMO streams in transmission | 1 to 8 | Number of MIMO streams in the transmission. Indicated in DL Basic Assignment A-MAP IE, DL Subband Assignment A-MAP IE, DL Persistent A-MAP IE |
| SI | Index of allocated pilot MIMO stream | 1 to 4 | SI shall be indicated if MEF is HE ME Indicated in DL Basic Assignment A-MAP IE, DL Subband Assignment A-MAP IE, DL Persistent A-MAP IE |
| MFM | MIMO feedback mode | Refer to Table 849 | To decide the feedback content and related MS processing Indicated in Feedback Allocation A-MAP IE, Feedback Polling A-MAP IE |
| MaxMt | Maximum number of MIMO streams | If Nt = 2(any MFM) 0b0:1 0b1:2 If Nt = 4(any MFM) 0b00:1 0b01:2 0b10:3 0b11:4 If Nt = 8(SU-MIMO MFM 0,1,2,3,4): 0b00:1 0b01:2 0b10:4 0b11:8 If Nt = 8(MU-MIMO MFM 5,6,7): 0b00:1 0b01:2 0b10:3 0b11:4 If Nt = 8(MU-MIMO MFM 6): 0b00:1 0b01:2 0b10:4 0b11:8 | If MFM indicates a SU feedback mode: the maximum STC rate scheduled for each user If MFM indicates a MU feedback mode: the maximum number of users total streams for all users scheduled on each RU Indicated in Feedback Allocation A-MAP IE, Feedback Polling A-MAP IE |
| CS indication | Codebook subset type for CL MIMO modes 2 and 4 | Base codebook or codebook subset | Depending on the MFM and CS indication, the MS shall feedback a PMI from the SU or MU base codebook, or from a subset of the SU or MU of the base codebook. Indicated in Feedback Allocation A-MAP IE, Feedback Polling A-MAP IE |

- ________________________________End proposed text change #4 _________________________________

- <Modify the section 16.3.8.4 on line 37, page 601 as shown >

- _______________________________Begin proposed text change #5 _________________________________

# 16.3.8.4 Pilot Structure

Uplink pilot is dedicated to each user and can be precoded or beamformed in the same way as the data subcarriers of the resource allocation. The pilot structure is defined for up to 4 transmission streams for SU-MIMO and up to 8 transmission streams for CSM.

The pilot pattern may support variable pilot boosting. When pilots are boosted, each data subcarrier should have the same Tx power across all OFDM symbols in a resource block.

Figure 547 and Figure 548 show the pilot structure for distributed LRUs where the number of streams is one or two, respectively. Figure 549 and Figure 550 contain the one and two-stream pilot patterns for the distributed PUSC LRU. Figure 551, Figure 552, Figure 553 and Figure 554 show the pilot structure for contiguous LRUs where the number of streams is one, two, three or four. Figure xxx shows the pilot structure for contiguous LRUs with 8 streams. Note that the pilot patterns for UL contiguous LRUs are same as in the downlink case.

________________________________End proposed text change #5 _________________________________

<Add the following figure after figure 554 on line 61, page 604 as shown >

- _______________________________Begin proposed text change #6 _________________________________

- Figure xxx – Pilot patterns for contiguous LRUs for 8 Tx streams

________________________________End proposed text change #6 _________________________________

- <Modify Table 924 in the section 16.3.10.1.3 on line 20, page 665 as shown >

- _______________________________Begin proposed text change #7 _________________________________

| Table 924 - UL MIMO parameters |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Number of transmit antennas | STC rate per MIMO layer | Number of MIMO streams | Number of subcarriers | Number of MIMO layers |
|  | Nt | R | Mt | NF | L |
| MIMO mode 0 | 2 | 1 | 2 | 2 | 1 |
|  | 4 | 1 | 2 | 2 | 1 |
| MIMO mode 1 | 1 | 1 | 1 | 1 | 1 |
| MIMO mode 1 and MIMO mode 2 | 2 | 1 | 1 | 1 | 1 |
|  | 2 | 2 | 2 | 1 | 1 |
|  | 4 | 1 | 1 | 1 | 1 |
|  | 4 | 2 | 2 | 1 | 1 |
|  | 4 | 3 | 3 | 1 | 1 |
|  | 4 | 4 | 4 | 1 | 1 |
| MIMO mode 3 and MIMO mode 4 | 1 | 1 | 1 | 1 | 1 |
|  | 2 | 1 | 1 | 1 | 1 |
|  | 2 | 2 | 2 | 1 | 1 |
|  | 4 | 1 | 1 | 1 | 1 |
|  | 4 | 2 | 2 | 1 | 1 |
|  | 4 | 3 | 3 | 1 | 1 |
| MIMO mode 4 | 4 | 4 | 4 | 1 | 1 |

- ________________________________End proposed text change #7 _________________________________