| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Contention-based Bandwidth Request Random Backoff Parameters (16.2.11, 16.2.12.8) |  |
| Date Submitted | 2010-03-16 |  |
| Source(s) | Prateek Kapadia, Abhay Karandikar, Srinadh B Shantidev Mohanty, Jie Hui, Shao-Cheng Wang | TICET (IIT Bombay) Intel |
| Re: |  |  |
| Abstract | MAC Management of Contention-based Bandwidth Request Random Backoff Parameters is specified |  |
| Purpose | Accept the revisions proposed in this contribution |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Random Backoff Parameters for Contention-based Bandwidth Request

Introduction

The Advanced WirelessMAN-OFDMA System defines three types of "QoS levels". Traffic Priority is used to distinguish priorities between service flows with the same QoS parameters. There are two priority classifications for contention-based bandwidth request - Access Class and Connection Priority. An access class is assigned to each flow, and a minimum access class is advertised by the ABS (in the AAI_SCD message) in order to restrict bandwidth request from flows with lower access class. Connection priority is meant to differentiate bandwidth request protocol parameters for different flows.

The bandwidth request protocol parameters differentiated by the connection priority are the initial backoff window size, maximum backoff window size, and window scaling factor. Another method for differentiating between bandwidth request protocol parameters is Differentiated Timer.

The present draft standard does not specify the method to assign connection priority parameters for a service flow.

The following solutions are proposed to clarify the text:

1. Define the bandwidth request protocol parameters for connection priority as service flow parameters.

2. Remove backoff timers from S-SFH SP3. (These are redundant, since the vanilla Truncated Binary Exponential Backoff can be implemented by assigning the same connection priority - with scaling factor equal to 2 - to all service flows.)

[Note to Editor: Please modify the text in section 16.2.12.8 and 16.3.6.5.1.2 as follows]

(Original Text: Black, New Text: Blue, Deleted Text: Red, strike-through)

------------------------- Start of text for NEW Section 16.2.12.8.x --------------------------

16.2.12.8.x Connection Priority Parameters

Connection priority parameters control the randomness in the exponential backoff timers chosen for each BR attempt. These parameters can be changed by the ABS depending on the system load and its distribution among the QoS requirements of all admitted flows.

The connection priority parameters are: initial backoff window size, maximum backoff window size, and scaling factor. The AMS shall follow an exponential backoff algorithm, i.e. after each failed BR attempt, the AMS scales its backoff window by the scaling factor and randomly selects a number within the new backoff window. The number thus selected represents a number of BR opportunities for backoff. The actual number of frames that are skipped by the MS before re-attempting BR is determined by a combination of the Differentiated timer (Section 16.2.12.8.3) and backoff.

------------------------- End of text for NEW Section 16.2.12.8.x --------------------------

------------ Start of text to add to Table 759 Service Flow Parameters, Section 16.2.12.8 -------------

| Initial Backoff Window Size | 4 | Window size expressed as a power of 2 |
| --- | --- | --- |
| Maximum Backoff Window Size | 4 | Window size expressed as a power of 2 |
| Backoff Scaling Factor | 4 | "0010" for Binary Exponential Backoff |

------------------------- End of text to add to Table 759, Section 16.2.12.8 --------------------------

------------------------- Start of text to delete from Table 813, Section 16.3.6.5.1.2 ------------------------

| Bandwidth request backoff start | 4 | Initial backoff window size for contention BRs, expressed as a power of 2. Values of n range 0-15 (the highest order bits shall be unused and set to 0) |
| --- | --- | --- |
| Bandwidth request backoff end | 4 | Final backoff window size for contention BRs, expressed as a power of 2. Values of n range 0- 15 |

-------------------------- End of text to delete from Table 813, Section 16.3.6.5.1.2 ------------------------