ost

| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Frequency partition for CINR calculation for scan reports (16.2.3) |  |
| Date Submitted | 2010-07-09 |  |
| Source(s) | Ariel Doubchak, Vladimir Yanover, Alvarion | vladimir.yanover@alvarion.com |
| Re: | Sponsor Ballot on the Draft Amendment (IEEE P802.16m/D6) |  |
| Abstract | Define the frequency partition for which CINR should be calculated for scan report purposes |  |
| Purpose | To be discussed and adopted by IEEE 802.16m TG |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Reuse factor for CINR calculation for scan report purposes (16.2.3)

Daniel Cohn, Ariel Doubchak
Alvarion

Jia Lin
Huawei

Problem Statement

802.16m/D6 supports AMS reporting neighbor ABS signal metrics to its serving ABS. These reports are typically used by the ABS to trigger ABS-initiated handover process, e.g. as a response to AMS changes in position (mobility-triggered handover) or to better distribute the load among ABSs (load-triggered handover). In particular CINR metrics allows the ABS to estimate spectral efficiency of the AMS if served by a neighbor ABS.

802.16m support flexible reuse schemes that can coexist in the same ABS (aka fractional frequency reuse or FFR) so there is a need for CINR reports from the AMS to reflect interference conditions in the different reuse schemes.

As an additional problem, 802.16m/D6 does not specify where the metric averaging parameter (as specified in 16.3.4.12) is taken from.

Suggested Solution

When CINR measurements are requested by the ABS, SCN-REP messages will include neighbor ABS CINR metrics reflecting conditions in the neighbor ABS for the frequency partition information requested by the ABS. If the SCN-REP is sent as response to a SCN-RSP message, the SCN-RSP will include the frequency partition information that should be used by the AMS to calculate neighbor ABS CINR. If the SCN-REP is sent as response to a trigger, the neighbor ABS CINR should be calculated according to FFR configuration in the AAI_NBR-ADV message.

Neighbor ABS FFR configuration in the AAI_NBR-ADV can be advertised as part of the SFH information for the neighbor ABS, or it can be explicitly advertised in the message fields (i.e. in case the serving ABS wishes to specify a different FFR configuration for CINR calculation purposes).

CINR calculations in different reuse schemes are specified in section 16.3.4.13 of 802.16m/D6..

Suggested Standard Modifications

1.- Modify table 692 in section 16.2.3.12 (AAI_NBR-ADV), page 115 after line 32, as follows:

| M/O | Attributes / Array of attributes | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
| ... | ... | ... | ... | ... |
| M | BSID | 48 |  | Shall be included for each neighbor BS |
| O | FFR partitioning configuration for neighbor ABS CINR calculations | 4 | DFPC of the FFR partitioning configuration, as defined in 16.3.5.2.3 | If not included, AMS will use FFR partitioning configuration advertised in the SFH information for the neighbor ABS in this message |
| O | FFR power configuration for neighbor ABS CINR calculations | 3 | fpPowerConfig, as defined in table 814 | If not included, AMS will use FFR power configuration advertised in the SFH information for the neighbor ABS in this message |
| M | Frequency partition for neighbor ABS CINR calculations | 2 | The frequency partition number according to whose FFR parameters CINR is calculated | Shall be included for each neighbor BS |

2.- Modify table 695 (AAI_SCN-RSP) in section 16.2.3.14 in page 125 after line 47, as follows:

| M/O | Attributes / Array of attributes |  | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- | --- |
| O | N_Recommended_SA_Preamble_Index |  | 4 | Number of preambles to be scanned [0..15] | Present if ABS requests AMS to scan SA preambles |
| O | preambleIndexArr[0..14] | SA Preamble Index | Variable ( 10 x N) | SA Preamble Index to be scanned | Present if N_Recommended_SA_Preamble_Index > 0 |
| O |  | Physical carrier index | 6 | As defined in AAI_Global-Config message | Shall be included when neighbor ABS is not included in AAI_NBR-ADV message and SCN-RSP is not sent as response to SCN-REQ which included this SA preamble index |
| O |  | Channel bandwidth of recommended ABS ID | 8 | Channel bandwidth of recommended ABS ID in units of 125 KHz. Field length is 1 byte. | Shall be included when neighbor ABS is not included in AAI_NBR-ADV message and SCN-RSP is not sent as response to SCN-REQ which included this SA preamble index |
| O |  | CP length | 2 | CP length of the recommended target ABS 00: 1/8 01: 1/16 10: 1/4 | Shall be included when neighbor ABS is not included in AAI_NBR-ADV message and SCN-RSP is not sent as response to SCN-REQ which included this SA preamble index |
| O |  | FFR partitioning configuration for neighbor ABS CINR calculation | 4 | DFPC value (0..15), as defined in 16.3.5.2.3 | Shall be included when recommended BS ID is not included in AAI_NBR-ADV |
| O |  | FFR power configuration for neighbor ABS CINR calculation | 3 | fpPowerConfig (0..8), as specified in table 814 | Shall be included when recommended BS ID is not included in AAI_NBR-ADV |
| O |  | Frequency partition for neighbor ABS CINR calculations | 2 | FP number (0..3) | Shall be included when recommended BS ID is not included in AAI_NBR-ADV |

3.- Modify table 696 (AAI_SCN-REP parameters) in section 16.2.3.15, after page 127 line 50, as follows:

| M/O | Attributes / Array of attributes |  |  |  | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- | --- | --- | --- |
| O |  |  | B) CINR mean |  | 8 | Indicates the CINR measured by the AMS from this SA preamble. The value shall be interpreted as a signed byte with units of 0.5 dB. When SCN-REP is sent as response to a SCN-RSP message, neighbor ABS CINR is calculated according to the frequency partition parameters (DFPC and fpPowerConfig) included in the SCN-RSP message if present. If these parameters are not present, or if the SCN-REP is sent as a result of a trigger, neighbor ABS CINR is calculated according to the frequency partition parameters included in the AAI_NBR-ADV message for this neighbor ABS. When this message is sent in response to trigger, the metric averaging parameter is taken from the relevant trigger. Otherwise, the metric averaging parameter is taken from the AAI_SCD message. | Present if AMS is asked to report CINR |
| ............................................................................................................................................................... |  |  |  |  |  |  |  |
| O |  |  |  | B) CINR mean | 8 | The ABS CINR Mean parameter indicates the CINR measured by the AMS from this carrier. The value shall be interpreted as a signed byte with units of 0.5 dB. When SCN-REP is sent as response to a SCN-RSP message, neighbor ABS CINR is calculated according to the frequency partition parameters (DFPC and fpPowerConfig) included in the SCN-RSP message if present. If these parameters are not present, or if the SCN-REP is sent as a result of a trigger, neighbor ABS CINR is calculated according to the frequency partition parameters included in the AAI_NBR-ADV message for this neighbor ABS. When this message is sent in response to trigger, the metric averaging parameter is taken from the relevant trigger. Otherwise, the metric averaging parameter is taken from the AAI_SCD message. | Present if AMS is asked to report CINR |