| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Paging Cycle and Paging Offset Values (16.2.3.21) |  |
| Date Submitted | 2010-04-30 |  |
| Source(s) | Ming-Hung Tao, Ying-Chuan Hsiao ITRI | Voice: +886-3-5914629 E-mail: MHTao@itri.org.tw |
| Re: | Category: LB31a / Area: Chapter 16.2.3 (MAC Control messages) |  |
| Abstract | This contribution proposes changes to the Section 16.2.3.21 of IEEE P802.16m/D5 |  |
| Purpose | To be discussed and adopted by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Paging Cycle and Paging Offset Values (16.2.3.22)

Ming-Hung Tao, Ying-Chuan Hsiao

ITRI

- Introduction

The current paging cycle values defined in D5 allows only 32 superframes (640 ms) as the maximum paging cycle length. However, in case of having huge numbers of Idle Mode AMSs in a paging group, the paging group may require a larger value of paging cycle to relax the paging message overheads. Similarly, if a paging group is concerning more about the power saving efficiency than paging latency, it may also require a larger paging cycle value.

Table 1 shows the impact of the paging cycle value on the AAI_PAG-ADV message overheads and paging latency. We can see that when there are 20000 Idle Mode AMSs in a paging group, 32-superframes paging cycle will result in almost 4% system throughput overheads which are used for broadcasting AAI_PAG-ADV messages. If we can apply the value of 64 superframes to the paging cycle, the paging message overheads can be reduced to occupy only 2.01 % system throughput.

The values for paging offset are still pending now in D5. If we can let the maximum value of paging cycle to be equal to 64 superframes, then the value for paging offset will be within the range from 0 to 63.

Table 1. Paging cycle values vs. AAI_PAG-ADV overheads and paging latency.

| Paging cycle | Number of Idle Mode AMSs in a paging group | Avg. paging latency | AAI_PAG-ADV throughput | System throughput (5MHz BW) | Ratio of AAI_PAG-ADV / system throughput |
| --- | --- | --- | --- | --- | --- |
| 32 superframes | 5000 AMSs | 320 ms | 136.6 kbps | 13000 kbps | 1.05 % |
|  | 10000 AMS | 320 ms | 261.6 kbps | 13000 kbps | 2.01 % |
|  | 20000 AMS | 320 ms | 511.6 kbps | 13000 kbps | 3.93 % |
| 64 superframes | 5000 AMS | 640 ms | 74.1 kbps | 13000 kbps | 0.57 % |
|  | 10000 AMS | 640 ms | 136.6 kbps | 13000 kbps | 1.05 % |
|  | 20000 AMS | 640 ms | 261.6 kbps | 13000 kbps | 2.01 % |
| 128 superframes | 5000 AMS | 1280 ms | 42.85 kbps | 13000 kbps | 0.32 % |
|  | 10000 AMS | 1280 ms | 74.1 kbps | 13000 kbps | 0.57 % |
|  | 20000 AMS | 1280 ms | 136.6 kbps | 13000 kbps | 1.05 % |

- Proposed Text for IEEE P802.16m/D5

============================ Start of Proposed Text ==================================

[Accept the following modifications marked in blue and red from line 17 on page 78.]

16.2.3.1 AAI_RNG-REQ

Table 680—AAI_RNG-REQ message Field Descriptions

| M/O | Attributes / Array of attributes | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
| O | Paging Controller ID | 48 | The Paging Controller ID which the AMS currently maintains in idle mode. | To be included when the AMS is attempting to perform reentry from Idle Mode or location update |
| O | Deregistration Identifier (DID) | 1012 | The ID which the AMS is assigned for idle mode and currently maintains. |  |
| O | PGID | 16 | The identification of the paging group that the AMS is previously belonging to. |  |
| O | Paging Cycle | 43 | PAGING_CYCLE applied to the AMS |  |
| O | Paging Offset | 46 | PAGING_OFFSET applied to the AMS |  |

[Accept the following modifications marked in blue and red from line 36 on page 81.]

16.2.3.2 AAI_RNG-RSP

Table 681—AAI_RNG-RSP message Field Descriptions

| M/O | Attributes / Array of attributes | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
| O | Paging Cycle | 43 | New Paging Cycle of the AMS 0b0000 - 0b1111 | It shall be included only if the Location Update Response is set to 0x00 (Success of Idle Mode Location Update) and the Paging Information has changed |
| O | Paging Group ID | 16 | New PGID of the AMS |  |
| O | Paging Offset | 46 | New PAGING_OFFSET of AMS |  |
| O | Paging Controller ID | 48 | The new Paging Controller ID which the AMS shall maintain in idle mode. | It shall be included only if the Location Update Response = 0x00 (Success of Idle Mode Location Update) and Paging Controller ID has changed |
| O | Deregistration Identifier (DID) | 1012 | The new DID which the AMS shall maintain in idle mod |  |

[Accept the following modifications marked in blue and red from line 52 on page 137.]

16.2.3.21 AAI_DREG-RSP message

Table 708—AAI_DREG-RSP message

| M/O | Attributes / Array of attributes | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
| O | Paging cycle | 43 | Used to indicate Paging cycle for the AMS 0x00: 8 superframes 0x01: 16 superframes 0x02: 32 superframes 0x03: 64 superframes 0x034-0x1507: reserved | Shall be present when Action code in AAI_DREG-RSP message is set to 0x05 or 0x07 |
| O | Paging offset | -6 | Used to indicate Paging offset for the AMS 0x00-0x63 | Shall be present when Action code in AAI_DREG-RSP message is set to 0x05 or 0x07 |
| O | Deregistration ID | 1012 | Used to indicate Deregistration ID used to identify the MS in idle mode 0…21012-1 | Shall be present when Action code in AAI_DREG-RSP message is set to 0x05 or 0x07 |

[Accept the following modifications marked in blue and red from line 37 on page 141.]

16.2.3.22 AAI_PAG-ADV (paging advertisement) Message

Table 709—AAI_PAG-ADV message

| M/O | Attributes / Array of attributes |  | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- | --- |
| M | Paged MS Info[1..32] | Deregistration Identifier | 1012 | Used to indicate Deregistration ID for the MS to be paged (Deregistration Identifier and Paging Cycle are used to identify each paged AMS) 0..210-1 | Shall be present if the Network Configuration indicates ABS is attached to the advanced network and unless Num_AMSs is 0 and |
| M |  | Paging Cycle | 43 | Used to indicate Paging cycle for the AMS to be paged 0x00: 8 superframes 0x01: 16 superframes 0x02: 32 superframes 0x03: 64 superframes 0x034-0x0F7: reserved | Shall be present unless Num_AMSs is 0 |
| M |  | Action Code | 1 | Used to indicate the purpose of the AAI_PAG-ADV message 0b0: perform network reentry 0b1: perform ranging for location update | Shall be present unless Num_AMSs is 0 |

============================== End of Proposed Text ================================

References

- IEEE P802.16m/D5. DRAFT Amendment to IEEE Standard for Local and metropolitan area networks—Part 16: Air Interface for Broadband Wireless Access Systems—Advanced Air Interface, Apr 2010.

- IEEE 802.16m-08/0034r2. IEEE 802.16m System Description Document, Sep 2009.

- IEEE 802.16m-07/002r9. IEEE 802.16m System Requirements Document, Sep 2009.