| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Text clean up for sleep operation in IEEE 802.16m (P802.16m/D5-16.2.17) |  |
| Date Submitted | 2010-04-30 |  |
| Source(s) | Lei Zhang ZTE Corporation Shantidev Mohanty Intel Corporation | zhang.lei13@zte.com.cn |
| Re: | Category: LB31a / Area: Chapter 16.2.17 (Sleep Mode) |  |
| Abstract | This contribution proposes changes to the Section 16.2.17 of IEEE P802.16m/D5 |  |
| Purpose | Discussion and adoption by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Text clean up for sleep operation in IEEE 802.16m (P802.16m/D5-16.2.17)

Lei Zhang

ZTE Corporation

Shantidev Mohanty

Intel Corporation

- Introduction

There are several editorial as well minor technical inconsistencies in the sleep mode operation text in Section 16.2.17. This contribution proposes changes to different parts of this section to clean up the text in this section.

- Proposed text

======================== Start of Proposed Text ========================

[Modify section 16.2.17 (from line 31 on page 371) of P802.16m/D5 as follows]

During the activation of Sleep Mode, the AMS is provided with a series of Sleep Cycles that typically consists of a Listening Window followed by a Sleep Window.

[Modify section 16.2.17 (from line 34 on page 371) of P802.16m/D5 as follows]

During Sleep Window in Sleep Mode, the ABS shall not transmit DL unicast MAC PDU to the AMS;, therefore the AMS may power down one or more physical operation components or perform other activities that do not require communication with the ABS.

[Modify section 16.2.17 (from line 47 on page 371) of P802.16m/D5 as follows]

If the AMS detects that the system configuration information it has is not up-to-date, then it shall not transmit in the Listening Window until it receives the up-to-date system configuration information.

[Modify section 16.2.17 (from line 58 on page 371) of P802.16m/D5 as follows]

For each AMS, the AMS and ABS shall keep up to 16 previously used negotiated Sleep Cycle settings and associated SCIDs.

[Modify section 16.2.17 (from line 61 on page 371) of P802.16m/D5 as follows]

If an AMS request to switch Sleep Cycle setting to the previous a pre-negotiated Sleep Cycle setting by the AAI_SLP-REQ including associated SCID, the ABS shall respond with the AAI_SLP-RSP including approval or other SCID.

[Modify section 16.2.17 (from line 64 on page 371) of P802.16m/D5 as follows]

Per AMS, a single Sleep Cycle setting is indicated by a Sleep Cycle ID (SCID) and it shall be applied across all the active connections of the AMS, and is indicated by the Sleep Cycle ID (SCID).

[Modify section 16.2.17 (from line 3 on page 372) of P802.16m/D5 as follows]

Sleep Mode activation/entry may be initiated either by an AMS or an ABS. When an AMS in the Active mode decides to enter the Sleep Mode, it shall negotiate parameters of the Sleep Cycle setting with the ABS. ABS makes the final decision regarding the AMS request and instructs the AMS to enter Sleep Mode.

[Modify section 16.2.17 (from line 13 on page 372) of P802.16m/D5 as follows]

The AMS may initiate the negotiation by sending an AAI_SLP-REQ message and shall expect an AAI_SLP-RSP message from the serving ABS in as response.

[Modify section 16.2.17 (from line 18 on page 372) of P802.16m/D5 as follows]

In the event that the ABS-initiated request (i.e., Unsolicited Sleep responseAAI_SLP-RSP) and an AMS-initiated request for Sleep Mode entry is being handled concurrently, the ABS-initiated request shall take precedence over the AMS-initiated Request.

[Modify section 16.2.17 (from line 33 on page 372) of P802.16m/D5 as follows]

The first sleep cycle on entry when entering to Sleep Mode from Active Mode contains only a sleep window which equals to the Initial Sleep Cycle.

[Modify section 16.2.17 (from line 36 on page 372) of P802.16m/D5 as follows]

A Sleep Cycle, except for the first initial cycle, shall begin with a Listening Window.

[Modify section 16.2.17 (from line 37 on page 372) of P802.16m/D5 as follows]

A Sleep Window shall follow the Listening Window and shall continue to the end of the current Sleep Cycle if the Listening Window does not occupy the full Sleep Cycle.

[Modify section 16.2.17 (from line 51 on page 372) of P802.16m/D5 as follows]

The length of the Listening Window length within a Sleep Cycle may be dynamically extended, as specified in section 16.2.16.2.3.2.

[Add the following item to line 20 on page 373 of P802.16m/D5]

•Next Sleep Cycle Flag (NSCF)

[Modify section 16.2.17 (from line 37 on page 373) of P802.16m/D5 as follows]

When NSCF is set to 0b10, current sleep cycle is reset to New Initial Sleep Cycle included in the AAI_SLP-RSP message, SCH or SCEH after positive traffic indicator. In this case, after receiving positive TRF_IND, the sleep cycle will be reset to some other value as defined by NSCF parameter.

[Modify section 16.2.17 (from line 48 on page 373) of P802.16m/D5 as follows]

Handling of MAC control signaling during sleep mode is specified in 16.2.16.2.6.

[Modify section 16.2.17 (from line 64 on page 373) of P802.16m/D5 as follows]

Listening window is measured in units of frames.

[Modify section 16.2.17 (from line 48 on page 378) of P802.16m/D5 as follows]

The length and phase frame location of the Sleep Cycles are not impacted by the interruption.

[Modify section 16.2.17 (from line 64 on page 378) of P802.16m/D5 as follows]

The occurrence of a scheduled Sleep Cycle interruption does not impact the length and phase frame location of the Sleep Cycle(s) to which it coincides.

======================== End of Proposed Text ========================

- References

[1] IEEE P802.16m/D5. DRAFT Amendment to IEEE Standard for Local and metropolitan area networks—Part 16: Air Interface for Broadband Wireless Access Systems—Advanced Air Interface, April 2010.