| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text for Broadcast A-A-MAP IE (Section 16.3.6.5) |  |
| Date Submitted | 2010-07-08 |  |
| Source(s) | Inuk Jung, Youngsoo Yuk, Kiseon Ryu and Jinsoo Choi LG Electronics | inuk.jung@lge.com, youngsoo.yuk@lge.com |
| Re: | Comment on IEEE P802.16m/D6 for Sponsor Ballot |  |
| Abstract | This contribution is to propose text changes for 16.3.6.5. |  |
| Purpose | Update P802.16m/D6 by discussion and adoption |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text for Broadcast A-A-MAP IE (Section 16.3.6.5)

Inuk Jung, Youngsoo Yuk, Kiseon Ryu and Jinsoo Choi

LG Electronics

- Introduction

The method for indexing the opportunity for normal ranging channel opportunity was determined at the session #67.

In case of dynamic ranging channel, the method is not clearly described in D6.

We have two methods for indicating the configuration of the dynamic ranging channel: by Broadcast Assignment A-MAP and by AAI_HO-CMD message.

In case of AAI_HO-CMD is used for assigning dynamic ranging channel, the opportunity index is included in the message. However Broadcast Assignment A-MAP does not contain the opportunity index. Because AAI_HO-CMD is unicast message, without the clear indication of the opportunity index in the B A A-MAP AMSs can have different interpretation for the opportunity index.

In order to removing the ambiguity, we propose to include the opportunity index of the allocated ranging channel to the Broadcast Assignment A-MAP.

- Proposed Text Changes

[Remedy: Change table 865, at line 50 in page 606, section 16.3.6.5.2.4.13, as the following]

------------------------------------------------Start proposed text----------------------------------------------------------------

Table 865 – Broadcast Assignment A-MAP IE*

| Syntax | Size in bits | Description/Notes |
| --- | --- | --- |
| Broadcast_Assignment_A-MAP_IE() { | - | - |
| A-MAP IE Type | 4 | Broadcast Assignment A-MAP IE |
| Function Index | 1 | 0: This IE carries broadcast assignment information 1: This IE carriers ranging channel allocation information |
| If (Function Index == 0){ |  |  |
| Burst Size | 6 | Burst size as indicated in the first 39 entries of Table 956 |
| Resource Index | 11 | 5 MHz: 0 in first 2 MSB bits + 9 bits for resource index 10 MHz: 11 bits for resource index 20 MHz: 11 bits for resource index Resource index includes location and allocation size |
| Long TTI Indicator | 1 | Indicates number of AAI subframes spanned by the allocated resource. 0b0: 1 AAI subframe (default) 0b1: 4 DL AAI subframes for FDD or all DL AAI subframes for TDD If number of Dl AAI subframes, D, is less than number of UL AAI subframes, U, Long TTI Indicator=0b1 |
| Transmission Format | 1 | 0b0: no time domain repetition 0b1: with time domain repetition |
| If (Transmission Format == 0b1){ |  |  |
| Repetition | 2 | 0b00: no more repetition of the same burst 0b01: the same burst shall be transmitted one more time 0b10: the same burst shall be transmitted two more times 0b11: the same burst shall be transmitted three more times |
| Reserved | 14 | Reserved bits |
| } else{ |  |  |
| Reserved | 16 | Reserved bits |
| } |  |  |
| } else if (Function Index == 1){ |  |  |
| Number of Ranging Opportunities (N) | 2 |  |
| For (i=0; i<N; i++){ |  |  |
| Subframe index | 3 |  |
| Ranging opportunity index | 2 | Opportunity index of the ranging channel |
| } |  |  |
| Reserved | TBD | Reserved bits |
| } |  |  |
| } |  |  |

-------------------------------------------------End proposed text----------------------------------------------------------------