| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed modifications for support of 8 streams in UL CSM in IEEE P802.16m/D6 |  |
| Date Submitted | 2010-07-08 |  |
| Source(s) | Sangheon Kim, Taeyoung Kim, Jaewon Cho, Heewon Kang Samsung Electronics | Sangheon.kim@samsung.com |
| Re: | Change Request on 802.16m Draft Amendment, IEEE P802.16m/D6 |  |
| Abstract | This contribution proposes change for 8 stream support for UL CSM in the 802.16m draft amendment, IEEE P802.16m/D6. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed modification to support 8 streams of UL CSM
in IEEE P802.16m/D6

Samsung Electronics

- Introduction

In UL MU-MIMO mode, the total number of streams is limited to four as UL SU-MIMO. The limitation of four streams is reasonable for SU-MIMO since it is not feasible for AMS to have five or more transmit antennas and the higher rank than 4 is not guaranteed in the small antenna spacing in AMS. However, the situation of CSM mode which mainly depends on the receiver structure of ABS is different from that of SU-MIMO mode. Because ABS can have eight receiver antennas and adopt an efficient receiving algorithm, we do not have to limit the number of streams to four for CSM. Therefore, this contribution proposes the modification for support of 8 streams in uplink CSM. For 8stream CSM, some fields of UL basic assignment A-MAP IE should be changed and 8stream pilot pattern should be additionally defined. In CSM mode in UL basic assignment A-MAP IE, the size of bits for CSM Format should be increased to 6bits from 4bits by using the reserved bits. In addition to the change in the basic assignment A-MAP IEs, we provide the corresponding changes in MIMO section. Regarding pilot pattern, we suggest the reuse of the downlink pilot pattern.

- Simulation Results

We evaluated the performance gain of the 8stream CSM over 4stream CSM by system level simulation. Simulation condition is shown in the following table. According to the proposed text change, TNS of 5, 6, and 7 cannot be indicated. Therefore, 8stream pilot pattern and overhead are assumed even for the cases that 5 to 7 AMSs are actually assigned.

- Table 1 Simulation Condition

| Parameter | Value |
| --- | --- |
| Channel configuration | Uma ( ITU-R M.2135) |
| Velocity | 3km |
| ISD | 500m |
| Cell configuration | 19 cells, 3 sector |
| System Bandwidth | 10 MHz |
| Frequency Reuse | 3/3/3 |
| Number of User | 20 |
| BS Rx Antenna | SLA 8 antennas with 4 Lamda spacing |
| MS Tx Antenna | 1 antenna |
| Permutation Type | SLRU |

Table 2 shows the simulation results. Although the pilot overhead is fixed for the cases of “TNS≥5”, we can see that 8stream CSM provides the 20% higher average throughput than 4stream CSM. Therefore, we recommend that 8stream CSM should be supported.

- Table 2 Simulation Results

|  | - Max 1stream | - Max 4 streams | - Max 8 streams |
| --- | --- | --- | --- |
| - Sector throughput(Mbps) | - 16.48 | - 46.65 | - 56.38 |
| - Edge throughput(Kbps) | - 581.27 | - 926.03 | - 896.89 |

- References

[1] DRAFT Amendment to IEEE Standard for Local and metropolitan area networks, P80216m/D6, May. 2010.

- Text Proposal

Proposed AWD text has been underlined in blue, AWD text that has been deleted has been marked in red and struck through and existing AWD text is shown in black.

<Modify the section 16.3.6.5.2.4.2 on line 50, page 562 as shown >

______________________________Start of the proposed text change #1 ______________________________

# 16.3.6.5.2.4.2 UL basic assignment A-MAP IE

Table 852 describes the fields in a UL Basic Assignment A-MAP IE used for resource assignment in the UL.

TNS: Total number of streams in the LRU for CSM

SI: First pilot index for CSM.

PF: Precoding flag to indicate adaptive or non-adaptive precoding.

Table 852 UL basic assignment A-MAP IE*

| Syntax | Size in bits | Description/Notes |
| --- | --- | --- |
| UL_Basic_Assignment_A-MAP_IE() { | - | - |
| A-MAP IE Type | 4 | UL Basic Assignment A-MAP IE |
| ISizeOffset | 5 | Offset used to compute burst size index |
| PMI Indicator | 1 | Flag to indicate if PMI is signaled explicitly 0b0: PMI not signaled explicitly 0b1: PMI signaled explicitly |
| CSM | 1 | Flag to indicate CSM 0b0: SU-MIMO 0b1: CSM |
| if (PMI Indicator == 0b0){ |  | PMI not signaled |
| if (CSM == 0b0){ |  | SU-MIMO (Non- adaptive / Adaptive precoding without PMI) |
| MEF | 1 | MIMO encoder format - 0b0: SFBC - 0b1: Vertical encoding Non-adaptive precoding shall be used at the AMS with SFBC. Mt shall be ignored with SFBC. |
| if (MEF == 0b1){ |  |  |
| Mt | 2 | Number of streams in transmission Mt <= Nt, up to 4 streams per AMS supported Nt: Number of transmit antennas at the AMS 0b00: 1 stream 0b01: 2 streams 0b10: 3 streams 0b11: 4 streams |
| PF | 1 | Precoding flag for SU-MIMO when PMI is not signaled 0b0: Non-adaptive precoding 0b1: Adaptive precoding using the precoder of rank Mt of the AMS's choice |
| Reserved | 4 | Reserved bits |
| }else{ |  |  |
| Reserved | 7 | Reserved bits |
| } |  |  |
| else if(CSM == 0b1) { |  | CSM (Non- adaptive/ Adaptive precoding without PMI) |
| CSM Format | 46 | For CSM modes the following combinations apply for upto 48 streams 0b0000: TNS = 2, Mt=1, SI =1 0b0001: TNS = 2, Mt =1, SI =2 0b0010: TNS = 3, Mt =1, SI =1 0b0011: TNS = 3, Mt =1, SI =2 0b0100: TNS = 3, Mt =1, SI =3 0b0101: TNS = 3, Mt =2, SI =1 0b0110: TNS = 3, Mt =2, SI =2 0b0111: TNS = 4, Mt =1, SI =1 0b1000: TNS = 4, Mt =1, SI =2 0b1001: TNS = 4, Mt =1, SI =3 0b1010: TNS = 4, Mt =1, SI =4 0b1011: TNS = 4, Mt =2, SI =1 0b1100: TNS = 4, Mt =2, SI =2 0b1101: TNS = 4, Mt =2, SI =3 0b1110: TNS = 4, Mt =3, SI =1 0b1111: TNS = 4, Mt =3, SI =2 0b000000: TNS = 2, Mt=1, SI =1 0b000001: TNS = 2, Mt =1, SI =2 0b000010: TNS = 3, Mt =1, SI =1 0b000011: TNS = 3, Mt =1, SI =2 0b000100: TNS = 3, Mt =1, SI =3 0b000101: TNS = 3, Mt =2, SI =1 0b000110: TNS = 3, Mt =2, SI =2 0b000111: TNS = 4, Mt =1, SI =1 0b001000: TNS = 4, Mt =1, SI =2 0b001001: TNS = 4, Mt =1, SI =3 0b001010: TNS = 4, Mt =1, SI =4 0b001011: TNS = 4, Mt =2, SI =1 0b001100: TNS = 4, Mt =2, SI =2 0b001101: TNS = 4, Mt =2, SI =3 0b001110: TNS = 4, Mt =3, SI =1 0b001111: TNS = 4, Mt =3, SI =2 0b010000: TNS = 8, Mt =1, SI =1 0b010001: TNS = 8, Mt =1, SI =2 0b010010: TNS = 8, Mt =1, SI =3 0b010011: TNS = 8, Mt =1, SI =4 0b010100: TNS = 8, Mt =1, SI =5 0b010101: TNS = 8, Mt =1, SI =6 0b010110: TNS = 8, Mt =1, SI =7 0b010111: TNS = 8, Mt =1, SI =8 0b011000: TNS = 8, Mt =2, SI =1 0b011001: TNS = 8, Mt =2, SI =2 0b011010: TNS = 8, Mt =2, SI =3 0b011011: TNS = 8, Mt =2, SI =4 0b011100: TNS = 8, Mt =2, SI =5 0b011101: TNS = 8, Mt =2, SI =6 0b011110: TNS = 8, Mt =2, SI =7 0b011111: TNS = 8, Mt =3, SI =1 0b100000: TNS = 8, Mt =3, SI =2 0b100001: TNS = 8, Mt =3, SI =3 0b100010: TNS = 8, Mt =3, SI =4 0b100011: TNS = 8, Mt =3, SI =5 0b100100: TNS = 8, Mt =3, SI =6 0b100101: TNS = 8, Mt =4, SI =1 0b100110: TNS = 8, Mt =4, SI =2 0b100111: TNS = 8, Mt =4, SI =3 0b101000: TNS = 8, Mt =4, SI =4 0b101001: TNS = 8, Mt =4, SI =5 0b101010~0b111111: reserved |
| PF | 1 | Precoding flag for CSM when PMI is not signaled 0b0: Non-adaptive precoding 0b1: Adaptive precoding using the precoder of rank Mt of the AMS's choice |
| Reserved | 31 | Reserved bits |
| } |  |  |
| } else { |  | PMI signaled |
| if (CSM == 0b0){ |  | SU-MIMO with PMI |
| Mt | 2 | Number of streams in transmission Mt <= Nt , up to 4 streams per AMS supported Nt: Number of transmit antennas at the AMS. 0b00: 1 stream 0b01: 2 streams 0b10: 3 streams 0b11: 4 streams |
| PMI | 6 | 4 bits PMI for Nt = 2, first 2 MSB set to 0 6 bits PMI for Nt = 4 |
| } else { |  | CSM with PMI |
| CSM Format | 4 | For CSM, modes the following combinations apply for upto 4 streams 0b0000: TNS = 2, Mt=1, SI =1 0b0001: TNS = 2, Mt =1, SI =2 0b0010: TNS = 3, Mt =1, SI =1 0b0011: TNS = 3, Mt =1, SI =2 0b0100: TNS = 3, Mt =1, SI =3 0b0101: TNS = 3, Mt =2, SI =1 0b0110: TNS = 3, Mt =2, SI =2 0b0111: TNS = 4, Mt =1, SI =1 0b1000: TNS = 4, Mt =1, SI =2 0b1001: TNS = 4, Mt =1, SI =3 0b1010: TNS = 4, Mt =1, SI =4 0b1011: TNS = 4, Mt =2, SI =1 0b1100: TNS = 4, Mt =2, SI =2 0b1101: TNS = 4, Mt =2, SI =3 0b1110: TNS = 4, Mt =3, SI =1 0b1111: TNS = 4, Mt =3, SI =2 |
| PMI | 4 | 4 bits PMI |
| } | - | - |
| } | - | - |
| Resource Index | 11 | 5 MHz: 0 in first 2 MSB bits + 9 bits for resource index 10 MHz: 11 bits for resource index 20 MHz: 11 bits for resource index Resource index includes location and allocation size |
| Long TTI Indicator | 1 | Indicates number of subframes spanned by the allocated resource. 0b0: 1 AAI subframe (default) 0b1: 4 UL AAI subframes for FDD or all UL AAI subframes for TDD If number of DL AAI subframes, D, is less than number of UL AAI subframes, U, Long TTI Indicator= 0b1 |
| HFA | 3 | HARQ Feedback Allocation |
| AI_SN | 1 | HARQ identifier sequence number |
| ACID | 4 | HARQ channel identifier |
| Reserved | 1 | Reserved bits |
| } | - | - |

______________________________End of the proposed text change #1_______________________________

- <Modify the section 16.3.8.4 on line 43, page 686 as shown >

- _______________________________Start of the proposed text change #2 ______________________________

# 16.3.8.4 Pilot Structure

Uplink pilot is dedicated to each user and can be precoded or beamformed in the same way as the data subcarriers of the resource allocation. The pilot structure is defined for up to 4 transmission streams for SU-MIMO and up to 8 transmission streams for CSM.

The pilot pattern may support variable pilot boosting. When pilots are boosted, each data subcarrier should have the same Tx power across all OFDM symbols in a resource block.

Figure 547 and Figure 548 show the pilot structure for distributed LRUs where the number of streams is one or two, respectively. Figure 549 and Figure 550 contain the one and two-stream pilot patterns for the distributed PUSC LRU. Figure 551, Figure 552, Figure 553 and Figure 554 show the pilot structure for contiguous LRUs where the number of streams is one, two, three or four. Figure xxx shows the pilot structure for contiguous LRUs with 8 streams. Note that the pilot patterns for UL contiguous LRUs are same as in the downlink case.

______________________________End of the proposed text change #2_______________________________

<Add the following figure after figure 554, on line 61, page 689 as shown >

- _______________________________Start of the proposed text change #3 ______________________________

- Figure xxx – Pilot patterns for contiguous LRUs for 8 Tx streams

______________________________End of the proposed text change #3_______________________________

- <Modify Table 952 in the section 16.3.10.1.3 on line 7, page 753 as shown >

- _______________________________Start of the proposed text change #4 ______________________________

- Table 952 - UL MIMO parameters

|  | Number of transmit antennas | STC rate per MIMO layer | Number of MIMO streams | Number of subcarriers | Number of MIMO layers |
| --- | --- | --- | --- | --- | --- |
|  | Nt | R | Mt | NF | L |
| MIMO mode 0 | 2 | 1 | 2 | 2 | 1 |
|  | 4 | 1 | 2 | 2 | 1 |
| MIMO mode 1 | 1 | 1 | 1 | 1 | 1 |
| MIMO mode 1 and MIMO mode 2 | 2 | 1 | 1 | 1 | 1 |
|  | 2 | 2 | 2 | 1 | 1 |
|  | 4 | 1 | 1 | 1 | 1 |
|  | 4 | 2 | 2 | 1 | 1 |
|  | 4 | 3 | 3 | 1 | 1 |
|  | 4 | 4 | 4 | 1 | 1 |
| MIMO mode 3 and MIMO mode 4 | 1 | 1 | 1 | 1 | 1 |
|  | 2 | 1 | 1 | 1 | 1 |
|  | 2 | 2 | 2 | 1 | 1 |
|  | 4 | 1 | 1 | 1 | 1 |
|  | 4 | 2 | 2 | 1 | 1 |
|  | 4 | 3 | 3 | 1 | 1 |
|  | 4 | 4 | 4 | 1 | 1 |

______________________________End of the proposed text change #4_______________________________

- <Modify Table 954 in the section 16.3.10.2.5.1 on line 20, page 755 as shown >

- _______________________________Start of the proposed text change #5 ______________________________

- Table 954 - UL MIMO control parameters

| Parameter | Description | Value | Notes |
| --- | --- | --- | --- |
| MEF | MIMO Encoding Format | SFBC Vertical encoding | MIMO encoding format |
| CSM | Collaborative Spatial Multiplexing | Disabled or enabled | SU MIMO if CSM is disabled MU-MIMO if CSM is enabled |
| Mt | Number of MIMO streams | 1 to 4 | Number of MIMO streams in the AMS transmission. |
| TNS | Total number of MIMO streams in the LRU | 1 to 4 8 | Enabled when CSM is enabled. Indication of the total number of MIMO streams in the LRU |
| SI | First pilot index | 1 to 4 8 | Enabled when CSM is enabled. 1 bit for 2Tx, 2bit for 4Tx |
| PF | Precoding flag | non adaptive precoding or adaptive codebook precoding | Cannot be applied to AMS with 1 transmit antenna |
| PMI Indicator | PMI indicator | 0b0: the AMS shall use the precoder of rank Mt of its choice 0b1: the inidicated PMI or rank Mt shall be used by the AMS for precoding | This field is relevant only when PF indicates adaptive codebook precoding. PMI indication = 0b0 may be used in TDD When PMI indication = 0b1, the ABS selects the precoder to use at the AMS. |
| PMI | Precoding matrix index in the UL base codebook | 0 to 9 when Nt =2 0 to 63 when Nt = 4 | Enabled when PF indicates adaptive codebook precoding, and PMI indication = 0b1. |

- ______________________________End of the proposed text change #5_______________________________