| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Modification to AAI_DSx-REQ (Section 16.2.3.47) |  |
| Date Submitted | 2010-09-08 |  |
| Source(s) | Hyunkyu Yu, Seho Kim, Sangheon Kim, Hyunjeong Kang Samsung Electronics Co., Ltd. | hk.yu@samsung.com |
| Re: | Comments on IEEE P802.16m/D8 for IEEE 802.16 WG SB_16m. |  |
| Abstract | The contribution proposes the modification to AAI_DSx-REQ. |  |
| Purpose | To be discussed and adopted by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Modification to AAI_DSx-REQ (Section 16.2.3.47)

Hyunkyu Yu, Seho Kim, Sangheon Kim, Hyunjeong Kang

Samsung Electronics

- Introduction

- HARQ channel mapping:

In uplink transmission, bandwidth allocation for the delay-sensitive traffic (e.g. VoIP) needs to be explicitly mapped to a flow. Otherwise, other service flows may use those resources, degrading the performance of that flow (VoIP). To guarantee the performance, we suggest mapping HARQ channels to a flow in DSx message. Note that persistent allocation cannot be always used for VoIP transmission, so we recommend adopting HARQ channel mapping concept, which is more general than just mapping persistent allocation to a flow.

- Padding PDU indicator:

Generally, if an ABS assigns the resource to AMS though an AMS has no data burst to be transmitted, the AMS is supposed to transmit the special data burst called padding PDU which consists of the first byte of 0xF0 and the remaining bytes of 0x00 according to padding rule of section 16.2.4.7. However, in terms of power saving problem in an AMS, the option that an AMS does not transmit the padding PDU can be considered. Therefore, this contribution proposes to add the parameter indicating whether or not an AMS transmits the padding PDU.

- Proposed Text Changes

[Remedy-1: Add the following text, at line 23 in page 377, section 16.2.12.8, as]

------------------------------------------------Start proposed text----------------------------------------------------------------

16.2.12.8 Service Flow/Convergence sublayer parameters

…

| … | … | … |
| --- | --- | --- |
| Authorization Token | variable | Authorization token that is used for authorizing the QoS for one or service flows generated by AMS-initiated higher level service flow creation or modification procedures. Refer to 11.13.32 |
| Number of HARQ Channels | 5 | Indicates the number of HARQ channels which are assigned to the flow. |
| fullNumHarqChArr[0…256] |  |  |
| A) HARQ Channel Mapping | 5 | Indicates the index (ACID) of each HARQ channel. |
| Padding PDU Indicator | 1 | Indicate whether or not an AMS transmits the padding PDU when an ABS assigned resource to the AMS but the AMS has no data to transmit. 0: AMS transmits padding PDU. 1: AMS may not transmit padding PDU. |
| ROHC Parameter Payload | variable | Refer to Table 784 |
| … | … | … |

-------------------------------------------------End proposed text----------------------------------------------------------------

[Remedy-2: Add the following text, at line 6 in page 382, after section 16.2.12.8.6, as]

------------------------------------------------Start proposed text----------------------------------------------------------------

16.2.12.8.7 HARQ channel mapping

This parameter specifies the set of HARQ channels for carrying data on this service flow acceptable to the sender of this message. The data of other flows shall not be carried through this set of HARQ channels. The absence of this parameter in any of AAI_DSA-REQ or AAI_DSC-REQ relevant for the flow means that this flow may use all HARQ channels except the HARQ channels already mapped to other flows through this parameter.

-------------------------------------------------End proposed text----------------------------------------------------------------

[Remedy-3: Add the following text, at line 31 in page 204, section 16.2.3.47.1, as]

------------------------------------------------Start proposed text----------------------------------------------------------------

16.2.3.47.1 AAI-DSA-REQ

…

- Predefined BR index parameters: Predefined BR index parameters define the mapping from predefined BR index(es) to BR action and BR size, which is used in 3-step Bandwidth Request procedure, and are only included in ABS initiated DSC-REQ. They are determined based on the QoS parameters of the service flow in the AAI-DSx messages. If BR action is 0b10 (BR), ABS shall assign different BR index to service flows whose UL Grant Scheduling Type is different and shall assign different BR index to different service flows whose UL Scheduling Type is same but BR size is different.

- Padding PDU Indicator: Indicates whether or not an AMS transmits the padding PDU when the AMS has no data to transmit though the resource assigned by ABS, where the padding PDU means the data burst of which the first byte is 0xF0 and the remaining bytes are 0x00. The absence of this parameter in any of AAI_DSA-REQ or AAI_DSC-REQ relevant for the flow indicates that the AMS shall transmit the padding PDU when the AMS has no data to transmit though the resource assigned by ABS.

…

Table 737 – AAI-DSA-REQ

| M/O | Attributes/Array of attributes | Size (bits) |  | Value/Notes | Conditions |  |
| --- | --- | --- | --- | --- | --- | --- |
| … | … | … |  | … | … |  |
| O | N.6) Vendor-specific PHS parameters | variable |  | Compound |  |  |
| O | Number of HARQ Channels | 8 | Indicates the number of HARQ channels which are assigned to the flow. |  |  |  |
| O | fullNumHarqChArr[0…256] |  |  |  |  |  |
| O | A) HARQ Channel Mapping | 5 | Indicates the index (ACID) of each HARQ channel. |  |  |  |
| O | Padding PDU Indicator | 1 | Indicate whether or not an AMS transmits the padding PDU when an ABS assigned resource to the AMS but the AMS has no data to transmit. 0: AMS transmits padding PDU. 1: AMS may not transmit padding PDU. |  | Present for UL |  |
| O | ROHC Parameter | variable |  |  |  |  |
| … | … | … | … |  | … |  |

-------------------------------------------------End proposed text----------------------------------------------------------------

[Remedy-4: Add the following text, at line 43 in page 220, section 16.2.3.47.4, as]

------------------------------------------------Start proposed text----------------------------------------------------------------

16.2.3.47.4. AAI_DSC-REQ

…

- Predefined BR index parameters: Predefined BR index parameters define the mapping from predefined BR index(es) to BR action and BR size, which is used in 3-step Bandwidth Request procedure, and are only included in ABS initiated DSC-REQ. They are determined based on the QoS parameters of the service flow in the AAI-DSx messages. If BR action is 0b10 (BR), ABS shall assign different BR index to service flows whose UL Grant Scheduling Type is different and shall assign different BR index to different service flows whose UL Scheduling Type is same but BR size is different.

- Indicates whether or not an AMS transmits the padding PDU when the AMS has no data to transmit though the resource assigned by ABS, where the padding PDU means the data burst of which the first byte is 0xF0 and the remaining bytes are 0x00. The absence of this parameter in any of AAI_DSA-REQ or AAI_DSC-REQ relevant for the flow indicates that the AMS shall transmit the padding PDU when the AMS has no data to transmit though the resource assigned by ABS.

When an ABS updates E-MBS service flow, the following parameters may be included in the AAI-DSC-REQ message.

…

Table 740 – AAI_DSC-REQ

| M/O | Attributes/Array of attributes | Size (bits) | Value/Notes | Conditions |
| --- | --- | --- | --- | --- |
| … | … | … | … | … |
| O | N.6) Vendor-specific PHS parameters | variable | Compound |  |
| O | Number of HARQ Channels | 5 | Indicates the number of HARQ channels which are assigned to the flow. |  |
| O | fullNumHarqChArr[0…256] |  |  |  |
| O | A) HARQ Channel Mapping | 5 | Indicates the index (ACID) of each HARQ channel. |  |
| O | Padding PDU Indicator | 1 | Indicate whether or not an AMS transmits the padding PDU when an ABS assigned resource to the AMS but the AMS has no data to transmit. 0: AMS transmits padding PDU. 1: AMS may not transmit padding PDU. | Present for UL |
| O | ROHC Parameter | variable |  |  |
| … | … | … | … | … |

-------------------------------------------------End proposed text----------------------------------------------------------------