| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | AAI_Global-Config Message and ASN.1 Clean Up (16.2.3.57) |  |
| Date Submitted | 2010-07-09 |  |
| Source(s) | I-Kang Fu, Kelvin Chou, Yih-Shen Chen, Alex Hsu, Paul Cheng MediaTek Inc. | IK.Fu@mediatek.com |
| Re: | P802.16m/D6 |  |
| Abstract | The existing ASN.1 code needs further clean up |  |
| Purpose | For TGm discussion and adoption into P802.16m/D7 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

AAI_Global-Config Message and ASN.1 Clean Up

I-Kang Fu, Kelvin Chou, Yih-Shen Chen, Alex Hsu, Paul Cheng

MediaTek Inc.

- Text Proposal

---------------------------------------------------------Start of the Text-----------------------------------------------------------

[Adopt the following text modifications from line#1, page#223 of P802.16m/D6]

16.2.3.57 AAI_Global-Config (global carrier configuration) Message

The AAI_Global-Config message provides the carrier information for all available carriers in the network. The ABS transmits the AAI_Global-Config message to an AMS after transmitting transmiting the AAI_REG-RSP message.

Table 762—AAI_Global-Config MAC Control Message Format

| Field | Size (bit) | Description |
| --- | --- | --- |
| MAC Control Message Type | 8 |  |
| Global carrier configuration change count | 3 | Indicates the value of Global carrier configuration change count of the network, will be incremented each time the contents of this message change. Value range is 1-7 |
| Number of Carrier Groups | 4 | Groups of contiguous carriers In case of FDD, only DL carrier groups are included. |
| For (i=0; i< Number of Carrier Groups; i++){ |  |  |
| Duplexing Mode | 1 | “0” for TDD “1” for FDD |
| Multi-Carrier Configuration Index Across the Network | 6 | Index associated to Table 803 |
| Start Frequency Assignment Index | 818 | Center frequency Frequency Assignment Index (in unit of 50kHz) of the first carrier located at lowest frequency position in the i-th carrier group |
| Number of Carriers | 63 |  |
| For(j=0;j<Number of Carriers; j++) { |  |  |
| Physical Carrier Index | 63 | Index of the physical carrier |
| If(Duplexing_Mode==1){ |  |  |
| Carrier Type | 1 | Indicates the type of this physical carrier. 0b0: Fully configured carrier 0b1: Partially configured carrier |
| } |  |  |
| Reference Carrier Indicator | 1 | 0b0: Non-reference carrier 0b1: Reference carrier of current carrier group |
| } |  |  |
| } |  |  |

[Adopt the following text modifications from line#38, page#896 of P802.16m/D6]

-- AAI_Global-Config message

-- Global Configuration Message

AAI-Global-Config ::= SEQUENCE {

}

---------------------------------------------------------Start of the Text-----------------------------------------------------------