| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Paging Offset Clean-Up (16.2.3.1) |  |
| Date Submitted | 2010-08-12 |  |
| Source(s) | Ming-Hung Tao, Ying-Chuan Hsiao, Yu-Min Chiu ITRI | Voice : + 886 3-5914629 E-mail: mhtao@itri.org.tw |
| Re: | Category: SB_16m / Area: Chapter 16.2.3 (MAC Control messages) |  |
| Abstract | The contribution cleans-up the length of Paging Offset. |  |
| Purpose | To be discussed and adopted by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Paging Offset Clean-Up (16.2.3.1)

Ming-Hung Tao, Ying-Chuan Hsiao, Yu-Min Chiu

ITRI

- Introduction

The lengths of paging offset are inconsistent in D7. We suggest making the length of paging offset equal to 9 bits through the whole document.

- Proposed Text for IEEE P802.16m/D7

[Remedy 1: Modify Paging offset in line 53 of page 74 in Table 680 as follows:]

| O | Paging offset | 612 | PAGING_OFFSET applied to the AMS | To be included when the AMS is attempting to perform reentry from Idle Mode or location update. In the legacy network mode, DID shall not be included, and the ABS performs a mapping for paging parameters between AAI air interface and legacy network interface. |
| --- | --- | --- | --- | --- |

[Remedy 2: Modify Paging offset in line 28 of page 77 in Table 681 as follows:]

| O | AMSID*/MAC address | 48 | A required parameter when the AMS confirms if the AAI_RNG-RSP is a response to the AAI_RNG-REQ message which the AMS sent. In the legacy network mode, the actual MAC address of the AMS is used instead. | It shall be included in the AAI_RNG-RSP message in response to the AAI_RNG-REQ message, which is not CMAC protected, when the AMS is not assigned its STID/DID yet. |
| --- | --- | --- | --- | --- |

[Remedy 3: Modify Paging offset in line 39 of page 78 in Table 681 as follows:]

| O | New Paging Cycle | 4 | New Paging Cycle of the AMS 0b0000 - 0b1111 | It shall be included only if the Location Update Response is set to 0x00 (Success of Idle Mode Location Update) and the Paging Information parameter has changed. The DID may be included only when the Network Configuration indicates ABS is attached to the advanced network. |
| --- | --- | --- | --- | --- |
| O | New Paging Group ID | 16 | New PGID of the AMS |  |
| O | New Paging Offset | 12 | New paging offset of AMS Determines the superframe within the paging cycle from which the paging listening interval starts. Shall be smaller than Paging cycle value. |  |
| O | Deregistration Identifier (DID) | 12 | The new DID which the AMS shall maintain in idle mode. |  |
| O | New Paging Controller ID | 48 | The new Paging Controller ID which the AMS shall maintain in idle mode. | It shall be included only if the Location Update Response = 0x00 (Success of Idle Mode Location Update) and Paging Controller ID has changed. The DID is included only when the Network Configuration indicates ABS is attached to the advanced network. |
| O | Deregistration Identifier (DID) | 12 | The new DID which the AMS shall maintain in idle mode. |  |

============================== End of Proposed Text ====================================

- References

- IEEE P802.16m/D7. DRAFT Amendment to IEEE Standard for Local and metropolitan area networks—Part 16: Air Interface for Broadband Wireless Access Systems—Advanced Air Interface, July 2010.

- IEEE 802.16m-08/0034r2. IEEE 802.16m System Description Document, Sep 2009.

- IEEE 802.16m-07/002r9. IEEE 802.16m System Requirements Document, Sep 2009.