| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text for paging carrier selection for E-MBS AMS in multi-carrier system (16.2.8.2) |  |
| Date Submitted | 2010-08-12 |  |
| Source(s) | Jeongki Kim, Youngsoo Yuk, Kiseon Ryu, and Jin Sam Kwak LG Electronics | E-mail: jeongki.kim@lge.com, youngsoo.yuk@lge.com, kiseon.ryu@lge.com |
| Re: | Sponsor Ballot comment on IEEE p802.16m D7 |  |
| Abstract | This contribution proposes the texts related to paging method for E-MBS AMS in multi-carrier system |  |
| Purpose | For E-MBS DG discussion |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text for paging carrier selection for E-MBS AMS in multi-carrier system (16.2.8.2)

Jeongki Kim, Youngsoo Yuk, Kiseon Ryu, and Jin Sam Kwak

- LG Electronics

- Jaejeong Shim

- Samsung

# Introduction

As described in the Section 16.2.8.2.10.2 in 16m Draft [1], the paging carrier of an AMS is selected as follows:

- The AAI_PAG-ADV message for an AMS shall be transmitted in only one of the carriers. An idle mode AMS determines the carrier index for monitoring the paging message within its paging-listening interval using the following equations:

  - Paging carrier index = DID modulo N – (5)

- For an E-MBS AMS the AAI_PAG-ADV message may be transmitted in the same carrier as the dedicated carrier for E-MBS. In this case, the AMS does not use equation (5).

For E-MBS AMS AAI_PAG-ADV needs to be transmitted in the carrier for Mixed E-MBS carrier with unicast service as well as E-MBS dedicated carrier.

An AMS may perform Location Update process to acquire its preferred carrier for E-MBS.

- References

[1] IEEE P802.16m/D5, “DRAFT Amendment to IEEE Standard for Local and metropolitan area networks”

# Text proposal for the IEEE 802.16m amendment

--------------Proposed Text Start------------------------------------------------------------------------

[Remedy #1: Modify the Section 16.2.8.2.10.2 as follows, on line 34,  page 318]

16.2.8.2.10.2 Idle mode

A multicarrier supporting AMS in idle state follows the same procedures defined in 16.2.18.

In a multi-carrier system the PGID_Info message is transmitted in all fully configured carriers. The AAI_PAG-ADV message for an AMS shall be transmitted in only one of the carriers. An idle mode AMS determines the carrier index for monitoring the paging message within its paging-listening interval using the following equations:

Paging carrier index = DID modulo N (5)

The value of N is the number of carriers per PGID per frequency band in an ABS that are used for transmitting paging message for idle mode AMS(s). The Paging Carrier Index corresponds to the ascending order of physical carrier index of the carriers which are used for transmitting paging message in the ABS. Paging carrier indication bit is used to specify if a carrier is a paging carrier or not. When the Paging carrier indication =1, then the corresponding carrier is a paging carrier. When the Paging carrier indication =0, then the corresponding carrier is not a paging carrier. The Paging carrier indication of different carriers is included in the PGID_Info message and may be included in the AAI_NBR-ADV message or the AAI_MC-ADV message. The paging carrier(s) included in a PGID_Info message is located on the same frequency band that the PGID_Info message is sent over. The ABS composes a PGID_Info message including the paging carrier(s) that belongs to a frequency band and indicated by Paging carrier indication bitmap and the ABS transmits the PGID_Info message over the fully configured carriers in the same frequency band. The AMS monitors the PGID_Info message at the same frequency band where it has most recently woken. When the AMS uses the Paging carrier indication included in the AAI_NBR-ADV message or the AAI_MC-ADV message, the AMS shall decides the value of N as the number of carriers per PGID in the frequency band where the AMS has most recently woken.

When an idle mode AMS moves to a new ABS and determines that the SFH change count of the new ABS is same as the information it received through the AAI_NBR-ADV message, then the AMS may use the paging carrier indication received through AAI_NBR-ADV message if this paging carrier indication is included in the AAI_NBR-ADV message.

The AAI_PAG-ADV message is transmitted in one or more of the frames starting from the second sub-frame in the super-frame.

For an E-MBS AMS the AAI_PAG-ADV message may shall be transmitted in the same carrier as the dedicated carrier for which E-MBS is provided. In this case, the AMS does not use equation (5).

An AMS may perform Location Update process to acquire its preferred carrier for the idle mode support when AMS cannot find the paging carrier. In this case, Ranging Purpose Indication = 0b0001 in AAI_RNG-REQ message is used for indicating paging carrier update in location update process, and the AMS may include Paging Carrier Update parameter in AAI_RNG-REQ message to inform its preferred carrier. The preferred carrier shall be continuously used in the subsequent paging until the preferred paging carrier is updated during the location update procedure.

--------------Proposed Text #1 ---------------------------------------------------------------------------