| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification Text on BR Header Transmission (Section 16.2.2.1) |  |
| Date Submitted | 2010-10-25 |  |
| Source(s) | Hyunkyu Yu, Jeongho Park, Sangheon Kim Samsung Electronics Co., Ltd. | hk.yu@samsung.com |
| Re: | Comments on IEEE P802.16m/D9 for IEEE 802.16 WG SB_16m. |  |
| Abstract | The contribution clarifies the text on BR header transmission. |  |
| Purpose | To be discussed and adopted by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clarification Text on BR Header Transmission (Section 16.2.2.1)

Hyunkyu Yu, Jeongho Park, Sangheon Kim

Samsung Electronics

- Introduction

- Bandwidth request header transmission:

In coverage loss detection section (16.2.26.1), there is a text specifying “Upon each expiration of the active_ABS_timer, to check whether an AMS is still alive in active mode, the ABS shall grant UL burst to the AMS and the AMS shall transmit a MAC PDU with data or if no data pending to be transmitted, then just with a BR header with BR size set to 0 on the UL grant.”.

Because the AMS does not know whether this UL grant is for coverage loss detection or not, for general cases (regardless of coverage detection), the AMS cannot help transmitting a BR header with BR size set to 0 in case of no data pending to be transmitted.

Thus, we’re proposing to add general description in BR header section to clarify the operation above: “In active mode, if the ABS grants UL bandwidth to the AMS and the AMS has no pending data to be transmitted, the AMS shall transmit a MAC PDU with a BR header with BR size set to 0 on the UL grant.”

- Proposed Text Changes

[Remedy-1: Add the following text, at line 7 in page 63, section 16.2.2.1.3.2, as]

------------------------------------------------Start proposed text----------------------------------------------------------------

16.2.2.1.3.2 Bandwidth Request (BR) without STID Header

BR without STID Header is sent through dedicated UL resource assigned to the AMS. BR type shall be included to indicate whether BR type is incremental or aggregate. BR format without STID header is defined in Table 658.

In active mode, if the ABS grants UL bandwidth to the AMS and the AMS has no pending data to be transmitted, the AMS shall transmit a MAC PDU with a BR header with BR size set to 0 on the UL grant.

-------------------------------------------------End proposed text----------------------------------------------------------------