| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposal for Text Addition for Context Retention Timer in AAI-RNG-REQ and AAI-DREG-REQ/RSP messages and ASN.1 code (16.2.3.1) |  |
| Date Submitted | 2010-12-18 |  |
| Source(s) | Danny Moses, Shantidev Mohanty, Shaocheng Wang, Joey Chou Intel | Voice : +972 3 920 5714 E-mail: danny.moses@intel.com |
| Re: | IEEE Standards Sponsor Ballot Recirculation-4 for P802.16m/D10. |  |
| Abstract | The contribution proposes Text Addition for Context Retention Timer in AAI-RNG-REQ and AAI-DREG-REQ/RSP messages and ASN.1 code (16.2.3.1), and some other DCR-related editorial fixes |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposal for Text Addition for Context Retention Timer in AAI-RNG-REQ and AAI-DREG-REQ/RSP messages and ASN.1 code (16.2.3.1)

Danny Moses, Shantidev Mohanty, Shaocheng Wang, Joey Chou

Intel

- Introduction

This contribution proposes some fixes to DCR-related sections.

The current 72 bit format of CRID comprises a 48 bit field uniquely identifying the DCR Controller in the ASN (like the Paging Controller’s ID) and a 24 bit field that uniquely identifies the MS which may be in DCR mode. This enables up to 16 Million Mobile Stations that may have an allocated CRID associated with one DCR controller.

Since mobile stations may be in DCR mode for a long time (days), it is not sure 16 million is enough.

Therefore, we would like to recommend to set the size of the CRID to 80 bits as follows:

•	48 bits for the DCR controller ID

•	32 bit for the MS unique number within the ASN

In addition, the Context Retention Timer is defined as a global value (without a value) in section 16.11, but no specifics on the size of the timer (hours, days, etc…) defined. We recommend to add a parameter that specifies the duration of the timer and be added to several MAC management messages, as detailed in the following.

•	AAI-RNG-REQ – when the AMS requests to perform either DCR entry (from Idle mode) or when the AMS requests to extend the timer

•	AAI-RNG-RSP – to indicate the CRT as specified by the network (it may except the AMS’s time period request or force a different value)

•	AAI-DREG-REQ – when the AMS requests to enter DCR mode from regular state

•	AAI-DREG-RSP – to indicate the CRT as specified by the network (just like the AAI-RNG-RSP)

- Suggested changes in the 802.16m/D10

The following is the proposed change in the 802.16m/D10. Note that the new text is marked with blue and underline; the deleted text are marked with red and strikethrough.

Suggested change #1:

Modify text in section 16.2.1.2.4 in page 94, line 37 with the following text:

The network shall assign a unique 72 80-bit CRID to each AMS during network entry.

Suggested change #2:

Modify table 678 in page 126, line 12 with the following text:

Table 678—AAI-RNG-REQ message Field Descriptions

| Field | Size (bits) | Value/Description | Conditions |
| --- | --- | --- | --- |
| SMS | Variable | Short Message content up to 140 bytes | May be presented when there is SMS content to be sent |
| } else if (Ranging Purpose Indication == 0b0100) { |  | // DCR mode extension |  |
| CRID | 7280 | AMS identifier which the AMS has been assigned for coverage loss or DCR mode and are currently maintained |  |
| CRT | 2 | New requested Context Retention Time value: 00 – 12 hours 01 – one day 10 – 3 days 11 - undefined |  |
| If (CMAC indicator == 0b1) { |  |  |  |

Suggested change #3:

Modify table 678 in page 126, line 38 with the following text:

Table 678—AAI-RNG-REQ message Field Descriptions

| Field | Size (bits) | Value/Description | Conditions |
| --- | --- | --- | --- |
| Initial Offset for uplink power control (OffsetInitial) | 5 | The bit size represents power level ranging from – 15dB (0x00) to 16 dB (0x1F) with 1 dB step. The value is determined by AMS after successful initial ranging process |  |
| } else if (Ranging Purpose Indication == 0b0111) { |  | // DCR mode from Idle mode |  |
| CRID | 80 | AMS identifier which the AMS has been assigned for coverage loss or DCR mode and are currently maintained |  |
| CRT | 2 | New requested Context Retention Time value: 00 – 12 hours 01 – one day 10 – 3 days 11 - undefined |  |
| } else if (Ranging Purpose Indication == 0b1000) { |  | // Reentry from DCR mode, coverage loss or detection of different ABS restart count |  |
| CRID | 7280 | AMS identifier which the AMS has been assigned for coverage loss or DCR mode and are currently maintained |  |
| If (CMAC indicator == 0b1){ |  |  |  |

Suggested change #4:

Modify table 679 in page 131, line 5 with the following text:

Table 679—AAI-RNG-RSP message Field Descriptions

| Field | Size (bits) | Value/Description | Conditions |
| --- | --- | --- | --- |
| CRID | 7280 | The AMS identifier newly assigned by the network for coverage loss or DCR mode | Shall be included when the AMS is attempting to perform network reentry from coverage loss or DCR mode or to perform network reentry/location update/ zone-switch with assigning a new CRID in response to AAI- RNG-REQ or it shall be included when the network assigns a new CRID unsolicitedly to the AMS. |
| CRT | 2 | New assigned Context Retention Time value: 00 – 12 hours 01 – one day 10 – 3 days 11 - undefined | Shall be included when the AAI-RNG-RSP is in response to a request to extend the DCR time or when a new CRID is assigned to the AMS. |
| Emergency Service FID | 4 | A FID number assigned within the Transport FID numbers 0010-1111 according to table 652 | Shall be included when this message is sent in response to an AAI- RNG-REQ with Ranging Purpose Indication set to code 0b0101 |

Suggested change #5:

Modify table 679 in page 131, line 35 with the following text:

Table 679—AAI-RNG-RSP message Field Descriptions

| Field | Size (bits) | Value/Description | Conditions |
| --- | --- | --- | --- |
| Location Update Response | 4 | 0x0= Success of Location Update 0x1= Failure of Location Update 0x2= Reserved 0x3= Success of location update and DL traffic pending 0x4-0x7: Reserved 0x4 0x8= Allow AMS’s DCR mode initiation request or DCR mode extension request 0x5 0x9= Reject AMS’s DCR mode initiation request of DCR mode extension request 0xA-0xF: Reserved | Shall be included when this message is sent in response to an AAI- RNG-REQ message used to perform location update or DCR mode initiation from Idle mode or DCR mode extension |

Suggested change #6:

Modify table 686 in page 155, line 53 with the following text:

Table 686—AAI-REG-RSP message Field Descriptions

| Field | Size (bits) | Value/Description | Conditions |
| --- | --- | --- | --- |
| CRID | 7280 | AMS identifier which the AMS has been assigned for coverage loss or DCR mode | Shall be included when AMS is attaching in the advanced network mode (i.e. network configuration bit in SFH = 0b0) |
| CRT | 2 | Default Context Retention Time value used by the network 00 – 12 hours 01 – one day 10 – 3 days 11 - undefined |  |
| If (Femto ABS LDM parameters are required to setup) { |  |  |  |

Suggested change #7:

Modify table 698 in page 197, line 5 with the following text:

Table 698—AAI-DREG-REQ message Field Descriptions

| Field | Size (bits) | Value/Description | Conditions |
| --- | --- | --- | --- |
| CRID | 80 | AMS identifier which the AMS has been assigned for DCR mode | When the AMS specifically requests to enter DCR mode |
| CRT | 2 | Requested Context Retention Time value: 00 – 12 hours 01 – one day 10 – 3 days 11 - undefined | When the AMS specifically requests to enter DCR mode and issues a request for setting the CRT value (informative only) |
| Idle Mode Retain Information element | 5 | Provided as part of this message indicative only. … |  |

Suggested change #8:

Modify table 699 in page 201, line 36 with the following text:

Table 699—AAI-DREG-RSP message Field Descriptions

| Field | Size (bits) | Value/Description | Conditions |
| --- | --- | --- | --- |
| Idle Mode Retain Information element | 5 | Provided as part of this message indicative only. … |  |
| CRT | 2 | New assigned Context Retention Time value: 00 – 12 hours 01 – one day 10 – 3 days 11 - undefined | Shall be included when the AAI-DREG-RSP is in response to a request to enter DCR mode in an AAI-DREQ-REQ message |
| } |  |  |  |

Suggested change #9:

Modify table 979 in page 1013, lines 26 by the following text:

Table 979—Parameters and constants

| System | Name | Time Reference | Minimum Value | Default Value | Maximum Value |
| --- | --- | --- | --- | --- | --- |
| AMS | BR timer | UL allocation waiting time for BR header after receiving a positive acknowledgement for a BR preamble sequence and a negative acknowledgement for quick access message | - | - | - |
| AMS | Context Retention Timer | Time valid to retain DCR mode | - | - | - |
| AMS | NLost-SFH | Number of lost SFHs for DL sync loss detection | 3 | 5 | 10 |

Suggested change #10:

Modify text in Annex R.2 in page 1030, line 32 with the following text:

MACVersion		::= INTEGER (0..255)

CRID			::= BIT STRING (SIZE(7280))

CRT			ENUMERATED {		-- encoding of two bits

halfDay,			-- 0b00

oneDay,			-- 0b01

threeDays			-- 0b10  (0b11 – reserved)

}

DID			::= BIT STRING (SIZE(12))

Suggested change #11:

Modify text in Annex R.2 in page 1031, line 65 with the following text:

LocationUpdateRsp ::= INTEGER {

successOfLocationUpdate (0),

failureOfLocationUpdate (1),

successOfLocationUpdateAndDLTrafficPending (3),

allowAmsDcrInitReqOrExtensionReq (48),

rejectAmsDcrInitReqOrExtensionReq (59)

-- 2, 0x5~0x7, 0x60xA~0xF: Reserved

} (0..15)

Suggested change #12:

Modify text in Annex R.2 in page 1035, line 6 with the following text:

smsMessage			SMS OPTIONAL,

…

}

LocationUpdateToDCRMode ::=	SEQUENCE {

crid						CRID,

crt						CRT

}

DCRModeExtension ::=		SEQUENCE {

crid						CRID,

crt						CRT,

akCount					AKCount OPTIONAL,

…

}

Suggested change #13:

Modify text in Annex R.2 (the ASN.1 definition of AAI-RNG_REQ) in page 1041, line 1 with the following text:

locationUpdatetoDcrMode		LocationUpdateToDCRMode,

Suggested change #14:

Modify text in Annex R.2 (the ASN.1 definition of AAI-RNG-RSP) in page 1042, line 10 with the following text:

crid					CRID OPTIONAL,

crt					CRT OPTIONAL,

emergencyServiceFID			INTEGER (2..15) OPTIONAL,

Suggested change #15:

Modify text in Annex R.2 (the ASN.1 definition of AAI-REG-RSP) in page 1048, line 14 with the following text:

crid					CRID OPTIONAL,

crt					CRT OPTIONAL,

femtoAbsLdm				SEQUENCE {

Suggested change #16:

Modify text in Annex R.2 (the ASN.1 definition of DeregToEnterDcrMode) in page 1049, line 49 with the following text:

DeregToEnterDcrMode  ::= SEQUENCE {

crid					CRID,

crt					CRT,

idleModeRetainInfo			IdleModeRetain

}

Suggested change #17:

Modify text in Annex R.2 (the ASN.1 definition of AAI-DREG-RSP) in page 1050, line 10 with the following text:

rejectConnectionInfoRetention NULL, -- 0x09

…

}

crt					CRT

}

- References

[1] IEEE Std 802.16-2009

[2] IEEE P802.16m/D10, “DRAFT Amendment to IEEE Standard for Local and metropolitan area networks”