IEEE P802.1

Proposed Extensions to the 802.1X MIB

Date:	July 10, 2002

Author:	David B. Nelson
	Enterasys Networks, Inc.
	50 Minuteman Road, Andover, MA 01810-1008
	Phone: (978) 684-1330 
	e-Mail:  dnelson@enterasys.com

Abstract

A proposed MIB, for application in 802.1X implementations using station based, or virtual port based, access control.  This MIB contains tables indexed by MacAddress rather than by ifIndex.

802.1X Extensions MIB using SMIv2

Status:  Current

Table of Contents

1. Introduction

2. The SNMP Management Framework

3. Background

4. How This MIB Works

5. The MIB

6. Scalability Considerations

7. Network Stability Considerations

8. Security Considerations

9. Applicability

10  Acknowledgments

11. References

12. Version History

1. Introduction

This document defines a portion of the Management Information Base

(MIB) for use with management protocols in the Internet community.  In

particular, it describes managed objects used for managing implementations of IEEE 802.1X authentication, based on station address.

This MIB supports the management of wireless LAN access points, and

other devices that provide station-based access control, by providing

station-based versions of IEEE 802.1X MIB objects.

Station-based objects augment the IEEE 802.1X MIB as it applies

to wireless LANs and other shared media that use station-based access

control.  Standard IEEE 802.1X MIB tables for Authenticator entities

use a 32-bit "port number" (actually, an InterfaceIndex) as an index.

The table objects for each conceptual row reflect the state of a PAE.

This arrangement presumes a 1:1 mapping between ports and PAEs.

In the wireless world, there isn't a one-to-one correlation between a

PAE and a radio port.  Instead, there are many virtual PAEs, each one

of which is created to handle a particular wireless Supplicant.  Many

port-based MIB objects can be interpreted as having a value that is a

rollup over all of the virtual PAEs for the port.  But in some cases,

like "backend state machine state", rollups don't make sense.  In

others, rollups may be meaningful, while failing to tell the complete

story.

It is possible to include station entries in the standard IEEE 802.1X

MIB tables, by making up artificial dot1xPaePortNumber values.  There

is also the option of simply not reporting station-level data at all.

Neither of these is particularly elegant; the second eliminates a lot

of troubleshooting data, and the first makes it hard to find.

Separating the station-based information out into parallel MIB tables

that are indexed by Supplicant MAC address lets a system offer useful

information in a way that is easier for a manager to find.

2. The SNMP Management Framework

The SNMP Management Framework presently consists of five major

components:

o   An overall architecture, described in RFC 2571 [RFC2571].

o   Mechanisms for describing and naming objects and events for the

purpose of management. The first version of this Structure of

Management Information (SMI) is called SMIv1 and described in

STD 16, RFC 1155 [RFC1155], STD 16, RFC 1212 [RFC1212] and RFC

1215 [RFC1215]. The second version, called SMIv2, is described

in STD 58, RFC 2578 [RFC2578], RFC 2579 [RFC2579] and RFC 2580

[RFC2580].

o   Message protocols for transferring management information. The

first version of the SNMP message protocol is called SNMPv1 and

described in STD 15, RFC 1157 [RFC1157]. A second version of the

SNMP message protocol, which is not an Internet standards track

protocol, is called SNMPv2c and described in RFC 1901 [RFC1901]

and RFC 1906 [RFC1906]. The third version of the message

protocol is called SNMPv3 and described in RFC 1906 [RFC1906],

RFC 2572 [RFC2572] and RFC 2574 [RFC2574].

o   Protocol operations for accessing management information. The

first set of protocol operations and associated PDU formats is

described in STD 15, RFC 1157 [RFC1157]. A second set of

protocol operations and associated PDU formats is described in

RFC 1905 [RFC1905].

o   A set of fundamental applications described in RFC 2573

[RFC2573] and the view-based access control mechanism described

in RFC 2575 [RFC2575].

A more detailed introduction to the current SNMP Management Framework

can be found in RFC 2570 [RFC2570].

Managed objects are accessed via a virtual information store, termed

the Management Information Base or MIB.  Objects in the MIB are

defined using the mechanisms defined in the SMI.

This memo specifies a MIB module that is compliant to the SMIv2.

A MIB conforming to the SMIv1 can be produced through the appropriate

translations. The resulting translated MIB must be semantically

equivalent, except where objects or events are omitted because no

translation is possible (use of Counter64). Some machine readable

information in SMIv2 will be converted into textual descriptions in

SMIv1 during the translation process. However, this loss of machine

readable information is not considered to change the semantics of the

MIB.

2.1  Requirement Level Keywords used in this Document

The MIB contained in this document uses SMIv2.

The key words "MUST", "MUST NOT", "REQUIRED", "SHALL", "SHALL NOT",

"SHOULD", "SHOULD NOT", "RECOMMENDED", "MAY", and "OPTIONAL" in this

document are to be interpreted as described in [RFC2119].

3. Background

This MIB is a result of field experience in implementing the IEEE 802.1X specification.  It addresses the need to provide station-based monitoring

and troubleshooting information for wireless Supplicants.

4. How This MIB Works

The MIB defined here supplements the IEEE 802.1X base MIB to provide

Better support for station-based access control -- the normal style of

Access control in an 802.11-based wireless LAN environment.

The station-based table objects in the 802.1X extensions MIB are much like their IEEE 802.1X MIB counterparts, except that they are indexed by MAC address.  Thus, if a system has two 802.11 radios, and 802.1X was enabled on a radio port with three stations, the MIB entries might look something like this:

IEEE 802.1X MIB (dot1xPaePortNumber)

------------------------------------

1  (Radio #1) -> authentication enabled/disabled, aggregate stats

2  (Radio #2) -> authentication enabled/disabled, aggregate stats

IEEE 802.1X Extensions MIB (dot1xAuthStationAddress)

--------------------------------------------------------------

08-00-2B-00-00-FF -> port 2, authenticated, user = JohnDoe, stats

08-00-2B-00-E0-2A -> port 2, authenticated, user = JaneDoe, stats

08-01-DC-98-21-0E -> port 2, held         , user = BadUser, stats

See the IEEE 802.1X specification for an introduction to authentication

concepts, for state machine definitions that provide a context for the

various MIB objects, and for documentation of the standard MIB.

There are some differences between station-based tables and port-based

tables of which network managers should be aware:

o   Station-based rows are likely to come and go much more frequently

than port-based rows.  Someone walking by an access point with a

new laptop can trigger the creation of a new station row.  Switch

ports don't come and go that easily.

o   Station-based tables are for monitoring and troubleshooting only.

Durable configuration of individual stations on the basis of MAC

addresses would not be scalable.  If individual configuration was

desirable, user-based configuration on the Authentication Server

would make more sense.  Thus, only the momentary toggle switches

to reinitialize a PAE or force reauthentication are writable.

One might ask "Why show configuration values for station-based virtual

PAEs if you cannot set values through the MIB?"  The answer is

that an Authentication Server (e.g. RADIUS/EAP server) may provide

suggested station-level configuration values as part of an Access-

Accept.  In this case, station-level settings may be something other

than just clones of the port-level ones.

5.  The MIB

8021X-EXTENSIONS-MIB DEFINITIONS ::= BEGIN

--  8021x-extensions-mib.txt

--

--  Part Number: <TBD>

--

--

--  This module provides authoritative definitions for the

--  IEEE 802.1X extensions MIB.

--

--  This module will be extended, as needed.

IMPORTS

MODULE-IDENTITY, OBJECT-TYPE, Counter32, Counter64, Unsigned32,

TimeTicks

FROM SNMPv2-SMI

TruthValue, MacAddress

FROM SNMPv2-TC

MODULE-COMPLIANCE, OBJECT-GROUP

FROM SNMPv2-CONF

InterfaceIndex

FROM IF-MIB

SnmpAdminString

FROM SNMP-FRAMEWORK-MIB

PaeControlledDirections, PaeControlledPortStatus,

PaeControlledPortControl

FROM IEEE8021-PAE-MIB;

8021xExtensionsMIB MODULE-IDENTITY

LAST-UPDATED “200207100000Z”

ORGANIZATION “IEEE 802.1 Working Group”

CONTACT-INFO

“http://grouper.ieee.org/groups/802/1/index.html”

DESCRIPTION

“The Port Access Entity module for managing IEEE

802.1X, using station based access control.”

::= { iso(1) std(0) iso8802(8802) ieee802dot1(n)

dot1xExtensionsObjects

OBJECT IDENTIFIER ::= { 8021xExtensionsMIB 1 }

-- ---------------------------------------------------------- --

-- Textual Conventions

-- ---------------------------------------------------------- --

-- ---------------------------------------------------------- --

-- Branches of the IEEE 802.1X Extensions MIB

-- ---------------------------------------------------------- --

dot1xSystemBranch

OBJECT IDENTIFIER ::= { dot1xExtensionsObjects 1 }

dot1xAuthenticatorBranch

OBJECT IDENTIFIER ::= { dot1xExtensionsObjects 2 }

dot1xSupplicantBranch

OBJECT IDENTIFIER ::= { dot1xExtensionsObjects 3 }

-- ---------------------------------------------------------- --

-- MIB Objects in the System Branch

-- ---------------------------------------------------------- --

-- This branch is a placeholder.

-- ---------------------------------------------------------- --

-- MIB Objects in the Authenticator PAE Branch

-- ---------------------------------------------------------- --

-- IEEE 802.1X extended MIB information for virtual Authenticator

-- PAEs that manage station-based, shared-media (e.g., wireless)

-- Supplicants.

dot1xAuthStationBranch

OBJECT IDENTIFIER ::= { dot1xAuthenticatorBranch 1 }

-- ---------------------------------------------------------- --

-- The Station-Based Authenticator PAE Table

-- ---------------------------------------------------------- --

dot1xAuthStationTable OBJECT-TYPE

SYNTAX        SEQUENCE OF Dot1xAuthStationEntry

MAX-ACCESS    not-accessible

STATUS        current

DESCRIPTION

"A table that contains basic status information for the

Authenticator PAEs associated with station-based virtual

ports."

::= { dot1xAuthStationBranch 1 }

dot1xAuthStationEntry OBJECT-TYPE

SYNTAX        Dot1xAuthStationEntry

MAX-ACCESS    not-accessible

STATUS        current

DESCRIPTION

"Each conceptual row contains basic information about a

station-based Authenticator PAE (such as a wireless access

point Authenticator PAE that manages a particular station

in a wireless workgroup).

The most natural label for such a PAE is the MAC address

of the Supplicant that it manages, and that, in some sense,

called the PAE into existence.  So we index the table by

the Supplicant MAC address."

INDEX { dot1xAuthStationAddress }

::= { dot1xAuthStationTable 1 }

Dot1xAuthStationEntry ::=

SEQUENCE {

dot1xAuthStationAddress             MacAddress,

dot1xAuthStationPaePort             InterfaceIndex,

dot1xAuthStationPaeState            INTEGER,

dot1xAuthStationBackendAuthState    INTEGER,

dot1xAuthStationUserName            SnmpAdminString

}

dot1xAuthStationAddress OBJECT-TYPE

SYNTAX        MacAddress

MAX-ACCESS    not-accessible

STATUS        current

DESCRIPTION

"The 48-bit IEEE media access control address of the

Supplicant associated with the logical station-based

access control port."

::= { dot1xAuthStationEntry 1 }

dot1xAuthStationPaePort OBJECT-TYPE

SYNTAX        InterfaceIndex

MAX-ACCESS    read-only

STATUS        current

DESCRIPTION

"The dot1xPaePortNumber of the shared-media port that

is associated with this Authenticator PAE."

::= { dot1xAuthStationEntry 2 }

dot1xAuthStationPaeState OBJECT-TYPE

SYNTAX        INTEGER {

initialize(1),

disconnected(2),

connecting(3),

authenticating(4),

authenticated(5),

aborting(6),

held(7),

forceAuth(8),

forceUnauth(9)

}

MAX-ACCESS    read-only

STATUS        current

DESCRIPTION

"The current value of the Authenticator PAE state machine."

REFERENCE  "IEEE 802.1X Section 9.4.1, Authenticator PAE state"

::= { dot1xAuthStationEntry 3 }

dot1xAuthStationBackendAuthState OBJECT-TYPE

SYNTAX        INTEGER {

request(1),

response(2),

success(3),

fail(4),

timeout(5),

idle(6),

initialize(7)

}

MAX-ACCESS    read-only

STATUS        current

DESCRIPTION

"The current state of the Backend Authentication state

machine."

REFERENCE  "IEEE 802.1X Section 9.4.1, Backend Authentication state"

::= { dot1xAuthStationEntry 4 }

dot1xAuthStationUserName OBJECT-TYPE

SYNTAX        SnmpAdminString

MAX-ACCESS    read-only

STATUS        current

DESCRIPTION

"The User-Name representing the identity of the

Supplicant PAE."

REFERENCE  "IEEE 802.1X Section 9.4.4, Session User Name"

::= { dot1xAuthStationEntry 5 }

-- ---------------------------------------------------------- --

-- The Station-Based Authenticator Configuration Table

-- ---------------------------------------------------------- --

dot1xAuthConfigTable OBJECT-TYPE

SYNTAX        SEQUENCE OF Dot1xAuthConfigEntry

MAX-ACCESS    not-accessible

STATUS        current

DESCRIPTION

"A table that contains configuration objects for the

Authenticator PAE associated with each station-based virtual

port.

Station-based configuration management poses several rather

major inconveniences, including a large number of rows, rows

indexed by MAC address (rather than by user), and rows that

come and go.

Port-level and Authentication-Server-based management are

quite a bit more convenient.  Thus, most of the objects in

this table are read-only; a way to see how other settings

have interacted if you need this information for

troubleshooting purposes."

::= { dot1xAuthStationBranch 2 }

dot1xAuthConfigEntry OBJECT-TYPE

SYNTAX        Dot1xAuthConfigEntry

MAX-ACCESS    not-accessible

STATUS        current

DESCRIPTION

"Configuration information for a station-based Authenticator

PAE."

INDEX { dot1xAuthStationAddress }

::= { dot1xAuthConfigTable 1 }

Dot1xAuthConfigEntry ::=

SEQUENCE {

dot1xAuthInitialize                 TruthValue,

dot1xAuthReauthenticate             TruthValue,

dot1xAuthAdminControlledDirections  PaeControlledDirections,

dot1xAuthOperControlledDirections   PaeControlledDirections,

dot1xAuthAuthControlledPortStatus   PaeControlledPortStatus,

dot1xAuthAuthControlledPortControl  PaeControlledPortControl,

dot1xAuthQuietPeriod                Unsigned32,

dot1xAuthTxPeriod                   Unsigned32,

dot1xAuthSuppTimeout                Unsigned32,

dot1xAuthServerTimeout              Unsigned32,

dot1xAuthMaxReq                     Unsigned32,

dot1xAuthReAuthPeriod               Unsigned32,

dot1xAuthReAuthEnabled              TruthValue,

dot1xAuthKeyTxEnabled               TruthValue

}

dot1xAuthInitialize OBJECT-TYPE

SYNTAX        TruthValue

MAX-ACCESS    read-write

STATUS        current

DESCRIPTION

"The initialization control for this Authenticator PAE.

Setting this attribute to TRUE causes the PAE to be

initialized.  The attribute value reverts to FALSE

once initialization has completed."

REFERENCE      "IEEE 802.1X Section 9.6.1.2, Initialize Port"

::= { dot1xAuthConfigEntry 1 }

dot1xAuthReauthenticate OBJECT-TYPE

SYNTAX        TruthValue

MAX-ACCESS    read-write

STATUS        current

DESCRIPTION

"The reauthentication control for this Authenticator

PAE.  Setting this attribute to TRUE causes the

Authenticator PAE state machine to reauthenticate the

Supplicant.  Setting this attribute FALSE has no

effect.  This attribute always returns FALSE when it

is read."

REFERENCE   "IEEE 802.1X Section 9.4.1.3 Reauthenticate"

::= { dot1xAuthConfigEntry 2 }

dot1xAuthAdminControlledDirections OBJECT-TYPE

SYNTAX        PaeControlledDirections

MAX-ACCESS    read-only

STATUS        current

DESCRIPTION

"The current value of the administrative controlled

directions parameter for the virtual port."

REFERENCE  "IEEE 802.1X Section 9.4.1, Admin Control Mode"

::= { dot1xAuthConfigEntry 3 }

dot1xAuthOperControlledDirections OBJECT-TYPE

SYNTAX        PaeControlledDirections

MAX-ACCESS    read-only

STATUS        current

DESCRIPTION

"The current value of the operational controlled

directions parameter for the virtual port."

REFERENCE  "IEEE 802.1X Section 9.4.1, Oper Control Mode"

::= { dot1xAuthConfigEntry 4 }

dot1xAuthAuthControlledPortStatus OBJECT-TYPE

SYNTAX        PaeControlledPortStatus

MAX-ACCESS    read-only

STATUS        current

DESCRIPTION

"The current value of the controlled Port

status parameter for the virtual port."

REFERENCE  "IEEE 802.1X Section 9.4.1, AuthControlledPortStatus"

::= { dot1xAuthConfigEntry 5 }

dot1xAuthAuthControlledPortControl OBJECT-TYPE

SYNTAX        PaeControlledPortControl

MAX-ACCESS    read-only

STATUS        current

DESCRIPTION

"The current value of the controlled Port

control parameter for the virtual port."

REFERENCE  "IEEE 802.1X Section 9.4.1, AuthControlledPortControl"

::= { dot1xAuthConfigEntry 6 }

dot1xAuthQuietPeriod OBJECT-TYPE

SYNTAX        Unsigned32

MAX-ACCESS    read-only

STATUS        current

DESCRIPTION

"The value, in seconds, of the quietPeriod constant

currently in use by the Authenticator PAE state

machine."

REFERENCE  "IEEE 802.1X Section 9.4.1, quietPeriod"

::= { dot1xAuthConfigEntry 7 }

dot1xAuthTxPeriod OBJECT-TYPE

SYNTAX        Unsigned32

MAX-ACCESS    read-only

STATUS        current

DESCRIPTION

"The value, in seconds, of the txPeriod constant

currently in use by the Authenticator PAE state

machine."

REFERENCE  "IEEE 802.1X Section 9.4.1, txPeriod"

::= { dot1xAuthConfigEntry 8 }

dot1xAuthSuppTimeout OBJECT-TYPE

SYNTAX        Unsigned32

MAX-ACCESS    read-only

STATUS        current

DESCRIPTION

"The value, in seconds, of the suppTimeout constant

currently in use by the Backend Authentication state

machine."

REFERENCE  "IEEE 802.1X Section 9.4.1, suppTimeout"

::= { dot1xAuthConfigEntry 9 }

dot1xAuthServerTimeout OBJECT-TYPE

SYNTAX        Unsigned32

MAX-ACCESS    read-only

STATUS        current

DESCRIPTION

"The value, in seconds, of the serverTimeout constant

currently in use by the Backend Authentication state

machine."

REFERENCE  "IEEE 802.1X Section 9.4.1, serverTimeout"

::= { dot1xAuthConfigEntry 10 }

dot1xAuthMaxReq OBJECT-TYPE

SYNTAX        Unsigned32

MAX-ACCESS    read-only

STATUS        current

DESCRIPTION

"The value of the maxReq constant currently in use by

the Backend Authentication state machine."

REFERENCE  "IEEE 802.1X Section 9.4.1, maxReq"

::= { dot1xAuthConfigEntry 11 }

dot1xAuthReAuthPeriod OBJECT-TYPE

SYNTAX        Unsigned32

MAX-ACCESS    read-only

STATUS        current

DESCRIPTION

"The value, in seconds, of the reAuthPeriod constant

currently in use by the Reauthentication Timer state

machine."

REFERENCE  "IEEE 802.1X Section 9.4.1, reAuthPeriod"

::= { dot1xAuthConfigEntry 12 }

dot1xAuthReAuthEnabled OBJECT-TYPE

SYNTAX        TruthValue

MAX-ACCESS    read-only

STATUS        current

DESCRIPTION

"The enable/disable control used by the

Reauthentication Timer state machine (IEEE 802.1X

Section 8.5.5.1)."

REFERENCE   "IEEE 802.1X Section 9.4.1, reAuthEnabled"

DEFVAL { false }

::= { dot1xAuthConfigEntry 13 }

dot1xAuthKeyTxEnabled OBJECT-TYPE

SYNTAX        TruthValue

MAX-ACCESS    read-only

STATUS        current

DESCRIPTION

"The value of the keyTransmissionEnabled constant

currently in use by the Authenticator PAE state

machine."

REFERENCE   "IEEE 802.1X Section 9.4.1, keyTransmissionEnabled"

::= { dot1xAuthConfigEntry 14 }

-- ---------------------------------------------------------- --

-- The Authenticator Statistics Table

-- ---------------------------------------------------------- --

dot1xAuthStatsTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot1xAuthStatsEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"A table that contains the statistics objects for the

Authenticator PAE associated with each Port.

An entry appears in this table for each port that may

authenticate access to itself."

REFERENCE   "IEEE 802.1X Section 9.4.2 Authenticator Statistics"

::= { dot1xAuthStationBranch 3 }

dot1xAuthStatsEntry OBJECT-TYPE

SYNTAX        Dot1xAuthStatsEntry

MAX-ACCESS    not-accessible

STATUS        current

DESCRIPTION

"The statistics information for an Authenticator PAE."

INDEX { dot1xAuthStationAddress }

::= { dot1xAuthStatsTable 1 }

Dot1xAuthStatsEntry ::=

SEQUENCE { dot1xAuthEapolFramesRx            Counter32,

dot1xAuthEapolFramesTx            Counter32,

dot1xAuthEapolStartFramesRx       Counter32,

dot1xAuthEapolLogoffFramesRx      Counter32,

dot1xAuthEapolRespIdFramesRx      Counter32,

dot1xAuthEapolRespFramesRx        Counter32,

dot1xAuthEapolReqIdFramesTx       Counter32,

dot1xAuthEapolReqFramesTx         Counter32,

dot1xAuthInvalidEapolFramesRx     Counter32,

dot1xAuthEapLengthErrorFramesRx   Counter32,

dot1xAuthLastEapolFrameVersion    Unsigned32,

dot1xAuthLastEapolFrameSource     MacAddress

}

dot1xAuthEapolFramesRx OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The number of valid EAPOL frames of any type

that have been received by this Authenticator."

REFERENCE   "IEEE 802.1X Section 9.4.2, EAPOL frames received"

::= { dot1xAuthStatsEntry 1 }

dot1xAuthEapolFramesTx OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The number of EAPOL frames of any type

that have been transmitted by this Authenticator."

REFERENCE   "IEEE 802.1X Section 9.4.2, EAPOL frames transmitted"

::= { dot1xAuthStatsEntry 2 }

dot1xAuthEapolStartFramesRx OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The number of EAPOL Start frames that have

been received by this Authenticator."

REFERENCE  "IEEE 802.1X Section 9.4.2, EAPOL Start frames received"

::= { dot1xAuthStatsEntry 3 }

dot1xAuthEapolLogoffFramesRx OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The number of EAPOL Logoff frames that have

been received by this Authenticator."

REFERENCE  "IEEE 802.1X Section 9.4.2, EAPOL Logoff frames received"

::= { dot1xAuthStatsEntry 4 }

dot1xAuthEapolRespIdFramesRx OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The number of EAP Resp/Id frames that have

been received by this Authenticator."

REFERENCE   "IEEE 802.1X Section 9.4.2, EAPOL Resp/Id frames

received"

::= { dot1xAuthStatsEntry 5 }

dot1xAuthEapolRespFramesRx OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The number of valid EAP Response frames

(other than Resp/Id frames) that have been

received by this Authenticator."

REFERENCE   "IEEE 802.1X Section 9.4.2, EAPOL Response frames

received"

::= { dot1xAuthStatsEntry 6 }

dot1xAuthEapolReqIdFramesTx OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The number of EAP Req/Id frames that have been

transmitted by this Authenticator."

REFERENCE   "IEEE 802.1X Section 9.4.2, EAPOL Req/Id frames

transmitted"

::= { dot1xAuthStatsEntry 7 }

dot1xAuthEapolReqFramesTx OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The number of EAP Request frames

(other than Rq/Id frames) that have been

transmitted by this Authenticator."

REFERENCE  "IEEE 802.1X Section 9.4.2, EAPOL Request frames

transmitted"

::= { dot1xAuthStatsEntry 8 }

dot1xAuthInvalidEapolFramesRx OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The number of EAPOL frames that have been

received by this Authenticator in which the

frame type is not recognized."

REFERENCE   "IEEE 802.1X Section 9.4.2, Invalid EAPOL frames

received"

::= { dot1xAuthStatsEntry 9 }

dot1xAuthEapLengthErrorFramesRx OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The number of EAPOL frames that have been received

by this Authenticator in which the Packet Body

Length field is invalid."

REFERENCE  "IEEE 802.1X Section 9.4.2, EAP length error frames

received"

::= { dot1xAuthStatsEntry 10 }

dot1xAuthLastEapolFrameVersion OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The protocol version number carried in the

most recently received EAPOL frame."

REFERENCE   "IEEE 802.1X Section 9.4.2, Last EAPOL frame version"

::= { dot1xAuthStatsEntry 11 }

dot1xAuthLastEapolFrameSource OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The source MAC address carried in the most

recently received EAPOL frame."

REFERENCE   "IEEE 802.1X Section 9.4.2, Last EAPOL frame source"

::= { dot1xAuthStatsEntry 12 }

-- ---------------------------------------------------------- --

-- The Authenticator Diagnostics Table

-- ---------------------------------------------------------- --

dot1xAuthDiagTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot1xAuthDiagEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"A table that contains the diagnostics objects for the

Authenticator PAE associated with each Port.

An entry appears in this table for each port that may

authenticate access to itself."

REFERENCE  "IEEE 802.1X Section 9.4.3 Authenticator Diagnostics"

::= { dot1xAuthStationBranch 4 }

dot1xAuthDiagEntry OBJECT-TYPE

SYNTAX Dot1xAuthDiagEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"The diagnostics information for an Authenticator PAE."

INDEX { dot1xAuthStationAddress }

::= { dot1xAuthDiagTable 1 }

Dot1xAuthDiagEntry ::=

SEQUENCE {

dot1xAuthEntersConnecting                     Counter32,

dot1xAuthEapLogoffsWhileConnecting            Counter32,

dot1xAuthEntersAuthenticating                 Counter32,

dot1xAuthAuthSuccessWhileAuthenticating       Counter32,

dot1xAuthAuthTimeoutsWhileAuthenticating      Counter32,

dot1xAuthAuthFailWhileAuthenticating          Counter32,

dot1xAuthAuthReauthsWhileAuthenticating       Counter32,

dot1xAuthAuthEapStartsWhileAuthenticating     Counter32,

dot1xAuthAuthEapLogoffWhileAuthenticating     Counter32,

dot1xAuthAuthReauthsWhileAuthenticated        Counter32,

dot1xAuthAuthEapStartsWhileAuthenticated      Counter32,

dot1xAuthAuthEapLogoffWhileAuthenticated      Counter32,

dot1xAuthBackendResponses                     Counter32,

dot1xAuthBackendAccessChallenges              Counter32,

dot1xAuthBackendOtherRequestsToSupplicant     Counter32,

dot1xAuthBackendNonNakResponsesFromSupplicant Counter32,

dot1xAuthBackendAuthSuccesses                 Counter32,

dot1xAuthBackendAuthFails                     Counter32

}

dot1xAuthEntersConnecting OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Counts the number of times that the state machine

transitions to the CONNECTING state from any other

state."

REFERENCE   "IEEE 802.1X Sections 9.4.2, 8.5.4.2.1"

::= { dot1xAuthDiagEntry 1 }

dot1xAuthEapLogoffsWhileConnecting OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Counts the number of times that the state machine

transitions from CONNECTING to DISCONNECTED as a result

of receiving an EAPOL-Logoff message."

REFERENCE   "IEEE 802.1X Sections 9.4.2, 8.5.4.2.2"

::= { dot1xAuthDiagEntry 2 }

dot1xAuthEntersAuthenticating OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Counts the number of times that the state machine

transitions from CONNECTING to AUTHENTICATING, as a

result of an EAP-Response/Identity message being

received from the Supplicant."

REFERENCE  "IEEE 802.1X Sections 9.4.2, 8.5.4.2.3"

::= { dot1xAuthDiagEntry 3 }

dot1xAuthAuthSuccessWhileAuthenticating OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Counts the number of times that the state machine

transitions from AUTHENTICATING to AUTHENTICATED, as a

result of the Backend Authentication state machine

indicating successful authentication of the Supplicant

(authSuccess = TRUE)."

REFERENCE   "IEEE 802.1X Sections 9.4.2, 8.5.4.2.4"

::= { dot1xAuthDiagEntry 4 }

dot1xAuthAuthTimeoutsWhileAuthenticating OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Counts the number of times that the state machine

transitions from AUTHENTICATING to ABORTING, as a

result of the Backend Authentication state machine

indicating authentication timeout (authTimeout =

TRUE)."

REFERENCE   "IEEE 802.1X Sections 9.4.2, 8.5.4.2.5"

::= { dot1xAuthDiagEntry 5 }

dot1xAuthAuthFailWhileAuthenticating OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Counts the number of times that the state machine

transitions from AUTHENTICATING to HELD, as a result

of the Backend Authentication state machine indicating

authentication failure (authFail = TRUE)."

REFERENCE  "IEEE 802.1X Sections 9.4.2, 8.5.4.2.6"

::= { dot1xAuthDiagEntry 6 }

dot1xAuthAuthReauthsWhileAuthenticating OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Counts the number of times that the state machine

transitions from AUTHENTICATING to ABORTING, as a

result of a reauthentication request (reAuthenticate

= TRUE)."

REFERENCE   "IEEE 802.1X Sections 9.4.2, 8.5.4.2.7"

::= { dot1xAuthDiagEntry 7 }

dot1xAuthAuthEapStartsWhileAuthenticating OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Counts the number of times that the state machine

transitions from AUTHENTICATING to ABORTING, as a

result of an EAPOL-Start message being received from

the Supplicant."

REFERENCE   "IEEE 802.1X Sections 9.4.2, 8.5.4.2.8"

::= { dot1xAuthDiagEntry 8 }

dot1xAuthAuthEapLogoffWhileAuthenticating OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Counts the number of times that the state machine

transitions from AUTHENTICATING to ABORTING, as a result

of an EAPOL-Logoff message being received

from the Supplicant."

REFERENCE  "IEEE 802.1X Sections 9.4.2, 8.5.4.2.9"

::= { dot1xAuthDiagEntry 9 }

dot1xAuthAuthReauthsWhileAuthenticated OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Counts the number of times that the state machine

transitions from AUTHENTICATED to CONNECTING, as a

result of a reauthentication request

(reAuthenticate = TRUE)."

REFERENCE  "IEEE 802.1X Sections 9.4.2, 8.5.4.2.10"

::= { dot1xAuthDiagEntry 10 }

dot1xAuthAuthEapStartsWhileAuthenticated OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Counts the number of times that the state machine

transitions from AUTHENTICATED to CONNECTING, as a

result of an EAPOL-Start message being received from the

Supplicant."

REFERENCE "IEEE 802.1X Sections 9.4.2, 8.5.4.2.11"

::= { dot1xAuthDiagEntry 11 }

dot1xAuthAuthEapLogoffWhileAuthenticated OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Counts the number of times that the state machine

transitions from AUTHENTICATED to DISCONNECTED, as a

result of an EAPOL-Logoff message being received from

the Supplicant."

REFERENCE  "IEEE 802.1X Sections 9.4.2, 8.5.4.2.12"

::= { dot1xAuthDiagEntry 12 }

dot1xAuthBackendResponses OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Counts the number of times that the state machine sends

an initial Access-Request packet to the Authentication

server (i.e., executes sendRespToServer on entry to the

RESPONSE state). Indicates that the Authenticator

attempted communication with the Authentication Server."

REFERENCE  "IEEE 802.1X Sections 9.4.2, 8.5.6.2.1"

::= { dot1xAuthDiagEntry 13 }

dot1xAuthBackendAccessChallenges OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Counts the number of times that the state machine

receives an initial Access-Challenge packet from the

Authentication server (i.e., aReq becomes TRUE,

causing exit from the RESPONSE state). Indicates that

the Authentication Server has communication with

the Authenticator."

REFERENCE  "IEEE 802.1X Sections 9.4.2, 8.5.6.2.2"

::= { dot1xAuthDiagEntry 14 }

dot1xAuthBackendOtherRequestsToSupplicant OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Counts the number of times that the state machine

sends an EAP-Request packet (other than an Identity,

Notification, Failure or Success message) to the

Supplicant (i.e., executes txReq on entry to the

REQUEST state). Indicates that the Authenticator chose

an EAP-method."

REFERENCE "IEEE 802.1X Sections 9.4.2, 8.5.6.2.3"

::= { dot1xAuthDiagEntry 15 }

dot1xAuthBackendNonNakResponsesFromSupplicant OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Counts the number of times that the state machine

receives a response from the Supplicant to an initial

EAP-Request, and the response is something other than

EAP-NAK (i.e., rxResp becomes TRUE, causing the state

machine to transition from REQUEST to RESPONSE,

and the response is not an EAP-NAK). Indicates that

the Supplicant can respond to the Authenticator's

chosen EAP-method."

REFERENCE "IEEE 802.1X Sections 9.4.2, 8.5.6.2.4"

::= { dot1xAuthDiagEntry 16 }

dot1xAuthBackendAuthSuccesses OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Counts the number of times that the state machine

receives an EAP-Success message from the Authentication

Server (i.e., aSuccess becomes TRUE, causing a

transition from RESPONSE to SUCCESS). Indicates that

the Supplicant has successfully authenticated to

the Authentication Server."

REFERENCE  "IEEE 802.1X Sections 9.4.2, 8.5.6.2.5"

::= { dot1xAuthDiagEntry 17 }

dot1xAuthBackendAuthFails OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Counts the number of times that the state machine

receives an EAP-Failure message from the Authentication

Server (i.e., aFail becomes TRUE, causing a transition

from RESPONSE to FAIL). Indicates that the Supplicant

has not authenticated to the Authentication Server."

REFERENCE  "IEEE 802.1X Sections 9.4.2, 8.5.6.2.6"

::= { dot1xAuthDiagEntry 18 }

-- ---------------------------------------------------------- --

-- The Authenticator Session Statistics Table

-- ---------------------------------------------------------- --

dot1xAuthSessionStatsTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot1xAuthSessionStatsEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"A table that contains the session statistics objects

for the Authenticator PAE associated with each Port.

An entry appears in this table for each port that may

authenticate access to itself."

REFERENCE  "IEEE 802.1X Section 9.4.4"

::= { dot1xAuthStationBranch 5 }

dot1xAuthSessionStatsEntry OBJECT-TYPE

SYNTAX Dot1xAuthSessionStatsEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"The session statistics information for an Authenticator

PAE. This shows the current values being collected for

each session that is still in progress, or the final

values for the last valid session on each port where

there is no session currently active."

INDEX { dot1xAuthStationAddress }

::= { dot1xAuthSessionStatsTable 1 }

Dot1xAuthSessionStatsEntry ::=

SEQUENCE { dot1xAuthSessionOctetsRx         Counter64,

dot1xAuthSessionOctetsTx         Counter64,

dot1xAuthSessionFramesRx         Counter32,

dot1xAuthSessionFramesTx         Counter32,

dot1xAuthSessionId               SnmpAdminString,

dot1xAuthSessionAuthenticMethod  INTEGER,

dot1xAuthSessionTime             TimeTicks,

dot1xAuthSessionTerminateCause   INTEGER

-- Session name is in dot1xAuthStationTable

}

dot1xAuthSessionOctetsRx OBJECT-TYPE

SYNTAX Counter64

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The number of octets received in user data

frames on this Port during the session."

REFERENCE   "IEEE 802.1X Section 9.4.4, Session Octets Received"

::= { dot1xAuthSessionStatsEntry 1 }

dot1xAuthSessionOctetsTx OBJECT-TYPE

SYNTAX Counter64

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The number of octets transmitted in user data

frames on this Port during the session."

REFERENCE  "IEEE 802.1X Section 9.4.4, Session Octets Transmitted"

::= { dot1xAuthSessionStatsEntry 2 }

dot1xAuthSessionFramesRx OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The number of user data frames received

on this Port during the session."

REFERENCE   "IEEE 802.1X Section 9.4.4, Session Frames Received"

::= { dot1xAuthSessionStatsEntry 3 }

dot1xAuthSessionFramesTx OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The number of user data frames transmitted

on this Port during the session."

REFERENCE  "IEEE 802.1X Section 9.4.4, Session Frames Transmitted"

::= { dot1xAuthSessionStatsEntry 4 }

dot1xAuthSessionId OBJECT-TYPE

SYNTAX SnmpAdminString

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"A unique identifier for the session, in the

form of a printable ASCII string of at least

three characters."

REFERENCE   "IEEE 802.1X Section 9.4.4, Session Identifier"

::= { dot1xAuthSessionStatsEntry 5 }

dot1xAuthSessionAuthenticMethod OBJECT-TYPE

SYNTAX INTEGER { remoteAuthServer(1),

localAuthServer(2)

}

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The authentication method used to establish the session."

REFERENCE "IEEE 802.1X Section 9.4.4, Session Authentication Method"

::= { dot1xAuthSessionStatsEntry 6 }

dot1xAuthSessionTime OBJECT-TYPE

SYNTAX TimeTicks

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The duration of the session in seconds."

REFERENCE  "IEEE 802.1X Section 9.4.4, Session Time"

::= { dot1xAuthSessionStatsEntry 7 }

dot1xAuthSessionTerminateCause OBJECT-TYPE

SYNTAX INTEGER { supplicantLogoff(1),

portFailure(2),

supplicantRestart(3),

reauthFailed(4),

authControlForceUnauth(5),

portReInit(6),

portAdminDisabled(7),

notTerminatedYet(999)

}

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The reason for the session termination."

REFERENCE   "IEEE 802.1X Section 9.4.4, Session Terminate Cause"

::= { dot1xAuthSessionStatsEntry 8 }

-- ---------------------------------------------------------- --

-- MIB Objects in the Supplicant Branch

-- ---------------------------------------------------------- --

-- This branch is a placeholder for any Supplicant-PAE-related

-- objects that might be added to this MIB in the future.

--

-- It is currently empty.

-- ---------------------------------------------------------- --

-- IEEE 802.1X Extensions MIB - Conformance Information

-- ---------------------------------------------------------- --

dot1xConformance

OBJECT IDENTIFIER ::= { 8021xExtensionsMIB 2 }

dot1xGroups      OBJECT IDENTIFIER ::= { dot1xConformance 1 }

dot1xCompliances OBJECT IDENTIFIER ::= { dot1xConformance 2 }

-- ---------------------------------------------------------- --

-- Units of conformance

-- ---------------------------------------------------------- --

dot1xAuthStationGroup OBJECT-GROUP

OBJECTS {

dot1xAuthStationPaePort,

dot1xAuthStationPaeState,

dot1xAuthStationBackendAuthState,

dot1xAuthStationUserName

}

STATUS current

DESCRIPTION

"A collection of objects providing basic status information

about Authenticator PAEs running on ports that use station-

based access control."

::= { dot1xGroups 1 }

dot1xAuthConfigGroup OBJECT-GROUP

OBJECTS {

dot1xAuthInitialize,

dot1xAuthReauthenticate,

dot1xAuthAdminControlledDirections,

dot1xAuthOperControlledDirections,

dot1xAuthAuthControlledPortStatus,

dot1xAuthAuthControlledPortControl,

dot1xAuthQuietPeriod,

dot1xAuthTxPeriod,

dot1xAuthSuppTimeout,

dot1xAuthServerTimeout,

dot1xAuthMaxReq,

dot1xAuthReAuthPeriod,

dot1xAuthReAuthEnabled,

dot1xAuthKeyTxEnabled

}

STATUS current

DESCRIPTION

"A collection of objects providing configuration information

about Authenticator PAEs running on ports that use station-

based access control."

::= { dot1xGroups 2 }

dot1xAuthStatsGroup OBJECT-GROUP

OBJECTS {

dot1xAuthEapolFramesRx,

dot1xAuthEapolFramesTx,

dot1xAuthEapolStartFramesRx,

dot1xAuthEapolLogoffFramesRx,

dot1xAuthEapolRespIdFramesRx,

dot1xAuthEapolRespFramesRx,

dot1xAuthEapolReqIdFramesTx,

dot1xAuthEapolReqFramesTx,

dot1xAuthInvalidEapolFramesRx,

dot1xAuthEapLengthErrorFramesRx,

dot1xAuthLastEapolFrameVersion,

dot1xAuthLastEapolFrameSource

}

STATUS current

DESCRIPTION

"A collection of objects providing statistics about

Authenticator PAEs running on ports that use station-

based access control."

::= { dot1xGroups 3 }

dot1xAuthDiagGroup OBJECT-GROUP

OBJECTS {

dot1xAuthEntersConnecting,

dot1xAuthEapLogoffsWhileConnecting,

dot1xAuthEntersAuthenticating,

dot1xAuthAuthSuccessWhileAuthenticating,

dot1xAuthAuthTimeoutsWhileAuthenticating,

dot1xAuthAuthFailWhileAuthenticating,

dot1xAuthAuthReauthsWhileAuthenticating,

dot1xAuthAuthEapStartsWhileAuthenticating,

dot1xAuthAuthEapLogoffWhileAuthenticating,

dot1xAuthAuthReauthsWhileAuthenticated,

dot1xAuthAuthEapStartsWhileAuthenticated,

dot1xAuthAuthEapLogoffWhileAuthenticated,

dot1xAuthBackendResponses,

dot1xAuthBackendAccessChallenges,

dot1xAuthBackendOtherRequestsToSupplicant,

dot1xAuthBackendNonNakResponsesFromSupplicant,

dot1xAuthBackendAuthSuccesses,

dot1xAuthBackendAuthFails

}

STATUS current

DESCRIPTION

"A collection of objects providing diagnostic statistics

about Authenticator PAEs running on ports that use station-

based access control."

::= { dot1xGroups 4 }

dot1xAuthSessionStatsGroup OBJECT-GROUP

OBJECTS {

dot1xAuthSessionOctetsRx,

dot1xAuthSessionOctetsTx,

dot1xAuthSessionFramesRx,

dot1xAuthSessionFramesTx,

dot1xAuthSessionId,

dot1xAuthSessionAuthenticMethod,

dot1xAuthSessionTime,

dot1xAuthSessionTerminateCause

}

STATUS current

DESCRIPTION

"A collection of objects providing statistics about the

current or last sessions for Authenticator PAEs running

on ports that use station-based access control."

::= { dot1xGroups 5 }

-- ---------------------------------------------------------- --

-- Compliance statements

-- ---------------------------------------------------------- --

dot1xCompliance MODULE-COMPLIANCE

STATUS current

DESCRIPTION

"The compliance statement for devices that support the

IEEE 802.1X extensions MIB."

MODULE

--  MANDATORY-GROUPS {}

GROUP dot1xAuthStationGroup

DESCRIPTION

"This group is conditionally mandatory for systems that

support both station-based 802.1X access control and the

Authenticator functions of the PAE, and that implement

any of the objects in the dot1xAuthStationBranch."

GROUP dot1xAuthConfigGroup

DESCRIPTION

"This group is conditionally mandatory for systems that

support both station-based 802.1X access control and the

Authenticator functions of the PAE, and that implement

any of the objects in the dot1xAuthStationBranch."

GROUP dot1xAuthStatsGroup

DESCRIPTION

"This group is conditionally mandatory for systems that

support both station-based 802.1X access control and the

Authenticator functions of the PAE, and that implement

any of the objects in the dot1xAuthStationBranch."

GROUP dot1xAuthDiagGroup

DESCRIPTION

"This group is optional."

GROUP dot1xAuthSessionStatsGroup

DESCRIPTION

"This group is optional."

OBJECT         dot1xAuthInitialize

MIN-ACCESS     read-only

DESCRIPTION    "Write access is not required."

OBJECT         dot1xAuthReauthenticate

MIN-ACCESS     read-only

DESCRIPTION    "Write access is not required."

::= { dot1xCompliances 1 }

END

6.  Scalability Considerations

On a system that uses station-based authentication, and has hundreds or

thousands of Supplicants, the tables in the dot1xAuthStationBranch

can grow quite large.  Walking them may take a significant time and may

burden the managed device.

It is recommended that network managers cherry-pick these tables (e.g.,

by scanning dot1xAuthStationPaeState to determine which table rows

are interesting), as opposed to always requesting all of the statistics

and configuration information for all of the stations.

7.  Network Stability Considerations

This memo does not discuss network stability considerations.

8.  Security Considerations

8.1  Security Concerns Relating To Sets

There are several management objects defined in this MIB that have a

MAX-ACCESS clause of read-write and/or read-create.  Such objects may

be considered sensitive or vulnerable in some network environments.

The support for SET operations in a non-secure environment without

proper protection can have a negative effect on network operations.

o   Failure to secure Set access to the station-level "reauthenticate"

and "initialize" objects could allow attackers to launch denial of

service attacks against individual users, the Authenticator (e.g.,

access point), or the Authentication Server.

8.2  Security Concerns Relating To Gets

There are some managed objects in this MIB that may contain sensitive

information.

o   The station-level tables reveal MAC addresses and authentication

states.  An attacker who saw this information might write down a

list of "MAC addresses that would be good to spoof" (on a shared

LAN such as a wireless LAN), as a way of bypassing 802.1X in the

future.

It should be noted that, for wireless LANs, an attacker can

already cull MAC addresses using a sniffer -- the use of 128-bit

encryption (and/or IPsec) is recommended to frustrate people

mounting a spoofing type of attack.

It may thus be important to control even GET access to station-based

MIB objects, and possibly to even encrypt the values of these objects

when sending them over the network via SNMP.  Not all versions of SNMP

provide features for such a secure environment.

8.3  SNMP Security

SNMPv1 by itself is not a secure environment.  Even if the network

itself is secure (for example by using IPSec), even then, there is no

control as to who on the secure network is allowed to access and

GET/SET (read/change/create/delete) the objects in this MIB.

It is recommended that the implementers consider the security

features as provided by the SNMPv3 framework.  Specifically, the use

of the User-based Security Model RFC 2574 [RFC2574] and the View-

based Access Control Model RFC 2575 [RFC2575] is recommended.

It is then a customer/user responsibility to ensure that the SNMP

entity giving access to instances of this MIB, is properly

configured to give access to the objects only to those principals

(users) that have legitimate rights to indeed GET or SET

(change/create/delete) them.

9.  Applicability

This MIB is technology-specific, designed to meet the needs of 802.11 wireless access points that implement IEEE 802.1X authentication.

A wired 802.3 switch that used non-standard (for Ethernet) station-

based IEEE 802.1X authentication could theoretically use the extensions

MIB defined in this document.

10.  Acknowledgments

This MIB is based on work by the technical staff of Enterasys

Networks, Inc.

For alignment with IEEE 802.1X, many objects in this extended MIB

are based on IEEE 802.1X base MIB objects.

11.  References:

[IEEE8021X] "Port-Based Network Access Control", IEEE Std 802.1X, 2001.

[RFC2119]   Bradner, S., "Key words for use in RFCs to Indicate

Requirement Levels", RFC 2119, March 1997.

[RFC2571]   Harrington, D., Presuhn, R., and B. Wijnen, "An Architecture

for Describing SNMP Management Frameworks", RFC 2571, April

1999.

[RFC1155]   Rose, M., and K. McCloghrie, "Structure and Identification

of Management Information for TCP/IP-based Internets", STD

16, RFC 1155, May 1990.

[RFC1212]   Rose, M., and K. McCloghrie, "Concise MIB Definitions", STD

16, RFC 1212, March 1991.

[RFC1215]   M. Rose, "A Convention for Defining Traps for use with the

SNMP", RFC 1215, March 1991.

[RFC2578]   McCloghrie, K., Perkins, D., Schoenwaelder, J., Case, J.,

Rose, M., and S. Waldbusser, "Structure of Management

Information Version 2 (SMIv2)", STD 58, RFC 2578, April

1999.

[RFC2579]   McCloghrie, K., Perkins, D., Schoenwaelder, J., Case, J.,

Rose, M., and S. Waldbusser, "Textual Conventions for

SMIv2", STD 58, RFC 2579, April 1999.

[RFC2580]   McCloghrie, K., Perkins, D., Schoenwaelder, J., Case, J.,

Rose, M., and S. Waldbusser, "Conformance Statements for

SMIv2", STD 58, RFC 2580, April 1999.

[RFC1157]   Case, J., Fedor, M., Schoffstall, M., and J. Davin, "Simple

Network Management Protocol", STD 15, RFC 1157, May 1990.

[RFC1901]   Case, J., McCloghrie, K., Rose, M., and S. Waldbusser,

"Introduction to Community-based SNMPv2", RFC 1901, January

1996.

[RFC1906]   Case, J., McCloghrie, K., Rose, M., and S. Waldbusser,

"Transport Mappings for Version 2 of the Simple Network

Management Protocol (SNMPv2)", RFC 1906, January 1996.

[RFC2572]   Case, J., Harrington D., Presuhn R., and B. Wijnen, "Message

Processing and Dispatching for the Simple Network Management

Protocol (SNMP)", RFC 2572, April 1999.

[RFC2574]   Blumenthal, U., and B. Wijnen, "User-based Security Model

(USM) for version 3 of the Simple Network Management

Protocol (SNMPv3)", RFC 2574, April 1999.

[RFC1905]   Case, J., McCloghrie, K., Rose, M., and S. Waldbusser,

"Protocol Operations for Version 2 of the Simple Network

Management Protocol (SNMPv2)", RFC 1905, January 1996.

[RFC2573]   Levi, D., Meyer, P., and B. Stewart, "SNMPv3 Applications",

RFC 2573, April 1999.

[RFC2575]   Wijnen, B., Presuhn, R., and K. McCloghrie, "View-based

Access Control Model (VACM) for the Simple Network

Management Protocol (SNMP)", RFC 2575, April 1999.

[RFC2570]   Case, J., Mundy, R., Partain, D., and B. Stewart,

"Introduction to Version 3 of the Internet-standard Network

Management Framework", RFC 2570, April 1999.

12. Version History

DRAFT -01

Initial version.