IANAifType ::= TEXTUAL-CONVENTION
       STATUS       current
       DESCRIPTION
               "This data type is used as the syntax of the ifType
               object in the (updated) definition of MIB-II's
               ifTable.

               The definition of this textual convention with the
               addition of newly assigned values is published
               periodically by the IANA, in either the Assigned
               Numbers RFC, or some derivative of it specific to
               Internet Network Management number assignments.  (The
               latest arrangements can be obtained by contacting the
               IANA.)

               Requests for new values should be made to IANA via
               email (iana@iana.org).

               The relationship between the assignment of ifType
               values and of OIDs to particular media-specific MIBs
               is solely the purview of IANA and is subject to change
               without notice.  Quite often, a media-specific MIB's
               OID-subtree assignment within MIB-II's 'transmission'
               subtree will be the same as its ifType value.
               However, in some circumstances this will not be the
               case, and implementors must not pre-assume any
               specific relationship between ifType values and
               transmission subtree OIDs."
       SYNTAX  INTEGER {
                   other(1),          -- none of the following
                   regular1822(2),
                   hdh1822(3),
                   ddnX25(4),
                   rfc877x25(5),
                   ethernetCsmacd(6), -- for all ethernet-like interfaces,
                                      -- regardless of speed, as per RFC3635
                   iso88023Csmacd(7), -- Deprecated via RFC-draft-ietf-hubmib-etherif-mib-v3  ethernetCsmacd (6) should be used instead
                   iso88024TokenBus(8),
                   iso88025TokenRing(9),
                   iso88026Man(10),
                   starLan(11), -- Deprecated via RFC-draft-ietf-hubmib-etherif-mib-v3  ethernetCsmacd (6) should be used instead
                   proteon10Mbit(12),
                   proteon80Mbit(13),
                   hyperchannel(14),
                   fddi(15),
                   lapb(16),
                   sdlc(17),
                   ds1(18),            -- DS1-MIB
                   e1(19),             -- Obsolete see DS1-MIB
                   basicISDN(20),
                   primaryISDN(21),
                   propPointToPointSerial(22), -- proprietary serial
                   ppp(23),
                   softwareLoopback(24),
                   eon(25),            -- CLNP over IP 
                   ethernet3Mbit(26),
                   nsip(27),           -- XNS over IP
                   slip(28),           -- generic SLIP
                   ultra(29),          -- ULTRA technologies
                   ds3(30),            -- DS3-MIB
                   sip(31),            -- SMDS, coffee
                   frameRelay(32),     -- DTE only. 
                   rs232(33),
                   para(34),           -- parallel-port
                   arcnet(35),         -- arcnet
                   arcnetPlus(36),     -- arcnet plus
                   atm(37),            -- ATM cells
                   miox25(38),
                   sonet(39),          -- SONET or SDH 
                   x25ple(40),
                   iso88022llc(41),
                   localTalk(42),
                   smdsDxi(43),
                   frameRelayService(44),  -- FRNETSERV-MIB
                   v35(45),
                   hssi(46),
                   hippi(47),
                   modem(48),          -- Generic modem
                   aal5(49),           -- AAL5 over ATM
                   sonetPath(50),
                   sonetVT(51),
                   smdsIcip(52),       -- SMDS InterCarrier Interface
                   propVirtual(53),    -- proprietary virtual/internal
                   propMultiplexor(54),-- proprietary multiplexing
                   ieee80212(55),      -- 100BaseVG
                   fibreChannel(56),   -- Fibre Channel
                   hippiInterface(57), -- HIPPI interfaces     
                   frameRelayInterconnect(58), -- Obsolete use either
                                       -- frameRelay(32) or 
                                       -- frameRelayService(44).
                   aflane8023(59),     -- ATM Emulated LAN for 802.3
                   aflane8025(60),     -- ATM Emulated LAN for 802.5
                   cctEmul(61),        -- ATM Emulated circuit          
                   fastEther(62),      -- Obsoleted via RFC-draft-ietf-hubmib-etherif-mib-v3  ethernetCsmacd (6) should be used instead
                   isdn(63),           -- ISDN and X.25           
                   v11(64),            -- CCITT V.11/X.21             
                   v36(65),            -- CCITT V.36                  
                   g703at64k(66),      -- CCITT G703 at 64Kbps
                   g703at2mb(67),      -- Obsolete see DS1-MIB
                   qllc(68),           -- SNA QLLC                    
                   fastEtherFX(69),    -- Obsoleted via RFC-draft-ietf-hubmib-etherif-mib-v3  ethernetCsmacd (6) should be used instead
                   channel(70),        -- channel                     
                   ieee80211(71),      -- radio spread spectrum       
           ibm370parChan(72),  -- IBM System 360/370 OEMI Channel
                   escon(73),          -- IBM Enterprise Systems Connection
                   dlsw(74),           -- Data Link Switching
                   isdns(75),          -- ISDN S/T interface
                   isdnu(76),          -- ISDN U interface
                   lapd(77),           -- Link Access Protocol D
                   ipSwitch(78),       -- IP Switching Objects
                   rsrb(79),           -- Remote Source Route Bridging
                   atmLogical(80),     -- ATM Logical Port
                   ds0(81),            -- Digital Signal Level 0
                   ds0Bundle(82),      -- group of ds0s on the same ds1
                   bsc(83),            -- Bisynchronous Protocol
                   async(84),          -- Asynchronous Protocol
                   cnr(85),            -- Combat Net Radio
                   iso88025Dtr(86),    -- ISO 802.5r DTR
                   eplrs(87),          -- Ext Pos Loc Report Sys
                   arap(88),           -- Appletalk Remote Access Protocol
                   propCnls(89),       -- Proprietary Connectionless Protocol
                   hostPad(90),        -- CCITT-ITU X.29 PAD Protocol
                   termPad(91),        -- CCITT-ITU X.3 PAD Facility
                   frameRelayMPI(92),  -- Multiproto Interconnect over FR
                   x213(93),           -- CCITT-ITU X213
                   adsl(94),           -- Asymmetric Digital Subscriber Loop
                   radsl(95),          -- Rate-Adapt. Digital Subscriber Loop
                   sdsl(96),           -- Symmetric Digital Subscriber Loop
                   vdsl(97),           -- Very H-Speed Digital Subscrib. Loop
                   iso88025CRFPInt(98), -- ISO 802.5 CRFP
                   myrinet(99),        -- Myricom Myrinet
                   voiceEM(100),       -- voice recEive and transMit
                   voiceFXO(101),      -- voice Foreign Exchange Office
                   voiceFXS(102),      -- voice Foreign Exchange Station
                   voiceEncap(103),    -- voice encapsulation
                   voiceOverIp(104),   -- voice over IP encapsulation
                   atmDxi(105),        -- ATM DXI
                   atmFuni(106),       -- ATM FUNI
   atmIma (107),       -- ATM IMA    
                   pppMultilinkBundle(108), -- PPP Multilink Bundle
                   ipOverCdlc (109),   -- IBM ipOverCdlc
   ipOverClaw (110),   -- IBM Common Link Access to Workstn
                   stackToStack (111), -- IBM stackToStack
                   virtualIpAddress (112), -- IBM VIPA
   mpc (113),          -- IBM multi-protocol channel support
   ipOverAtm (114),    -- IBM ipOverAtm
   iso88025Fiber (115), -- ISO 802.5j Fiber Token Ring
   tdlc (116),        -- IBM twinaxial data link control
   gigabitEthernet (117), -- Obsoleted via RFC-draft-ietf-hubmib-etherif-mib-v3  ethernetCsmacd (6) should be used instead
   hdlc (118),         -- HDLC
   lapf (119),        -- LAP F
   v37 (120),        -- V.37
   x25mlp (121),       -- Multi-Link Protocol
   x25huntGroup (122), -- X25 Hunt Group
   trasnpHdlc (123),   -- Transp HDLC
   interleave (124),   -- Interleave channel
   fast (125),         -- Fast channel
   ip (126),        -- IP (for APPN HPR in IP networks)
   docsCableMaclayer (127),  -- CATV Mac Layer
   docsCableDownstream (128), -- CATV Downstream interface
   docsCableUpstream (129),  -- CATV Upstream interface
   a12MppSwitch (130), -- Avalon Parallel Processor
   tunnel (131),       -- Encapsulation interface
   coffee (132),       -- coffee pot
   ces (133),          -- Circuit Emulation Service
   atmSubInterface (134), -- ATM Sub Interface
   l2vlan (135),       -- Layer 2 Virtual LAN using 802.1Q
   l3ipvlan (136),     -- Layer 3 Virtual LAN using IP
   l3ipxvlan (137),    -- Layer 3 Virtual LAN using IPX
   digitalPowerline (138), -- IP over Power Lines 
   mediaMailOverIp (139), -- Multimedia Mail over IP
   dtm (140),        -- Dynamic syncronous Transfer Mode
   dcn (141),    -- Data Communications Network
   ipForward (142),    -- IP Forwarding Interface
   msdsl (143),       -- Multi-rate Symmetric DSL
   ieee1394 (144), -- IEEE1394 High Performance Serial Bus
   if-gsn (145),       --   HIPPI-6400 
   dvbRccMacLayer (146), -- DVB-RCC MAC Layer
   dvbRccDownstream (147),  -- DVB-RCC Downstream Channel
   dvbRccUpstream (148),  -- DVB-RCC Upstream Channel
   atmVirtual (149),   -- ATM Virtual Interface
   mplsTunnel (150),   -- MPLS Tunnel Virtual Interface
   srp (151), -- Spatial Reuse Protocol 
   voiceOverAtm (152),  -- Voice Over ATM
   voiceOverFrameRelay (153),   -- Voice Over Frame Relay 
   idsl (154), -- Digital Subscriber Loop over ISDN
   compositeLink (155),  -- Avici Composite Link Interface
   ss7SigLink (156),     -- SS7 Signaling Link 
   propWirelessP2P (157),  --  Prop. P2P wireless interface
   frForward (158),    -- Frame Forward Interface
   rfc1483 (159), -- Multiprotocol over ATM AAL5
   usb (160), -- USB Interface
   ieee8023adLag (161),  -- IEEE 802.3ad Link Aggregate
                   bgppolicyaccounting (162), -- BGP Policy Accounting
                   frf16MfrBundle (163), -- FRF .16 Multilink Frame Relay 
                   h323Gatekeeper (164), -- H323 Gatekeeper
                   h323Proxy (165), -- H323 Voice and Video Proxy
                   mpls (166), -- MPLS                   
                   mfSigLink (167), -- Multi-frequency signaling link
                   hdsl2 (168), -- High Bit-Rate DSL - 2nd generation
                   shdsl (169), -- Multirate HDSL2
                   ds1FDL (170), -- Facility Data Link 4Kbps on a DS1
                   pos (171), -- Packet over SONET/SDH Interface
                   dvbAsiIn (172), -- DVB-ASI Input
                   dvbAsiOut (173), -- DVB-ASI Output 
                   plc (174), -- Power Line Communtications
                   nfas (175), -- Non Facility Associated Signaling
                   tr008 (176), -- TR008
                   gr303RDT (177), -- Remote Digital Terminal
                   gr303IDT (178), -- Integrated Digital Terminal
                   isup (179), -- ISUP
                   propDocsWirelessMaclayer (180), -- Cisco proprietary Maclayer
                   propDocsWirelessDownstream (181), -- Cisco proprietary Downstream
                   propDocsWirelessUpstream (182), -- Cisco proprietary Upstream
                   hiperlan2 (183), -- HIPERLAN Type 2 Radio Interface
                   propBWAp2Mp (184), -- PropBroadbandWirelessAccesspt2multipt
                   sonetOverheadChannel (185), -- SONET Overhead Channel
                   digitalWrapperOverheadChannel (186), -- Digital Wrapper
                   aal2 (187), -- ATM adaptation layer 2
                   radioMAC (188), -- MAC layer over radio links
                   atmRadio (189), -- ATM over radio links   
                   imt (190), -- Inter Machine Trunks
                   mvl (191), -- Multiple Virtual Lines DSL
                   reachDSL (192), -- Long Reach DSL
                   frDlciEndPt (193), -- Frame Relay DLCI End Point
                   atmVciEndPt (194), -- ATM VCI End Point
                   opticalChannel (195), -- Optical Channel
                   opticalTransport (196), -- Optical Transport
                   propAtm (197), --  Proprietary ATM       
                   voiceOverCable (198), -- Voice Over Cable Interface
                   infiniband (199), -- Infiniband
                   teLink (200), -- TE Link
                   q2931 (201), -- Q.2931
                   virtualTg (202), -- Virtual Trunk Group
                   sipTg (203), -- SIP Trunk Group
                   sipSig (204), -- SIP Signaling   
                   docsCableUpstreamChannel (205), -- CATV Upstream Channel
                   econet (206), -- Acorn Econet
                   pon155 (207), -- FSAN 155Mb Symetrical PON interface
                   pon622 (208), -- FSAN622Mb Symetrical PON interface
                   bridge (209), -- Transparent bridge interface
                   linegroup (210), -- Interface common to multiple lines    
                   voiceEMFGD (211), -- voice E&M Feature Group D
                   voiceFGDEANA (212), -- voice FGD Exchange Access North American
                   voiceDID (213), -- voice Direct Inward Dialing
                   mpegTransport (214), -- MPEG transport interface
                   sixToFour (215), -- 6to4 interface (DEPRECATED)
                   gtp (216), -- GTP (GPRS Tunneling Protocol)
                   pdnEtherLoop1 (217), -- Paradyne EtherLoop 1
                   pdnEtherLoop2 (218), -- Paradyne EtherLoop 2
                   opticalChannelGroup (219), -- Optical Channel Group
                   homepna (220), -- HomePNA ITU-T G.989    
                   gfp (221), -- Generic Framing Procedure (GFP)
                   ciscoISLvlan (222), -- Layer 2 Virtual LAN using Cisco ISL
                   actelisMetaLOOP (223), -- Acteleis proprietary MetaLOOP High Speed Link 
                   fcipLink (224), -- FCIP Link 
                   rpr (225), -- Resilient Packet Ring Interface Type
                   qam (226), -- RF Qam Interface
                   lmp (227), -- Link Management Protocol
                   cblVectaStar, (228) -- Cambridge Broadband Limited VectaStar
                   docsCableMCmtsDownstream, (229) -- CATV Modular CMTS Downstream Interface
                   adsl2 (230) -- Asymmetric Digital Subscriber Loop Version 2
       }