17.3.2.2 Relationship to the IF-MIB

<<Add to the end of this clause>>

The BRIDGE MIB also defines a new interface type called an internal LAN (I-LAN).  This interface is identified by an ifIndex and is intended to help manage internal associations between bridging components.  The BRIDGE-MIB provides the ability to create and destroy I-LAN interfaces.  When an I-LAN interface is created, a corresponding row is created in the ifTable.  The ifType of this interface is XXX.

I-LAN interfaces support stacking through the ifStackTable.  Through this mechanism, the I-LAN interface can create internal connections between components.  For example, an I-LAN interface can connect two C-components in a system.  To complete this process, the following steps are required:

- Create C-component 1

- Create C-VLAN port 1 on component 1

- Create a bridge port interface (type 209) identified by ifIndex 1 using a external mechanism

- Associate C-VLAN port 1 on component 1 to bridge port interface 1

- Create an I-LAN interface identified with ifIndex 2

- Stack bridge port interface 1 on the I-LAN interface 2

- Create C-component 2

- Create C-VLAN port 1 on component 2

- Create a bridge port interface (type 209) identified by ifIndex 3 using a external mechanism

- Associate C-VLAN port 1 on component 2 to bridge port interface 3

- Stack bridge port interface 3 on the I-LAN interface 2

The result of the previous configuration steps are displayed in the Figure XXX below.

Figure 17-XXX

As a result, traffic can now be forwarded between C-Component 1 and C-Component 2 through the internal LAN.

17.3.8 Relationship of the IEEE8021-PBB MIB to other MIB modules

<<Add to the end second last paragraph>>

Therefore, when a PIP is created, a row is also created in the ifTable identified by the PIP’s ifIndex.  The ifType of this interface is XXX.

<<Add to the end of the clause>>

A PIP supports stacking through the ifStackTable.  For external PIP interfaces, a PIP can be stacked on an Ethernet interface (type 6).  If the PIP is internal, it can be stacked on an I-LAN interface to create a connection between an I-Component and a B-component.  An example of how this process is completed is described below:

- Create I-component 1

- Create VIP 1 on component 1

- Create a PIP identified by ifIndex 1

- Associate VIP 1 on component 1 to the PIP

- Create an I-LAN interface identified with ifIndex 2

- Stack PIP 1 on the I-LAN interface 2

- Create B-component 2

- Create CBP 1 on component 2

- Associate CBP 1 on component 2 to I-LAN interface 2

The result of the previous configuration steps are displayed in the Figure XXX below.

Figure 17-XXX

As a result, traffic can now be forwarded between I-Component 1 and B-Component 2 once a service is provisioned.