| Title: | Liaison to IEEE 802.1 Higher Layer Working Group & ITU-T SG12/Q17, SG13/Q5 and SG15/Q9 from MEF Accurate Frame Loss Ad Hoc Group |
| --- | --- |
| Date: | 24 July 2008 |
| Location: | Boston MA, USA |
| - Contacts: | - Nan Chen, President MEF (nan@chen.net) - Bill Bjorkman, VP, TC Co-Chair (bill@metroethernetforum.net) - Paul Bottorff, TC Co-Chair (pbottorf@nortel.com) |
| - To: | - IEEE 802.1 Higher Layer Working Group |
|  | - ITU-T Study Group 12, Question 17 - ITU-T Study Group 13, Question 5 - ITU-T Study Group 15, Question 9 |
|  | - Tony Jeffree, 802.1 Chair (tony@jeffree.co.uk) - Al Morton, Rapporteur Q17/SG12 (acmorton@att.com) - Huub van Helvoort, Rapporteur Q5/SG13 (hhelvoort@huawei.com) - Ghani Abbas, Rapporteur Q9/SG15 (ghani.abbas@ericsson.com) |
| From: | Metro Ethernet Forum |

Mr. Jeffree,

Mr. Morton,

Mr. Helvoort,

Mr. Abbas:

The Accurate Frame Loss ad hoc committee of the Metro Ethernet Forum was established to review a problem for reconciling customers’ and providers’ measurements of frame loss in a point-to-point Metro Ethernet service.  We believe that IEEE & ITU-T should be made aware of the issue and can help solve this problem, to the benefit of the users of 802.1Q and 802.1ad bridges and users of Y.1731 OAM employed by the customers and providers of Metro Ethernet services.  The particular problem is described more fully in the attached MEF contributions, but can be summarized using the following scenario:

- IEEE 802.1Q bridges are the customer equipment employed at each end of a point-to-point Metro Ethernet service supplied by a provider network consisting of IEEE 802.1ad provider bridges.

- In each direction (near-end and far-end), the customer wishes to measure frame loss using synthetic and/or bare traffic frame loss.  The customer will use either LMM/LMR and/or CCM frame loss measurements standardized in Y.1731 and the ETH-LM function from G.8021.

- In each direction (near-end and far-end), the service provider will also measure frame loss using synthetic and/or bare traffic frame loss.  The service provider will use either LMM/LMR and/or CCM frame loss measurements standardized in Y.1731 and the ETH-LM function from G.8021.

- The customer and provider need to reconcile those counts, in order to verify that the Service Level Agreement (SLA) has been met.  This frame loss measurement can also be used to perform frame loss fault isolation.

- Frames that are counted at the Up/Down VLAN MEP (slide 6 & 7 – Queues A, B, E & F) are subject to the queue for potential discard due to various reasons defined in IEEE 802.1Q Queue Management 8.6.7. These frames, which are lost in the queues within the UNI-C or UNI-N, are counted by the Customer [internal MEP counter] but not by the Provider’s MEP using Y.1731/G.8021.

At present, the IEEE 802.1 Bridge Port Stack specifies that the queue resides below the “Down VID MEP”.  Within the MEP, the Internal MEP counter counts all VID ETH_CI_D/P/DE frames within the ETH Diagnostic Flow Termination Function.  These same “counted” frames are then subject to discard according to the rules in the 8.6.7 Queue Management IEEE 801.Q.

This local loss manifests itself as a difference in the frame counts between the MEPs on the Subscriber MA, and the Subscriber MEP on an 802.1Q bridge and the Service Provider MEP on an 802.1ad bridge.  The loss in the former case is detected through the use of the ETH-LM function.  However, the loss cannot be isolated without involving the Service Provider and a period of an understood measurement period.  By providing visibility of local loss, it can be isolated by the Subscriber prior to involving the Service Provider.

The ad hoc group has reviewed a number of possible solutions but is unable to agree on the best course of action.  The solutions can be summarized by the following:

- Down VID MEP internal counter adjustment: The transmit counters should exclude frames lost in the EVC ingress UNI-C between the Down VID MEP and the media from the count of frames sent (Queue A & F).  On the other hand, an Up VID MEP should exclude frames lost between the Up VID MEP and the EVC egress UNI-N media from the count of frames sent (Queue B & E). The internal counter could use the IEEE 802.1Q 12.6.1.1.3 Frame Discard counters to adjust its internal transmit count.  This would become impossible in all cases as the IEEE 802.1Q 12.6.1.1.2 (b) states that the Frame Discard counter per VLAN ID is optional. Without the per VIDs Frame Discard counters, there would be no way to adjust the correct Down or Up VID MEP internal counter.

- Locate another VLAN MEP below the queue.

  - While it is understood the precautionary reasoning for placing the VLAN MEP above the queue, such that the SOAM frames that pass through the 802.1 stack are prevented from flooding the EVC capacity, the VLAN MEP’s location with respect to the queue permits the counted frames to be subject to discard.

  - However, by placing an additional MEP below the queue it is unclear how the counters above and below the queues would be coordinated.

- Implement the appropriate counters in the UNI-C and UNI-N to count any frames lost on transmission or reception between the MEP and the media. The IEEE 802.1Q 12.6.1.1.3 Frame Discard counter is one such counter. Some of these counters may already be defined and others may need to be added.  These Counters should be defined as part of the Bridging (802.1), MAC (802.3 et.al.), and PHY standards where the losses can occur.  Implementation of these counters identifies loss, and where it is located, but they do not correct the ETH-LM counters. Nor would these counters be synchronized with the MEP counters used in ETH-LM.

These possible solutions are not ideal and a consideration of new counters may be the only logical course of action.

Attach: Accurate Frame Loss Adhoc Findings and Recommendations - MEF 26050_002