| INTERNATIONAL TELECOMMUNICATION UNION |  | STUDY GROUP 15 |
| --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2009-2012 |  | TD 650 Rev.1 (PLEN/15) |
|  |  | English only Original: English |
| Question(s): | 10/15 | 10-21 September 2012 |
| TD |  |  |
| Source: | Editor G.8011/Y.1307 |  |
| Title: | Draft Recommendation ITU-T G.8011/Y.1307 (version 3) (for Consent, September 2012) |  |

- Draft ITU-T Recommendation G.8011/Y.1307 version 3 (for Consent)

- Ethernet Service Characteristics

- Revision history:

| Rev | Document number | Date | Description |
| --- | --- | --- | --- |
| 0.1 | wd13 TD655/WP3 | Sept 30, 2011 | Published version of G.8011-2009 (version 2) - Addition of Editor`s notes based on wd9 proposal |
| 0.2 | Wd27 | Dec 12, 2011 | Initial drafting result based on editor’s notes |
| 0.3 | wd13 | Apr 13, 2012 | Additional drafting (including figures 6-x) |
| 0.4 0.5 | wd13r1 wd13r2 | Apr 24, 2012 Apr 27, 2012 | Result of drafting from interim meeting implementing all editor’s notes aligning with MEF 10.2 (and deferring to its service attribute definitions) and additional clarifications including - Removal of “traffic separation” attribute - Change Annex A to Appendix and update - Delete Appendix I as it is outdated - Update table 7-4 on Ethernet OAM |
| 0.6 | TD650/PLEN | May 11, 2012 | Document submitted for Consent |
| 0.7 |  | Sept 17, 2012 | Modifications based on MEF liaison (TD 722/3) including - Removal of “UNI Type” EC attribute - Removal of “connectivity monitoring” EC attributes - Addition of new connectivity monitoring overview clause |

Draft Recommendation ITU-T G.8011/Y.1307 version 3 (for Consent) Ethernet service characteristics

Summary Recommendation ITU-T G.8011/Y.1307 describes a framework for defining network-oriented characteristics of Ethernet services that is aligned with MEF 10.2. The framework is based on the modelling of Ethernet layer networks described in Recommendation ITU-T G.8010/Y.1306. The attribute sets introduced in this framework (Ethernet virtual connection (EVC), Ethernet connection (EC), user-to-network interface (UNI) and network-to-network interface (NNI)) are intended to be used to create numerous specific Ethernet services.

Keywords Ethernet, Ethernet connection, Ethernet service, framework, network-to-network interface, user-to-network interface.

CAUTION !

Draft  RECOMMENDATION

-

CONTENTS

Page

1	Scope		1

2	References		1

3	Definitions		2

4	Abbreviations		3

5	Conventions		4

6	Ethernet services		4

6.1	Ethernet service areas		4

6.2	Ethernet service aspects		7

6.3	Ethernet services views		8

7	Ethernet virtual connection attributes		9

7.1	EVC		10

7.2	EC 		13

8	Ethernet UNI attributes		17

8.1	UNI		18

8.2	EC per UNI		25

9	Ethernet NNI attributes		26

9.1	INNI		26

9.2	ENNI		27

Annex A – Extended UNI		30

A.1	Introduction		30

A.2	Extended UNI-N		30

Appendix I – Relationship of G.8011/Y.1307 attributes to [ITU-T G.8010]		28

A.1	Introduction		28

A.2	Ethernet connection attributes		28

A.3	Interface attributes		29

Appendix II – EC topology		31

Recommendation ITU-T G.8011/Y.1307

Ethernet service characteristics

# 1	Scope

This Recommendation defines a framework to describe a set of Ethernet services based on [MEF 10.2]. The framework consists of a set of attributes for each of Ethernet virtual connection, Ethernet connection, Ethernet UNI and Ethernet NNI. The resulting services that can be defined do not refer to a particular network technology implementation and are supported by the Ethernet layer architecture model presented in [ITU-T G.8010] and [MEF 12.1].

This Recommendation provides the framework to define different services to carry an Ethernet link connection. The Ethernet subnetwork for each of the services introduced in this Recommendation is defined in a companion set of Recommendations ITU-T G.8011.x/Y.1307.x based on [MEF 6.1].

-

2	References

The following ITU-T Recommendations and other references contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published. The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

[ITU-T G.809]	Recommendation ITU-T G.809 (2003), Functional architecture of connectionless layer networks.

[ITU-T G.8001]	Recommendation ITU-T G.8001/Y.1354 (2011), Terms and definitions for Ethernet frames over Transport.

[ITU-T G.8010]	Recommendation ITU-T G.8010/Y.1306 (2012), Architecture of Ethernet layer networks.

[ITU-T G.8011.1]	Recommendation ITU-T G.8011.1/Y.1307.1 (2009), Ethernet private line service.

[ITU-T G.8011.2]     	Recommendation ITU-T G.8011.2/Y.1307.2 (2009): Ethernet virtual private line service

[ITU-T G.8011.3]      	Recommendation ITU-T G.8011.3/Y.1307.3 (2010): Ethernet virtual private LAN service

[ITU-T G.8011.4]      	Recommendation ITU-T G.8011.4/Y.1307.4 (2012): Ethernet virtual private tree service

[ITU-T G.8011.5]      	Recommendation ITU-T G.8011.5/Y.1307.5 (2010): Ethernet private LAN service

[ITU-T G.8012]	Recommendation ITU-T G.8012/Y.1308 (2004), Ethernet UNI and Ethernet NNI.

[ITU-T G.8012.1]	Recommendation ITU-T G.8012.1/Y.1308.1 (2012), Interfaces for the Ethernet Transport Network.

[ITU-T G.8021]	Recommendation ITU-T G.8021/Y.1341 (2010), Characteristics of Ethernet transport network equipment functional blocks.

[ITU-T G.8021.1]	Recommendation ITU-T G.8021.1/Y.1341.1 (2012), Types and Characteristics of Ethernet equipment.

[ITU-T Y.1731]				Recommendation ITU-T G.8013/Y.1731 (2011), OAM functions and mechanisms for Ethernet based networks.

[IEEE 802.1Q]	IEEE 802.1Q (2011), IEEE standard for Local and Metropolitan Area Networks – Virtual Bridged Local Area Networks.

[IEEE 802.1AX]	IEEE 802.1X (2008), IEEE Standard for Local and Metropolitan Area Networks – Link Aggregation.

[IEEE 802.3]	IEEE 802.3 (2008), IEEE Standard for Information technology – Telecommunications and Information exchange between systems – Local and metropolitan area networks – Specific requirements – Part 3: Carrier Sense Multiple Access with Collision Detection (CSMA/CD) Access Method and Physical Layer Specifications.

[MEF 6.1]	The Metro Ethernet Forum MEF (2008), Technical Specification MEF 10.2 – Ethernet Services Definitions – Phase 2.

[MEF 10.2]	The Metro Ethernet Forum MEF (2009), Technical Specification MEF 10.2 – Ethernet Services Attributes – Phase 2.

[MEF 12.1]	The Metro Ethernet Forum MEF (2010), Carrier Ethernet Network Architecture Framework Part 2: Ethernet Services Layer - Basic Elements.

[MEF 26.1]	The Metro Ethernet Forum MEF (2012), External Network Network Interface (ENNI)–Phase 2

[MEF 30]	The Metro Ethernet Forum MEF (2012), Service OAM Fault Management Implementation Agreement

[MEF 35]	The Metro Ethernet Forum MEF (2012), Service OAM Performance Monitoring Implementation Agreement

# 3	Definitions

This Recommendation uses the following terms defined in [ITU-T G.8010]:

3.1	ETH link

3.2	Subnetwork connection

3.3	Link

3.4	Link connection

3.5	Subnetwork

3.6	Traffic conditioning function

This Recommendation uses the following terms defined in [ITU-T G.809]:

3.7	Flow point

3.8	Flow termination

3.9	Termination flow point

This Recommendation uses the following terms defined in [ITU-T G.8001]:

3.10	Access link

3.11	Customer

3.12	Ethernet connection (EC)

3.13	Ethernet service

3.14	Ethernet service area

3.15	Ethernet service instance

3.16	NNI

3.17	UNI

This Recommendation uses the following terms defined in [MEF 10.2]:

3.18	Ethernet virtual connection (EVC)

3.19	Service frame

# 4	Abbreviations

This Recommendation uses the following abbreviations:

CFM		Connectivity Fault Management

CI 		Characteristic Information

EC		Ethernet Connection

ETH		Ethernet MAC Layer Network

ETY		Ethernet PHY Layer Network

EVC		Ethernet Virtual Connection

FP		Flow Point

GARP		Generic Attribute Registration Protocol

GMRP		GARP Multicast Registration Protocol

GVRP		GARP VLAN Registration Protocol

ID		Identification

LACP		Link Aggregation Control Protocol

LAMP		Link Aggregation Marker Protocol

LBM		LoopBack Message

LBR		LoopBack Reply

LC		Link Connection

LTM		Link Trace Message

LTR		Link Trace Reply

MAC		Media Access Control

MEG		Maintenance Entity Group

MEP		MEG End Point

MIP		MEG Intermediate Point

MSTP		Multiple Spanning Tree Protocol

MVRP		Multiple VLAN Registration Protocol

NNI		Network-to-Network Interface

PHY		Physical device

RSTP		Rapid Spanning Tree Protocol

SN		Subnetwork

SNC		Subnetwork connection

STP		Spanning Tree Protocol

TFP		Termination Flow Point

UNI		User-to-Network Interface

UNI-C		Customer side of UNI

UNI-N		Network side of UNI

VLAN		Virtual Local Area Network

# 5	Conventions

The diagrammatic convention for Ethernet services described in this Recommendation is that of [ITU-T G.8010].

Further, the use of ETH link in this Recommendation is that of [ITU-T G.8010]. Specifically, ETH link is a generalization that collectively refers to ETH link and ETH link connection.

# 6	Ethernet services

This Recommendation does not define Ethernet services, but provides a framework from which services can be defined. This framework is based on the Ethernet transport architecture described in [ITU-T G.8010]. Unlike the original version of this Recommendation, the base Ethernet services attributes and their definitions are imported from [MEF 10.2] to ensure alignment. This Recommendation adds further explanation to some of these attributes to clearly show the relationship to [ITU-T G.8010]. In addition, several ITU-T specific attributes are defined to provide a superset of attributes.

## 6.1	Ethernet service areas

Ethernet service areas identify the various portions of a network that support an Ethernet service instance. A simple model of an Ethernet network (for a single carrier's network) is shown in Figure 6-1. Three Ethernet service areas are identified: access (UNI-C to UNI-N), edge-to-edge (the Ethernet connection from UNI-N to UNI-N), and end-to-end (UNI-C to UNI-C).

Figure 6-1 also shows a three-tier relationship. The three tiers (equipment at the top, ETH layer in the middle and ETY layer at the bottom) allow a clear identification of how the equipment functions map onto the ETH and ETY layers. Note that the customer equipment is shown as a flow point on the right and a subnetwork (SN) on the left of the diagram, to illustrate that both are possible.

It is further shown that the UNI reference point occurs in the middle of the access link, or more precisely that the UNI is a reference point whose functionality is split into customer (UNI-C) and network (UNI-N) components. Additional UNI details are defined in [ITU-T G.8012] and [ITU-T G.8012.1].

The relationship between the Ethernet service areas illustrated in Figure 6-1 (and in Figures 6-2 and 6-3), and the maintenance entities (ME) described in [ITU-T G.8010] is shown in Table 6-1:

Table 6-1 – Ethernet areas and maintenance entities

| Ethernet area | Maintenance entity |
| --- | --- |
| Access link | Access link |
| End-to-end | UNI-C to UNI-C |
| Edge-to-edge | UNI-N to UNI-N |

Note that a ME is point to point whilst the Ethernet service may be multipoint to multipoint. Therefore, in general, an Ethernet service area will contain multiple ME instances (these are referred to as a maintenance entity group).

Figure 6-1 – Single-provider view of Ethernet service areas

Figure 6-2 shows a simple network where the service provider's network is a single link. It introduces the NNI link, which is the link between the UNI-Ns and the NNI reference point.

Similar to the UNI, a demarcation occurs in the middle of the NNI link, or more precisely the NNI is a reference point whose functionality is split in half – either between different providers or within the same provider. Additional NNI details are defined in [ITU-T G.8012] and [ITU-T G.8012.1].

Figure 6-2 – Single-provider with NNI view of Ethernet service areas

Figure 6-3 shows the case of two interconnected operators and illustrates the implications to the NNI. Notably, this is an inter-domain NNI as defined in [ITU-T G.8012] and [ITU-T G.8012.1].

Figure 6-3 – Multi provider with NNI view of Ethernet service areas

An additional network view is shown in Annex A. Note that more complex networks beyond those described are for further study.

## 6.2	Ethernet service aspects

An Ethernet service provider provides Ethernet services between the UNI reference points shown in Figure 6-1, using the topology of the Ethernet network. This topology may be composed of a simple Ethernet link or alternatively one or more subnetworks and the links between them. Five sets of attributes can be derived from the Ethernet service areas figures to form a framework to define a specific Ethernet service. The locations of these five attribute sets (UNI-C port, UNI-N port, NNI port, Ethernet connection and Ethernet virtual connection) are also shown in Figures 6-1, 6-2 and 6-3.

The remainder of this Recommendation defines Ethernet virtual connection and Ethernet connection attributes for the support of Ethernet services for UNI-N to UNI-N (clause 7), Ethernet UNI-N port attributes (clause 8), and Ethernet NNI port attributes for intra- and inter-carrier handoff (clause 9). The attributes for the UNI-C port are for further study. Details on the structures and mappings of the UNI and NNI to specific server layers are specified in [ITU-T G.8012] and [ITU-T G.8012.1]. The equipment functions of these interfaces are defined in [ITU-T G.8021] and [ITU-T G.8021.1].

Note that in most cases, EC is functionally equivalent to the EC defined by [MEF 12.1]. -The EC is used to describe the network underlying the Ethernet service.  To describe the Ethernet service, this Recommendation will use the term EVC as equivalent to the MEF EVC.

The permitted values for the sets of attributes will be specified for each of the Ethernet services that are defined in other Recommendations:  [ITU-T G.8011.1] , [ITU-T G.8011.2] , [ITU-T G.8011.3] , [ITU-T G.8011.4], [ITU-T G.8011.5] .  These Recommendations are aligned with similar services defined in [MEF 6.1].  The result is that this Recommendation allows classification of many Ethernet services.

## 6.3	Ethernet services views

It is important to understand the perspective of an Ethernet service. The full lists of attributes and their values may differ depending on whether the service is viewed from the network looking out or from the customer looking in.

The end result is that a service defined from the customer's perspective with MEF EVC and UNI attributes can be deployed over a network infrastructure service defined with ITU-T G.8011 EVC, EC and UNI attributes. This is especially true since the base attributes of this Recommendation are defined by MEF  [MEF 10.2].

In [MEF 12.1], there is a clear relationship described between the “Service Model” constructs and the “Architecture Model” constructs, as well as a relationship between the reference points of the Service Model and the Flow Points/Flow Point Pools of the Architecture Model. In particular, the term EVC is exclusively used as a Service Model construct that specifies an association between certain types of reference points, UNIs, while the term EC is used exclusively as an Architecture Model construct that identifies connection(s) created by Network Operators to instantiate the requested association of reference points.

### 6.3.1	Network view

This Recommendation presents the framework for a series of Ethernet services from the perspective of the network, that is the operator or service provider. As a result, various topology, service and performance characteristics are visible that may not be visible from other views. This can result in a wide variety of services based on these parameters.

In addition, each service will have attributes that describe the behaviour of the network connection. These attributes may be simple (e.g., connectivity – pt-pt) or may be a grouping of attribute elements (e.g., characteristics – address, priority, etc.). It should be noted that in the network view, for example, the Ethernet connection (per Figure 6-1) set of attributes will have a number of infrastructure specific attributes. That is, the EC is not network agnostic.

### 6.3.2	Customer view

Ethernet services can also be described from the perspective of the customer, that is the subscriber. Such a description does not provide any details about how a service is realized. That is, the EVC is network agnostic.

This Recommendation is complementary to the definitions of Ethernet services from the customer viewpoint looking into the network. For example, the MEF EVC may be carried over an EC as described by ITU-T.

# 7	Ethernet virtual connection and Ethernet connection attributes

## 7.1	EVC

This clause describes Ethernet virtual connection (or EVC) attributes that characterize a particular instance of an Ethernet service. The area of applicability of these EVC attributes is identified in Figure 6-1 as being equivalent to the ETH connection or ETH connectivity (per clause 6.6 of [ITU-T G.8010]). The base set of ITU-T G.8011 EVC attributes is the same as the Ethernet virtual connection (EVC) attributes defined in [MEF 10.2] Table 13 and they are summarized in Table 7-1.

| Table 7-1 – EVC service attributes |  |  |
| --- | --- | --- |
| EVC service attribute | Service attribute parameters and values | MEF 10.2 reference |
| EVC type | Point-to-point, multipoint-to-multipoint, or rooted-multipoint | 6.1 |
| EVC ID | An arbitrary string, unique across the MEN, for the EVC supporting the service instance | 6.2 |
| UNI list | A list of <UNI Identifier, UNI Type> pairs | 6.3 |
| Maximum number of UNIs | Integer. MUST be 2 if EVC type is point-to-point. MUST be greater than or equal to 2 otherwise. | 6.4 |
| EVC maximum transmission unit size | 2000  Integer  1522. | 6.10 |
| CE-VLAN ID preservation | Yes or No | 6.6.1 |
| CE-VLAN CoS preservation | Yes or No | 6.6.2 |
| Unicast service frame delivery | Discard, deliver unconditionally, or deliver conditionally. If deliver conditionally is used, then the conditions MUST be specified. | 6.5.1.1 |
| Multicast service frame delivery | Discard, deliver unconditionally, or deliver conditionally. If deliver conditionally is used, then the conditions MUST be specified. | 6.5.1.2 |
| Broadcast service frame delivery | Discard, deliver unconditionally, or deliver conditionally. If deliver conditionally is used, then the conditions MUST be specified. | 6.5.1.3 |
| Layer 2 control protocols processing | A list of Layer 2 control protocols labelled tunnel or discard. | 6.7 |
| EVC performance | Performance objectives for frame delay performance, frame delay variation performance, frame loss ratio performance, and availability performance and associated class of service identifier(s) as defined in clause 6.8 of MEF 10.1 | 6.8, 6.9 |

The relationship of these attributes to [ITU-T G.8010] is shown in Appendix I.

The values for these attributes will be specified for each of the Ethernet services defined in the G.8011.x/Y.1307.x series of Recommendations.

## 7.2	EC

This clause describes Ethernet connection (or EC) attributes that characterize a particular instance of an Ethernet service.  The area of applicability of these EC attributes is identified in Figure 6-1 as being equivalent to the ETH link (per clause 6.6 of [ITU-T G.8010]).  These attributes are in addition to those defined in MEF and summarized in the previous clause.

| Table 7-2 – EC service attributes |  |  |
| --- | --- | --- |
| EC service attribute | Service attribute parameters and values | clause |
| Link type | Dedicated, shared | 7.2.1 |
| Survivability | None, specify | 7.2.2 |

The relationship of these attributes to [ITU-T G.8010] is shown in Appendix I.

The values for these attributes will be specified for each of the Ethernet services defined in the G.8011.x/Y.1307.x series of Recommendations.

## ontrol protocols

This attribute is related to the L2 control protocol attribute of the UNI

7.2.1	Link type

This ITU service attribute indicates the characteristics of the server layer that is used to transport the Ethernet service. There are two options: dedicated and shared.

This attribute describes the bandwidth competition that an Ethernet service instance will encounter within the network. The use of the link type attribute in the context of a customer is for further study.

### 7.2.1.1	Dedicated

A dedicated link type indicates that all ETH links supporting the EC have the following characteristics:

•	Each ETH link is exclusively allocated to transport the ETH_CI of a single service instance.

•	The ETH_CI transported by an ETH link does not compete for resources with the CI of other service instances.

This attribute is referring to an EC; an EC does not necessarily map onto a single link. As a result, if the link type is dedicated it follows that all links supporting the EC must be dedicated and have the corresponding characteristics.

### 7.2.1.2	Shared

A shared link type indicates that one or more ETH links supporting the EC have the following characteristics:

•	The ETH link is allocated to transport the ETH_CI of one or more service instances.

•	The ETH_CI transported by an ETH link competes for resources with the CI of other service instances.

## Traffic separation

### 7.

### --

### Connectivity monitoring

## 7.2.2	Survivability

The transport network can provide survivability for each service. The survivability alternatives for protection and restoration are related to Ethernet layer protection switching and/or server layer specific protection switching. As a result, the protection or restoration mechanism associated with the appropriate server layer as defined in [ITU-T G.8012] would be specified.  Any additional relevant details on the server layer survivability would be listed in the definition of the service

The options are: none or specify

The options include:

- ITU-T G.8031 – Ethernet linear protection

- ITU-T G.8032 – Ethernet ring protection

- IEEE 802.1AX – Link aggregation

- IEEE 802.1Q (RSTP, MSTP, GVRP, MVRP) – Spanning Tree Restoration

- IEEE 802.1aq – Shortest path bridging

MEF defines metrics for resiliency and availability performance as part of [MEF 10.2] and [MEF 10.2.1].

8	Ethernet UNI attributes

# 8.1	UNI attributes

This clause describes service UNI attributes that characterize a particular instance of an Ethernet service at the demarc of the UNI noted in Figure 6-1. There is a UNI defined at each of the ETH and ETY layers. The base set of ITU-T G.8011 UNI attributes is the same as the UNI attributes defined in [MEF 10.2] Table 12 and they are summarized in Table 8-1.

Table 8-1 – UNI service attributes

| Layer | UNI service attribute | Service attribute parameters and values | [MEF 10.2] reference |
| --- | --- | --- | --- |
| ETH | UNI identifier | Any string | 7.1 |
|  | MAC layer | IEEE 802.3 – 2012 | 7.3 |
|  | UNI maximum transmission unit size | 2000  Integer  1522. | 7.4 |
|  | Service multiplexing | Yes or No | 7.5 |
|  | UNI EVC ID | A string formed by the concatenation of the UNI ID and the EVC ID | 7.6.2 |
|  | CE-VLAN ID for untagged and priority tagged service frames | A number in 1, 2, …, 4094. | 7.6.1 |
|  | CE-VLAN ID/EVC map | Map | 7.7 |
|  | Maximum number of EVCs | Integer  1 | 7.8 |
|  | Bundling | Yes or No | 7.9 |
|  | All to one bundling | Yes or No | 7.10 |
|  | Ingress bandwidth profile per ingress UNI | No, or parameters | 7.11.2.1, 7.11.1 |
|  | Ingress bandwidth profile per class of service identifier | No, or parameters for each class of service identifier | 7.11.2.3, 7.11.1 |
|  | Ingress Bandwidth Profile Per EVC | No, or parameters for each EVC | 7.11.2.2, 7.11.1 |
|  | Egress bandwidth profile per egress UNI | No, or parameters | 7.11.3.1, 7.11.1 |
|  | Egress bandwidth profile per class of service identifier | No, or parameters for each class of service identifier | 7.11.3.3, 7.11.1 |
|  | Egress Bandwidth Profile Per EVC | No, or parameters for each EVC | 7.11.3.2, 7.11.1 |
|  | Layer 2 control protocols processing | A list of layer 2 control protocols with each being labelled with one of Discard, Peer, Pass to EVC, Peer and Pass to EVC | 7.13 |
| ETY | Physical medium | A Standard Ethernet PHY | 7.2 |
|  | Speed | 10 Mbit/s, 100 Mbit/s, 10/100 Mbit/s Auto-negotiation, 1 Gbit/s, or 10 Gbit/s | 7.2 |
|  | Mode | Full duplex | 7.2 |

-

ontrol protocols

For Tables 8-2 and 8-3, note that

ontrol protocols

Table 8-4 – CFM protocol processing at UNI-N

9	Ethernet NNI attributes

9.1	INNI attributes

As it is internal to a network operator’s network, this Recommendation does not specify service INNI attributes.  However, an operator may specify attributes to characterize a particular instance of an Ethernet service at the demarc line of the Internal NNI noted in Figure 6-2. There is a INNI defined at each of the ETH and server layers. These are described in [ITU-T G.8012] and [ITU-T G.8012.1].

# 9.2	ENNI attributes

This clause describes service ENNI attributes that characterize a particular instance of an Ethernet service at the demarc line of the External NNI noted in Figure 6-3. There is a ENNI defined at each of the ETH and server layers. These are summarized in Table 9-1.

Table 9-1 – ENNI service attributes

| ENNI service attribute | Service attribute parameters and values | MEF 26.1 reference |
| --- | --- | --- |
| Operator ENNI Identifier | A string that is unique across the Operator MEN. | 7.1.1 |
| Physical Layer | One of the PHYs listed in [R5] of [MEF 26.1] | 7.1.2 |
| Frame Format | Frame formats as specified in 7.1.3 of [MEF 26.1] | 7.1.3 |
| Number of Links | An integer with value 1 or 2 | 7.1.4 |
| Protection Mechanism | Link Aggregation, none, or other | 7.1.5 |
| ENNI Maximum Transmission Unit Size | An integer number of bytes greater than or equal to 1526 | 7.1.6 |
| End Point Map | A table with rows of the form <S-VLAN ID value, End Point Identifier, End Point Type> | 7.1.7 |
| Maximum Number of OVCs | An integer greater than or equal to 1 | 7.1.8 |
| Maximum Number of OVC End Points per OVC | An integer greater than or equal to 1 | 7.1.9 |

The relationship of these attributes to [ITU-T G.8010] is shown in Appendix I.

Other options for the Protection mechanisms attribute are listed in clause 7.2.3

10	Connectivity Monitoring

Connectivity monitoring can be achieved via Ethernet OAM mechanisms defined in [ITU-T Y.1731].

Additional specifications on the use of CFM as Service OAM for fault management are defined in [MEF 30]  and for performance monitoring  are defined in [MEF 35].

Service OAM Fault Management [MEF 30] and Service OAM Performance Monitoring [MEF 35] are used to implement the “EVC performance” EVC attribute

CFM [ITU-T Y.1731] may also be used to implement the “Survivability” EC attribute.

Annex A

Extended UNI

(This annex forms an integral part of this Recommendation)

## A.1	Introduction

This Recommendation introduced a simple network model in clause 6 while indicating that more complex models are possible. The UNI shown in Figure 6-1 is a simple or collapsed case. This annex introduces additional models that are possible.

## A.2	Extended UNI-N

This case introduces an access network or private line inside the operator network that 'extends' the UNI link towards the demarc. The extended UNI would result in the UNI-N function that is shown in Figure A.1. The functions and attributes of the UNI-N are located in the device closest to the demarc thus extending from the device closest to the operator's network across an intermediate access network.

Figure A.1 – Single-provider view of Ethernet service areas
with distributed UNI-N

-

Appendix I

Relationship of G.8011/Y.1307 attributes to ITU-T G.8010

(This annex does not form an integral part of this Recommendation)

## I.1	Introduction

This annex describes the direct relationship of the attributes defined in this Recommendation and the architecture of [ITU-T G.8010].

## I.2	Ethernet connection attributes

All Ethernet connection services are built by interconnecting ETH links. The service or EVC attributes are related to a set of ETH links, or an ETH subnetwork. They define restrictions on interconnection, or on the attributes of links to be used. The relationship of these attributes to [ITU-T G.8010] is shown in Table I.1 and Table I.2.  Note that many of the architectural realtionships listed below shown are in Figure 8-4.

| Table I.1 – G.8011/Y.1307-G.8010/Y.1306 EVC attribute relationship |  |  |  |  |
| --- | --- | --- | --- | --- |
| EVC service attribute (Table 7-1) | Service attribute parameters and values |  | G.8010/Y.1306 relationship |  |
| EVC type | p2p, mp2mp, rooted mp |  | EC type |  |
| EVC ID | Arbitrary text string to identify associated EVC |  | Management plane identifier of EVC (not defined in [ITU-T G.8010]) |  |
| UNI list | UNI ID + UNI type |  | Management plane list of addresses of FP (not defined in [ITU-T G.8010]) |  |
| Preservation | VLAN – yes or no CoS – yes or no |  | Allowing for insertion/extraction of VLAN tag in ETH/ETH-m adaptation with the same VID and PCP/DEI values as on the UNI |  |
| Service frame delivery | Discard, deliver unconditionally, or deliver conditionally |  | Address – Determines whether EC uses filtering when forwarding traffic units between interconnecting links Priority – Determines whether all Queuing process should differentiate based on priority (PCP/DEI bits) |  |
| L2CP processing | Discard, or tunnel |  | Determines whether EC uses filtering when interconnecting links. Filtering is performed in ETH/ETH or ETH/ETH-m adaptation sink |  |
| Performance | Specify |  | Performance associated with ETH SNC |  |
| Bandwidth profile | Specify |  | Bandwidth associated with the EC at the UNI-N |  |

| Table I.2 – G.8011/Y.1307-G.8010/Y.1306 EC attribute relationship |  |  |
| --- | --- | --- |
| EC service attribute (Table 7-2) | Service attribute parameters and values | G.8010/Y.1306 relationship |
| Link type | Dedicated, shared | Determines the mapping of EC to ETH links. Dedicated means exactly 1 EC per ETH link, Shared means 1 or more ECs per ETH link |
| Survivability | None, specify | ETH and server layer survivability. |

## I.3	Interface attributes

Table I.3 and I.4 show the relation between [ITU-T G.8010] and the UNI and ENNI attributes defined in this Recommendation. Note that many of the architectural functions listed are in Figure 15 of [ITU-T G.8010].

Table I.3 – G.8011/Y.1307-G.8010/Y.1306 UNI attribute relationship

| UNI attributes (Table 8-1) | G.8010/Y.1306 description | G.8010/Y.1306 architectural function |
| --- | --- | --- |
| UNI ID | Management identifier of ETH FP (not defined in [ITU-T G.8010]) | One or more FP |
| UNI EVC ID | Management identifier of UNI EVC (not defined in [ITU-T G.8010]) | - |
| MAC layer | ETH layer | ETH layer network |
| CE-VLAN ID /EVC mapping | Flow point mapping to configure EC function | ETH_FP to/from SNC |
| Service multiplexing | ETH multiplexing | ETY/ETH-m adaptation function used |
| Bundling |  |  |
| Bandwidth profile | Traffic conditioning and shaping | ETH_TCS function |
| Layer 2 control protocol processing | 802.3 L2 protocol generation and termination | 802.3 protocols process in Srv/ETH adaptation |
|  | GARP & reserved -address filtering | Filter process in Srv/ETH adaptation |
| PHY speed/mode/medium | Server layer technology | ETY Adaptation and TT |

Table I.4 shows the relation between [ITU-T G.8010] and the ENNI attributes defined in this Recommendation.

Table I.4 – G.8011/Y.1307-G.8010/Y.1306 ENNI attribute relationship

| EC per UNI attributes (Table 9-1) | G.8010/Y.1306 description | G.8010/Y.1306 architectural function |
| --- | --- | --- |
| Operator ENNI Identifier | Management identifier of ETH FP (not defined in [ITU-T G.8010]) | FP |
| Physical Layer | Server layer technology | ETY Adaptation and TT |
| Frame Format | ETH_CI | ETH_CI |
| Number of Links | Number of ETY trails | ETY trail |
| Protection Mechanism | survivability | survivability |
| End Point Map | Flow point mapping to configure EC function | ETH_FP to/from SNC |

Appendix II

EC topology

(This appendix does not form an integral part of this Recommendation)

## II.1	EC topology

The EC topology instantiated to support an EVC indicates the connectivity between Ethernet endpoints in the transport service. There are three options, as defined in clause 6.1 of [MEF 10.2]: pt-pt, mp-mp, pt-mp.  They are also defined in [ITU-T G.8010]. The following clauses show how these are modelled.

### II.1.1	Point-to-point

ELINE EVC is supported by a point-to-point (pt-pt) EC which has exactly two points. The pt-pt EC  is supported by:

•	a link connection within the provider network (non-extendible), or

•	a subnetwork connection with only two flow points in use (extendible).

The topology for the network portion of the non-extendible pt-pt service is shown in Figure II-1.

The ETH link connection may be supported by a server layer technology that is connection oriented (circuit switched or packet switched) or connectionless.

Figure II-1 – Network portion of the point-to-point topology

### II.1.2	Multipoint-to-multipoint

ELAN EVC is supported by a multipoint to multipoint (mp-mp) EC which has two or more points.

The topology for the network portion for LAN services consists of one or more subnetworks with ETH links between them, as shown in Figure II-2.

Each of the ETH links may be supported by a server layer technology that is connection oriented (circuit switched or packet switched) or connectionless. Additional ETH termination points can be added/deleted to/from this service topology.

A special case of the mp-mp construct is where the subnetwork has only two flow points in use. In this case, it supports a pt-pt service as described in clause II.1.1.

Figure II-2 – Network portion of the multipoint-to-multipoint topology

### II.1.3	Rooted-multipoint

ETREE EVC is supported by a rooted multipoint (rooted mp) EC which is between one or more root points and one or more leaf points.

For E-Tree service, each leaf point can only exchange data with the root point, while a root point can exchange data with each leaf point and other root points.

The topology for the rooted multipoint EC consists of one or more subnetworks with ETH links between them, as shown in Figure II-3.

Each of the ETH links may be supported by a server layer technology that is connection oriented (circuit switched or packet switched) or connectionless. Additional ETH termination points can be added or deleted to/from this service topology.

Figure 7-3 – Network portion of the point-to-multipoint topology

### II.2	Service multiplexing

Service multiplexing, as defined in clause 7.5 of [MEF 10.2], indicates if the access to the Ethernet transport service is multiplexed (i.e., contains multiple service instances) or not.

The topology illustrated in Figure II-4 consists of N point-to-point connections presented to a single physical interface.

Figure II-4 – Network portion of the service multiplexing line topology

In the case of service multiplexing at the service UNI (demarc), one ETH link is used between the provider and the customer to transport ETH_CI of multiple customers' service instances. On ingress, the onus is on customer equipment to shape the service instance that will be multiplexed to ensure sufficient fairness to avoid congestion of the access link. The network can ensure service instance bandwidth on the ETH link with traffic conditioning.

__________

NOTE – The G.8011-2004 Ethernet connection (EC) definition excluded the UNI-N. This was a significant difference from the MEF EVC at the time. As a result, G.8011-2009 clarified that the EVC starts in the UNI-N making the EC and EVC essentially equivalent.  However, additional clarifications from MEF 12.1 Figure 10 align with the original G.8011.  As a result, this Recommendations reinstates the difference between EC and EVC. Exactly where the start of the EVC is located depends on the underlying server layer.