IEEE 802.11

Wireless Access Method and Physical Layer Specifications

╶───────────────────────────────────────────────────────────────

Title:	IEEE Requirements Document comments on doc # 92/01

Presented by:

Wim Diepstraten

NCR SE-Utrecht

NCR/AT&T Network Product Group

Nieuwegein

Netherlands

31-3402-76482 (V)

31-3402-39125 (Fax)

Wim.Diepstraten@Utrecht.ncr.com (Email)

╶───────────────────────────────────────────────────────────────

Abstract:	The following is a list of comments, questions or remarks on the IEEE Project 802.11 "Wireless Local Area Network Requirements" document.

C-1:	Introduction

Q - How living is the document after sign-off.

.	Synchronous exported services to be specified.

Q -X Should several conformance levels be specified, to cover for instance different moving speed ranges.

p4:X Need a reference MSDU size specification for the transfer delay numbers. Note: 2 msec transfer delay for a 64 byte octet is much different from a 2 msec transfer delay for a 1024 octet MSDu size. Suggest reference to be 128 octets, as suggested in page 24.

p4-X Is it correct that "Service Initiation Time" is only applicable for connection oriented services?

p7-X Clarification of "Interference distance" needed. Is this the "Signal-to-Interference" ratio (SIR) in dB?

We need to have a term for the SIR at which a good service quality can be obtained.

p10-	The Multicast/Broadcast transfer reliability cannot be guaranteed in a wireless environment. When is this deviation from the general 802 MAC service definition to be covered and how?

Q-	What kind of applications use group/Multicast addressing?

p11:	How about IEEE 802.9 IVD LAN draft standard reference?

C-2:	General Requirements

p14:	What are distribution system standards for Sync traffic? No 802 equivalent standard available?

p14:	Security/Integrity

The method used should be world-wide applicable without restrictions.

p14:	Isn't "Roaming" support a to be specified general requirement?

p15:	Shouldn't the 802.2 "Bridge" functions be limited to "single leave" support only. We should prevent bridging from one network to another via the wireless medium.

Q: 	What should this mean for a Directional Antenna point-to-point link and its co-existence with the 802.11 LAN.

p15:	What kind of "Bridge" functionality is supported? I Would expect that the "Access Point" would use a MAC Bridge approach.

p15:	Perhaps delete the "Complete Architectural Scope" (Fig 2.2) from this document altogether to keep it general enough. To cover also sync and bridging services, much more pictures are needed.

C-3:	Media Access Control Layer

-	Sections 3.2.1 through 3.2.3 should better be deleted, because they relate to implementation, rather than requirements.

-	Also section 3.3 could better be deleted because it does relate too much to implementation.

p19:	All "To be supplied" sections should be specified before "ballot".

p20:	Shouldn't support for the different NOS's and protocol stacks be a prime requirement? Why is NDIS and ODI referenced here? They are more interface specifications than NOS protocol stacks.

p23:	Don't see the median jitter conclusion?

p25:	Sync maximum transfer delay of 10 msec is equal to the nominal delay specification. I would expect 20 msec.

p27:	Internetworking: Does this sufficiently cover the Sync internetworking requirements. Suggested extension:

"belonging to a given LAN. However, this requirement does not apply to interconnection with other networks e.g. those using ISDN protocol stacks." (JK)

p28:	Add Arcnet

Also SNMP is a industry standard

C-4 Physical Media Layer:

p29:	Suggest to delete end p29, p30 and p31, because they are not requirements, but more implementation details.

p32:	Delete 4.3.2 if possible

Delete 4.3.4 because 18 GHz requires license, so is not in accordance with the general requirements.

Appendices:

A:	Education:	Video anticipated for "Field Study", why then not for other education applications.

B:	Finance:	Only trading floor application described.

Need description of Bank/Branch transaction applications, or at least a reference to possible "office" requirements covering this type of traffic characteristic.

C:	Industrial:	No Comments

D:	Medical:	No Comments

E:	Meetings:	No Comments

F:	Office:

-	F2.3 SNA and DECNET are no NOS's. Suggest to generalize to mention support for different protocol stacks.

-	Why no Banyan support?

-	F2.6 use # of stations per hectare

G:	Retail:

-	Change "enterprise" to premises/store

-	Need hypermarket application with much higher station density.

-	Need to cover different (future) applications related to automatic shelf labeling. This is documented in contribution # IEEE P802.11-92/15

H:	Warehousing:	No comments

I:	Multi Media:	No comments

Issue's in deleted sections:

p18:	end of section 3.2.2. Why full duplex characteristic required?

p19:	Quality of Service info should also include receive level information.

p20:	TxClock should be supplied by PHY. Usually needed for synchronous sampled systems to keep sampling clock in sync with the bit and symbol clocks.

p31:	-	Point 5 statement is not clear in defining the "Interference distance".

-	Point 9 should be much stronger. A measurement of interference level at one station is no indicator of the interference level of another station.

p32:	Section 4.3.3 item 1. I don't understand why item 9 can be removed from the general PHY characteristics.