IEEE P802.11

Wireless Access Method and Physical Specifications

Title:		Radio LAN System Standardization Activities in Japan

Date:		November 9, 1992

Source:	"Report on Technical Requirements for Radio LAN System"

(submitted by the Telecommunications Technology Council)

This paper reports on  standardization activities for Radio  LAN System in Japan.

In Japan,  the  technical  requirements  of  Government, in order to promote efficient frequency use and interference protection of radio systems, are considered by the  Telecommunications  Technology  Council of MPT ( Ministry of Posts and Telecommunications ).   Radio LAN System is also discussed at this council.

On the other hand, concrete and detailed R&D of  Radio LAN System  is being conducted by  Radio  LAN System study group of the RCR ( Research & Development Center for Radio Systems, since May 1991 ).  The voluntary technical standards will be established by the Standard Committee of  RCR.

Radio  LAN  System is  being considered by dividing to following three types.

Type A : 	High-Speed, more than 10Mbps. Millimetre wave and/or quasi-microwave will be used. Common air interface will be discussed.

Type B : 	High-Speed, approx.10 - 15Mbps. 18 - 20 GHz.

Type C : 	Medium-Speed, approx.256 kbps - 2Mbps.

Spread spectrum on ISM band.

This July, a partial report on the  " Technical  Requirements for  Radio  LAN

System " was submitted by the TTC.

The partial report deals  with  the  technical  requirements for Medium-Speed

Radio LAN systems (Type C) and for High-Speed Radio LAN systems (Type B).

The main technical requirements for Midium- and High-Speed Radio LAN systems are shown in following paper.

Based  on  the  findings  in  this  partial report,  MPT expects to revise the

necessary ministerial ordinances.

Remained type A system is now being considered by TTC and RCR study group.

"Technical Requirements of Radio LAN Systems"

Technical Requirements of Medium-Speed Radio LAN Systems(Type C)

1.     General Requirements

(i)  Communication system

One-way, simplex, half-duplex or full-duplex.

(ii)  Spectrum scattering system

The spectrum scattering system shall be the

direct scattering (DS) system the frequency hopping (FH)

system or the hybrid system between DS and

FH systems (DS/FH system).

(iii)  Radio frequency band

A frequency band shall be selected from the frequency band

of 2400 to 2500MHz (ISM band) designated for industrial,

scientific and medical (ISM) purposes as a new spectrum

scattering system will be commercialized.

(iv)  Antenna power

It will be appropriate to specify in density power as the

ISM band will be used and it must be less than

or equal to 10mW per MHz.

(v)  Control of illegal use

The structure of the principal parts of the transmitting

equipment must be such that they cannot be opened easily.

2.     Technical requirements of radio equipment

(i) Transmitting equipment

(1)  Allowable frequency deviation

Must be less than or equal to +/-50 ppm.

(2)  Spurious emission intensity

The spurious emission intensity at frequency "f" which excludes

the designated frequency shall be as follows:

a. Below 25µW at fL-fH =<f<fL and fU<f =<fU + fH

b. Below 2.5µW at fL-fH>f and fU+fH < f

where    fL:  Lower-limit frequency (MHz) of designated

frequency band .  (MHz)

fU:  Upper-limit frequency of designated frequency

band (MHz)

fH:  Value obtained by multiplying the designated

frequency bandwidth (MHz) by 0.5.

(3)  Allowable antenna power deviation

20% above nominal and 80% below nominal.

(4)  Tolerance of occupied frequency bandwidth

The tolerance of the occupied frequency bandwidth must be

the required frequency bandwidth below the designated

frequency bandwidth.

(5)  Scattering bandwidth (90% frequency bandwidth of total power)

The scattering bandwidth shall be more than or equal to 500kHz.

(6)  Scattering factor

The scattering factor (ratio of symbol rate of scattering

bandwidth to clock frequency) should be more than or equal to 10.

(ii) Receiving equipment

(1)  Undesired radiation

Undesired radiation must be less than or equal to 4nW at frequencies

below 1GHz and less than or equal to 20nW at frequencies higher

than 1GHz.

High-Speed Radio LAN System Technical Requirements (Type B)

1. General Conditions

(i)  Communication system

oneway, simplex, half-duplex or full-duplex.

(ii) Transmission method

Time division duplex (TDD) system shall be used.

(iii)Modulation speed

Must be 10Mbps or higher.

(iv) Modulation system

Quadrature Amplitude Modulation (QAM), 4 level frequency

shift keying (4-level-FSK), or Quadrature Phase Shift

Keying (QPSK).

(v) Carrier frequency spacing

20MHz.In the case of interleave it is 10 MHz.

(vi) Radio frequency band

A frequency should appropriately be selected from the

quasi-millimeter band (18 to 20GHz band).

(vii) Antenna Power

Antenna power shall be less than or equal to 300 mW.

(viii) Control of illegal use

The structure of the principal parts of the transmitting

equipment must be such that they cannot be opened easily.

2. Technical Requirements of Wireless Equipment

(i) Transmitting equipment

(1)  Allowable frequency deviation

Must be less than or equal to +/-10ppm.

(2)  Spurious emission intensity

Must be less than or equal to 100 µW.

(3)  Allowable antenna power deviation

20% above nominal and 80% below nominal.

(4)  Allowable occupied band-width

Must be less than or equal to 17MHz.

(5)  Adjacent channel leak power

Must be more than or equal to 40dB to the mean power within

+/- 8.5 MHz band detuned 20 MHz frequency from the carrier.

(ii) Receiving equipment

(1)  Receiving sensitivity

The receiver input power with a data packet error rate of

5 x 10**(-2) shall be less than or equal to -71dBm.

(2)  Spurious response

Shall be higher than or equal to 10dB.

(3)  Adjacent channel selectivity

Must be more than or equal to 25dB at a frequency detuned 20MHz

from the carrier.

(4)  Undesired radiation

Must be below or equal to 4 nW less than 1GHz. Must be below or

equal to 20 nW above 1 GHz and 10 GHz.  Must be below or equal to

20 µW above 10 GHz.

(iii)  Antenna

Must be less than or equal to 20 dBi.