Frequency Hopping Spread Spectrum PHY

Nathan Silberman

California Microwave Inc.

985 Almanor Ave.

Sunnyvale, CA 94086

Introduction

This contribution is intended to provide an overview of a FHSS PHY and outlines the criterias for defining this PHY.  The final specifications have to be worked out in conjunction with the MAC definition and the agreed channel model.  Committee members are welcomed to provide their inputs to the completion of this document.

At the end of this paper, there are some experimental facts and a link calculation example.

Requirements outline:

§	Compliance with Regulatory Agencies for unlicensed operation

§	Compliance with 802.11 PAR (Data Rate at least 1 Mbps, etc.)

§	Operation in a multinetwork environment (multiple collocated networks)

§	Minimum Area coverage

§	Suitable for low power consumption implementations

§	Cost effective

§	Modes of operation: 
- peer to peer with no prior knowledge
- node to AP and AP to node

§	Support asynchronous and time deterministic connectivity.

§	Support a specified number of stations per cell (Access Point)

§	Suitable for small size implementation

§	Robust operation in narrow band and partial band interference as well as multipath fading.

§	Graceful degradation under load and interference.

What FH can offer:

§	Being an avoidance type system offers better robustness than other SS techniques with low processing gain (as the ones that could be used in the ISM bands).

§	Less susceptibility to "Near Far" problem.

§	Can provide the largest amount of band spreading.

§	Relatively short acquisition time.

§	Non coherent modulation techniques used, lend themselves towards simpler implementations therefore low cost.

§	Can provide best distance performance with an optimized receiver

§	Robust operation in presence of Narrow band or partial band interference.

§	Multipath environment operation: "Natural frequency Diversity"

§	Multiple Networks through CDMA

Specifications

The following table presents an example of Frequency Hopping PHY specification.  I left several blanks for those items that can be determined only after the channel model is agreed by 802.11 and PHY MAC interface requirements determined; Other parameters have to be worked out between PHY and MAC groups.

PARAMETER

Required Spec.

Proposed Spec.

Comments

Frequency Range

2.4 to 2.4835 Ghz

2.4 to 2.4835 Ghz

Other frequency bands will follow.

Minimum number of frequencies

75

75

per FCC part 15.247

No of hops per sec.

2.5 min. (FCC 15.247)

4

Transmitted power

1000, 250, 100, 50 mW

Power levels dependent on the specific applications

Max. Radiated EIRP

6 dBW in US
? dBW in Europe

Total radiated power including antenna gain

Normally defined by regulatory agencies in each country.

Transmitted power variation (tolerance)

TBD dB

+0 /-3 dB max. (total)

Receiver Sensitivity

-80 to -90 dBm

Final number TBD after channel and link model agreed

Max. Input signal level at antenna connector with no performance degradation

TBD dBm

0 dBm

Adjacent Channel selectivity

-25 dB at channel boundary

-65 dB @Df=2 MHz

Channel bandwidth (allocated)

1 MHz

Occupied channel bandwidth (spectrum shape)

30 dBc  @ Df=.5 MHz

86 dBc @ Df=2 MHz

Data Rate

1 Mbps

1 Mbps

Higher data rates not recommended because of delay spread and bandwidth constraints

Spurious emissions in band (  @ Df§1 MHz from Fc)

-80 dBc @ 100 mW TX power

Spurious emissions out of band

§ -20 dBW in US

§ - 60 dBW/100 kHz @ 1 to 10 Ghz

§ -66 dBW/100 kHz @ 30 MHz to 1 Ghz

dependent on country

Switching time
 TX to RX

Time from full power transmission to full sensitivity receiver availability.  This parameter should include any preamble time used for receiver synchronization.

Switching time 
 RX to TX

time from full sensitivity reception to full power transmitter availability

Channel switching time
(hop settling time)

typically 100 to 200 §S

Elapsed time from receipt of hop command until unit frequency settles within +/- §F= Receiver Acceptance range or TX frequency tolerance (whichever is tighter)

Modulation

4 level FSK

Demodulation

Non coherent

BER at specified Eb/No

10 exp. -6

10 exp. -6 
@  Eb/No=17 dB

Channel availability

99.5 %

Could also be specified as probability of outage.  With no interference.

TX Frequency Stability

15 ppm

Receiver Frequency Acceptance Range

+/- TBD Hz

Data Line/ Clock Input/Output Jitter

Include static and dynamic jitter (see 802.3 definition)

This parameter has to be negotiated with MAC group.

Min. reception range

333 feet

Conditions TBD

Max. Required range

( For applications where no AP's are available)

1000 feet

Conditions TBD

Antenna port impedance

50 ohms

VSWR

Devices shall stand  
0§VSWR§§ with no damage.

-Operational VSWR =TBD

Interface to MAC

RX Data
TX Data
RX/TX clock
Frequency control
TX/RX switching
Signal Quality
Net Management info.

Timing and logic levels TBD.

Safety Requirements

Compliance with applicable Safety Agencies requirements

Field Experience

§	Superior performance in presence of interference, both narrow band and partial band.  Up to 60 dB I/S in adjacent channels.

§	Good propagation characteristics in indoors environments.  @ 1 W. TX power indoors range up to 1000 ft., outdoors range 6 miles with std. monopole antennas and retries protocol.

§	Guaranteed data delivery with retries ( Several  systems tested in the field over more than nine months @250 Kbps data rate with interference present.  In some sites a number of microwave ovens were in normal use)

Conclusions:

Frequency Hopping Spread Spectrum provides an economical, technically feasible solution meeting the requirements for a wireless LAN.  When coupled with a suitable protocol it exhibits superior performance in the presence of narrow band and partial band Interference and Multipath propagation as proven by field experience.

Link Calculations Example

See attached spread sheet.