IEEE 802.11

Wireless Access Method and Physical Layer Specifications

TITLE:	A Draft Proposal for Direct Sequence Spread Spectrum PHY Standard

Ryan Tze

Max Sheng

Toshiba America Information Systems, Inc.

2 Musick,

Irvine, CA 92713-9724

Tel: (714) 587-6769

Fax:(714) 583-3893

Email: RyanT@tais.com

MaxS@tais.com

Introduction:	This paper proposes a Direct Sequence Spread Spectrum (DSSS) physical layer that operates at 2.4 GHz ISM band. To support a single MAC/multiple PHY interface, the proposed specification have parameters  similiar to those proposed in the Frequency Hopping systems.

The specification is put in a table format as long as it is a living document.

Requirements outline:

·	Compliance with 802.11 PAR.

·	Compliance with Regulatory Agencies for unlicensed operation.

·	Support operation in a multinetwork environment. (multiple collocated networks)

·	Minimum area coverage.

·	Suitable for low power consumption implementations.

·	Suitable for small size implementation

·	Cost effective

·	Ensure interoperability between conformant DSSS stations.

·	Modes of operation:

- peer to peer

- node to AP and AP to node

·	Support asychnronous and time deterministic services.

·	Robust operation in narrow band and partial band interference as well as multipath fading

·	Graceful degradation under load and interference

Specifications:

The following table uses a template made by Nathan Silberman in the IEEE P802.11-92/127r for the 2.4GHz ISM band. The following table represents a template for DSSS PHY specification. Several areas are left to be determined after the channel model is agreed and after MAC-PHY interface is determined. Other parameters have to be worked out between PHY and MAC groups.

|  | Parameter | Proposed | Notes |
| --- | --- | --- | --- |
| 1. | Frequency range | 2.4 - 2.4835 GHZ (USA) 2.4 - 2.5 GHZ (ETSI) 2.471 - 2.497 GHZ (Japan) |  |
| 2. | Max. transmitted power level | 250 mw | IEEE c.95.1-1991 |
| 3. | Min. transmitted power level | 1 mw |  |
| 4. | Power control | above 100 mw |  |
| 5. | Level of power controls | 4 levels | 2-bit control |
| 6. | Number of Channels | 3 | 1 channel for Japanese market |
| 7. | Data rate | 1 M bit /sec | support fallback data rates. see also #8, #10. |
| 8. | Spreading chip ratio | >= 12, programmable | maintain same channel bandwidth with different data rate. i.e. #7 * #8 is constant. |
| 9. | Modulation | Binary MSK | modulation index 0.5 |
| 10. | *Receiver Demodulator | Two methods: a. FM descriminator b. Coherent receiver | a. For Post-despreading. b. For classical despreading. |
| 11. | Spreading sequence (main code ) | 16 bits, 64k length | Choose from sets of Maximal linear code. Longer code makes the information less susceptible to undesirable interception. |
| 12. | ** Spread code preamble: synchornisztion code ( sync code ) | 5 bits, 31 chips. repeat with inverted sync code for total of 120 us duration. | For code synchronization, antenna selection. |
| 13. | Channel bandwidth | 18 MHZ |  |
| 14. | Maximum Radiated EIRP | Controlled by regulatory agencies. |  |
| 15. | Spurious emissons in band | TBD |  |
| 16. | Spurious emissions out of band | Controlled by regulatory agencies. |  |
| 17. | Transmitted power variation | TBD |  |
| 18. | Frequency tolerance | 100 ppm |  |
| 19. | Minimum receiver sensitivity | -80 dBm | at #23 BER. |
| 20. | Adjacent channel rejection margin | 40 dB |  |
| 21. | Switching time Tx to Rx | 20 usec max |  |
| 22. | Switching time Rx to Tx | 20 usec max |  |
| 23. | BER (Bit error rate) | 1*10e-7 |  |
| 24. | Minimum reception distance | 50 ft. |  |
| 25. | Maximum reception distance | 1000 ft. |  |
| 26. | Channel availability | 99.5% |  |
| 27. | Antenna impedence | 50 W |  |
| 28. | Interface lines to convergence layer (when exposed) | a. RX data b. TX data c. RX/TX clock d. RTS/CTS e. Data valid f. Control line g. Status line h. Ctl/Sta Clock |  |
| 29. | PHY-MAC management info/control variables | a. PHY type (FH, DS or IR) b. Number of frequency channels c. Signal quality d. Number of RX signal strength levels e. Current RX signal strength f. Number of TX power levels g. Get current TX power level h. Set TX power level i. Channel in use (if more than one) j. Sleep mode k. Wake-up l. Standby mode m. Data rate n. Jabber control function o. Jabber indication | TBD |
| 30. | VSWR | Device shall stand infinite transmit VSWR without damage. | Operation VSWR TBD |
| 31. | Receiver noise figure | 6 db |  |
| 32. | Safety Requirements | Compliance with applicable Safety Agencies requirements |  |
| 33. | DTE/DCE interface | TBD |  |
| 34. | ACK protocol support | TBD |  |

Notes:

* Receiver demoulator:

The direct sequence system can have two different versions of receiver design, target for different market requirements. One is post-despreading, which is economical approach at minimum cost and least real-estate on the circuit board. The other one is the classical despreading, which has better jamming margin and processing gain. Certainly, the latter is costly to build. These two versions should be inter-operable.

* *Code structure and Data frame Alignment:

½¬		120 us				®½

| 31 chips | · · · | 31chips |  |
| --- | --- | --- | --- |

Sync code	· · · 			Sync code	 	 Main Code