IEEE 802.11

Wireless Access Method and Physical Specification

title:	PROPOSAL FOR 2 Mbit/s DSSS PHY

date:	March 8, 1993

author:	Jan Boer

NCR Wireless Communications and Networking Division

Zadelstede 1-10

3431 JZ Nieuwegein-Holland

Telephone: (+31) 3402-76483

Fax: (+31) 3402-39125

E-mail: jan.boer@utrecht.ncr.com

AUTONUMLGL§	INTRODUCTION

This contribution is intended to provide a proposal for the definition of the IEEE 802.11 DSSS PHY standard. The proposal is based on existing technology that is available today.

The document is more than just a specification, it will also describe the reasons and  the benefits of certain design choices. The specification framework can be found in the appendix.

The Physical layer that is presented here has the following characteristics:

- Each channel has a bitrate of 2 Mbit/s.

- Multichannel: three channels are defined in the 2.4 GHz ISM band and one channel can be

defined in the 915 MHz ISM band.

AUTONUMLGL§	REQUIREMENTS

The basic requirement for the physical layer that is to be standardised by the IEEE is compliance with 802.11 PAR. For the PHY this means that the bitrate has to be between 1 and 20 Mbit/s.

Secondly  the PHY has to be in compliance with Regulatory Agencies for unlicensed operation.

An outline of other requirements is listed below:

. Operation in a multinetwork environment (multiple co-located networks)

. Suitable for low power, cost effective and small size implementations

. Targeted environments: indoor offices, retail, industrial, medical.

. Robust operation with all kinds of interferers.

. High medium reuse efficiency.

- AUTONUMLGL§	SYSTEM CONSIDERATIONS

- Operation in a radio environment makes the system different from a cable environment in many aspects. The most important considerations for defining a Radio system are listed:

- - CARRIER SENSING

- Compared to a 802.3 Ethernet LAN with CSMA/CD the collision detection in a radio environment is not possible. Therefor the emphasis is on the avoidance of collisions. The most promising MAC protocols are based on a Listen Before Talk function. Before a frame can be sent into the air, the receiver must examine if there is something received above a certain threshold level: the carrier sense (CS) function. Fast carrier sensing is crucial for the performance of a Radio network. The sooner it is known that an other station started transmission the lower the probability for a collision, the so called collision window must be as short as possible. The length of the carrier sense  delay effects heavily the 'normalised propagation' delay (this is the ratio carrier sense delay/packet transmission time) in a CSMA environment (see a.o. doc IEEE P802.11/91-67). As known the normalised propagation delay is a parameter for the CSMA performance.

- - COVERAGE

- The covered range depends on environment, interference situation, receiver sensitivity, regulatory power limits and the outage requirements. High receiver sensibility and diversity provisions are utilised to provide a maximum coverage range. The actual coverage range depends on:

- - Path loss characteristics.

- Attenuation coefficients of n=2 up to to 6 can be expected.

- - Multipath.

- The radio system must be designed such that it is robust against delay spread. In normal indoor environments delay spread figures up to 200 ns occur.

- - Fading.

- Radio systems suffer from fading and shadowing. The fading margin that is to be taken into account can be improved significantly by applying an antenna diversity scheme.

- - Interferers.

- In a radio environment there are all kinds of interferers. If there is a neighbouring system on the same frequency a receiver must be capable to capture a frame of its own network to overcome this co-channel interference. Other interferers are e.g. narrow band interferers. The carrier sense function should not be sensitive for interference sources.  The characteristics of the system must be chosen such that it is robust against interferers.

- - POWER CONTROL

- Radio systems transmitting on the same frequencies do not have the physical separation like cables. To use the medium as optimally as possible special arrangements must be designed in order to optimise the reuse of the medium. If two transmitters are sending to two different receivers at the same time, the interference at the receiver of the wanted signal by the not wanted is determined by the power with which the signals are sent and the attenuation of the two signals. Medium reuse (or parallel transmission) can be optimised by having the arrangement that no transmitter is emitting more power than necessary for reliable reception  by the receiver. The optimal amount of power can change and depends on mobility. The control of this power is a dynamic function that differs per destination or that can have a system dependent limit. This proposal contains provisions to support Dynamic Power Control.

- If a station is sending with less power than inherently the threshold for carrier sensing must be adapted. In this case the threshold can be less sensitive to prevent that a station defers for another transmitter, while this was not necessary.

- AUTONUMLGL§	MAJOR CHARACTERISTICS

- The major characteristics for the Direct Sequence Spread Spectrum PHY are listed below:

- - FREQUENCY BAND

- The system is designed to provide multiple channels in the 2.4 GHz ISM band. Also a single channel can be supported in the 915 MHz ISM band.

- - SPREADING SEQUENCE

- An 11-chip Barker sequence is proposed for spreading. Spreading with a factor 11 is conformant to the FCC requirement for a processing gain of 10 dB. In addition the sequence has very good odd and even autocorrelation properties. It makes the system robust against interferers.

- - SYMBOL RATE

- The symbol rate is 1 MBaud. Together with the 11-chip spreading sequence this 11 Mchip/s system shows good robustness against delay spread.

- - MODULATION

- The modulation method is differential QPSK. The four phases define 2 bit per symbol resulting in a bitrate of 2 Mbit/s. QPSK has been analysed as very efficient with regard to medium reuse and modulation bandwidth (doc IEEE 802.11/91-22) . With differential schemes the need for accurate carrier tracking is less than with coherent detection.

- - CARRIER SENSING

- The system should have a fast carrier sense capability. Carrier detection must be more sensitive than the receiver sensitivity. This is needed to limit co-channel interference in an overlapping environment.

- - CARRIER OFFSET TRACKING

- The system is capable to determine and track the carrier offset caused by a frequency difference between the Local Oscillators of the transmitter and receiver. The system is designed to allow the use of 50 ppm crystal accuracy for operation in the 2.4 GHz band to allow low cost implementation.

- - CAPTURE CAPABILITY

- While receiving a frame from one transmitter, the receiver should be capable to capture a frame from another transmitter if the level of this second frame is  above the minimum capture ratio. A minimum capture ratio of  8 dB is feasible.

- AUTONUMLGL§	OVERVIEW

- The proposed Physical Layer will be one of the multiple PHY's that will be part of the IEEE 802.11 standard. This document will concentrate on the Medium Dependent Layer (see figure 1).

- EMBED MSDraw   \* mergeformatµ §

- fig 1:  General overview of the 802.11 MAC and PHY Layer.

- Figure 2 shows the functional blocks of the PHY.

- Different functional blocks can be identified in the PHY. The TX block performs all the analog and /or digital signal processing transmitter functions. It includes a TX control block that controls the sequence of all these functions: e.g. start of transmission, generation of the training sequence etc. It interfaces to the MAC.

- The same blocks can be distinguished in the receiver part. In addition to the RX and TX control blocks a management block is defined. All static and per packet handling are controlled here. Examples are TX level,  RX threshold or bitrate. The management block is interfaced to the MAC through the Mngt I/F

- Finally there is an antenna interface. This I/F connects the TX or RX to the antenna.

- EMBED MSDraw   \* mergeformatµ §

- Figure 2  Basic Blocks in the PHY and the Interfaces.

- AUTONUMLGL§	THE TRANSMITTER AND RECEIVER BLOCK DIAGRAMS

- This chapter gives the functional blocks that allow the description of the important characteristics of transmitter and receiver. They are described in a generic model; it is not the intention to specify the implementation of transmitter or receiver.

- AUTONUMLGL§	THE TRANSMIT PATH

- Prior to the MAC-frame a PHY-preamble (training sequence) for the training and synchronisation of the receiver is generated (See Training requirements).

- The MAC-frame is scrambled. The FCC requires this if the spreading code length is less than 127.  The now formed bit stream is input of the differential encoder. Dibits are formed from the data bits and mapped to I (in phase) and Q (quadrature) signals in the differential encoder.  Differential encoding is chosen, which makes the requirements for carrier recovery in the receiver less stringent.

- EMBED MSDraw   \* mergeformatµ §

- Figure 3. Transmitter functional block diagram.

- The resulting I and Q data signals are still symbol based. In the next step spreading is introduced: The I and Q signals are both replaced by a sequence of 11 chips. In both paths a +1 is replaced directly by the sequence, a  -1 is replaced by the sign-inverted sequence. The I and Q output signals of the   spreading block have a chiprate of 11 Mchips/s. Now the I and Q signal are filtered and upconverted to the RF-band. This can be the 915 MHz band or the 2.4 GHz band. The resulting RF signal is amplified by the power amplifier (PA) before it  is send out by the antenna.

- AUTONUMLGL§	THE RECEIVE PATH

- The functionality of the receiver and the associated requirements for the PHY preamble generated in the transmitter are described  with reference to the following block diagram.

- EMBED MSDraw   \* mergeformatµ §

- Figure 4. Receiver functional block diagram

- After bandpass filtering the modulated RF signal is fed to the downconverter which delivers the I and Q baseband signals. The baseband signals are filtered and gain controlled. The AGC should take care of a huge dynamic range.

- Both I and Q signals are fed into the correlators and correlated with the 11 bit sequence. The correlator is followed by a timing recovery circuit. The function is to find and track   the peak in the combined output of the I and Q correlator. The peak position defines the symbol timing. Tracking is necessary to compensate for TX and RX clock differences.

- Because of the use of differential encoding two correlator output peak signals undergo in the differentiator a phase difference determination. This phase difference is translated in the databits. However, caused by a frequency difference of the local oscillators (LO) of the transmitter and the receiver an additional phase turn is introduced for each symbol. The carrier compensation circuit should take care of this. If the carrier compensation circuit is capable of determine average phase turns per symbol of about 90 degrees (this is 250 KHz in a 1 MBaud system). This means for the 2.4 GHz crystals an accuracy of 50 ppm for both TX and RX.  In addition this circuit must track the carrier offset during the reception of a frame because of LO jitter.

- Finally the phase jumps are fed into the decision unit to retrieve the bits. This bit stream is descrambled in the descrambler block. The descrambler must be synchronised (or initialised).

- As described above an important receiver function is a reliable carrier sense function. The best place to perform carrier sensing is behind the correlator, where a data signal can be distinguished from any other signal and a signal quality measurement can be performed.

- For each incoming frame different circuits have to be trained. Therefor a training sequence (preamble) has to be defined which is generated by the transmitter. In this training also the decision is to be made which antenna is to be selected.

- In the next paragraph the requirements for the training will be described.

- AUTONUMLGL§	TRAINING REQUIREMENTS

- The above described blocks that need training are:

- - AGC

- - Timing recovery

- - Carrier compensation

- - Descrambler

- The following characteristics of the receiver define the requirements for the training sequence (preamble):

- - Fast carrier sensing

- - Antenna selection

- - Carrier offset training

- - Descrambler training

- - Capturing capability

- They are described in the next paragraphs.

- AUTONUMLGL§	FAST CARRIER SENSING

- For fast carrier sensing the following functions must be performed on the incoming training sequence:

- - Automatic Gain Control.  This can be done either in the analog RF part or in the baseband signal processing part or both. The required performance of the AGC function is dependent on the rest of the receiver . Initially the adaptation of the AGC can be relatively fast. (Once the training is done, the AGC adaptation must be slower to avoid AM distortion)

- - Carrier sensing indicates that a valid input frame is present. Validity is checked by correlating the input frame with the spreading sequence in the correlator. If there is a clear peak in the correlator output compared to other values in the symbol, this indicates a valid frame. The proposed carrier sensing approach has the big advantage that the receiver will not react on an incoming noise signal or (non co-channnel) interferer.

- - The peak can be fast identified and with high reliability if the rectified output samples of the correlator is integrated based on a modulo symbol time (sample 12 is added to sample 1, sample 13 added to sample 2, etc.)

- - In the carrier detection circuit the height of the peak is compared to the other samples within the symbol. The ratio can be used as a measure for the quality of the incoming signal (SQ).

- Carrier sense time is dependent on the performance of the above described functions.

- AUTONUMLGL§	ANTENNA SELECTION.

- For antenna selection two scenarios can be employed:

- - Parallel measurement at the antennas is performed (at the same time) and the best is selected. Parallel measurement need also parallel receivers ( see figure 5).

- - The alternative is consecutive measurement via the