IEEE Project 802.11

Wireless Local Area Networks

Proposed Standard:

Physical Layer Specification for

2.4 GHz Direct Sequence Spread Spectrum

Wireless LAN

This is an unapproved draft of a portion of a proposed IEEE Standard, subject to change.  Permission is hereby granted for IEEE Standards Committee participants to reproduce this document for purposes of IEEE standardization activities.  Permission is also granted for member bodies and technical committees of ISO and IEC to reproduce this document for the purposes of developing a national position.  Other entities seeking permission to reproduce this document for standardization or other activities, or to reproduce portions of this document for these or other uses, must contact the IEEE Standards Department for the appropriate license.  Use of information contained in this unapproved draft is at your own risk.

IEEE Standards Department

Copyright and Permissions

445 Hoes Lane, P.O. Box 1331

Piscataway, NJ  08855-1331, USA

DO NOT SPECIFY OR CLAIM CONFORMANCE

TO THIS DOCUMENT

Editor:			Paul Struhsaker

AIRONET Wireless Communications, Inc.

367 Ghent Road, Suite 300

Akron, Ohio  44333

Email: pstru@telxon.com

# 101.0 Physical Layer Specification for 2.4 GHz Direct Sequence Wireless LAN

### 101.1.1 Scope

This document describes the physical layer services provided by the 802.11 wireless LAN MAC for the 2.4 GHz Direct Sequence Spread Spectrum (DS) system. The DS PHY layer consists of two protocol functions as follows:

a)	A physical layer convergence function which adapts the capabilities of the physical  medium dependent system into the Physical Layer service. This function is supported by the Physical Layer Convergence Procedure (PLCP) which defines a method of mapping the 802.11 MAC layer Protocol Data Units (MPDU) into a framing format suitable for sending and receiving user data and management information between two or more nodes using the associated physical medium dependent system.

b)	A Physical Medium Dependent (PMD) system whose function defines the characteristics of, and method of transmitting and receiving data via wireless media between two or more nodes.

Each physical medium dependent sublayer for the DS PMD may require the definition of a unique PLCP. If the PMD sublayer already provides the defined Physical Layer services, the physical layer convergence function might be null.

### 101.1.2 DSSS Physical Layer Functions

The 2.4 GHz DS PHY architecture is shown in Figure 10-1. The DS physical layer contains three functional entities: the physical medium dependent function, the physical layer convergence function, and the layer management function. Each of these functions is described in detail in the following subsections.

The DS Physical Layer service is provided to the Media Access Control through the physical layer service primitives described in Section 8.

µ §

Figure 101.1-1, Protocol Reference Model

#### 101.1.2.1 Physical Layer Convergence Procedure Sublayer

In order to allow the 802.11 MAC to operate with minimum dependence on the PMD sublayer, a physical layer convergence sublayer is defined. This function simplifies the physical layer service interface to the 802.11 MAC services.

#### 101.1.2.2 Physical Medium Dependent Sublayer

The physical medium dependent sublayer provides a transmission to send or receive data between  two or more nodes. In this case, the 2.4 GHz ISM bands using Direct Sequence modulation.

#### 101.1.2.3 Physical Layer Management Entity (LME)

The Physical LME performs management of the local Physical Layer Functions in conjunction with the MAC Management entity.

### 101.1.3 Definitions

This section defines the terms used in this standard. Words in italics indicate terms that are defined elsewhere in the lists of definitions

### 101.1.4 Acronyms

BPDU		Burst Protocol Data Unit

CCA		Clear Channel Assessment

CRC		Cyclic Redundancy Check

DBPSK		Differential Binary Phase Shift Key

DQPSK		Differential Quadrature Phase Shift Key

DS/DSSS	Direct Sequence Spread Spectrum

ED		Energy Detection

FCS		Frame Check Sequence

LME		Layer Management  Entity

MAC		Media Access Control

MIB		Managment Information Base

MPDU		MAC Protocol Data Unit

PDU		Protocol Data Unit

PHY_SAP	Physical Layer Service Access Point

PLCP		Physical Layer Convergence Procedure

PMD		Physical Medium Dependent

PMD_SAP	Physical Medium Dependent Service Access Point

PN		Pseudo Noise (PN code sequence)

RSSI		Received Signal Strength Indication

SQ		Signal Quality (PN correlation strength)

SAP		Service Access Point

TXE		Transmit Enable

### 101.1.5 Service Specification Method and Notation

The models represented by figures and state diagrams are intended as the illustrations of functions provided. It is important to distinguish between a model and a real implementation. The models are optimized for simplicity and clarity of presentation, the actual method of implementation is left to the discretion of the 802.11 DS PHY compliant developer.

The service of layer or sublayer is a set of capabilities that it offers to a user in the next higher layer (or sublayer). Abstract services are specified here by describing the service primitives and parameters that characterize each service. This definition is independent of any particular implementation.

## 101.2 DSSS Physical Layer Convergence Procedure Sublayer

[ Editors note: DS PHY Template items 3 and 18 are covered in this section]

### 1011.2.1 Introduction

This section provides a convergence procedure in which MPDUs  are converted to and from BPDUs. During transmission, the MPDU is appended with a PLCP preamble (radio header) to create the BPDU. At the receiver, the PLCP preamble is processed and the internal data fields are processed to aid in demodulation and delivery of the MPDU.

### 101.2.2 Physical Layer Convergence Procedure Frame Format

Figure 101.2.1 shows the format for the BPDU including the PLCP preamble and the MPDU. The PLCP preamble contains the following fields: synchronization, unique word, 802.11 signaling, 802.11 service, length, CRC16. Each of these  fields will be described in detail in section 101.2.3.

µ §

Figure 101.2.1, PLCP Frame Format

### 101.2.3 PLCP Field Definitions

The entire PLCP preamble shall be transmitted using the 1 MBPS DBPSK  modulation described in section 101.4.7. All the transmitted data bits shall be scrambled using the feedthrough scrambler described in section 101.4.

#### 101.2.3.1 PLCP Synchronization (Sync)

The synchronization field consists of 128 bits of scrambled 1 bits. This field is provided so that the receive can perform  the necessary operations for synchronization.

#### 101.2.3.2 PLCP Unique Word Field

The Unique Word (or start frame delimiter)  is provided  to indicate the start of PHY dependent parameters within PLCP preamble. The Unique Word consist of a 16 bit word h05CFfield,  hF3A0 (MSB to LSB). In binary, the Unique Word is: 0000010111001111. The left most bit (LSB) shall be transmitted first in time.

#### 101.2.3.3 PLCP 802.11 Signal Field

The 8 bit 802.11 signal  field indicates to the PHY the modulation which will be used for transmission or reception of the MPDU. Each bits reprepresent 100KBPS increment  in the data rate. The DS PHY currently supports two mandatory modulation services given by the following 8 bit words, where the left most bit is transmitted first in time:

a)	h0A (MSB to LSB) / binary  01010000 for 1 MBPS DBPSK

b)	h14 (MSB to LSB) /  binary  00101000 for 2 MBPS DQPSK

The DS PHY rate change capability will be described in section 101.2.5. This field is protected by the CRC16 frame check sequence described in section 101.2.3.6

#### 101.2.3.4 PLCP 802.11 Service Field

The 8 bit 802.11 service field  is reserved for future use. This field must be set  to h00 / binary 00000000.  The left most 802.11 service field bit shall be transmitted first in time. This field is protected by the CRC16 frame check sequence described in section 101.2.3.6

#### 101.2.3.5 PLCP Length Field

The PLCP length field is an unsigned 16 bit integer which indicates the number of octets ( 1 to a maximum of  [2048 data payload octets + the octets for mac over head]) to be transmitted in the MPDU. The transmitted value is provided by the LENGTH parameter in the PHY_DATA.request primitive described in section 8. The LSB (least significant bit) shall transmitted first in time. The bits shall be transmitted in increasing sequential order with the MSB (most significant bit) being transmitted last. This field is protected by the CRC16 frame check sequence described in section 101.2.3.6

#### 101.2.3.6 PLCP CRC16 Field

The 802.11 signal, 802.11 service, and length field shall be protected with a CCITT CRC16 FCS (frame check sequence). The CRC16 FCS is the ones compliment of the remainder generated by the module 2 division of the protected PLCP fields by the polynomial:

x16 +  x12   +  x5 + 1

The protected bits will be processed in transmit order. All FCS calculations shall be made prior to data scrambling.

As a typical implementation at the transmitter, the initial contents of the register of the device computing the division is preset to all 1's and is then modified by the division by the generator polynomial (as described above) of the PLCP data fields. The ones complement of the resulting remainder is transmitted as the 16 bit FCS.

As a typical implementation at the receiver, the initial contents of the register of the device computing the remainder is preset to all 1's. The final remainder, after multiplication by x16 and then module 2 division by the generator polynomial (above) of the serial incoming protected bits (after randomizer has been removed) will be  0001 1101 0000 1111 (x15 to x0 ) in the absence of transmission errors.

As an example, the signal, PLCP service, and Length fields for a DQPSK signal with a packet length of 3192 would be given by the following:

001101 0000 0000 0000 1100 0000 0011 0000 0000 (left most bit transmitted first in time)

The ones complement FCS for these protected PLCP preamble bits would be the following:

0100 0001 1000 00101 1011 0101 0111 (left most bit transmitted first in time)

### 101.2.4 PLCP / DS PHY Data Scrambler and Descrambler

The polynomial 1 +  x-4   +  x-7  shall be used to scramble ALL data transmitted by the DS PHY. The feedthrough configuration of the scrambler requires is self initializing. No prior initialization of the scrambler is required for either transmit or receive processing. Figures 10.2.2 and 101.2.2 and 11.2.3  shows typical implementation of the data scrambler/descrambler. Other implementations are possible.

µ §

Figure 101.2.2, Data Scrambler

µ §

Figure 101.2.3, Data Decrambler

### 101.2.5 PLCP Data Modulation and Modulation Rate Change

The PLCP preamble shall be transmitted using the 1 Mbps DBPSK  modulation. The 802.11 signal field will indicate the modulation which will be used to transmit the MPDU. The transmitter and receiver will initiate the modulation indicated by the 802.11 signal field starting with the first symbol (1bit for DBPSK 2 bits for DQPSK) of the MPDU. The MPDU transmission rate is set by the RATE parameter in PHY_DATA.request primitive.

### 101.2.6 PLCP Transmit Procedure

The PLCP transmit procedure is shown if figure 101.2.4. All commands issued by MAC require PHY-DATA.confirm primitives to be issued by the PHY. The PHY-DATA.confirm primitives provide flow control between the MAC and the PHY. The PHY-DATA.confirm primitives are not shown in figure 101.2.4.

Based on the status of  CCA  indicated by PHY_DATA.indicate(START-OF-ACTIVITY) the MAC will assess that the channel is clear. A clear channel is indicated by PHY_DATA.indicate(END-OF-ACTIVITY). If the channel is clear, transmission of the MPDU is initiate by issuing the PHY_DATA.request (START-OF-ACTIVITY) primitive. The TXVECTOR elements for the PMD Parameters associated with this primitive areHY_DATA.request(STATR_OF_ACTIVITY) are the PMD Parameters  LENGTH, ANTSEL, RATE (SIGNALING), SERVICE, CHNL_ID, and TXPWR_LEVEL.

The PLCP will issue PMD_ANTSEL, PMD_RATE, PMD_SERVICE, PMD_FREQ, and PMD_TXPWRLVL primitives to configure the PHY. The PLCP will then issue a PMD_TXE enable and the PHY entity will immediately initiate data scrambling and transmission of the PLCP preamble based on the parameters passed in the PHY_DATA.request primitive. The time required  for TX power on ramp described in section 101.4.7 is included in the PLCP synchronization field. Once the PLCP preamble transmission is completed, data  is exchanged  between the  MAC and the PHY  by a series of PH_DATA.request(DATA, octet) issued by the MAC and PHY_DATA.confirm primitives. The modulation rate change, if any, is initiated with the first data symbol of the octet of the MPDU as described in section 101.2.5. The PHY proceeds with MPDU transmission through a series of data octet transfers form the MAC. At the PMD layer, each octet  is divided into symbols (1 or 2 bits) and presented to the PHY layer through PMD_DATA.request primitives.  The data octets are transmitted  In LSB to MSB order with the LSB being transmitted first in time. Transmission is terminated by the MAC  through the primitive PHY_DATA.request(END-OF-DATA-AND-ACTIVITY).) followed by PHY_DATA.request(END_OF_ACTIVITY).  PMD_TXE is disabled with the end of activity. The termination occurs after the transmission of the final bit of the last MPDU octet. The number of octets transmitted must equal the number supplied in the DS PHY preamble LENGTH. The packet transmission is completed and the PHY entity enters the default receive state (i.e. PMD_TXE is disabled).

[ Editors note: are the octets being transmitted LSB to MSB or MSB to LSB?  This is very important for interoperablility! put in confirms ]

µ  §

Figure 101.2.4, PLCP Transmit Procedure

#### 101.2.6.1 Transmit State Machine

A typical state machine implementation of the PLCP transmit procedure is provided in figure 101.2.5.

µ §µ §µ §

Figure 101.2.5, PLCP Transmit State Machine

### 101.2.7 PLCP Receive Procedure

The PLCP receive procedure is shown if figure 101.2.6. All commands issued by MAC require PHY-DATA.confirm primitives to be issued by the PHY. The PHY-DATA.confirm primitives provide flow control between the MAC and the PHY. The PHY-DATA.confirm primitives are not shown in figure 101.2.6.

In order to receive data, PMD_TXE must be disabled so that the PHY entity is in the default receive state. Further, the PHY must be set to operate at the appropriate CHNL_ID through Station Management via the Layer Management Service Access Point (LMSAP). Other receive parameters such as RSSI,  SQ (signal quality, and indicated RATE may also be accessed via the LMSAP.

Upon receiving the transmitted energy, the PMD_ED will be enabled as the RSSI strength reaches the ED_THRESHOLD. If the transmitted energy is associated with a DS PHY PLCP preamble synchronization, then if the PHY entity acquires PN code lock based on the SQ_THRESHOLD, and PMD_CS will be enabled. Based on the CCA (see section 101.4.8.4) these conditions can be used to indicate activity to the MAC. by issuing the PHY-DATA.indicate(START-OF-ACTIVITY)is issued for energy detection or code lock prior to correct reception of the PLCP frame. The PMD primitives PMD_SQ and PMD_RSSI are issued to update the  RSSI and SQ parameters reported to the MAC.

The PHY entity begins searching for the Unique Word field. Once the Unique word field is detected, CRC16 processing is initiated and the PLCP 802.11 signal, 802.11 service, and Length fields are received. The CRC16 FSC will be processed. If the CRC16 FCS check fails or no match is found for the 802.11 signal or service fields, a PHY-DATA.indicate(END-OF-ACTIVITYDATA) primitive will be issued and PHY receiver will be reset. Should code lock be lost during reception prior to completion of either the PLCP header or the full MDPU as indicted by the length field, the PLCP will issue a PHY-DATA.indicate(END-OF-ACTIVITYDATA,carrier_lost).

If the PLCP preamble reception is successful a PHY_DATA.indicate(START_OF_DATA) will be issued,. The RXVECTOR associted with this primitive includes the signaling field, the service field, the length field, PMD_RSSI, and PMD_SQ.

Tthe received MPDU bits are assembled into octets and presented to the MAC using  a series of PH-DATA.indicate(DATA, octet)  and PHY_DATA.confirm primitive exchanges. The rate change indicated in the 802.11 signal field is initiated with the first symbol  of the MPDU as described in section 101.2.5. The PHY proceeds with MPDU reception. Reception is terminated after the reception of the final bit of the last MPDU octet indicated by the PLCP preamble LENGTH field. Depending on the state of energy detection and code lock, either a PHY-DATA.indicate(END-OF-DATA, no_error) or a PHY-DATA.indicate(END-OF-DATA-AND-ACTIVITY) primitive is issued to the MAC. Should code lock / synchronization  be lost prior to the end of the MPDU as indicated in the PLCP LENGTH field, the error condition PHY-DATA. indicate(END-OF-ACTIVITYDATA, carrier_lost) will be reported to the MAC.

µ  §

Figure 101.2.6, PLCP Receive Procedure

#### 101.2.7.1 Receive State Machine

A typical state machine implementation of the PLCP transmit procedure is provided in figure 101.2.7.

µ  §

Figure 101.2.7, PLCP Receive State Machine

## 101.3 DSSS Physical Layer Management Entity

This section will be completed after the Management [ Information Block structure has been determined at the July Plenary Session.]

### 1011.3.1 MPHY_SAP Sublayer Managment primitives

The following message may be sent  betwwen the PHY sublayer entities and intra layer of higher Layer Managment Entities (LME).

| Primitive | Request | Indicate | Confirm | Response |
| --- | --- | --- | --- | --- |
| MPHY_RESET | X |  | X |  |

Table 101.3.1 MPHY_SAP Sublayer Managment Primitives

### 101.3.2 MPHY_SAP Managment Service Primitive Parameters

There are currently no parameters associated with  the MPHY_SAP service primitives

### 101.3.3 MPHY_ SAP Detailed Service Specification

#### 101.3.3.1 MPHY_RESET.request

##### 101.3.3.1.1 Function

This primitive is a request by the LME to reset the PHY. The PHY is always reset to the receive state to avoid accidential data transmission

##### 101.3.3.1.2 Sematics of the Service Primitive

The primitive shall provide the following parameters:

MPHY_RESET.request

There are no parameters associated with this primitive

##### 101.3.3.1.3 When Generated

This primitive is generated at any time to reset the PHY

##### 101.3.3.1.4 Effect of Receipt

Receipt of this primitive by the PHY sublayer will cause the PHY entity to reset both the transmit and the receive state machines and place the PHY into the default receive state.

#### 101.3.3.2 MPHY_RESET.confirm

##### 101.3.3.2.1 Function

This primitive is a  confirmation of request by the LME to reset the PHY.

##### 101.3.3.2.2 Sematics of the Service Primitive

The primitive shall provide the following parameters:

MPHY_RESET.confirm

There are no parameters associated with this primitive

##### 101.3.3.2.3 When Generated

This primitive is generated as a response  to the MPHY_RESET.request once the PHY has sucessfully completed the reset.

##### 101.3.3.2.4 Effect of Receipt

The effect of the receipt of this primitive by the LME is unspecified.

### 101.3.4 DSSS Physical Layer Managment Information Base

Text for this section is  incomplete and under revistion

The following parameters will be included in the DS PHY MIB. All fields are 8 bits in length and represent either integers form 0 to 255 or  as the exact field definition in the PLCP or PMD sections of this documents

Static PHY capabilities reported to the MAC

| Parameter | Definition |
| --- | --- |
| NO_ANT | Number of Antennas |
| NO_TXPWRLVLS | Number of transmit power levels |
| NO_CHANNELS | Number of channels |
| NO_RATES | Number of Rates |
| RATE(0) - RATE(255) | RATE list of PLCP signal fields which indicate the rates supported |

Operational Parameters set by the MAC.

| Parameter | Definition |
| --- | --- |
| CHNL_ID | The current DS PHY channel which may be set by either Layer management or by a PHY_DATA.request primitive issued by the MAC |
| SQ_THRESHOLD | Signal Quality / Correlation Threshold for the PMD_CS function |
| RSSI_THRESHOLD | RSSI threshold for the PMD_ED |

Operational PHY Status.

| Parameter | Definition |
| --- | --- |
| RSSI | RSSI level measured at the point of PMD_CS enable (code lock) |
| SQ | Signal Quality / Correlation level at the point of PMD_CS enable (code lock) |
| ANT_STATE | Current Antenna Position |

## 101.4. DSSS Physical Medium Dependent Sublayer

### 101.4.1 Scope and Field of Application

This section describes the PMD services provided to the PLCP for the DS Physical Layer. Also defined in the section are the functional, electrical, and RF characteristics required for interoperablity of implementations conforming to this specification. The relationship of this specification  to the entire DS OHY Layer is shown in figure 101.4.1

µ §

Figure 101.4.1, PMD Layer Reference Model

### 101.4.2 Overview of Service

The DS Physical Medium Dependent Sublayer accepts Physical Layer Convergence Procedure sublayer service primitives and provides the actual means by which data is transmitted or received from the media. The combined function of  DS PMD sublayer primitives and parameters  for the receive function  results in a data stream, timming information, and associated received signal parameters being delivered to the PLCP sublayer. A like functionality is provided for data transmission.

### 101.4.3 Overview of Interactions

The primitives associated with the 802.11 PLCP sublayer to the DS PMD falls into two basic categories:

a)	Service primitives that support PLCP per-to-per interactions

b)	Service primitives that have local significance and support sublayer-to-sublayer interactions.

### 101.4.4 Basic Service and Options

All of the service primitives described in this section are considered mandatory unless otherwise specified.

#### 101.4.4.1 PMD_SAP Peer-to-Peer Service Primitives

The following table indicates the primitives for peer-to-peer interactions.

| Primitive | Request | Indicate | Confirm | Response |
| --- | --- | --- | --- | --- |
| PMD_DATA | X | X |  |  |

Table 101.4.1, PMD_SAP Peer-to-Peer Service Primitives

#### 101.4.4.2 PMD_SAP Sublayer-to-Sublayer Service Primitives

| Primitive | Request | Indicate | Confirm | Response |
| --- | --- | --- | --- | --- |
| PMD_TXE | X |  |  |  |
| PMD_ANTSEL | X |  |  |  |
| PMD_TXPWRLVL | X |  |  |  |
| PMD_FREQ | X |  |  |  |
| PMD_RATE | X | X |  |  |
| PMD_RSSI |  | X |  |  |
| PMD_SQ |  | X |  |  |
| PMD_CS |  | X |  |  |
| PMD_ED |  | X |  |  |

Table 101.4.2, PMD_SAP Sublayer-to-Sublayer Service Primitives

#### 101.4.4.3 PMD_SAP Service Primitive Parameters

| Parameter | Associate Primitive | Value |
| --- | --- | --- |
| TXD_UNIT | PMD_DATA.request | One(1), Zero(0): BPSK di bit combinations 00,01,11,10: QPSK |
| RXD_UNIT | PMD_DATA.indicate | One(1), Zero(0): BPSK di bit combinations 00,01,11,10: QPSK |
| RF_STATE | PMD_TXE.request | Receive, Transmit |
| ANT_STATE | PMD_ANTSEL.request | 1 to 256 |
| TXPWR_LEVEL | PMD_TXPWRLVL | 0,1,2,3 (max of 4 levels) |
| CHNL_ID | PMD_FREQ.request | 0-6 for channel set |
| RATE | PMD_RATE.indicate PMD_RATE.request | h0A for 1 MBPS DBPSK h14 for 2 MBPS DQPSK |
| RSSI | PMD_RSSI.indicate | 0-7 bits of RSSI |
| SQ | PMD_SQ.indicate | 0-7 bits of Signal Quality |

Table 101.4.3, List of Parameters for the PMD Primitives

### 101.4.5 PMD_SAP Detailed Service Specification

The following section describes the services provided by each PMD primitive.

#### 101.4.5.1 PMD_DATA request

##### 101.4.5.1.1 Function

This primitive defines the transfer of data from the PLCP sublayer to the PMD entity.

##### 101.4.5.1.2 Semantic of the Service Primitive

The primitive shall provide the following parameters:

PMD_DATA.request(TXD_UNIT)

The TXD_UNIT parameter takes on the value of either ONE(1) or ZERO(0) for BPSK modulation or the di-bit combination 00, 01, 11, or 10 for modulation. This parameter represents a single symbol of data bit which in turn is used by the PHY to be differential encoded into a DBPSK or DQPSK transmitted symbol. The symbol itself is spread by the PN code prior to transmission.

##### 101.4.5.1.3 When Generated

This primitive is generated by the PLCP sublayer to request transmission of a symbol. The data clock for this primitive is supplied by PMD layer based on the PN code repetition.

##### 101.4.5.1.4 Effect of Receipt

The PMD performs the differential coding, PN code modulation, and transmission of the data.

#### 101.4.5.2 PMD_DATA.indicate

##### 101.4.5.2.1 Function

This primitive defines the transfer of data from the PMD entity to the PLCP sublayer.

##### 101.4.5.2.2 Semantic of the Service Primitive

The primitive shall provide the following parameters:

PMD_DATA.indicate(RXD_UNIT)

The RXD_UNIT parameter takes on the value of  ONE(1) or ZERO(0) for BPSK modulation or as the di-bit 00, 01, 11, or 10 for QPSK modulation. This parameter represents a single symbol  which has been demodulated by the PMD entity.

##### 101.4.5.2.3 When Generated

This primitive is generated by the PMD entity  forwards received data to the PCLP sublayer. The data clock for this primitive is supplied by PMD layer based on the PN code repetition.

##### 101.4.5.2.4 Effect of Receipt

The PLCP sublayer either interperates the bit or bits which are recovered as part of the PLCP convergence procedure or pass the data to the MAC layer as part of the MPDU.

#### 101.4.5.3 PHY_TXE.indicate

##### 101.4.5.3.1 Function

This primitive, generated by the PHY PLCP sublayer, initiates BPDU transmission by the PMD layer.

##### 101.4.5.3.2 Semantic of the Service Primitive

The primitive shall provide the following parameters:

PMD_TXE.request(RF_STATE)

RF_STATE. The RF_STATE parameter indicates one of two possible PHY operating conditions: Receive or Transmit. This primitive is a Transmit Enable. The default PHY state is receive.

##### 101.4.5.3.3 When Generated

This primitive is generated  by the PLCP sublayer to initiate the PMD layer transmission of the BPDU. The PHY_DATA.request primitive must be provided to the PLCP sublayer prior to setting RF_STATE = transmit within the PMD_TXE command.

##### 101.4.5.3.4 Effect of Receipt

PMD_TXE initiates or terminates transmission of a BPDU by the PMD sublayer.

#### 101.4.5.4 PMD_ANTSEL.request

##### 101.4.5.4.1 Function

This primitive, generated by the PHY PLCP sublayer, selects the antenna used by the PHY for transmission or reception.

##### 101.4.5.4.2 Semantic of the Service Primitive

The primitive shall provide the following parameters:

PMD_ANTSEL.request(ANT_STATE)

ANT_STATE. ANT_STATE selects which of the available antennas should be used for transmit. The number of available antenna is determined by the PHY_INFO parameters NO_ANT. ( Note: receive antenna selection, if any, is a funtion of the vendor implemented PHY)

##### 101.4.5.4.3 When Generated

This primitive is generated  by the PLCP sublayer to select a specific antenna for transmission. While not required,  multiple antennas can be  used to implement a receive antenna diversity scheme. The PMD  parameters RSSI and SQare provided for optional antenna diversity implementation.

##### 101.4.5.4.4 Effect of Receipt

PMD_ANTSEL immediatly selects the antenna specified by ANT_STATE

#### 101.4.5.5 PMD_TXPWRLVL.request

##### 101.4.5.5.1 Function

This primitive, generated by the PHY PLCP sublayer, selects the power level used by the PHY for transmission.

##### 101.4.5.5.2 Semantic of the Service Primitive

The primitive shall provide the following parameters:

PMD_TXPWRLVL.request(TXPWR_LEVEL)

TXPWR_LEVEL. TXPWR_LEVEL selects which of the optional transmit power levels provide antennas should be used for the current paclet transmission. The number of avaliable power levels  is determined by the MIB parameter NO_TXPWRLVLS. Section 101.4.7.3 provides further information on the optional DS PHY power level control capabilities.

##### 101.4.5.5.3 When Generated

This primitive is generated  by the PLCP sublayer to select a specific transmit power. This primitive is applied prior to setting PMD_TXE into the transmit state.

##### 101.4.5.5.4 Effect of Receipt

PMD_TXPWRLVL immediatly sets the transmit power level given by TXPWR_LEVEL.

#### 101.4.5.6 PMD_FREQ.request

##### 101.4.5.6.1 Function

This primitive, generated by the PHY PLCP sublayer, selects the channel frequency which will be used by the DS PHY for transmission or reception.

##### 101.4.5.6.2 Semantic of the Service Primitive

The primitive shall provide the following parameters:

PMD_FREQ.request(CHNL_ID)

CHNL_ID. CHNL_ID selects which of the DS PHY channels frequencies will be used for transmission or reception. Section 101.4.6.2 provides further information on the DS PHY channel plan.

##### 101.4.5.6.3 When Generated

This primitive is generated  by the PLCP sublayer to change or set the current DS PHY channel frequency.

##### 101.4.5.6.4 Effect of Receipt

The receipt of  PMD_FREQ immediatly changes the frequency channel as set by the CHNL_ID parameter.

#### 101.4.5.7 PMD_RATE.request

##### 101.4.5.7.1 Function

This primitive, generated by the PHY PLCP sublayer, selects the modulation RATE which will be used by the DS PHY for transmission.

##### 101.4.5.7.2 Semantic of the Service Primitive

The primitive shall provide the following parameters:

PMD_RATE.request(RATE)

RATE. RATE selects which of the DS PHY data rates will be used for MPDU transmission  Section 101.4.6.4 provides further information on the DS PHY modulation rates. The DS PHY ratechange capability is fully described in section 101.2.

##### 101.4.5.7.3 When Generated

This primitive is generated  by the PLCP sublayer to change or set the current DS PHY modulation rate used for MPDU portion of a BPDU.

##### 101.4.5.7.4 Effect of Receipt

The receipt of  PMD_RATE selects the rate which will be used for all transmissions. This rate will be used for transmission only. The DS PHY will still be capable of receiving aal the required DS PHY modulation rates.

#### 101.4.5.8 PMD_RATE.indicate

##### 101.4.5.8.1 Function

This primitive, generated by the PMD sublayer, indicates which modulation rate was used to receive the MPDU portion of the BPDU. The modulation is indicated in the PLCP preamble 802.11 SIGNALLING field.

##### 101.4.5.8.2 Semantic of the Service Primitive

The primitive shall provide the following parameters:

PMD_RATE.indicate(RATE)

RATE. In receive mode, the RATE parameter informs the PLCP layer which of the DS PHY data rates was used to process the MPDU portion of the BPDU.  Section 101.4.6.4 provides further information on the DS PHY modulation rates. The DS PHY rate change capability is fully described in section 101.2.

##### 101.4.5.8.3 When Generated

This primitive is generated  by the PMD sublayer when the PLCP preamble 802.11 SIGNALLING field has been properly detected.

##### 101.4.5.8.4 Effect of Receipt

This parameter is provided to the PLCP layer for information only.

#### 101.4.5.9 PMD_RSSI.indicate

##### 101.4.5.9.1 Function

This primitive, generated by the PMD sublayer, provides to the PLCP and MAC entity the Received Signal Strength..

##### 101.4.5.9.2 Semantic of the Service Primitive

The primitive shall provide the following parameters:

PMD_RSSI.indicate(RSSI)

RSSI. The RSSI is a measure of the RF energy received by the DS PHY.  RSSI indications up to 8 bits (256 levels) are supported.

##### 101.4.5.9.3 When Generated

This primitive is generated  by the PMD when the DS PHY is in the receive state. It is continuously avaliable to the PLCP which inturn provides the parameter to the MAC entity.

##### 101.4.5.9.4 Effect of Receipt

This parameter is provided to the PLCP layer for information only. The RSSI may be used in conjuction with SQ as part of a Clear Channel Assessment scheme.

#### 101.4.5.10 PMD_SQ.indicate

##### 11.4.5.10.1 Function

This primitive, generated by the PMD sublayer, provides to the PLCP and MAC entity the Signal Quality of the DS PHY PN code correlation. The signal quallity is sampled when the DS PHY achieves code lock and held until the next code lock acquisition.

##### 11.4.5.10.2 Semantic of the Service Primitive

The primitive shall provide the following parameters:

PMD_SQ.indicate(SQ)

SQ. The SQ is a measure of the PN code correlation quality  received by the DS PHY.  SQ indications up to 8 bits (256 levels) are supported.

##### 11.4.5.10.3 When Generated

This primitive is generated  by the PMD when the DS PHY is in the receive state and code lock is achieved. It is continuously avaliable to the PLCP which inturn provides the parameter to the MAC entity.

##### 11.4.5.10.4 Effect of Receipt

This parameter is provided to the PLCP layer for information only. The SQ may be used in conjuction with RSSI as part of a Clear Channel Assessment scheme. SQ may also be used a a code lock indication based on the SQ_THRESHOLD parameter in the 101.4.5.11 PMD_CS.in

#### 101.4.5.11 PMD_CS.indicate

This primitive, generated by the PMD, indicates to the PLCP layer that the receiver has acquired (locked) the PN code and data is being demodulated.

##### 101.4.5.11.1 Function

This primitive, generated by the PMD, indicates to the PLCP layer that the receiver has acquired (locked) the PN code and data is being demodulated.

##### 101.4.5.11.2 Semantic of the Service Primitive

The  PMD_CS (Carrier Sense) primitive in conjunction with PMD_ED provide CCA status through the PLCP layer  PHY_DATA primitive.   PMD_CS  indicates a binary status of ENABLED or DISABLED. PMD_CS is ENABLED when the correlator signals quality indicated in PMD_SQ is greater than the CS_THRESHOLD parameter. PHY_CS is DISABLED when the PMD_SQ falls below the correlation threshold.

##### 101.4.5.11.3 When Generated

This primitive is generated  by the PHY sublayer when the PHY is receiving  BPDU and the PN code has been acquired.

##### 101.4.5.11.4 Effect of Receipt

This indicator is provided to the PLCP  for forwarding to the MAC entity for information purposes through the PHY_CS indicator. This parameter indicates that the RF media is busy and occupied by a DS PHY signal. The DS PHY sould not be placed into the transmit state when PMD_CS is ENABLED.

#### 101.4.5.12 PMD_ED.indicate

##### 101.4.5.12.1 Function

This primitive, generated by the PMD, indicates to the PLCP layer that the receiver has detected RF energy indicated by the PMD_RSSI  primitive which is above a predefined threshold.

##### 101.4.5.12.2 Semantic of the Service Primitive

The PMD_ED (Energy Detect) primitive along with the PMD_SQ provide CCA  status at the  PLCP layer through the PHY_DATA primitive.   PMD_ED  indicates a binary status of ENABLED or DISABLED. PHY_ED is ENABLED when the RSSI indicated in PMD_RSSI  is greater than the ED_THRESHOLD parameter. PMD_ED is DISABLED when the PMD_RSSI falls below the energy detect threshold.

##### 101.4.5.12.3 When Generated

This primitive is generated  by the PHY sublayer when the PHY is receiving   RF enegy from any source which exceedes the ED_THRESHOLD parameter.

##### 101.4.5.12.4 Effect of Receipt

This indicator is provided to the PLCP  for forwarding to the MAC entity for information purposes through the PHY_ED indicator. This parameter indicates that the RF media may be busy with an RF energy source which is not DS PHY compliant. If a DS PHY source is being received, the PMD_CS function will be enabled shortly after the PMD_ED function is enabled. .

### 101.4.6 PMD Operating Specifications General

The following sections provide general specifications for the DS Physical Medium Dependent sublayer. These specifications apply to both  the receive and transmit functions and general operation of a compliant DS PHY.

#### 101.4.6.1 Operating Frequency Range [Templet 1]

The DS PHY will operate in the frequency range of 2.4 to 2.4835 GHz as allocated by regulatory bodies in the USA and Europe.

The 2.471 to 2.497 GHz frequency band has been allocated by regulatory authority in Japan.

Additional frequency bands will follow as they are made available.

#### 101.4.6.2 Number of Operating Channels [Templet 2]

For the 2.4 to 2.4835 GHz  frequency band, 7 channels are specified. These channels are divided into four channel groups identified as groups 1 to 4. The group channel center frequencies  and CHNL_ID numbers a shown in table 101.4.5 for FCC (North America), ETSI  (Europe), and Japan.

| CHNL_ID | Group | FCC Channel Frequencies | ETSI Channel Frequencies | Japan Frequency |
| --- | --- | --- | --- | --- |
| 1 | 1a | 2412 KGHz | N/A | N/A |
| 2 | 1b | 2442 KGHz | 2442 KGHz | N/A |
| 3 | 2a | 2422 KGHz | 2422 KGHz | N/A |
| 4 | 2b | 2452 KGHz | 2452 KGHz | N/A |
| 5 | 3a | 2432 KGHz | 2432 KGHz | N/A |
| 6 | 3b | 2462 KGHz | 2462 KGHz | N/A |
| 7 | 4 | N/A | N/A | 2484 KHz |
| 7 | 4 | N/A | N/A | 2483 GHz |

Table 101.4.5, DS PHY Frequency Channel Plan

In a multiple cell network topology, adjacent cells using different group 1 channels can operate simultaneously without interference.  In a similar fashion, group 2 and 3 channels can operate simultaneously with other group 2 and 3 channels.  Group 1, 2, and 3 channels may be used simultaneously in a cellular frequency plan where the interference effects have been factored into the cell plan. The Group 4 channel is designated specifically for operation in Japan.

#### 101.4.6.3 Spreading Sequence [Templet 3]

The following 11 chip Barker sequence  shall be used :

+1, -1,+1,+1, -1,+1, +1, +1, -1, -1, -1

The left most chip is output first in time.  The first chip is aligned as the start of a transmitted symbol.  The symbol duration is exactly 11 chips long.

#### 101.4.6.4 Modulation and Channel Data Rates [Templet 14 and 15]

Two modulation formats and data rates are specified for the DS PHY: a Basic Access Rate  and an Enhanced Access Rate. The Basic Access Rate is based on 1 MBPS  DBPSK  modulation. The DBPSK encoder is specified in Table 101.4.6. The Enhanced Access Rate is based on 2 MBPS DQPSK.  The DQPSK encoder is specified in Table 101.4.6.

| Bit Input | Phase Change (+jw) |
| --- | --- |
| 0 | 0 |
| 1 | P |

Table 101.4.6 1, MBPS DBPSK   Encoding Table

| Dibit pattern (d0,d1) d0 is first in time | Phase Change (+jw) |
| --- | --- |
| 00 | 0 |
| 01 | P/2 |
| 11 | P |
| 10 | 3P/2 (-P/2) |

Table 101.4.6 2, MBPS DQPSK   Encoding Table

#### 101.4.6.5 Transmit and Receive In Band and Out of Band Spurious Emissions [Templet 22]

The DS PHY shall conform with in band and out of band spurious emissions as set by regulatory bodies in the USA and Europe.  For the USA,  refer to FCC 15.247, 15.205, and 15.209.  For Europe, refer to ETSI res 02-09.

#### 101.4.6.6 Transmit to Receive Turnaround Time [Templet 23]

The TX to RX turnaround time shall be less than 10 usec including the power down ramp specifiend in section 101.4.7.7.

Conformance shall be demonstraited as The time from transition of PMD_TXE from the TX state to the RX  state until the radio is in receive mode as indicated by the exposed CCA signal  being  less than or equal to 325 usec. A reciever input signal 3dB above the ED threshold described in section 101.4.8.4 shall be present at the receiver.

#### 101.4.6.7 Receive to Transmit Turnaround Time [Templet 22]

The time from transition  of the PMD_TXE control  from the RX state to the TX state until the radio is in transmit mode shall be less than or equal to 105 usec.  This includes the transmit power up ramp described in section 10.4.8.6.1.4.8.6.

The slot time for the DS PHY is the sum of  the RX to TX turnaround time  (5 usec) and the energy detetct time (15 usec  specified in section 11.4.8.4)

#### 101.4.6.8 Transmit and Receive Antenna Port Impedance [Templet 30]

The transmit and receive antenna port(s) impedance shall be 50W if the port is exposed.

#### 101.4.6.9 Transmit and Receive VSWR [Templet 31]

DS PHY devices shall withstand no damage and remain stable over the range 0£ VSWR£¥.

#### 101.4.6.10 Transmit and Receive Operating Temperature Range

Two temperature ranges for full operation compliance to the DS PHY are specified.  Type 1 is defined as 0o C to 40o C is designated for office environments.  Type 2 is defined as -30o C to 70o C and is designated for industrial environments.

### 101.4.7 PMD Transmit Specifications

The following sections describe the transmit functions and parameters associated with the Physical Medium Dependent sublayer.

#### 101.4.7.1 Transmit Power Levels [Templet  4 and  6]

The maximum Equivalent Radiated Power (EIRP) as mesured in accordance  with practices specified by the regulatory bodies is shown in Table 101.4.7. In the USA, the radiated emissions should also conform with the ANSI uncontrolled radiation emission standards (ANSI document C95.1 publised in 1991).

| Maximum EIRP [mw] | Geographic Location | Compliance Document |
| --- | --- | --- |
| 1000 | USA | FCC 15.247 |
| 100 | EUROPE | ETSI res 02-09 |

Table 101.4.7, Transmit Power Levels

#### 101.4.7.2 Minimum Transmitted Power Level [Templet 4]

The minimum transmitted power shall be no less than 10 mW.

#### 101.4.7.3 Transmit Power Level Control [Templet 5]

Power control shall be provided for transmitted power greater than 100 mW.  A maximum of 4 power levels shall be provided between 100 mW and 1000 mW.  At  a minimum, a radio capable of transmission greater than 100 mW must be capable of switching power back to 100 mW or less.

#### 101.4.7.4 Transmit Spectrum Mask [Templet 12]

The Transmitted spectral products shall be less than -30 dbr (dB relative to  the SINx/x  peak) for fc -22 MHz < f < fc -11 MHz and  fc +11 MHz < f < fc+ 22 MHz and - 50 dBr for  f < fc -22 MHz and f > fc + 22 MHz where fc is the channel center frequency. The Transmit spectral mask is shown in figure 101.4.1. The measurements shall be made using 100 KHz resolution bandwidth at the antenna port.

µ §

Figure 101.4.1, Transmit Spectrum Mask

Figure 101.4.2, Transmit Spectrum Baseband Eye Diagram

#### 101.4.7.5 Transmit Center Frequency Tolerance [Templet 13]

The transmitted center frequency tolerance is +/- 25 ppm.

#### 101.4.7.6 Chip Clock Accuracy [Templet 17]

The PN code chip clock accuracy sall be less than +/- 25ppm.

#### 101.4.7.7 Transmit Power on and Power Down Ramp [Templet 23 and 24]

The Transmit power on ramp for 10% to 90% of maximum power shall be no greater than 2 usec. The transmit power on  ramp is shown in figure 101.4.2.

µ §

Figure 101.4.2, Transmit Power On Ramp

The Transmit power down ramp for 90% to 10% maximum power shall be no greater than 2 usec. The transmit power down ramp is shown in figure 101.4.3.

µ §

Figure 101.4.3, Transmit Power Down Ramp

The transmit power ramps are measured from the transition of the PMD_TXE control signal. The transmit power ramps shall be constructed such that the DS PHY emissions conform with spurious frequency product specification defined in section 101.4.6.5.

#### 101.4.7.8 RF Carrier Suppression

The RF carrier suppression, measured at the channel center frequency, shall be at least 15 dB below the peak SIN(x)/x power spectrum. The RF Carrier Suppression shall be measured transmitting a repetative 01 data sequence with the scrambler disabled using QPSK modulation. A 100 kHz resoultion bandwidth shall be used to  perform this measurment.

#### 101.4.7.9 Transmit Modulation Accuracy

The transmit modulation accuracy requirement for the DS PHY is based on the difference between actual transmitted waveform  and the ideal signal waveform. Modulation accuracy shall be determined by measuring the  peak  vector error magnitude  measured during each chip period. Worst case vector error magnitude shall not exceeded  0.35  for  the normalized  sampled chip data. The ideal complex  I and Q constellation points associated with QPSK modulation ( 0.707,0.707), (0.707, -0.707), (-0.707, 0.707),    (-0.707, -0.707) shall be  used as the reference. These measurements shall be from baseband  I and Q sampled data after recovery through a reference receiver system.

Figure 101.4.4 illustrates the ideal QPSK constellation points and range of worst case error specified for modulation accuracy.

µ §

Figure 101.4.4, Modulation Accuracy Measurement Example

Error vector measurement requires a reference receiver capable of carrier lock. All measurements will be made under carrier lock conditions . The distortion induced in the constellation by the reference receiver shall be calibrated and measured. The test data error vectors described below will be corrected to compensate for the reference receiver distortion.

The Vendor 802.11 compatible radio shall provide an exposed chip clock which is used to sample the I and Q outputs of the reference receiver.

The measurement is made under the conditions of continuous DQPSK transmission using scrambled all 1's.

The EYE pattern of the I channel will be used to determine the I and Q sampling point. The chip clock provided by the vendor radio will be time delayed such that the samples fall at a 1/2 chip period offset from the mean of the zero crossing positions of the EYE (see figure 101.4.5 below). This is the ideal center of the EYE and may not be the point of maximum EYE OPENING.

µ §

Figure 101.4.5, Chip Clock Alignment with Baseband Eye pattern

Using the aligned chip clock, 1000 samples of the I and Q baseband outputs from the reference receiver are captured. The vector error magnitude error magnitudes are  calculated as follows:

Calculate the DC offsets for I and Q samples

Imean =ån=01000I(n)/1000

Qmean =ån=01000Q(n)/1000

.Calculate the DC corrected I and Q samples for all N =1000 samples pairs

IDC(n)= I(n) - Imean

QDC(n)= Q(n) - Qmean

.Calculate the average magnitude  of  I and Q  samples

Imag = ån=01000 |IDC(n)|/1000

Qmag = ån=01000 |QDC(n)|/1000

.Calculate the normalized error vector magnitude for the IDC(n)/QDC(n) pairs

VERR(n) = sqrt[ 1/2 *( (|IDC(n)|/ Imag)^2 + (|QDC(n)|/ Qmag)^2) ] - VCORRECTION				with VCORRECTION = error induced by the reference receiver system

A vendor DS PHY  implementation shall be compliant if for all N =1000 samples the following condition is meet:

VERR(n) < 0.35

### 101.4.8 PMD Receiver Specifications

The following sections describe the receive functions and parameters associated with the Physical Medium Dependent sublayer.

#### 101.4.8.1 Receiver Minimum Input Level Sensitivity [Templet 4]

The Bit Error Rate (BER) shall be less than 10-5 for an input level of -80 dBm measured at the antenna connector.  This BER is specified for 2 MBPS QPSK modulated with the 11 chip Barker spreading sequence.

#### 101.4.8.2 Receiver Maximum Input Level [Templet 9]

The receiver shall provide a minimum  BER of 10-5 at a maximum input level of -4 dBm measured at the antenna.

#### 101.4.8.3 Receiver Adjacent Channel Rejection [Templet 10]

Adjacent channel rejection is defined between the two channels in each channel group defined in section 101.4.6.2.

The adjacent channel rejection shall be equal to or better than 35 dB with a BER of 10-5 using 2 MBPS QPSK with the 11 chip Barker spreading code.

The adjacent channel rejection shall be measured using the following method:

Input a 2 MBPS QPSK signal spread with the 11 chip Barker spreading code at a level 6 dB greater than specified in section 101.4.8.1.  In an adjacent channel (as defined by the channel groups),  input  a signal modulated in a similar fashion which adhears to the transmit mask specified in section 101.4.7.3 to a level 41 dB above the level specified in 101.4.8.1.  The adjacent channel signal must be derived from a separate signal source.  It can not be a frequency shifted version of the reference channel.Under these conditions, the BER shall be no worse than 10-5.

#### 101.4.8.4 Clear Channel Assessment

The DS PHY shall provide the capability to perform Clear Channel Assessment (CCA).

The CCA shall be based on the status of energy detection above a given threshold and by detection of code lock based on correlation quality ( i.e. carrier sense). The energy detection status is given by PMD primitive, PMD_ED.   The carrier sense status is given by PMD_CS. The status of PMD_ED and PMD_CS are used in the PLCP convergence procedure to indicate activity to the MAC through the PHY interface primitive PHY_DATA.indicate. The primitive class indicates the  CCA status

A Busy channel is indicated by PHY_DATA.indicate of class START-OF-ACTIVITY, END-OF-DATA, or DATA.

Clear Channel is indicated by PHY_DATA.indicate of class END-OF-DATA-AND-ACTIVITY or END-OF-ACTIVITY.

The DS PHY CCA state machine  is shown in Figure 101.4.6.  CCA operation shall be based on the following parameters:

a)	The energy detection threshold shall be set to a minimum value ofless than or equal to  -80 dBm for TX power > 100 mW, -76 dBm  for 50 mW < TX power <= 100 mW, and  -70 dBm <= 50 mW.

b)	The watchdog timer period will be 11 msec +/-10%

c)	The watchdog timer count  for detection of continuous energy sources will be 2 timer periods. This results in a worst case access period of 22 msec in the presence a continuous interference source.

d)	The worst case latency of  the detection of an energy change across the ED threshold incident on the receiver antenna to the reporting of  channel busy  shall be less than or equal to 215 usec. This implies that the CCA signal is available as an exposed test point.

e) 	The CCA state machine shall be reset  upon transition from the  TX state to the RX state.

µ §

Figure 101.4.6, DS PHY CCA State Machine

Conformance to DS PHY CCA shall be demonstrated by applying continuous and pulsed CW ( above and below the appropriate  energy detect threshold) and DS PHY compliant signals such that all states shown in figure 101.4.6 are demonstrated.

# Appendix A Approved DS PHY Sub-issues

The DS PHY specification is a single 802.11 issue consisting of a number of subissues which document individual PHY specification. Document 93/232, The DS PHY Templet,  contains a complete and upto date  summary of the the DS PHY sub-issues and there status. Templet numbers are indicated in the DS PHY draft standard in italics.